| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0714 |
| --- | --- |

| Title: | CE8.f: ALF subjective testing results |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ali Tabatabai, Madhukar Budagavi Sony, Texas Instruments David Flynn BBC Peisong Chen, Wade Wan Broadcom Koohyar Minoo Motorola Mobility Teruhiko Suzuki, Cheung Auyeung, Jun Xu Sony Do-Kyoung Kwon, Minhua Zhou Texas Instruments Inc. | Email: | Ali.Tabatabai@am.sony.com, madhukar@ti.com |
| Source: | CE8 |  |  |

_____________________________

# Abstract

This contribution presents subjective testing results for CE8.f on ALF visual quality testing. In this test, ALF-On is compared against ALF Off and ALF post filter configurations. Viewing experts from the following companies participated in the test: NEC, MediaTek, Toshiba, Qualcomm, Sony, InterDigital, Broadcom, Motorola, Panasonic, NTT Docomo, TI, Samsung, Magnum Semiconductor.

# Introduction

Bitstream Generation and Cross-Check

Bitstreams were generated with following options:

|  | SAO | ALF | Postfilter(*) | Testers (**) |
| --- | --- | --- | --- | --- |
| HM5.0 | ON | ON | OFF | - |
| ALF-Off | ON | OFF | OFF | TI, MMI, Qualcomm, Toshiba |
| ALF-Post | ON | OFF | ON | TI, SONY, Toshiba |

(*) Postfilter scheme adopts off-loop ALF. (i.e., Just apply ALF as a post filter.)

(**) Testers may share the task or cross-check the results.

Following bitstreams were used for ALF-On v/s HM-5.0 test:

| Sequence | Class | QP | Coding config | Bitrate |  | Percentage Difference (%) |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | ALF ON | ALF Off |  |
| BasketballDrive | B | 32 | LB | 3150.01 | 3164.24 | 0.5% |
|  |  | 37 | LB | 1608.99 | 1611.41 | 0.2% |
| Cactus | B | 32 | LP | 2665.96 | 2657.03 | -0.3% |
|  |  | 37 | LP | 1311.12 | 1303.20 | -0.6% |
| Kimono | B | 32 | LP | 1177.35 | 1183.68 | 0.5% |
|  |  | 37 | LP | 573.75 | 578.08 | 0.8% |
| BasketballDrill | C | 32 | RA | 825.75 | 817.28 | -1.0% |
|  |  | 37 | RA | 436.87 | 433.08 | -0.9% |
| RaceHorses | C | 32 | LP | 1020.78 | 1019.07 | -0.2% |
|  |  | 37 | LP | 462.77 | 460.81 | -0.4% |
| Vidyo1 | E | 32 | LB | 359.1312 | 355.1344 | -1.1% |
|  |  | 37 | LB | 189.9432 | 188.3352 | -0.8% |
| Riverbed | 1080p | 32 | LB | 6046.5856 | 6033.3408 | -0.2% |
|  |  | 37 | LB | 2930.2464 | 2926.9696 | -0.1% |
| SpinCalendar | 720p | 32 | LB | 807.2099 | 814.6403 | 0.9% |
|  |  | 37 | LB | 388.8419 | 384.247 | -1.2% |

Following bitstreams were used for ALF-Post v/s HM-5.0 test:

| Sequence | Class | QP | Coding config | Bitrate |  | Percentage Difference (%) |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | ALF ON | ALF-Post |  |
| BasketballDrive | B | 32 | LB | 3150.01 | 3205.19 | 1.8% |
|  |  | 37 | LB | 1608.99 | 1638.01 | 1.8% |
| Cactus | B | 32 | LP | 2665.96 | 2696.69 | 1.2% |
|  |  | 37 | LP | 1311.12 | 1327.66 | 1.3% |
| Kimono | B | 32 | LP | 1177.35 | 1195.64 | 1.6% |
|  |  | 37 | LP | 573.75 | 584.76 | 1.9% |
| BasketballDrill | C | 32 | RA | 825.75 | 834.91 | 1.1% |
|  |  | 37 | RA | 436.87 | 444.49 | 1.7% |
| RaceHorses | C | 32 | LP | 1020.78 | 1029.34 | 0.8% |
|  |  | 37 | LP | 462.77 | 467.54 | 1.0% |
| Vidyo1 | E | 32 | LB | 359.1312 | 373.3160 | 3.9% |
|  |  | 37 | LB | 189.9432 | 198.5416 | 4.5% |
| Riverbed | 1080p | 32 | LB | 6046.59 | 6047.82 | 0.0% |
|  |  | 37 | LB | 2930.25 | 2936.18 | 0.2% |
| SpinCalendar | 720p | 32 | LB | 807.21 | 852.03 | 5.6% |
|  |  | 37 | LB | 388.84 | 410.65 | 5.6% |

Test equipments and settings

Hardware for subjective testing:

| System | Dell Studio XPS Desktop 435T/9000 |
| --- | --- |
| CPU | Intel Core i7 975 @3.33GHz |
| RAM | 12.0 GB |
| OS | Windows 7 Ultimate 64 bit |
| Video Disk | OCZ Tech REVODRIVE 3 X2 SERIES SSD 240GB (Blackmagic Design Video Disk Speed Read Test > 600 MB/s) (CrystalDiskMark Sequential read speed > 600 MB/s) |
| Graphic Card | EVGA GEFORCE GTX590 3GB GDDR5 |
| Monitor | Sony professional calibrated 2D monitor ( at least 42 in) Final monitor model is to be determined later. |

Software for subjective testing:

| Video Player | David Flynn’s video player was used. |
| --- | --- |
| Scripts for displaying test videos | David Flynn’s scripts |

The bitstreams used for testing were made available at: ftp://ftp.kw.bbc.co.uk/hevc/incoming/hevc_ce8f/ .

Test method (Designed by Vittorio)

BASIC TEST CELL

The Basic Test Cell (BTC) will be done of:

- The message “ Reference” on the screen for 1 second (white letter on black background or, better, black letters on mid gray background);

- Play the uncompressed reference video clip;

- The message “A” on the screen for 1 second (white letter on black background or, better, black letters on mid gray background);

- Play the Anchor video clip for a given QP and coding condition;

- The message “B” on the screen for 1 second (white letter on black background or, better, black letters on mid gray background);

- Play the Test video clip for the same QP and coding condition;

- The message “VOTE” on the screen for 5 second (white letter on black background or, better, black letters on mid gray background);

IMPORTANT RULES:

1) Change in a random way (and hidden way to the viewing experts) the order of presentation of the Anchor and of the Test video clips at any BTC.

[E.g. “A” Anchor / “B” Test; “A” Test / “B” Anchor; “A” Test / “B” Anchor; “A” Anchor / “B” Test; …]

2) Run the above “basic test cell” (BTC) for all the test conditions and for a given video source.

3) Repeat all the test conditions for all the video sources.

4) Viewing experts must ignore the order of presentation of the “A” and “B” cases

5) Select three BTCs of a session (with very low, mid and very good quality) and show them also at the beginning of the viewing session (results of these BTCs will be discarded).

TIME REQUIRED

(3 (training) + (2(tests) x 16 test conditions)) x [ 1s(Reference message) + 10s(uncompressed video) + 1s(A message) + 10s(anchor sequence) +  1s(B message) + 10s(test video) + 5s(VOTE message)

= (3 + 2 x 16) x 38s = 1330s ~ 23 min

VIEWING CONDITIONS

This “expert viewing” protocol requires the following viewing conditions to be satisfied:

- Quite office with low ambient light (no light directly on the screen, no external windows light))

- two / three expert seated at the same desk;

- 32” or bigger HD display;

- Viewing distance 2H (H is the height of the active part of the screen)

- Paper sheet vote collection

- Voting scale from 0 [worst quality] to 10 [best quality]

- One voting scale for each BTC

- A and B are scored separately comparing each to the reference. (2 votes for each BTC)

Visual testing logistics

Date: January 31, 2012

Location: SONY lab.

Three sessions were conducted.

Session 1 time: 3:00 – 3:30

Session 2 time: 3:45 – 4:15

Session 3 time: 4:30 – 5:00

There were six viewing experts per viewing session. Viewing experts from the following companies participated in the test: NEC, MediaTek, Toshiba, Qualcomm, Sony, InterDigital, Broadcom, Motorola, Panasonic, NTT Docomo, TI, Samsung, Magnum Semiconductor.

# Visual testing results

## ALF-Off v/s ALF-On (HM5), QP=32

## ALF-Off  v/s ALF-On (HM5), QP=37

## ALF-Postfilter v/s ALF-On (HM5), QP=32

## ALF-Postfilter v/s ALF-On (HM5), QP=37

# BD-Rate results

## ALF-Off v/s ALF-On (HM5)

### Common condition sequences:

Summary:

| AI-HE | 1.6% |
| --- | --- |
| RA-HE | 3.3% |
| LB-HE | 2.8% |
| LP-HE | 4.2% |
| RA-HE10 | 4.7% |

Details:

|  | All Intra HE |  |  | All Intra HE-10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A (8bit) | 3.3% | 1.2% | 1.1% |  |  |  |
| Class B | 1.7% | 1.6% | 1.2% |  |  |  |
| Class C | 1.1% | 2.7% | 3.0% |  |  |  |
| Class D | 0.7% | 2.3% | 2.7% |  |  |  |
| Class E | 2.3% | 3.6% | 4.6% |  |  |  |
| Overall | 1.6% | 2.3% | 2.5% |  |  |  |
|  | 1.6% | 2.2% | 2.5% |  |  |  |
| Class F | 0.8% | 1.4% | 1.3% |  |  |  |
| Enc Time[%] | 85% |  |  |  |  |  |
| Dec Time[%] | 90% |  |  |  |  |  |
|  | Random Access HE |  |  | Random Access HE-10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A (8bit) | 4.5% | 0.2% | -0.2% | 5.7% | 6.3% | 6.9% |
| Class B | 3.4% | 0.5% | -0.3% | 3.9% | 3.1% | 1.9% |
| Class C | 2.3% | 0.5% | 1.0% |  |  |  |
| Class D | 3.4% | 0.4% | 0.7% |  |  |  |
| Class E |  |  |  |  |  |  |
| Overall | 3.3% | 0.5% | 0.3% | 4.7% | 4.5% | 4.1% |
|  | 3.3% | 0.5% | 0.3% | 4.7% | 4.4% | 4.1% |
| Class F | 1.2% | 0.9% | 0.7% |  |  |  |
| Enc Time[%] | 95% |  |  | 97% |  |  |
| Dec Time[%] | 89% |  |  | 85% |  |  |
|  | Low delay B HE |  |  | Low delay B HE-10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 2.9% | 0.4% | -0.4% |  |  |  |
| Class C | 2.7% | 0.4% | 1.0% |  |  |  |
| Class D | 2.3% | 0.6% | 1.9% |  |  |  |
| Class E | 3.3% | 0.4% | -2.4% |  |  |  |
| Overall | 2.8% | 0.5% | 0.2% |  |  |  |
|  | 2.8% | 0.5% | 0.2% |  |  |  |
| Class F | 2.3% | 0.7% | 0.2% |  |  |  |
| Enc Time[%] | 103% |  |  |  |  |  |
| Dec Time[%] | 95% |  |  |  |  |  |
|  | Low delay P HE |  |  | Low delay P HE-10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 6.3% | 1.2% | 0.3% |  |  |  |
| Class C | 2.6% | 0.5% | 0.7% |  |  |  |
| Class D | 1.1% | 0.8% | 1.3% |  |  |  |
| Class E | 6.9% | 1.0% | -1.5% |  |  |  |
| Overall | 4.2% | 0.9% | 0.3% |  |  |  |
|  | 4.2% | 0.9% | 0.3% |  |  |  |
| Class F | 2.1% | 1.0% | 0.6% |  |  |  |
| Enc Time[%] | 97% |  |  |  |  |  |
| Dec Time[%] | 89% |  |  |  |  |  |

### Riverbed and SpinCalendar:

Summary:

| AI-HE | 2.6% |
| --- | --- |
| RA-HE | 3.6% |
| LB-HE | 4.8% |
| LP-HE | 6.8% |
| RA-HE10 | 5.3% |

Details:

|  | All Intra HE |  |  | All Intra HE-10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| New | 2.6% | 1.7% | 1.7% |  |  |  |
| Overall | 2.6% | 1.7% | 1.7% |  |  |  |
|  | 2.6% | 1.6% | 1.7% |  |  |  |
| Enc Time[%] | 75% |  |  |  |  |  |
| Dec Time[%] | 87% |  |  |  |  |  |
|  | Random Access HE |  |  | Random Access HE-10 |  |  |
|  | Y | U | V | Y | U | V |
| New | 3.6% | 1.9% | 1.3% | 5.3% | 3.9% | 10.0% |
| Overall | 3.6% | 1.9% | 1.3% | 5.3% | 3.9% | 10.0% |
|  | 3.6% | 1.9% | 1.4% | 5.4% | 3.9% | 9.7% |
| Enc Time[%] | 96% |  |  | 102% |  |  |
| Dec Time[%] | 92% |  |  | 78% |  |  |
|  | Low delay B HE |  |  | Low delay B HE-10 |  |  |
|  | Y | U | V | Y | U | V |
| New | 4.8% | 1.4% | 1.4% |  |  |  |
| Overall | 4.8% | 1.4% | 1.4% |  |  |  |
|  | 4.8% | 1.5% | 1.2% |  |  |  |
| Enc Time[%] | 95% |  |  |  |  |  |
| Dec Time[%] | 81% |  |  |  |  |  |
|  | Low delay P HE |  |  | Low delay P HE-10 |  |  |
|  | Y | U | V | Y | U | V |
| New | 6.8% | 2.7% | 2.5% |  |  |  |
| Overall | 6.8% | 2.7% | 2.5% |  |  |  |
|  | 6.8% | 2.6% | 2.7% |  |  |  |
| Enc Time[%] | 95% |  |  |  |  |  |
| Dec Time[%] | 78% |  |  |  |  |  |

## ALF-Postfilter v/s ALF-On (HM5)

### JCTVC sequences:

Summary:

| AI-HE | 0.0% |
| --- | --- |
| RA-HE | 1.2% |
| LB-HE | 2.3% |
| LP-HE | 2.7% |
| RA-HE10 | 1.2% |

Details:

|  | All Intra HE |  |  | All Intra HE-10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A (8bit) | 0.0% | 0.0% | 0.0% |  |  |  |
| Class B | 0.0% | 0.0% | 0.0% |  |  |  |
| Class C | 0.0% | 0.0% | 0.0% |  |  |  |
| Class D | 0.0% | 0.0% | 0.0% |  |  |  |
| Class E | 0.0% | 0.0% | 0.0% |  |  |  |
| Overall | 0.0% | 0.0% | 0.0% |  |  |  |
|  | 0.0% | 0.0% | 0.0% |  |  |  |
| Class F | 0.0% | 0.0% | 0.0% |  |  |  |
| Enc Time[%] | 96% |  |  |  |  |  |
| Dec Time[%] | 98% |  |  |  |  |  |
|  | Random Access HE |  |  | Random Access HE-10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A (8bit) | 0.4% | -0.5% | -0.6% | 1.1% | 1.9% | 2.5% |
| Class B | 1.0% | -0.3% | -0.6% | 1.3% | 0.7% | 0.4% |
| Class C | 1.2% | -0.6% | -0.2% |  |  |  |
| Class D | 1.7% | 0.5% | 0.5% |  |  |  |
| Class E |  |  |  |  |  |  |
| Overall | 1.2% | -0.2% | -0.2% | 1.2% | 1.2% | 1.3% |
|  | 1.2% | -0.2% | -0.2% | 1.3% | 1.2% | 1.3% |
| Class F | 1.1% | 0.8% | 0.6% |  |  |  |
| Enc Time[%] | 101% |  |  | 102% |  |  |
| Dec Time[%] | 113% |  |  | 109% |  |  |
|  | Low delay B HE |  |  | Low delay B HE-10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 1.2% | -0.1% | -0.3% |  |  |  |
| Class C | 2.1% | 0.0% | 0.3% |  |  |  |
| Class D | 2.2% | 1.0% | 2.0% |  |  |  |
| Class E | 4.3% | 2.1% | -1.6% |  |  |  |
| Overall | 2.3% | 0.6% | 0.2% |  |  |  |
|  | 2.2% | 0.6% | 0.2% |  |  |  |
| Class F | 3.3% | 1.8% | 1.7% |  |  |  |
| Enc Time[%] | 101% |  |  |  |  |  |
| Dec Time[%] | 113% |  |  |  |  |  |
|  | Low delay P HE |  |  | Low delay P HE-10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 2.6% | 0.8% | 0.5% |  |  |  |
| Class C | 2.1% | 0.1% | -0.1% |  |  |  |
| Class D | 1.3% | 1.1% | 1.3% |  |  |  |
| Class E | 5.4% | 2.2% | -1.2% |  |  |  |
| Overall | 2.7% | 1.0% | 0.2% |  |  |  |
|  | 2.7% | 1.0% | 0.2% |  |  |  |
| Class F | 3.2% | 2.2% | 2.1% |  |  |  |
| Enc Time[%] | 101% |  |  |  |  |  |
| Dec Time[%] | 113% |  |  |  |  |  |

### Riverbed and SpinCalendar:

Summary:

| AI-HE | 0.0% |
| --- | --- |
| RA-HE | 2.5% |
| LB-HE | 3.6% |
| LP-HE | 4.4% |
| RA-HE10 | 2.7% |

Details:

|  | All Intra HE |  |  | All Intra HE-10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| New | 0.0% | 0.0% | 0.0% |  |  |  |
| Overall | 0.0% | 0.0% | 0.0% |  |  |  |
|  | 0.0% | 0.0% | 0.0% |  |  |  |
| Enc Time[%] | 99% |  |  |  |  |  |
| Dec Time[%] | 105% |  |  |  |  |  |
|  | Random Access HE |  |  | Random Access HE-10 |  |  |
|  | Y | U | V | Y | U | V |
| New | 2.5% | 1.1% | 1.0% | 2.7% | 1.8% | 2.9% |
| Overall | 2.5% | 1.1% | 1.0% | 2.7% | 1.8% | 2.9% |
|  | 2.5% | 1.1% | 1.2% | 2.8% | 1.9% | 2.7% |
| Enc Time[%] | 93% |  |  | 97% |  |  |
| Dec Time[%] | 98% |  |  | 94% |  |  |
|  | Low delay B HE |  |  | Low delay B HE-10 |  |  |
|  | Y | U | V | Y | U | V |
| New | 3.6% | 1.3% | -0.7% |  |  |  |
| Overall | 3.6% | 1.3% | -0.7% |  |  |  |
|  | 3.6% | 1.3% | -0.9% |  |  |  |
| Enc Time[%] | 98% |  |  |  |  |  |
| Dec Time[%] | 116% |  |  |  |  |  |
|  | Low delay P HE |  |  | Low delay P HE-10 |  |  |
|  | Y | U | V | Y | U | V |
| New | 4.4% | 2.5% | 0.7% |  |  |  |
| Overall | 4.4% | 2.5% | 0.7% |  |  |  |
|  | 4.4% | 2.5% | 0.9% |  |  |  |
| Enc Time[%] | 106% |  |  |  |  |  |
| Dec Time[%] | 118% |  |  |  |  |  |