| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0715 |
| --- | --- |

| Title: | BoG report on Decoded picture buffering and reference picture signaling |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Jill Boyce Vidyo 433 Hackensack Ave Hackensack, NJ USA | Tel: Email: | jill@vidyo.com |
| Source: | BoG |  |  |

_____________________________

# Abstract

The BoG met on Feb 3 and Feb 4.

The documents were classified into the following categories.

- POC (4)

- Reference picture lists unification/combination (7)

- Long term reference pictures (5)

- Other reference picture list topics (10)

# Recommendations

The BoG recommends to adopt:

- JCTVC-H0137 Proposal #1 , to add a condition to send ref_idx_list_curr in ref_pic_list_combination( )

- JCTVC-H0138 Unification of reference picture list modification processes, to make list 0 and list 1 modification the same as list c modification

- JCTVC-H0200 Proposal #1, changing element ‘delta_poc_lsb_lt_minus1 [i]’ to  ‘delta_poc_lsb_lt [i]

- JCTVC-H0531 For long term reference pictures, add delta_poc_msb_present_flag, so same POC LSB may be used for multiple long term reference pictures

- JCTVC-H0412 Add SPS flags to indicate that all slices in a picture use identical reference picture lists

- JCTVC-H0565 Add restriction to DeltaPocS0[ idx ][ i ] to fix text bug that if there are more than one short term reference picture with the same poc lsb value, not clear which one is referenced when changed to long term, unless H0449 is adopted.

The BoG also proposes an output document H0725, editor Yan Ye, on reference picture related test conditions to update/replace JCTVC-G1036, adding conditions for reference picture list reordering and using different list ordering per slice..

One document remains for revisit, JCTVC-H0449.

Further study encouraged on refererence picture list combination signaling and absorbing ref_pic_list_combination() into the ref_pic_list_modification().

# Document review notes

JCTVC-H0021 JCT-VC AHG report: Reference picture buffering and list construction (AHG21) [R. Sjöberg (chair), Y. Chen, Hendry, T. K. Tan, W. Wan, Y.-K. Wang (vice chairs)]

The d6 version of the HEVC working draft 5 (JCTVC-G1103) was the first version to contain reference picture sets. It was made available on January 19.

It was noted that there was a change to the G1036 test conditions document case 2.6 that should be reflected in an official document.  This change was in the software, so experimental results to this meeting incorporated it, so the test conditions document should reflect it.

Software changes to support the specified picture coding patterns were hard coded in some cases, enabled by config files parameters in the HM branch HM-5.0-dev-misc-ahg21 branch, updated to HM-5.1-dev-ahg21.

Bit counting functionality was added to the HM-5.1-dev-ahg21 source code, and total bitrate percentage of RPS for the tested conditions are reported in the AHG report.

Reference picture buffering and list construction AHG recommends for the JCT-VC to review all picture buffer management and list construction related input.

## POC

JCTVC-H0449 AHG21: A constraint to simplify and improve the robustness of POC computation [G. J. Sullivan (Microsoft)]

The current draft of HEVC, like POC mode 0 in AVC, requires the decoder to infer MSB values for the picture order count associated with each decoded picture. These MSB values are inferred from the history of the decoding process from picture-to-picture throughout the entire coded video sequence, and the removal of one or more pictures from the bitstream can result in corruption of these MSB values, resulting in out-of-order playback of the remaining pictures, incorrect reference picture list construction, and incorrect temporal weighted prediction. It is asserted that this inference specification also poses difficulties for random access, since a decoder that performs random access into the bitstream does not have access to the preceding pictures in the coded video sequence.

This contribution proposes to impose a constraint on the values of the POC LSB values of the pictures in the bitstream (as expressed by the pic_order_cnt_lsb syntax element). It is asserted that the imposition of this constraint would have a negligible effect on coding efficiency, simplify the decoding process for establishing relative POC order, eliminate the need for inference of POC MSB values, and improve robustness to picture losses. This proposal is similar to part 15 of the prior proposal JCTVC-G788 revision 1; however, this aspect of the prior proposal was deferred at the previous meeting.

Could potentially have a minor bitrate efficiency overhead when the number of pictures in the DPB is large compared the poc lsb.

Possible interaction with long term reference picture contributions.

Would this require a change to the reference software?  Proponent asserted that it would be nice to do but not necessary, but other participants believe that a decoding process change would be required. We need to better understand the decoding process changes and interaction with long term reference pictures.  Interest was expressed in having software available that matches the proposed semantics.

For long term reference picture referencing, only the LSB is required.  We already have the restriction that LSB of a long term reference picture cannot be reused.

A participant noted that a dependency on the HRD is imposed, which may be undesirable.

Revisit.

JCTVC-H0425 POC granularity for reference picture sets and slice header [M. M. Hannuksela, S. M. Gopalakrishna (Nokia)]

It is asserted that encoders may choose to keep the POC value difference of any two consequent pictures in output order having the same temporal_id remains unchanged. For example, encoders may use a regular GOP pattern and assign POC values proportional to a constant picture sampling interval. It is therefore proposed that a POC granularity value is provided for each temporal_id in the sequence parameter set. The POC granularity value for a particular temporal_id indicates the POC interval between any two consecutive pictures in output order having that temporal_id. It is noted that POC granularity value equal to 1 can be indicated for bitstreams having an irregular POC assignment pattern.

This contribution proposes to use the temporal_id-based POC granularity for POC difference coding in reference picture sets and POC value signaling in slice headers.

Revision 1 of the document includes a bug fix on the specification text for inter-RPS prediction.

A participant noted that if the H0423 were adopted, may be able to save bits in the SPS by inferring the POC granularity from the SOP description.

Software is available in the zip package, but not integrated with the AHG21 HM branch.  Experimental results are not available. Bitrate savings are expected from the POC signaling in the slice header and non-inter-RPS-predicted reference picture sets, but not in inter-RPS-predicted reference picture sets.

Some relationship to H0501.

No action.

JCTVC-H0257 Cyclic POC [J. Koyama, K. Kazui, S. Shimada, A. Nakagawa (Fujitsu)]

This contribution proposes to change the definition of picture order count (POC) to a cyclic pattern to support a functionality requirement for surveillance. The range of POC value remains 32-bit and no syntax change is required.

Proponent would like to be able to have the functionality to code an individual video sequence that lasts for longer than one year (when a 32-bit POC would wrap) without having to insert an IDR, because of the spike in bitrate associated with an IDR.

A participant suggested that H0449 would address the functionality with a different solution.

No text is available.

No action.

JCTVC-H0501 Efficient POC signaling for temporal scalability [Y. Chen, Y. -K. Wang (Qualcomm)]

In this proposal, an adaptive POC signaling is proposed. Unlike the current HEVC design which signals for each picture a fixed number of Least Significant Bits (LSB) of the POC, with this proposal, pictures having different temporal_id values can be signaled with different number of bits for the POC LSB.

Some relationship to H0425, with which it may be combined.  In H0425 the POC step size varies by temporal layer.  In this contribution, the POC LSB range changes per temporal layer.  The result is that higher temporal layers would wrap more frequently than lower temporal layers.

It is asserted that this would solve an error resiliency problem, when the temporal layer 0 POC LSB is too small.  The problem could also be solved by using a larger POC LSB for all layers, but the proposal would save bits.  No experimental data quantifying the savings has been provided.

No action.

## Reference picture lists unification/combination

JCTVC-H0137 AHG21: Reference picture lists combination syntax [Y Ye, Y He (InterDigital)]

In this contribution, the reference picture list combination process is slightly modified to remove redundant signalling. In order to enable bit counting for ref_pic_list_combination(), an encoder-only reordering method for the combined list based on POC distance and QP is implemented in the HM5.0 software.

Two proposed syntax changes:

- Add condition to send ref_idx_list_curr

- Use te(v) instead of ue(v) for ref_idx_list_curr

Encoder only change was implemented to utilize QP in ref pic list ordering for the purpose of testing the modified syntax.  The current test conditions in G1036 would not benefit from the syntax modification.

Simulation results reported using HM5.0 random access testing conditions show that the proposed syntax  ~14% reduction in signalling overhead for ref_pic_list_combination(), due to change #2, use of te(v). Gains not reported for change #1 alone.  It was noted that te(v) is not used in the current specification.  #1 seems to be a design cleanup.

Recommend: Adopt #1

JCTVC-H0653 AHG21: Cross-verification of Reference picture lists combination syntax from InterDigital (JCTVC-H0137) [L. Guo (Qualcomm)] [late]

Noted.

JCTVC-H0138 AHG21: Unification of reference picture list modification processes [Y He, Y Ye (InterDigital)]

Currently HEVC WD5 specifies two different methods to modify the reference picture lists List0/List1 and the combined list. In this contribution, unification of reference picture list modification process for List0/List1 and combined list is proposed to simplify the syntax and modification process of reference picture lists List 0/List 1. Additionally, an encoder-only L0/L1 reordering method was implemented in the HM5.0 software. Simulation results using HM5.0 RA testing conditions reportedly show proposed syntax consistently achieves 47% reduction in signalling overhead for ref_pic_list_modification().

Propose syntax change, and semantics and decoding process.

- Changed conditions for sending ref_pic_set_idx, removal of list_modification_idc

- Use te(v) instead of ue(v) to send ref_pic_set_idx

Makes list modification for individual lists to be more similar to the medication process for combined list. Semantics are simplified significantly, due to proposal  #1.  Experimental results are for the two proposals combined.

A temporary list is formed that is a superset of the list initialization, with the beginning of the list identical, but extended to include the other pictures in the RPS.  A participant noted that the decoding process could be rewritten so that the longer temp list is only filled when list modification is used.  Proponent is encouraged to provide revised text.

Currently combined list is a subset of the combination of the final list 0 and list 1.

A participant noted that the temporary lists could also be used for the construction of the combined list.   In that case, the combined list would no longer be constrained to be a subset of the combination of the final list 0 and list 1.   There may be a problem, for example, if the neighboring block uses list 0 prediction and the current block uses list C prediction.  There may be some relation to JCTVC-H0466.

A participant noted that u(v) coding may perform similarly to te(v).  It was suggested that the proponent provide hand calculations of expected bitrate differences of ue(v), te(v), u(v).

Revisit once revised text and calculations of bit rate differences are available.

REVISIT notes:

u(v) turns out to be more efficient than te(v).  Proposal change to use u(v).

Decoding process changed to be more efficient when no ref pic list modification flag is 0.

Proponents agreed to provide software implementation (as per normal adoption) and additionally a mechanism for configurable reordering.

The group should define some test conditions for the next meeting that include reordering, and different lists for individual slices in picture.

Recommend: Adopt.

JCTVC-H0679 AHG21: Cross-check of H0138 on Unification of reference picture list modification processes [Edouard.Francois (Canon)] [late]

Noted.

JCTVC-H0466 AHG21: Unified reference list for single-directional prediction [V. Wahadaniah, C. S. Lim (Panasonic)]

This contribution proposes a number of modifications on the construction of reference picture list for single-directional prediction in P and B pictures. It is purported that the benefits of the proposed design include unification, reduced complexity and improved flexibility of the reference picture list construction and modification processes.

Proposals

- Create initial RefPiclist0 from RPS in P slices

- For P and B slices,

- Modification to initial combined list construction

- Same list reordering process for list 0, list 1 and list C.  (Similar to H0138)

#1 aimed at sequences with P frames with reordering, which is not a common use case.  Adds semantics.  No experimental data reporting gains was provided.  Could use ref pic list modification to achieve same final list functionality.

Proponent reports that proposal #2 requires further study.

No action.

JCTVC-H0504 Comments on Reference Picture Lists Combination Syntax [Hendry, Y. J. Jeon, S. W. Park, S. O. Jeong, B. M. Jeon (LG), Y. Chen, Y. -K. Wang, W. -J. Chien (Qualcomm)]

It was reported that there is some redundancy on the reference picture list combination syntax in the current HEVC WD. More specifically when the ref_pic_list_combination_flag is equal to 0, it indicates that two reference picture lists are identical, but both lists are still signaled separately. In this document, it is proposed that the location of this flag be moved to an earlier place of the slice header, such that the redundancy can be removed by not signaling the reference picture list 1 when ref_pic_list_combination_flag is equal to 0. Furthermore, the name of this flag is changed to identical_lists_flag avoid confusion.

Related to the problem identified in JCTVC-G549.

Don’t need to signal num_ref_idx_l1_active_minus1 if identical_lists_flag is equal to 1.

Seems to be a cleanup to the existing design.  With the existing design, could be a problem if the flag indicates that the lists are identical, but the list 1 parameters are also transmitted but are different.  Either they have to be constrained to be identical (and the constraint expressed in the spec text), or problems could exist.

Concern expressed about applying the list 1 to the pred_weight_table().

Proposal

- Slice header change to make num_ref_idx_l1_active_minus1 conditional

- ref_pic_list_modification() change to not send list 1 if identical

Also suggested editorial name change of the ref_pic_list_combination_flag to identical_lists_flag.

Discussion if the ref_pic_list_combination_flag could simply be removed.

Some concerns about on the fly design. No decision this meeting, but further study encouraged.

JCTVC-H0292 Syntax on List Combination [Y. Park, T. Lee, J. H. Kim, J. H. Park (Samsung)]

This contribution proposes syntax to set the default number of List Combination in PPS and syntax to signal the active number of List Combination in slice header. In addition, one syntax which has additional information is proposed to be removed and syntax which has similar context to be gathered together.

- Define the default number of LC (List Combination) in PPS as like List0 and List1

- Remove syntax that has additional information

- Move LC reordering syntax to ref_pic_list_modification() in slice header

A participant proposed combination of the override flag in the slice header, and there was support for that suggestion.

Some syntax was removed, but other previously optional syntax was made mandatory.  No bitrate count data is provided.

REVISIT Notes:

For Proposal #1, want to move it to the PPS, and then override in the slice header.

Option #1, combined override flag for list1 and list C

Option #2, separate flags for list 1 and list C

It isn’t clear that any change is necessary.  Just potential bitrate improvement, but it is not clear that bits will actually be saved.  Don’t want to move to the PPS.

For Proposal # 1

Move num_ref_idx_lc_active_minus1.  Concerns about putting this in the PPS.

For Proposal #2 and 3

Some support expressed for identical list flag and merging the combination into the ref_pic_list_modification().  Proponents encouraged to further study and bring contribution to next meeting.

## Long Term Ref Pics

JCTVC-H0694 Long-term picture coding [K. Suehring, H. Schwarz, T. Wiegand (Fraunhofer HHI)] [late]

This contribution proposes a modified long-term picture coding using bit strings to identify long-term pictures in the decoded picture buffer. Although a small overhead is introduced for long-term pictures that are near to the current decoded picture, approx. 45% of the necessary bits can be saved in scenarios where the long-term picture has a large picture order count difference.

Replace syntax in the slice header to send a bit string that uniquely identifies the indicated long term ref pic from the pool of possible entries in the buffer that haven’t already been marked.

Introduces a new unary code entropy coding method, un(v), which does binarization.  No data provided to indicate how much of the gain is due to this entropy coding method vs. the unique bit string identification.

The uploaded document experimental results appears to have some copy-paste errors and will be corrected and uploaded.

Error resiliency is negatively impacted.  An encoder could make more resilient by increasing bitrate.

The long term picture identification would become dependent on the short term ref pic list.

Possible conflict with H0449.

No action.

JCTVC-H0200 AHG21: Comments on Signaling of Long-term Reference Pictures [Hendry, S. Jeong, S. Park, B. Jeon (LG)]

In the 7th JCT-VC meeting, method for signaling short-term and long-term reference picture was adopted. This document proposes some changes to the current syntax and semantics elements for signaling long-term reference pictures. The proposed changes are:

- Replace syntax element ‘delta_poc_lsb_lt_minus1 [i]’ with ‘delta_poc_lsb_lt [i]’ to enable handling of two long-term reference pictures with the same delta POC LSB relative to current  slice

- Allow skip signaling delta POC LSB of long-term reference pictures without removing it from decoded picture buffer.

It is suggested that the proposed changes may improve signaling efficiency without changing the main concept of current signaling approach.

For #1, if multiple long term pics have the same POC, can remove only the furthest away long term ref pic, have no way to remove the ones that are closer.  May be possible to combine H0531.

For #2, don’t need to explicitly signal long term ref pic POC.  Has a negative error resiliency impact. Have a flag to indicate if long term ref pic changes are present, otherwise send nothing.

Recommendation: Adopt #1, changing element ‘delta_poc_lsb_lt_minus1 [i]’ to ‘delta_poc_lsb_lt [i].

JCTVC-H0468 AHG21: Flexible signalling of long term reference pictures [V. Wahadaniah, C. S. Lim (Panasonic)]

This contribution proposes a number of modifications on the signalling of long term reference pictures in the reference picture set design of HEVC. It is purported that the benefits of the proposed design include improved flexibility and reduced redundancy of the reference picture set design. It is suggested that the JCT-VC consider the aspects proposed in this contribution and refine the current design of reference picture set.

Propose to put long term ref pic parameters in the PPS, using absolute POC.  Can optionally also send long term ref pic indications in the slice header.  If a long term ref pic indication is put in the slice header, for as long as the long term ref pic is used in the sequence, it must be sent in every slice.

A participant noted a problem for random access with CRA, where absolute POC is not known.  A possible solution would be to transmit absolute POC in CRA pictures.

Possible error resiliency impacts if the PPS were lost, although PPS could be repeated or could be sent out of band.

No software implementation available.  No bitrate counts available, but may see some bitrate savings when long term ref pics are used, although repeated PPS may offset this.

Similar to G714 and H0502.

Some support expressed for including long term ref pic parameters in the PPS.

Revisit.  Proponents encouraged to provide bitrate counts.

REVISIT notes (also apply to H0502):

H0502 experimental results provided

Suggest to change i(v) to have a sign bit plus u(v), to avoid adding a new entropy coding method.

Concern about on the fly design.

The applicable use case needs to be better understood.  Low delay application for which long term ref pics were originally designed, don’t know in advance when a long term ref pic will be marked.  Random access applications would kick out long term ref pics whenever a CRA arrived anyway.

Random access possible problems in determination of the absolute POC.

A participant suggested that the long term ref picture parameters could go into the APS instead of the PPS.

More accurate bitrate counts are requested with actual simulations which consider the additional bits are needed for the PPS.

JCTVC-H0502 Signaling of long-term reference pictures in the PPS [Y. -K. Wang, Y. Chen (Qualcomm)]

This document proposes to enable the inclusion of candidate long-term reference pictures, as part of the reference picture set signaling in the picture parameter set.

Slice header can refer to the long term ref pics sent in the PPS and can add new long term ref pics.

No software implementation.  No bitrate counts available, but expect to see bitrate savings when long term ref pics are used.

Similar to G714 and H0468.

Some support expressed for including long term ref pic parameters in the PPS.

Revisit.  Proponents encouraged to provide bitrate counts.

JCTVC-H0531 AHG21: Comments on Signaling of Reference Pictures [Sachin Deshpande, Andrew Segall (Sharp)] [late]

This document provides comments on signaling of reference pictures. This document proposes syntax to signal long-term reference pictures.

Send long term poc delta minus 1.

Have a software implementation available, to be uploaded soon with the contribution.

Provided a function that could go into the encoder model (but shouldn’t go in the WD) for determining when it is necessary to signal the MSB POC cycle.  Perhaps a Note in spec may be helpful.

Relaxes the restrictions on use of long term ref pics with same POC value as the current pic.  Can save bits by allowing for shorter POC LSB.

Encoder is responsible to determine when the delta POC MSB needs to be sent.

A participant suggested to change delta_pic_lsb_lt_minus1 to delta_poc_lsb_lt in order to signal the same POC.

Interaction with other proposals?  Seem to be orthogonal.  Could be combined with moving long term ref pic signaling to PPS.  Could be combined with an absolute doc.

Proposed semantics text doesn’t describe that the delta_poc_msb_check_flag must be equal to 1 when removing a long term ref pic when there are more than one long term ref pic with the same poc LSB value. The text can be updated to reflect the restriction.

A suggestion was made to not send delta_poc_msb_check_flag, but switch delta_poc_msb_cycle_lt_minus1 to delta_poc_msb_cycle_lt.

A suggestion was made to not send  delta_poc_msb_check_flag, but if the value of delta_poc_lsb_lt is zero, always send the delta_poc_msb_cycle.

A participant suggested that experimental results be provided comparing with using a larger POC LSB to avoid the need to use the same POC LSB for a long term ref picture.

Quality of text is immature and needs improvements.

Revisit.  Proponents encourage to provide bitrate counts and to improve text.

REVIST notes:

Revised text available, has been reviewed by several other participants.

Sugestion to change the name of the delta_poc_msb_check_flag to delta_poc_msb_present_flag.

A participant noted that this provides additional flexibility by allowing more than one long term reference picture to use the same poc lsb.

Experimental results were provided comparing with using a larger POC LSB to avoid the need to use the same POC LSB for a long term ref picture, showing small gains.  In some other cases a small loss occurs if there were no long term ref picture with the same POC.

Recommendation: Adopt.

## Other reference picture list topics

JCTVC-H0412 AHG21: Reference picture list restrictions [R. Sjöberg, J. Samuelsson (Ericsson)]

This document proposes to add a flag identical_ref_pic_lists_flag to the SPS to enable indicating whether the reference picture lists for all slices that belong to the same picture and have the same slice type are identical or not. If this flag is set, the decoder may perform reference picture list construction once per picture regardless of the number of slices that are used. This document claims that the list construction process is a burden for the decoder if there are many slices for each picture since list construction is generally not part of decoding hardware but done on relatively slow general purpose processors.

This document also proposes to add a flag to the SPS to signal whether there is any reference picture list modification syntax in the bitstream at all. Besides indicating to the decoder that there are no modifications of the lists, the presence of reference picture list modification syntax is conditioned on this flag, saving a few bits.

This document finally proposes that identical_ref_pic_lists_flag shall be equal to 1 for HEVC profiles targeting broadcast TV.

Recommendation: Adopt .

Suggest to rename the identical_ref_pic_lists_flag, to include slice in the name.

Semantics for the ref_pic_list_modification_not_present_flag need to be updated to indicate that default combination will take place if the flag is set.

JCTVC-H0467 AHG21: Signalling periodic reference picture lists [V. Wahadaniah, S. M. T. Naing, C. S. Lim (Panasonic)]

This contribution proposes the signalling of periodic reference picture list reordering parameters together with predefined reference picture sets in the PPS. It is purported that the proposed design removes redundancy in reference picture list construction when the same reordering operations are repeated over a number of group of pictures (GOPs). It is suggested that the JCT-VC consider the aspects proposed in this contribution and refine the current design of reference picture list.

Allow the ref pic reordering to be done in the PPS, and can be overwritten in the slice header.  Expect to get coding efficiency gains, but have no data.

No action.

JCTVC-H0426 AHG21: Removal of reference picture list modification [M. M. Hannuksela, S. M. Gopalakrishna (Nokia)]

The contribution proposes that:

- Reference picture list modification syntax structure is removed from the slice header. The syntax, semantics and decoding process of the reference picture list modification are also removed.

- Signed POC difference values can be used conditionally in the reference picture set (RPS) syntax structure to allow specifying reference picture sets flexibly. A default negative sign is inferred for set 0 and a default positive sign is inferred for set 1, unless signs are provided explicitly for each item in reference picture sets.

- Reference picture list 0 is formed by 1-to-1 copy of short-term reference picture set S0 followed by short-term reference picture set S1. Reference picture list 1 is formed by 1-to-1 copy of short-term reference picture set S1 followed by short-term reference picture set S0. The same process was used earlier for initialisation of reference picture lists.

- Each long-term reference picture may be inserted at an indicated position in reference picture list 0 or 1, as controlled by the slice header.

It is asserted that the page count of the specification reduces by about two pages due to the changes above. Moreover, it is asserted that there is no essential change in bit count overhead of the proposal relative to the current RPS design (JCTVC-G1002 and JCTVC-G198r2) according to the common conditions in JCTVC-G1036. It is believed that no test case in JCTVC-G1036 requires signed POC differences proposed in this contribution or reference picture list modification included in the current RPS design – hence for the test cases in JCTVC-G1036 the bitstreams according to this contribution are reportedly essentially identical to the bitstreams according to the current RPS design.

In revision 1 of the document, the specification text has been updated with the following changes:

- It is clarified that the marking of pictures as “used for reference” and “unused for reference” caused by referring or including reference picture set in a slice header shall be identicall in all slices of a coded picture. This facilitates the use of different reference picture lists in different slices of the same picture.

- The decoding process for reference picture sets is invoked for each slice header.

The proposal combines together the RPS with the modifications, and simplifies the specification text.

If an encoder wants to modify the lists differently for individual slices in the picture, for example for weighted prediction,  would need to send a full RPS for each one.

No experimental data when reordering is actually used, although no test cases are defined.  We might wish to define a test case that includes reordering.

No action.

JCTVC-H0433 AHG21: Reference Picture Set Signaling Method [Sachin Deshpande, Kiran Misra, Andrew Segall]

This document proposes syntax to signal a delete operation for removing select reference pictures from a previously signaled reference picture set (RPS). This approach can be used to signal a new partial RPS by referring to a RPS signaled in PPS. It is asserted that this change supports efficient signaling of reference picture set for the current picture if the desired reference picture set differs from that defined in the picture parameter set for only a small subset of entries.

In existing design could delete using inter RPS.  The proposal can save bits in the RPS, but possibly at the expense of bits in the slice.

Use case that has feedback from decoder to encoder.

Many new syntax elements for very small gains.   Doesn’t add any functionality.

No action.

JCTVC-H0565 AHG21: Short-term reference picture range [J. Samuelsson, R. Sjöberg (Ericsson)]

This contribution presents a proposal for a minor change in the reference picture set semantics regarding short-term reference pictures.

Propose to add the restriction that DeltaPocS0[ idx ][ i ] shall be in the range of −1 to −MaxPicOrderCntLsb/2, inclusive and that DeltaPocS1[ idx ][ i ] shall be in the range of 1 to MaxPicOrderCntLsb/2 − 1, inclusive. That way there will only be one short-term reference picture with a specific value of pic_order_cnt_lsb in the DPB.

Has identified a bug in the current design that it if there are more than one short term reference picture with the same poc lsb value, not clear which one is referenced when changed to long term.

Similar to H0449.  Two different approaches to solve the same problem.  H0449 also addresses other problems as well.  This contribution imposes an encoder restriction with no decoder process change.

Recommendation: Adopt unless H0449 is adopted.

JCTVC-H0482 Improvement to Intra RPS Prediction [J. Zhao, S. Deshpande, A. Segall (Sharp)]

This contribution proposes a method to reduce the bits required to code RPS by intra RPS prediction. The proposed method exploits symmetry between the first positive delta POC in an RPS and the largest,  negative delta POC. Furthermore, the proposal reports that the method can be combined with other RPS intra prediction techniques and provide further bit reduction.  Combined with the POC granularity technique in JCTVC-G643, achieves a bit-rate reduction of 48 bits (20.7%) compared to 232 bits for the existing intra RPS method when inter RPS is off.  This is for random access conditions with a GOP size of eight.  Alone, the symmetry approach achieves a bit-rate reduction of 14 bits (6%) for the same simulation.  It is asserted that the method is most beneficial for applications where slice level transmission of the RPS is used frequently.

Can save bits in RA conditions where regular hierarchical coding patterns are used, but only if inter RPS is not used.  (WD does not require use of inter RPS).  No savings for LD because no positive POC ref pics.

No action.

JCTVC-H0551 Parameterized RPS Models [J. Zhao, S. Deshpande, A. Segall (Sharp)]

This contribution proposes a method to signal the RPS of common prediction structures efficiently.  A parameterized model is employed, and the parameters are transmitted from the encoder to the decoder in the PPS.  At the decoder, an RPS set is then generated from the bit-stream information and addressed in the same way as existing, explicit sets.  The result is improved coding efficiency.  For example, for HM5.0 RA and LD common test conditions, 18 bits are needed to signal the parameterized model instead of 142 bits for RA and 135 bits for LD to send a RPS set explicitly. Here, it is proposed to support parameterized models for hierarchical B and IPPP configurations.  Arbitrary configurations are still supported through the use of existing syntax.

Similar observation as in H0423 that regular patterns typically used, but very different approach.

A participant proposed that we need a few default RPS sets, with some parameters.

Text is immature.  Would need an RPS list generation process.

Further study encouraged.

JCTVC-H0196 AHG21: Comments on signaling of short-term reference pictures [Hendry, S. Jeong, S. Park, B. Jeon (LG)]

In the 7th JCT-VC meeting, method for signaling short-term and long-term reference picture was adopted. This document proposes some changes to the current syntax and semantics elements for signaling short-term reference pictures. The proposed changes are:

- Modification to signaling of number of reference pictures in RPS

- Making signaling of ‘used_by_curr_pic_sX_flag’ conditional on a new no_unused_ref_pic_flag

It is suggested that the proposed changes improve signaling efficiency without changing the main concept of the current signaling.

For #1, get rid of num_negative_ref_pics, instead use num_ref_pics.  It introduces PPS dependency on SPS, which is a problem.

#2 adds a flag to the PPS to sometimes avoid sending a bit in the RPS for each picture in the buffer.

Would need some semantic restriction to the no_unused_ref_pic_flag.

What is the default behavior if the conditionally sent flags are not sent?  Especially in the case when temporal layers are used, and if different slice types are present in the same picture.

Revisit.

REVISIT notes

Should rename to be a present flag.

Doesn’t provide any additional functionality, but saves some bits.

No action.

JCTVC-H0548 Signaling of collocated picture for HEVC [Y. Yu, K. Panusopone, L. Wang (Motorola Mobility)]

In the current working draft of HEVC, a flag, collocated_from_l0_flag, is used to explicitly indicate the collocated picture is a reference picture specified in either list0 or list1. In this proposal, we propose to remove collocated_from_l0_flag for low delay case because this flag is redundant. Furthermore, we propose to add a flag at sequence level to indicate that only one picture motion information is kept for collocated picture purpose instead of keeping multiple pictures in the current working draft. This way may save memory for low delay application without any performance impact if multiple reference pictures are used. Simulation results show that there is no any PSNR change for every slice while we save N-1 bits where N is number of pictures.

H0442 was already adopted this meeting, is related.

Decided this is not the appropriate BoG for this contribution.  Needs to be reallocated.

JCTVC-H0695 Cross check report of Motorola Mobility's proposal JCTVC-H0548 [?? (??)] [late] [miss]

Decided this is not the appropriate BoG for this contribution.  Needs to be reallocated.

## General discussion

What problems are we trying to change with the current design?

- Design cleanliness – prioritize

- Coding efficiency

Test conditions: We should  update the G1036 doc to contain extra conditions for ref pic reordering, and different ref pic reordering for each slice in flag.

Yan Ye to make a draft of proposed reordering conditions to add to G1036, will circulate via email to interested parties.  We will try to have an output document at this meeting.  Yan also agreed to provide config files available that correspond to the decided test conditions.

Suggestion to remove some test cases, but didn’t decide to do so at this time.

Karsten to email Yan the bug fix in case 2.6 to integrate.

Agree to not require running test cases that are not impacted by a proposal, for contributions to the next meeting.