- Draft specification of field indication SEI

D.1.X	Field indication SEI message syntax

| field_indication( payloadSize ) { | Descriptor |
| --- | --- |
| sequence_type_flag | u(1) |
| progressive_source_flag | u(1) |
| bottom_field_flag | u(1) |
| top_field_first_flag | u(1) |
| duplicate_flag | u(1) |
| reserved_zero_3bits /* equal to 0 */ | u(3) |
| } |  |

D.2.X	Field indication SEI message semantics

The field indication SEI message applies to the current access unit only.

The presence of field indication SEI message in the bitstream is specified as follows.

- If field_indication_presence_flag is equal to 1, one field indication SEI message shall be present in every access unit of the current coded video sequence.

- Otherwise (field_indication_presence_flag is equal to 0), one field indication SEI message may be present in any access unit of the current coded video sequence.

Note – In the absence of the field indication SEI message or other indication to the contrary, the current picture should be inferred to be a progressive frame coded as a frame picture.

sequence_type_flag equal to 0 indicates the coded video sequence conveys frame pictures. When sequence_type_flag is equal to 1, the coded video sequence conveys field pictures. If the SEI message is not present, the value may be inferred to be equal to 0. This flag shall have an identical value in each field indication SEI message of the coded video sequence.

NOTE – The video coding layer of HEVC does not treat access units conveying fields or frames differently. A field sequence would therefore be coded with picture dimensions of a field. For example, access units containing 1080i fields would commonly have dimensions of 1920 x 540, while the sequence picture rate would express the rate of the source fields (typically between 50 Hz and 60 Hz) instead of the source frame rate (25 to 30 Hz).

progressive_source_flag equal to 1 indicates the scan type of the access unit is progressive. When progressive_source_flag is equal to 0 the scan type of the access unit is indicated to be interlaced scanned source. The interpretation of combinations of field_indication_presencesequence_type_flag and progressive_source_flag values is defined in Table D-X.

- Table D-X – Interpretation of combinations of field_indication_presencesequence_type_flag and progressive_source_flag values

| - Interpretation | - field_indication_presencesequence_type_flag | - progressive_source_flag |
| --- | --- | --- |
| Progressive frame as frame picture | - 0 | - 1 |
| Interleaved fields in frame picture | - 0 | - 0 |
| Field picture | - 1 | - 0 |
| Field picture extracted from progressive frame | - 1 | - 1 |

bottom_field_flag indicates the parity of the field contained within the access unit when sequence_typefield_indication_presence_flag is equal to 1. bottom_field_flag equal to 1 specifies bottom field parity. Otherwise, bottom_field_flag equal to 0 specifies top field parity. When field_indication_presencesequence_type_flag is equal to 0, bottom_field_flag has no meaning and shall be equal to 0.

top_field_first_flag indicates field output order. When top_field_first_flag equal to 1, the top field is indicated to be temporally first, followed by the bottom field. When top_field_first_flag is equal to 0, the bottom field is indicated to be temporally first, followed by the top field. When field_indicationsequence_type_presence_flag is equal to 0 and progressive_source_flag is equal to 1, top_field_first_flag has no meaning and shall be equal to 0.

duplicate_flag equal to 1 indicates that the current picture is indicated to be a duplicate of a previous picture in display order. duplicate_flag equal to 0 indicates that the current picture is not indicated to be a duplicate picture.

NOTE1 – This flag should be used to mark coded pictures known to have osriginated from a repetition process such as 3:2 pull-down or other duplication and interpolation methods. This flag would commonly be used when a video feed is encoded in a "transport pass-through" fashion, with known duplicate pictures tagged by setting duplicate_flag equal to 1.

NOTE2 –When sequence_typefield_indication_presence_flag is equal to 1 and duplicate_flag is equal to 1, it is assumed the access unit contains a duplicated field of the previous field in output order with the same parity as the current field.

D.2.X	The nominal vertical and horizontal sampling locations for interlaced scanned 4:2:0, 4:2:2, and 4:4:4 formats (Informative) [Ed. (AHG13): This section should be placed in the end of Field indication SEI message semantics or VUI.]

The nominal vertical and horizontal sampling locations of samples in top and bottom fields for 4:2:0, 4:2:2, and 4:4:4 chroma formats are shown in Figure D-1, Figure D-2, and Figure D-3, respectively.

Figure D-1 – Nominal vertical and horizontal sampling locations of 4:2:0 samples in top and bottom fields

Figure D-2 – Nominal vertical and horizontal sampling locations of 4:2:2 samples in top and bottom fields

Figure D-3 – Nominal vertical and horizontal sampling locations of 4:4:4 samples in top and bottom fields

VUI modification (high-lighted with red font with yellow background)

VUI parameters syntax

| vui_parameters( ) { | C | Descriptor |
| --- | --- | --- |
| aspect_ratio_info_present_flag | 0 | u(1) |
| if( aspect_ratio_info_present_flag ) { |  |  |
| aspect_ratio_idc | 0 | u(8) |
| if( aspect_ratio_idc = = Extended_SAR ) { |  |  |
| sar_width | 0 | u(16) |
| sar_height | 0 | u(16) |
| } |  |  |
| } |  |  |
| overscan_info_present_flag | 0 | u(1) |
| if( overscan_info_present_flag ) |  |  |
| overscan_appropriate_flag | 0 | u(1) |
| video_signal_type_present_flag | 0 | u(1) |
| if( video_signal_type_present_flag ) { |  |  |
| video_format | 0 | u(3) |
| video_full_range_flag | 0 | u(1) |
| colour_description_present_flag | 0 | u(1) |
| if( colour_description_present_flag ) { |  |  |
| colour_primaries | 0 | u(8) |
| transfer_characteristics | 0 | u(8) |
| matrix_coefficients | 0 | u(8) |
| } |  |  |
| } |  |  |
| chroma_loc_info_present_flag | 0 | u(1) |
| if( chroma_loc_info_present_flag ) { |  |  |
| chroma_sample_loc_type_top_field | 0 | ue(v) |
| chroma_sample_loc_type_bottom_field | 0 | ue(v) |
| } |  |  |
| timing_info_present_flag | 0 | u(1) |
| if( timing_info_present_flag ) { |  |  |
| num_units_in_tick | 0 | u(32) |
| time_scale | 0 | u(32) |
| fixed_frame_rate_flag | 0 | u(1) |
| } |  |  |
| nal_hrd_parameters_present_flag | 0 | u(1) |
| if( nal_hrd_parameters_present_flag ) |  |  |
| hrd_parameters( ) | 0 |  |
| vcl_hrd_parameters_present_flag | 0 | u(1) |
| if( vcl_hrd_parameters_present_flag ) |  |  |
| hrd_parameters( ) | 0 |  |
| if( nal_hrd_parameters_present_flag ¦ ¦ vcl_hrd_parameters_present_flag ) |  |  |
| low_delay_hrd_flag | 0 | u(1) |
| pic_struct_present_flag | 0 | u(1) |
| field_indication_presence_flag | 0 | u(1) |
| bitstream_restriction_flag | 0 | u(1) |
| if( bitstream_restriction_flag ) { |  |  |
| motion_vectors_over_pic_boundaries_flag | 0 | u(1) |
| max_bytes_per_pic_denom | 0 | ue(v) |
| max_bits_per_mb_denom | 0 | ue(v) |
| log2_max_mv_length_horizontal | 0 | ue(v) |
| log2_max_mv_length_vertical | 0 | ue(v) |
| num_reorder_frames | 0 | ue(v) |
| max_dec_frame_buffering | 0 | ue(v) |
| } |  |  |
| } |  |  |

field_indication_presence_flag equal to 1 specifies that a field indication SEI message shall be present in every access unit of the current coded video sequence. field_indication_presence_flag equal to 0 specifiesindicates that a field indication SEI message may or may not be present in any access unit of the current coded video sequence. When field_indication_presence_flag is not present, its value shall be inferred to be equal to 0.