- Draft specification of field indication SEI

D.1.X	Field indication SEI message syntax

| field_indication( payloadSize ) { | C | Descriptor |
| --- | --- | --- |
| progressive_source_flag | 5 | u(1) |
| bottom_field_flag | 5 | u(1) |
| top_field_first_flag | 5 | u(1) |
| duplicate_flag | 5 | u(1) |
| reserved_zero_4bits /* equal to 0 */ | 5 | u(4) |
| } |  |  |

D.2.X	Field indication SEI message semantics

The field indication SEI message applies to the current access unit only.

The presence of field indication SEI message in the bitstream is specified as follows.

- If FieldSequenceFlag is equal to 1, one field indication SEI message shall be present in every access unit of the coded video sequence.

- Otherwise (FieldSequenceFlag is equal to 0), one field indication SEI message may be present in any access unit of the coded video sequence.

In the absence of the field indication SEI message, the current picture is a progressive frame coded as a frame picture.

progressive_source_flag equal to 1, indicates the scan type of the access unit is progressive. When progressive_source_flag is equal to 0, the scan type of the access unit is of a non-progressive source typical of content captured as fields by an electronic sensor.

| Interpretation | FieldSequenceFlag | progressive_source_flag |
| --- | --- | --- |
| Progressive frame as frame picture | 0 | 1 |
| Interleaved fields in frame picture | 0 | 0 |
| Field picture | 1 | 0 |
| Field picture extracted from progressive frame (PsF) | 1 | 1 |

bottom_field_flag indicates the parity of the field contained within the access unit when FieldSequenceFlag is equal to 1. bottom_field_flag equal to 1 specifies bottom field parity. Otherwise, bottom_field_flag equal to 0 specifies top field parity. When FieldSequenceFlag is equal to 0, bottom_field_flag shall be ignored.

The nominal vertical and horizontal sampling locations of samples in top and bottom fields for 4:2:0, 4:2:2, and 4:4:4 chroma formats are shown in Figure D-1, Figure D-2, and Figure D-3, respectively. [Ed. (AHG13): Is it possible to move these into the VUI subclause that describes chroma phases?]

top_field_first_flag indicates field output order. When top_field_first_flag equal to 1, the top field is output first, followed by the bottom field. When top_field_first_flag is equal to 0, the bottom field is output first, followed by the top field. When FieldSequenceFlag is equal to 0 and progressive_source_flag is equal to 1, top_field_first_flag shall be ignored.

duplicate_flag equal to 1 indicates that the current picture is known to be a duplicate of the previous picture in display order. duplicate_flag equal to 0 indicates that the current picture is not known to be a duplicate picture.

NOTE1 – This flag should be used to mark coded pictures known to have originated from a repetition process such as 3:2 pull-up or other duplication and interpolation methods. This flag would be commonly applied when a video feed is encoded in a "transport pass-through" fashion, with known duplicate pictures tagged by setting duplicate_picture equal to 1.

NOTE2 –When FieldSequenceFlag is equal to 1 and duplicate_flag is equal to 1, it is assumed the access unit contains a duplicated field of the previous field in display order of the parity indicated by top_field_first_flag.

VUI modification (high-lighted with red font with yellow background)

VUI parameters syntax

| vui_parameters( ) { | C | Descriptor |
| --- | --- | --- |
| aspect_ratio_info_present_flag | 0 | u(1) |
| if( aspect_ratio_info_present_flag ) { |  |  |
| aspect_ratio_idc | 0 | u(8) |
| if( aspect_ratio_idc = = Extended_SAR ) { |  |  |
| sar_width | 0 | u(16) |
| sar_height | 0 | u(16) |
| } |  |  |
| } |  |  |
| overscan_info_present_flag | 0 | u(1) |
| if( overscan_info_present_flag ) |  |  |
| overscan_appropriate_flag | 0 | u(1) |
| video_signal_type_present_flag | 0 | u(1) |
| if( video_signal_type_present_flag ) { |  |  |
| video_format | 0 | u(3) |
| video_full_range_flag | 0 | u(1) |
| colour_description_present_flag | 0 | u(1) |
| if( colour_description_present_flag ) { |  |  |
| colour_primaries | 0 | u(8) |
| transfer_characteristics | 0 | u(8) |
| matrix_coefficients | 0 | u(8) |
| } |  |  |
| } |  |  |
| chroma_loc_info_present_flag | 0 | u(1) |
| if( chroma_loc_info_present_flag ) { |  |  |
| chroma_sample_loc_type_top_field | 0 | ue(v) |
| chroma_sample_loc_type_bottom_field | 0 | ue(v) |
| } |  |  |
| timing_info_present_flag | 0 | u(1) |
| if( timing_info_present_flag ) { |  |  |
| num_units_in_tick | 0 | u(32) |
| time_scale | 0 | u(32) |
| fixed_frame_rate_flag | 0 | u(1) |
| } |  |  |
| nal_hrd_parameters_present_flag | 0 | u(1) |
| if( nal_hrd_parameters_present_flag ) |  |  |
| hrd_parameters( ) | 0 |  |
| vcl_hrd_parameters_present_flag | 0 | u(1) |
| if( vcl_hrd_parameters_present_flag ) |  |  |
| hrd_parameters( ) | 0 |  |
| if( nal_hrd_parameters_present_flag ¦ ¦ vcl_hrd_parameters_present_flag ) |  |  |
| low_delay_hrd_flag | 0 | u(1) |
| pic_struct_present_flag | 0 | u(1) |
| field_sequence_flag | 0 | u(1) |
| bitstream_restriction_flag | 0 | u(1) |
| if( bitstream_restriction_flag ) { |  |  |
| motion_vectors_over_pic_boundaries_flag | 0 | u(1) |
| max_bytes_per_pic_denom | 0 | ue(v) |
| max_bits_per_mb_denom | 0 | ue(v) |
| log2_max_mv_length_horizontal | 0 | ue(v) |
| log2_max_mv_length_vertical | 0 | ue(v) |
| num_reorder_frames | 0 | ue(v) |
| max_dec_frame_buffering | 0 | ue(v) |
| } |  |  |
| } |  |  |

field_sequence_flag equal to 1 indicates that the current sequence is a field sequence. field_sequence_flag equal to 0 indicates that the current sequence is a frame sequence. A field sequence consists of a series of access units each containing lines of either top or bottom parity. A frame sequence is a series of access units that contain either progressive frames or frames consisting of a line-interleaved top and bottom fields. When field_sequence_flag is not present, its value shall be inferred to be equal to 0. FieldSequenceFlag is set to field_sequence_flag.

NOTE – The video coding layer of HEVC does not treat access units conveying fields or frames differently. A field sequence would therefore be coded with picture dimensions of a field. For example, access units containing 1080i fields would commonly have dimensions of 1920 x 540 ( pic_width_in_luma_samples=1920, pic_height_in_luma_samples=540 ), while the sequence picture rate would express the rate of the source fields (typically between 50 Hz and 60 Hz) instead of the source frame rate (25 to 30 Hz).