| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0721 |
| --- | --- |

| Title: | Intra mode coding : Combination of H0075 and H0175 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Edouard FRANCOIS Chuohao Yeo | Tel: Email: | +33 2 99 87 68 83 edouard.francois@crf.canon.fr chyeo@i2r.a-star.edu.sg |
| Source: | Canon, Institute for Infocomm Research |  |  |

_____________________________

# Abstract

This contribution summarizes the combined proposal from JCTVC-H0075 and JCTVC-H0175 on intra mode coding. The solution is built on top of H0075, and includes the following additional features:

1.	35 modes for 4x4 PUs

2.	MPM sorting moved in remaining branch

3.	Simplified mode numbering (DC=0, then angular indexes 1..33, Planar=34)

4.	3 MPMs (Planar as 3rd MPM)

5.	Remove all contexts (including 1 context of HM5)

These modifications purportedly simplify the intra mode coding process and HEVC specification. An average BD-Rate gain of 0.2% and 0.3% for AI-HE and AI-LC configurations respectively is reported.

# Proposal description

The proposed in a modified version of H0075. The decoding tree of H0075 has been modified as follows:

All bins are bypass coded, which results in 1 less context than in current HM.

In addition the proposal includes the following features:

1.	35 modes for 4x4 PUs

2.	MPM sorting moved in remaining branch

3.	Simplified mode numbering (DC=0, then angular indexes 1..33, Planar=34)

4.	3 MPMs (Planar as 3rd MPM)

MPMs derivation:

- If left/top are non available or not intra or Planar, left/top are set to DC

- MPM0 is set to Left

- MPM1 is derived as follows

  - if (Top != Left)            MPM1 = Top

  - else if (Left != DC)      MPM1 = DC

  - else                              MPM1 = Vert

Remaining mode decoding:

- if (prev_intra)                		mode = MPM[idx]

- else if (planar_flag)       		mode = Planar

- else

  - if (MPM0>MPM1)       swap MPM0 & MPM1

  - mode = remaining

  - if (MPM0<= mode)      mode++

  - if (MPM1<= mode)      mode++

The WD changes are attached to the contribution.

# Results

The complete results are in the attached xls file.

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A (8bit) | -0.2% | 0.2% | 0.3% | -0.4% | 0.0% | 0.0% |
| Class B | -0.1% | -0.1% | -0.1% | -0.3% | -0.2% | -0.2% |
| Class C | -0.2% | -0.1% | -0.1% | -0.3% | -0.2% | -0.2% |
| Class D | -0.2% | 0.0% | 0.0% | -0.2% | -0.2% | -0.2% |
| Class E | -0.1% | 0.1% | 0.3% | -0.3% | -0.1% | 0.0% |
| Overall | -0.2% | 0.0% | 0.0% | -0.3% | -0.2% | -0.1% |
|  | -0.2% | 0.0% | 0.0% | -0.3% | -0.2% | -0.1% |
| Class F | -0.2% | 0.2% | 0.2% | -0.3% | 0.2% | 0.0% |
| Enc Time[%] | 97% |  |  | 98% |  |  |
| Dec Time[%] | 100% |  |  | 101% |  |  |

# Patent rights declaration(s)

Canon Research Centre France may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Institute for Infocomm Research may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).