| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 | Document: JCTVC-H0724 |
| --- | --- |

| Title: | BoG report on coefficient level coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Vivienne Sze | Email: | sze@ti.com |
| Source: | BoG on coefficient level coding |  |  |

# Abstract

The BoG met on February 5, 2012 from 8:45pm – 10:55pm to review 7 proposals (14 documents) on coefficient level coding.

# Recommendations

The BoG recommends to adopt the following

- Level luma context reduction (JCTVC-H0130)

- Simplified coeff_abs_level_minus3 coding (JCTVC-H0498 8-bit look up table with 4 parameters)

Further investigation in CE is recommended on

- Context assignment for parallel coefficient level (JCTVC-H0550)

- Context Set Selection for Coefficient Level Coding (JCTVC-H0404)

# Reviewed Proposals

| JCTVC-H0130 | CE11 Subtest 2: Level luma context reduction | Samsung |
| --- | --- | --- |
| JCTVC-H0160 | CE11:Context reduction for coefficient level | Panasonic |
| JCTVC-H0550 | Non-CE11: context assignment for parallel coefficient level coding | QCOM |
| JCTVC-H0497 | Improving coeff_abs_level_minus3 coding performance under high bitrate | Motorola |
| JCTVC-H0498 | Simplified coeff_abs_level_minus3 coding | Motorola |
| JCTVC-H0404 | Context Set Selection for Coefficient Level Coding | RIM |
| JCTVC-H0062 | Context Simplification on coeff_abs_level_greater1_flag | BJUT |

JCTVC-H0130 CE11 Subtest 2: Level luma context reduction [Y. Piao, J. Min, J. H. Park (Samsung)]

JCTVC-H0446 CE11: Cross-check of Samsung’s context number reduction for level coding in CABAC [J. Sole (Qualcomm)] [late]

This contribution presents a simplified luma context set for coeff_abs_level_greater1_flag and coeff_abs_level_greater2_flag to remove some contexts used in level coding. A total of 56 contexts are used for level coding in current software, 32 for coeff_abs_level_greater1_flag and 24 for coeff_abs_level_greater2_flag. With simplified context set proposed in this contribution, 14 contexts can be removed with negligible performance change in BD-rate in common test condition (0.0% in all configurations).

JCTVC-H0160 CE11:Context reduction for coefficient level [K. Terada, H. Sasai, T. Nishi (Panasonic)]

JCTVC-H0355 CE11: Cross-check results for subtest 3.2.2 [V. Sze (TI)] [late]

This proposal presents a technique for context reduction of coeff_abs_level_greater1_flag and coeff_abs_level_greater2_flag. In this proposal, if context set index is equal to 1, 2, 4 or 5, the contexts for each coeff_abs_level_greater1/2_flag are reduced by changing the threshold of c1 and c2. c1 threshold is changed from 3 to 2. c2 threshold is changed from 2 to 1. This proposal was implemented on HMv5.0 and evaluated in the CE11. Contexts of coeff_abs_level_greater1/2_flag were reportedly reduced from 56 to 46 without major impact on coding efficiency.

Both H0130 and H0160 propose context reduction for coeff_abs_level_greater1_flag and coeff_abs_level_greater2_flag.

H0130 reduces the number of ctxSet for luma from 6 to 4.  Context reduction of 14.

Some support expressed for H0130. Pending adoption following review of non-CE proposals. Recommend Adopt

H0160 reduces number of context for certain ctxSet for both luma and chroma; coeff_abs_level_greater1_flag reduced from 3 to 2, and coeff_abs_level_greater2_flag from 2 to 1.  One extra step in order to change step depending on ctxSet. Context reduction of 10.

Comment from expert that while context reduction is desirable, additional logic may not be worth the trade-off of context reduction.

Negligible loss for both approaches at both common conditions and low QP is reported.

Verbally reported that combined results H0160 and H0130 with loss of 0.04%, with 20 context reduction.

JCTVC-H0062 Context Simplification on coeff_abs_level_greater1_flag [Wenpeng Ding, Yunhui Shi, Baocai Yin (BJUT)]

JCTVC-H0690 Cross-check of JCTVC-H0062 on context simplification of coeff_abs_level_greater1_flag [H. Yang (Huawei)] [late] [miss]

The proposed technique in this contribution is aimed to simplify contexts using in coefficients entropy coding of coeff_abs_level_greater1_flag. In this proposal, the number of contexts in the same group is reduced . The number of contexts for coeff_abs_level_greater1_flag is reduced from 32 to 24. All tests about this contribution are based on HM5.0. Experimental results show that the BD-Rate increase is negligible compared with HM5.0 with all test configurations.

0.07% loss in Intra LC and 0.05% RA LC case. No low QP results. Reduce by 8 contexts.

Change context calculation formula from

ctxIdxInc  =  ( ctxSet  *  4)  +   min(greater1Ctx, 3)

to

ctxIdxInc  =  ( ctxSet  *  3)  +   min((greater1Ctx, 3)+1)>>1

What is coding efficiency of H0062 with H0130? Additional reduction of 6 on top of H0130.

Experts comment loss is of concern and not worth trade-off context reductions.

JCTVC-H0404 Context Set Selection for Coefficient Level Coding [N.Nguyen, T.Ji, D.He, G.Martin-Cocher (RIM)]

JCTVC-H0358 Non-CE11: Cross-check of proposal H0404 on coefficient level context selection [V. Sze (TI)] [late] [miss]

A refined context set selection algorithm for coefficient level coding is proposed. The proposed algorithm uses information from neighbouring coefficient groups and therefore, is more consistent with multi-level significant coefficient flag coding, while providing a small improvement in BD-rate. On average, under common test conditions against HM-5.0 anchor, the proposal improves the BD-rate by 0.1% for the AI-HE configuration and suffers no loss in all other configurations.  For low QP settings, the proposal improves the BD-rate by 0.1% for the AI-HE, AI-LC and RA-HE10 configurations.

Change context selection for determining ctxSet to be based on neighbors rather the previous scan set. In HM-5.0, neighbor is far when moving to next diagonal scan.

f(R, L) = Q(R) + Q(L)
Q(k) = 0 if k = 0,

Q(k) = 1 if 0 < k <= 3, and Q(k) = 2 otherwise

Gains of -0.1% AI_HE; loss of 0.4% in class F LB_LC for low QP

Experts comment need to check and buffer neighbors. As compared to HM-5.0 one counter.  Question about how many bits per buffer: 2-bits per sub-block x 8 (16 bits for 32x32)

Comment that approach of the proposal seems to be logical, but no coding gains.

While increasing complexity, no clear benefit in coding efficiency. Extra read/write operations.

How will this perform with H0130?

Further study to demonstrate coding gains. (CE)

JCTVC-H0550 Non-CE11: context assignment for parallel coefficient level coding [W. -J. Chien, J. Chen, J. Sole, M. Karczewicz (Qualcomm)]

JCTVC-H0659 Non-CE11: Crosscheck for Qualcomm's context assignment for parallel coefficient level coding in JCTVC-H0550 [T.-D. Chuang, Y.-W. Huang (MediaTek)] [late]

In current HM, the context modeling of the syntax element coeff_abs_level_greater1_flag is dependent on the value of previous coded coeff_abs_level_greater1_flag. Up to 16 coeff_abs_level_greater1_flag might be coded consecutively. This dependency hinders the throughput of the entropy coding and allegedly, it is a main bottleneck in current CABAC design. This contribution removes the dependency of previous coded coeff_abs_level_greater1_flag for context modeling. The coding performance impact of the proposed scheme is an average 0.1% BD-rate increase for all common test configurations.

Scheme 1: For coeff_abs_level_greater1_flag use count of non-zero significant_coeff_flag for both chroma and luma; for luma, 4 context sets with 6 contexts per ctxSet.  Reduced ctxSets, increase contexts/ctxSet = same number of contexts as in HM-5.0

Scheme 2: Scheme 1 + coeff_abs_level_greater2_flag: 4 ctxSet for luma with 4 contexts per ctxSet (using 4 more contexts for coeff_abs_level_greater2_flag than H0130)

Proposing Scheme 2

Coding loss for Scheme 1 and Scheme 2 is 0.1%.

Parallelism desirable but loss is a concern if they losses add up from other simplifications. Noted that additional context required relative to H0130.

Seems desirable however no software and WD available. Suggest combine with H0130 and revisit.

Further study in CE highly encouraged.

JCTVC-H0497 Improving coeff_abs_level_minus3 coding performance under high bitrate [J. Lou, X. Fang, L. Wang (Motorola Mobility)]

JCTVC-H0607 Cross-check of JCTVC-H0497: Improving coeff_abs_level_minus3 coding performance under high bitrate [X. Zhang, S. Liu (Mediatek)] [late]

In the current CABAC of HEVC, syntax coeff_ab_level_minu3 is used to represent the absolute value of a quantized transform coefficient level minus 3. Adaptive truncated Rice code with parameter 0, 1, 2, and 3 are used. This document proposes to add another parameter 4 to improve the coding performance under high bitrate. Cross-check will be provided by MediaTek. The attached spreadsheet contains detailed data of the results.

JCTVC-H0498 Simplified coeff_abs_level_minus3 coding [J. Lou, L. Wang (Motorola Mobility)]

JCTVC-H0633 Cross-verification of Simplified coeff_abs_level_minus3 coding from Motorola Mobility (H0498) [L. Guo (Qualcomm)] [late] [miss]

In the current CABAC of HEVC, syntax coeff_ab_level_minu3 is used to represent the absolute value of a quantized transform coefficient level minus 3. Adaptive truncated Rice code with parameter 0, 1, 2, and 3 are used. This document proposes to simplify the truncated Rice code with 8 bit codeword, so that memory could be saved if lookup table is used for implementation. Cross-check will be provided by Qualcomm. The attached spreadsheet contains detailed data of the results.

H0497: Add parameter 4 for high bitrate - low QP (12, 17, 22, 27) – -0.36% AI_HE, -0.35% AI_LC

H0498: Use 8-bit codeword to reduce memory of lookup table, no loss on common condition; loss at low QP (0.16 for AI_HE, 0.17 AI_LC, ~0.5% class F)

With parameter 4, low QP (-0.22 for AI_HE, -0.21 AI_LC)

Complexity: 294 bytes in HM-5.0, 181 bytes in proposal + extra logic to switch to parameter 4

Several experts express trade-off of complexity and coding gain desirable.

WD text provided; experts suggest changes to draft are simple.

Cross-check document H0633 states that code and proposal match as well as results (cross-checker not present).

Recommend Adopt.