| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0727 |
| --- | --- |

| Title: | JCT-VC BoG report: tiles and wavefront parallel processing |  |  |
| --- | --- | --- | --- |
| Status: | [Input Document to JCT-VC] |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Michael Horowitz 1055 W. Hastings Street Vancouver, BC V6E 2E9 Canada | Tel: Email: | +1 778 731-3432 x323 michael@ebriskvideo.com |
| Source: | eBrisk Video, Inc. |  |  |

_____________________________

# Abstract

This BoG report contains notes including recommendations to JCT-VC on the topic of tiles, wavefront parallel processing and entropy slices.

# Introduction

This BoG report is organized into three sections: tiles, wavefront parallel processing, and entropy slices. The following text color coding conventions are applied:

- Red text indicates the result of the BoG discussion

- Blue text indicates additional conditions that should be considered in connection with the BoG discussion result

- Yellow highlighting indicates a BoG recommendation to the JCT-VC.

*JCTVC-H0179 Storage reduction for dependent Tiles [E. François (Canon), S. Pautet]

This contribution proposes a modification of the dependent tiles design to limit the storage issue due to dependencies across vertical tile boundaries. The proposal is similar to the solution used for horizontal LCU borders. Motion data of 4x4 blocks adjacent to right borders of tiles are downsampled. Regarding intra mode, mode of PUs on left borders of a tile is forced to Planar. The proposal has been evaluated using tests conditions used in JCTVC-F335. It is reported that no coding impact is observed for all test configurations, while half of the memory is saved for motion data, and no memory is required for intra mode data.

Comment: mitigates some of the single core decoder related implementation concerns associated with to dependent tiles, but does not address all concerns (e.g., it is still necessary to save data at vertical tile boundaries).

Note: Sub sampling MV data further was not investigated.

Note: The software implementation for the proposal was based on software associated with JCTVC-F060 which was adopted in Geneva (G145: Elimination of line buffer, G229: Reducing motion data line buffers)

If dependent tiles remains in our design, this BoG recommends adoption of both elements of this proposal.

*JCTVC-H0658 AHG4: Crosscheck for Canon's storage reduction for dependent tiles in JCTVC-H0179 [T.-D. Chuang, Y.-W. Huang (MediaTek)] [late]

Note: Cross check matched results completely.

*JCTVC-H0203 AHG4: Further latency reduction of CABAC initialization for dependent Tiles [Hendry, S. Jeong, S. Park, B. Jeon (LG)]

When dependent tiles are processed, a low latency CABAC initialization scheme was proposed. In that proposal, the CABAC probabilities of the first LCU in each tile are inherited from those of the adjacent left LCU. This contribution builds on the proposal in that the CABAC probabilities of the first LCU in each tile of the first tile column, from the second tile row, are inherited from those of the adjacent upper LCU, on top of the probability inheritance from the left LCU in the proposal. It is claimed that the proposed method can further reduce processing latency in the proposal, by not having to wait for all the tiles in the upper LCU row of the row boundaries to be processed before inheriting the CABAC probabilities. Result from experiments report that the proposed method does not cause loss when 4 tiles (2 tile rows and 2 tile columns) and causes maximum 0.3% loss when 16 tiles (4 tile rows and 4 tile columns) is used.

Comment: there is a need to store information to initialize tiles in the second tile row (and below) in the case of a single core decoder implementation.

Related comment: depending on implementation buffer space may be reused obviating the need for additional memory.

Comment: the proposal directly addresses the latency issue only to the first tile in each tile row.

Question: How much coding efficiency is gained by dependent tiles vs. independent tiles under the same conditions.

Question: what is the maximum loss for a given sequence for 16 tiles? Answer: Basketball drill 0.5%.

The BoG did not reach consensus to recommend adoption this proposal.

*JCTVC-H0414 Cross-check of “AHG4: Further latency reduction of CABAC initialization for dependent Tiles” (JCTVC-H203) [M. Horowitz, S. Xu (eBrisk)]

Note: Results 100% matched those reported by proponent.  Three minor comments were discussed regarding the software implementation.

*JCTVC-H0329 AHG4: Crosscheck for LG's further latency reduction of CABAC initialization for dependent tiles in JCTVC-H0203 [C.-W. Hsu, Y.-W. Huang (MediaTek)] [late]

Note: Results matched 100%.

*JCTVC-H0348 Simplification on tiles and slices [T. Lee, B. Choi, J. Park (Samsung)]

This document proposes a design for simplification of slices and tiles. In the design, there is no separation between independent tile and dependent tile and at every ends of tiles, a slice is terminated. Therefore, every slice is included in a tile with no crossing tile boundaries. To emulate independent tile, slice is used and its boundary is coincided with tile boundary while to emulate dependent tile, entropy slice is used and its boundary is coincided with tile boundary.

With the proposed design, tile boundary is not needed to check availability and only slice (or entropy slice) boundary is enough to determine availability for in-picture prediction and entropy context prediction. QP and cabac_init_idc are initialized from the (first) slice header in tiles. Tile entry points in the bitstream are known by parsing slice header’s slice address without explicit location information for markers.

Comment: There is concern over slice header and NAL unit header overhead per tile when one slice is required per tile compared with the case when many tiles may be contained in one slice.

Comment: Tiles make parallelism independent of the slice structure so the degree of parallelism depends on the number of tiles. If one wishes to remove the overhead discussed in the above comment, then one needs to constrain the degree of parallelism. This is particularly true for lower bit rates.

Comment: there may be other ways (in addition to independent tiles) to reduce losses resulting from slice headers.

Comment: support was expressed for this idea.

Comment: This proposed design reduces the number of condition checking when retrieving neighbour information possibly for every TU and PU.

Comment: WPP may be used to reduce slice header overhead for very low bit rate cases.

Comment: WPP may not be applicable for low delay applications.

The BoG did not reach consensus to recommend adoption this proposal at this time. Other proposals make similar recommendations and some of these proposals are to be considered in the profile and level discussions.  We defer decision to the plenary.

*JCTVC-H0270 AHG4: Avoiding lower left LCUs at dependent tile column boundaries [C.-W. Hsu, C.-Y. Tsai, Y.-W. Huang, S. Lei (MediaTek)]

In this contribution, a constraint is proposed for reducing processing latency that avoids using lower left LCUs at dependent tile column boundaries. As a result of a bug, the latest HM includes the proposed constraint. It is proposed therefore, that the WD be modified to match the HM and that the HM bug report and associated software patch be closed without action.

If dependent tiles remains in our design, the BoG recommends adoption of this contribution pending the test results comparing results with and without the proposed constraint.

*JCTVC-H0345 Constrained Tile for parallel decoding [H. Sasai, K. Uchibayashi, T. Nishi (Panasonic)]

In order to entropy decode bit streams in parallel, decoder needs to search the beginning of each parallel processing units. In this contribution, 1-bit flag in slice header is proposed to indicate whether the slice starts at the top-left position of tile for easier implementation of tile level parallel processing. Furthermore, a set of restrictions on tile and slice for the same purpose is also proposed as a starting point of profile discussion.

Comment: The information conveyed by this flag can be derived in other ways.

Comment: There is a comment that “For HD and larger size, it is assumed to use tile for parallel processing”.  It was clarified that this statement is not an integral part of the proposal.

Comment: this proposal shares a commonality with H0348 in that slices are not allowed to cross tile boundaries.

The idea that each tile shall start with a slice is covered in the comment for H0463.

The BoG recommends no action to the proposal of adding a bit to the slice header to indicate whether the slice is the first in a tile.

*JCTVC-H0593 Cross-verification of JCTVC-H0348 on simplification on tiles and slices [V. Seregin, M. Coban (Qualcomm)] [late]

The results matched the proponents.

*JCTVC-H0463 Comments on specifying tile support in HEVC Profiles and Levels [W. Wan, T. Hellman (Broadcom)]

Several techniques have been proposed in previous JCT-VC meetings and adopted into the general HEVC framework to enable parallelism for multi-core architectures.  As the committee attempts to reach CD (Committee Draft) status, it is unclear how these techniques map into appropriate Profile and Level definitions.  An example of these parallelism techniques is tiles.  This submission discusses several aspects related to supporting tiles in decoders to determine whether and how tiles would fit within a particular Profile/Level definition where every decoder conformant to that definition is required to support decoding of a stream encoded using tiles and some applications would not use tiles at all.

Several constraints are clearly described in the contribution.

BoG recommends the adoption of the constraint that a picture shall not be composed of N tiles and M wavefronts where N > 1 and M > 1.

BoG recommends the study of whether in-loop deblocking is sufficient to eliminate visual artefacts when SAO and ALF are active, but do not filter across vertical and/or horizontal tile boundaries.

A few BoG participants felt that dependent tiles should be considered for a separate profile (outside the “main” profile), the others felt that dependent tiles should be prohibited.  No consensus was reached on this point.

Comment: it was suggested that coding efficiency be investigated comparing dependent and independent tiles.  In particular, making certain that all recent dependent tile adoptions are included in the investigation.

Comment: it was stated that it would be helpful to have an implementation of a decoder that decodes a bitstream containing tiles in picture raster scan to ensure there are no parsing issues.

BoG recommends that a maximum number of tile columns constraint be considered when defining profile(s) and levels.

BoG recommends: Tiles may be composed of multiple complete slices.

With respect to the availability of the slice header in the context of a single core decoder wishing to reconstruct LCUs in picture scan order, two solutions were identified:

- All LCUs in a slice shall lie “beyond” the first LCU of the slice in the picture raster scan. (i.e., this constraint would not prevent multiple tiles to be contained in one slice)

Three examples of slice and tile combinations, two allowed combinations (top) and one dis-allowed combination (bottom)

- Slices shall not cross tile boundaries (i.e., this constraint would prevent multiple tiles to be contained in one slice).  Note that this constraint implies the constraint above.

The BoG recommends constraint #1.

The BoG was unable to reach consensus regarding a position on constraint #2. We recommend further discussion on this topic in the JCT-VC plenary.

*JCTVC-H0520 Tile groups [Y. -K. Wang, A. K. Ramasubramonian, Y. Chen, M. Coban, M. Karczewicz (Qualcomm)]

This document reproposes the proposal in JCTVC-G318, with less types of tile groups. A tile grouping mechanism is proposed, wherein tiles are assigned to different tile groups. Each tile group is identified by a tile group ID value. The decoding order of the tile groups is in the ascending order of the group ID value. Inside each tile group, the tiles are decoded in a raster scan order. Four tile group types are defined, some of which may remind readers of some slice group types in AVC. It is asserted that tile groups may be used for improved parallel processing, improved error resilience and flexible region of interest (ROI) coding.

Comment: the proposed parallel processing use case, introduces latency.

BoG recommends no action on this contribution

*JCTVC-H0521 Flexible tile dependency and loop filter control [Y. -K. Wang, J. Chon, Y. Chen, I. S. Chong, M. Coban, M. Karczewicz (Qualcomm)]

This document reproposes the proposal in JCTVC-G317, with some changes. It is proposed to enable the control of in-picture prediction and loop filtering between tiles individually for each vertical or horizontal tile boundary. In-picture prediction includes pixel value prediction, motion prediction, coding mode prediction, and entropy coding context prediction. Loop filtering operations include deblocking filtering, adaptive loop filtering, and sample adaptive offset.

Comment: the proposed use case may be achieved through clever use of combinations of loop filter independence flag for tile and slice boundaries.

The BoG recommends no action.

*JCTVC-H0556 AHG4: Harmonized method for signalling entry points of tiles and WPP substreams [Hendry, Sang Oh Jeong, Seung Wook Park, Byeong Moon Jeon (LG), Kiran Misra, Andrew Segall (Sharp)]

In the last meeting, a harmonized method for signaling entry points for tiles and wavefront parallel processing was proposed (JCTVC-G722).  Here, we provide new results with improvements on the method.  Specifically, the proposal contains the following features: (1) unified syntax and semantics for tiles and WPP entry points, (2) byte alignment of the WPP substreams to facilitate signaling all entry points as bytes, (3) coding the entry point offsets as u(v), and (4) simplifying the signaling of the entry_point_type flag.

It is claimed that the proposed unified syntax and semantic for tiles’ and WPP substreams’ entry points provides simpler signaling method rather than having separate signaling method for tiles and WPP substreams.  It is also reported that the remaining modifications improve signaling efficiency by requiring less bits compare to current method of signaling WPP substreams’ and tiles’ entry points, respectively. Gains because of these modifications are about 0.1% for Y, Cb, and Cr for all configurations except intra only.

Comment: the BoG has recommended that tiles and wavefronts not be used together. Consequently, some of the proposed syntax would need modification to account for this.

The BoG recommends adoption of this proposal.

It is assumed that the above comment will be addressed if this proposal is adopted.

It is assumed that the cross-check comments will be taken into account if this proposal is adopted.

*JCTVC-H0370 Crosscheck of LGE/Sharp proposal for byte-aligned substreams (JTCVC-Hxxxx) [Felix Henry, Gordon Clare (Orange)]

The code appears to do what is claimed by the proponent.

The results matched 100% except in one configuration (LBHE) where the reference was very slightly different (cross check gains are larger than those reported by the proponent by 0.1%)

The crosscheck contains two comments suggesting improvement to the text proposed for the WD.

*JCTVC-H0293 Syntax on parallel processing information signaling [S. S. Jeong, Y. Park, C. Kim, J. H. Kim, J. H. Park (Samsung)]

Current syntax design related to the techniques supporting parallel processing is dispersed in SPS and PPS. So, decoder system have to decode a lot of syntax related to parallel processing for knowing whether techniques supporting parallel processing are used or not. This contribution changes current syntax design to more readable and simple merged design fitting to the environment for multi-core. This contribution also proposes new flags for informing whether a sequence or picture uses slice or entropy slice.

Comment: the slice_enabled_flag which indicates whether the picture contains more than 1 slice cannot be set until the first slice is completely encoded which introduces delay.

Comment: changing the value of the slice_enabled_flag would require sending a new (or referencing a different) PPS for each picture.

Comment: an SEI per picture may be more reliable.

Comment: perhaps the APS is a more appropriate place, but this would break the concept of collecting all parallelism tools together.

Comment: if slice_enabled_flag were not proposed this proposal would possibly be more palatable.

Comment: the indication of slice_enabled_flag and entropy_slice_enabled_flag may be better suited to and SEI message.

The BoG recommends investigating the placing of WPP parameters into the SPS in a manner consistent with the way tiles parameters are signaled.

# Wavefront parallel processing

*JCTVC-H0226 On derivation of luma QP predictor for Wavefront Parallel Processing [S. Shimada, K. Kazui, J. Koyama, A.Nakagawa (Fujitsu)]

This contribution proposes to fix the bug in the derivation process of the QP predictor of the first quantization group (QG) of each TreeBlock (TB) line when Wavefront Parallel Processing (WPP) is used. In the WD5/HM5, the QP predictor of the first QG in the N-th TB line is derived from QP of the last QG in the (N-1)-th TB line. This QP derivation significantly prevents TB line based parallel processing using WPP. It is proposed that the QP predictor of the first QG in the N-th TB line is set to QP of the last QG in M-th TB line where M = N－(num_substreams_minus1 + 1). As the result, the parallelism of WPP is improved.

The BoG acknowledges the problem identified in the proposal is real.  In addition to the proposed solution, we identified 2 additional candidate solutions.

- proposed solution: inherit QP value of last QG in bitstream order

- inherit QP value of one of the QGs in the LCU immediately above

- inherit QP value from slice in which the LCU belongs (as is done for independent tiles)

The solution proposed in H0226 (i.e. #1) requires the decoder to use the same (or lower) degree of parallelism as contained in the bitstream. Solutions #2 and #3 remove that constraint.

The BoG recommends adoption of #3 since it addresses the issue raised in the contribution and for consistency of the specification.

*JCTVC-H0349 Simplification on WPP [T. Lee, J. Choi, J. Park (Samsung)]

This document proposes (1) Remove non-flush mode (2) No adaptation on synchronization for copying initial CABAC probabilities. There are two modes of non-flush and flush modes to determine CABAC reset at the end of each LCU line and at the first LCU in a thread, CABAC probabilities are copied from the specified LCU indicated by synchronization offsets. The main purpose of non-flush mode and   adaptive synchronization is performance improvement over flush mode and non-adaptive synchronization. However, according to test results, flush mode has only 0.2% loss over non-flush mode and synchronization with the LCU above and to the right by 2 LCUs has 0.0% improvement over the one with the LCU above and to the right by 1 LCU.

BoG recommends adoption of the idea in this proposal which are identical to those proposed in items 2 and 3a of H0517.

*JCTVC-H0328 AHG4: Crosscheck for Samsung's simplification on WPP in JCTVC-H0349 [C.-Y. Tsai, Y.-W. Huang (MediaTek)] [late]

The cross-checker was not present.

*JCTVC-H0513 On parallel processing schemes [Y. -K. Wang, M. Coban (Qualcomm)]

Lots of concerns regarding different possible combinations of tiles, wavefront parallel processing (WPP), and entropy slices were reportedly heard during the previous JCT-VC meeting. In response to those concerns, this document proposes to disallow the combinations of more than one of tiles, wavefront parallel processing (WPP), and entropy slices.

WPP is simplified and considered as one mode of tiles. Entropy slices associated with a particular regular slice are contained in one NAL unit instead of their own individual NAL units. A unified entry point byte offset signaling is used for all the three parallel processing schemes. Byte alignment bits padding is based on the first CU address in a partition (tile or entropy slice).

The required text changes are included in an attachment of this document, based on HEVC WD5d2 in JCTVC-G1103_d2.

More discussions on the proposed changes specific for entropy slices are also found in JCTVC-H0515, and more discussions on the proposed changes specific for WPP are also found in JCTVC-H0517.

The BoG recommends to disallow the combination of WPP and entropy slices

The BoG could not reach consensus to recommend to disallow the combination of tiles and entropy slices.  One participant discussed his concern about entropy decoding load balancing when tiles are in use if entropy slices were disallowed at the same time (assuming WPP is also disallowed at the same time). A related comment: entropy decoding load balancing can be achieved using normal slices (with some coding efficiency loss). The BoG recommends further discussion on this topic in a JCT-VC plenary session.

*JCTVC-H0517 On wavefront parallel processing [Y. -K. Wang, M. Coban (Qualcomm)]

This document proposes to simply the wavefront parallel design in the following three sub-proposals:

1) bitstream causality is kept to avoid bitstream pointer jumping back and forth within a coded slice;

2) the entropy coding synchronization point is fixed to be the second LCU in the above LCU row; 3a) CABAC flushing

3b) and byte alignment at the end of LCU row is mandated.

Comment: For proposal #1 (to solve bitstream causality concern), there is a fundamental trade-off between entry point overhead and the degree of freedom of parallelism. Maximum loss in common conditions is in class E and is around 1%.

Regarding sub-proposal #1, the BoG recommends that the proponents evaluate the overhead due to the addition of entry points as proposed.

The BoG recommends adoption of sub-proposal #2: The entropy coding synchronization point is fixed to be the end of the second LCU in the above LCU row. H349 recommends the same idea. This adoption will require the removal of the entropy_coding_synchro syntax element and an accompanying modification to the decoding process. The proponents are asked to work together to propose how entropy_coding_synchro syntax element shall be removed.

Comment: Worst case bit rate overhead for cabac flush, byte alignment and entry point signalling for entry point at every LCU row is up to 2% for class E for classes A and B 0.4% and class C 0.6%.

The BoG recommends adoption of sub-proposal #3a: CABAC flushing at the end of each LCU row is mandated.  The idea is also included in H0349. This adoption will require the removal of the cabac_istate_reset_flag syntax element and an accompanying modification to the decoding process.

The BoG recognizes the need to investigate whether the value of having byte alignment is worth the 0.6% BD-rate average loss (across all sequences) that is incurred for sub-proposal #3b: mandating byte alignment at the end of each LCU row.

# Entropy slices

*JCTVC-H0515 On entropy slices [M. Coban, Y. -K. Wang, J. Chon, M. Karczewicz (Qualcomm)]

Entropy slices provide independently parsable units by disabling usage of context elements across slices and resetting of CABAC states at the beginning of each entropy slice, while allowing in-picture prediction across slices for decoding. Each entropy slice is encapsulated in its NAL unit with its light weight slice header. Tiles and wavefront parallel processing methods that offer alternative parallel processing mechanisms signal entry points to parallel decodable units in the main slice header. In a similar approach this document proposes that all entropy slices associated with the same “parent” slice are encapsulated into one coded slice NAL unit and the entry points of the entropy slices are signalled in the slice header. Simulation results show 0.1% to 0.2% BD rate coding gain for common test configurations.

Summary: Proposal places all entropy slices in a slice into a single NALU and introduces entry points to signal the start of each entropy slice.

Comment: if we had a unified entry point signalling (as proposed in H0556) this proposal would need to be harmonized into that.

Comment: the proposal introduces latency (at the encoder) as a result of adding entry point into the slice header which is not the case in the current design in the WD.

Comment: this proposal has potential to improve error resilience.

The BoG recommends no action.

*JCTVC-H0642 Cross-verification of JCTVC-H0515 On Entropy Slices [T. Lee, J. Park (Samsung)] [late]

The results match the results of the proponent 100%.