# WD text Changes (changes marked in yellow)

# Table 7.3.10 residual coding syntax

| residual_coding ( x0, y0, log2TrafoWidth, log2TrafoHeight, scanIdx, cIdx ) { | Descriptor |
| --- | --- |
| last_significant_coeff_x_prefix | ae(v) |
| last_significant_coeff_y_prefix | ae(v) |
| if ( last_significant_coeff_x_prefix > 3 ) |  |
| last_significant_coeff_x_suffix | ae(v) |
| if ( last_significant_coeff_y_prefix > 3 ) |  |
| last_significant_coeff_y_suffix | ae(v) |
| numCoeff = 0 |  |
| do { |  |
| xC = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ numCoeff ][ 0 ] |  |
| yC = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ numCoeff ][ 1 ] |  |
| numCoeff++ |  |
| } while( ( xC != LastSignificantCoeffX ) ¦¦ ( yC != LastSignificantCoeffY ) ) |  |
| numLastSubset = (numCoeff − 1) >> 4 |  |
| for( i = numLastSubset; i >= 0; i− − ) { |  |
| offset = i << 4 |  |
| if( max( log2TrafoWidth, log2TrafoHeight ) > 3) { |  |
| xCG = ScanOrder[ log2TrafoWidth – 2 ][ log2TrafoHeight – 2 ][ scanIdx ][ i ][ 0 ] |  |
| yCG = ScanOrder[ log2TrafoWidth – 2 ][ log2TrafoHeight – 2 ][ scanIdx ][ i ][ 1 ] |  |
| rightCGFlag = (xCG = = (1<< (log2TrafoWidth – 2)) − 1) ? 0 : significant_coeff_group_flag[ xCG + 1 ][ yCG ] |  |
| bottomCGFlag = (yCG = = (1 << (log2TrafoHeight – 2)) − 1) ? 0 : significant_coeff_group_flag[ xCG ][ yCG + 1 ] |  |
| if( (i = = numLastSubset) ¦ ¦ (rightCGFlag + bottomCGFlag = = 2) ¦ ¦ (i = = 0) ) |  |
| significant_coeff_group_flag[ xCG ][ yCG ] = 1 |  |
| Else |  |
| significant_coeff_group_flag[ xCG ][ yCG ] | ae(v) |
| for( n = 15; n >= 0; n− − ) { |  |
| xC = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ n + offset ][ 0 ] |  |
| yC = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ n + offset ][ 1 ] |  |
| if ( (n + offset) < (numCoeff − 1) && significant_coeff_group_flag[ xCG ][ yCG ] ) { |  |
| numNZInCG = (i = = numLastSubset) ? 1 : 0 |  |
| if ( (n > 0) ¦ ¦ (rightCGFlag + bottomCGFlag = = 2) ¦ ¦ (i = = 0) ¦ ¦ (numNZInCG > 0) ) { |  |
| significant_coeff_flag[ xC ][ yC ] | ae(v) |
| numNZInCG += significant_coeff_flag[ xC ][ yC ] |  |
| Else |  |
| significant_coeff_flag[ xC ][ yC ] = 1 |  |
| } |  |
| } |  |
| } else { |  |
| for( n = 15; n >= 0; n− − ) { |  |
| xC = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ n + offset ][ 0 ] |  |
| yC = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ n + offset ][ 1 ] |  |
| if( (n + offset) < (numCoeff − 1) ) |  |
| significant_coeff_flag[ xC ][ yC ] | ae(v) |
| } |  |
| } |  |
| numSigCoeff = 0 |  |
| firstGreater1CoeffIdx = -1 |  |
| for( n = 15; n >= 0; n− − ) { |  |
| xC = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ n + offset ][ 0 ] |  |
| yC = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ n + offset ][ 1 ] |  |
| if( significant_coeff_flag[ xC ][ yC ] && numSigCoeff < 8 ){ |  |
| coeff_abs_level_greater1_flag[ n ] | ae(v) |
| numSigCoeff++ |  |
| if (coeff_abs_level_greater1_flag[ n ] && firstGreater1CoeffIdx == -1) |  |
| firstGreater1CoeffIdx = n |  |
| } |  |
| } |  |
| if(firstGreater1CoeffIdx != -1) |  |
| coeff_abs_level_greater2_flag[ firstGreater1CoeffIdx] | ae(v) |
| for( n = 15; n >= 0; n− − ) { |  |
| xC = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ n + offset ][ 0 ] |  |
| yC = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ n + offset ][ 1 ] |  |
| if( significant_coeff_flag[ xC ][ yC ] ) { |  |
| coeff_sign_flag[ n ] | ae(v) |
| } |  |
| numSigCoeff = 0 |  |
| for( n = 15; n >= 0; n− − ) { |  |
|  | ae(v) |
| xC = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ n + offset ][ 0 ] |  |
| yC = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ n + offset ][ 1 ] |  |
| if( significant_coeff_flag[ xC ][ yC ] ) { |  |
| baseLevel = 1 + coeff_abs_level_greater1_flag[ n ] + coeff_abs_level_greater2_flag[ n ] |  |
| if(baseLevel == numSigCoeff < 8 ? 2 + (n == firstGreater1CoeffIdx) : 1 ){ |  |
| coeff_abs_level_remaining[ n ] |  |
| transCoeffLevel[ x0 ][ y0 ][ cIdx ][ xC ][ yC ] = ( coeff_abs_level_remaining[ n ] + baseLevel ) * ( 1 − 2 * coeff_sign_flag[ n ] ) |  |
| numSigCoeff++ |  |
| } else |  |
| transCoeffLevel[ x0 ][ y0 ][ cIdx ][ xC ][ yC ] = 0 |  |
| } |  |
| } |  |
| } |  |

7.4.10 Residual coding semantics

coeff_abs_level_remaining[ n ] is the remained value of a transform coefficient level that is coded with golomb-Rice code at the scanning position n. The value of coeff_abs_level_remaining is constrained by the limits in subclause XX.When coeff_abs_level_remaining [ n ] is not present, it is inferred as 0.

Change in section 9.2.1.1 Initialization process for context variables

Table 9-1 – Association of ctxIdx and syntax elements for each slice type in the initialisation process

|  | Syntax element | ctxIdxTable | Slice Type |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | I | P | B |
| slice_header() | alf_cu_flag | Error: Reference source not found | 0 | 1 | 2 |
| … | … | … | … | … | … |
| residual_coding() | last_significant_coeff_x_prefix | Error: Reference source not found | 0..29 | 30..59 | 60..89 |
|  | last_significant_coeff_y_prefix | Error: Reference source not found | 0..29 | 30..59 | 60..89 |
|  | significant_coeff_group_flag | Table 9-26 | 0..3 | 4..7 | 8..11 |
|  | significant_coeff_flag | Table 9-28 | 0..47 | 48..95 | 96..143 |
|  | coeff_abs_level_greater1_flag |  | 0.. 23 | 24.. 47 | 48.. 71 |
|  | coeff_abs_level_greater2_flag | Error: Reference source not found | 0..5 | 6..11 | 12..17 |

Table 9-2 – Values of variable initValue for coeff_abs_level_greater1_flag ctxIdx is updated as

| Initialisation variable | coeff_abs_level_greater1_flag ctxIdx |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 |
| initValue | 104 | 68 | 116 | 86 | 104 | 132 | 86 | 87 | 102 | 66 | 114 | 68 | 87 | 84 | 100 | 101 |
|  | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 |
| initValue | 104130 | 130 | 147 | 149 | 104 | 196 | 100 | 165 | 119 | 179 | 179 | 164 | 119 | 85 | 117 | 149 |
|  | 32 | 33 | 34 | 35 | 36 | 37 | 38 | 39 | 40 | 41 | 42 | 43 | 44 | 45 | 46 | 47 |
| initValue | 133 | 98 | 114 | 115 | 118 | 99 | 115 | 116 | 135 | 146 | 147 | 164 | 119 | 148 | 116 | 133 |
|  | 48 | 49 | 50 | 51 | 52 | 53 | 54 | 55 | 56 | 57 | 58 | 59 | 60 | 61 | 62 | 63 |
| initValue | 119 | 179 | 148 | 164 | 119 | 85 | 117 | 149 | 133 | 98 | 114 | 115 | 118 | 99 | 115 | 100 |
|  | 64 | 65 | 66 | 67 | 68 | 69 | 70 | 71 |  |  |  |  |  |  |  |  |
| initValue | 135 | 177 | 147 | 164 | 119 | 132 | 148 | 149 |  |  |  |  |  |  |  |  |

Table 9-3 – Values of variable initValue for coeff_abs_level_greater2_flag ctxIdx is updated as

| Initialisation variable | coeff_abs_level_greater2_flag ctxIdx |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 |
| initValue | 86 | 102 | 115 | 117 | 101 | 101 | 84 | 117 | 83 | 100 | 84 | 117 |
|  | 12 | 13 | 14 | 15 | 16 | 17 |  |  |  |  |  |  |
| initValue | 84 | 117 | 83 | 84 | 84 | 117 |  |  |  |  |  |  |

Table 9-31 – Syntax elements and associated types of binarization, maxBinIdxCtx, ctxIdxTable, and ctxIdxOffset

| coeff_abs_level_greater1_flag | I | FL, cMax = 1 | 0 |  | 0 |
| --- | --- | --- | --- | --- | --- |
|  | P |  | 0 |  | 24 |
|  | B |  | 0 |  | 48 |
| coeff_abs_level_greater2_flag | I | FL, cMax = 1 | 0 | Error: Reference source not found | 0 |
|  | P |  | 0 | Error: Reference source not found | 6 |
|  | B |  | 0 | Error: Reference source not found | 12 |

9.2.2.9 Binarization process for coeff_abs_level_remaining

Input to this process is a request for a binarization for the syntax element coeff_abs_level_remaining[ n ].

Output of this process is the binarization of the syntax element.

The variables cLastSE and cLastRiceParam are derived as follows.

- If n is equal to 0 or all previous syntex elements coeff_abs_level_remaining[ pos ] with pos less than n are derived to be equal to 0 instead of being explicitly parsed, cLastSE and cLastRiceParam are set equal to 0.

- Otherwise ( n is not equal to 0 ), cLastSE is set equal to coeff_abs_level_remaining[ n  − 1 ] and cLastRiceParam is set equal to the value of cRiceParam that has been derived during the invocation of this subclause for the syntax element coeff_abs_level_remaining[ n – 1 ] of the same transform block.

- – Specifcation of cRiceParam depending on cLastSE and cLastRiceParam

The variable cRiceParam is derived from cLastSE and cLastRiceParam equivalent to the following pseudo-code

cRiceParam = cLastRiceParam

+ (cLastSE >= 3   &&  cLastRiceParam == 0)

+ (cLastSE >= 6   &&  cLastRiceParam <= 1)

+ (cLastSE >= 12  &&  cLastRiceParam <= 2);

+ (cLastSE >= 23  &&  cLastRiceParam <= 3);

The variable cTRMax is derived from cRiceParam as given in Table 9-36.

Table 9-36 – Specifcation of cTRMax depending on cRiceParam

| cRiceParam | cTRMax |
| --- | --- |
| 0 | 7 |
| 1 | 14 |
| 2 | 26 |
| 3 | 46 |
| 4 | 78 |

###### 9.2.3.1.1.6 Derivation process of ctxIdxInc for the syntax element coeff_abs_level_greater1_flag

Inputs to this process are the color component index cIdx, the 16 coefficient subset index i and the current coefficient scan index n within the current subset.

Output of this process is ctxIdxInc.

The variable ctxSet specifies the current context set and for its derivation the following applies.

- If n is equal to 15 or all previous syntax elements coeff_abs_level_greater1_flag[ pos ] with pos greater than n are derived to be equal to 0 instead of being explicitly parsed, the following applies.

- The variable ctxSet is initialized as follows.

- If the current subset index i is equal to 0 or cIdx is greater than 0, the following applies.

ctxSet  =  0		(9-1)

- Otherwise (i is greater than 0 and cIdx is equal to 0), the following applies.

ctxSet  =  2		(9-2)

- When the subset i is not the first one to be processed in this subclause, the following applies.

- The variable numGreater1 is set equal to the variable numGreater1 that has been derived during the last invocation of subclause  for the syntax element coeff_abs_level_greater2_flag for the subset i + 1.

- When ( numGreater1  >>  1 ) is greater than 0, ctxSet is incremented by one as follows.

ctxSet  =  ctxSet  +  1		(9-3)

-

- The variable greater1Ctx is set equal to 1.

- Otherwise (coeff_abs_level_greater1_flag[ n ] is not the first to be parsed within the current subset i),.for the derivation of ctxSet and greater1Ctx the following applies.

- The variable ctxSet is set equal to the variable ctxSet that has been derived during the last invocation of this subclause.

- The variable greater1Ctx is set equal to the variable greater1Ctx that has been derived during the last invocation of this subclause.

- When greater1Ctx is greater than 0, the variable lastGreater1Flag is set equal to the syntax element coeff_abs_level_greater1_flag that has been used during the last invocation of this subclause and greater1Ctx is modifed as follows.

- If lastGreater1Flag is equal to 1, greater1Ctx is set equal to 0.

- Otherwise (lastGreater1Flag is equal to 0), greater1Ctx is incremented by 1.

The context index increment ctxIdxInc is derived using the current context set ctxSet and the current context greater1Ctx as follows.

ctxIdxInc  =  ( ctxSet  *  4 )  +  Min( 3, greater1Ctx )	(9-5)

When cIdx is greater than 0, ctxIdxInc is modified as follows.

ctxIdxInc =  ctxIdxInc  +  16		(9-6)

###### 9.2.3.1.1.7 Derivation process of ctxIdxInc for the syntax element coeff_abs_level_greater2_flag

Inputs to this process are the color component index cIdx, the 16 coeffcient subset index i and the current coefficient scan index n within the current subset.

Output of this process is ctxIdxInc.

The variable ctxSet specifies the current context set and for its derivation the following applies.

- If n is equal to 15 or all previous syntex elements coeff_abs_level_greater2_flag[ pos ] with pos greater than n are derived to be equal to 0 instead of being explicitly parsed, the following applies.

- The variable ctxSet is initialized as follows.

- If the current subset index i is equal to 0 or cIdx is greater than 0, the following applies.

ctxSet  =  0		(9-7)

- Otherwise (i is greater than 0 and cIdx is equal to 0), the following applies.

ctxSet  =  2		(9-8)

- The variable numGreater1 is set equal to 0.

- When the subset i is not the first one to be processed in this subclause, the following applies.

- The variable numGreater1 is set equal to the variable numGreater1 that has been derived during the last invocation of this subclause for the subset i + 1 and is modifed as follows.

numGreater1  =  numGreater1  >>  1		(9-9)

- When numGreater1 is greater than 0, ctxSet is incremented by one as follows.

ctxSet  =  ctxSet  +  1		(9-10)

-

-

ctxIdxInc = ctxSet 		 (9-13)

When cIdx is greater than 0, ctxIdxInc is modified as follows.

ctxIdxInc =  ctxIdxInc  +  4		(9-14)