| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC- H0728 WG11 number: |
| --- | --- |

| Title: | BoG report on High throughput binarization schemes for CABAC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Alberto Dueñas 2315 N. First Street, San Jose, CA 95131 USA | Email: | alberto@cavium.com |
| Source: | Cavium Inc. |  |  |

_____________________________

# Abstract

This BoG report contains notes including recommendations to JCT-VC on the topic of high throughput binarization for CABAC Entropy codecs. The BoG met on February 5, 2012 from 3:00 pm – 5:30 5pm to review 2 proposals for high throughput binarization schemes and reached the conclusion to adopt one of propose schemes.

- Mandates

The BoG mandate was to perform further assessment of JCTVC-H0554 and JCTVC-H0510 including maturity and fulfillment of purpose to limit the throughput in the expected way while not overly penalizing the compression performance or adding complexity to the design by adding additional methods of entropy coding, operations, etc.

- Recommendations

The BoG recommends adoption of the following:

- Scheme II from Throughput improvement on CABAC coefficients level (JCTVC-H0554)

pending satisfactory results and review of working draft text after integration with JCTVC-H0130 and JCTVC-H0498.

It was expressed by some of the experts that there is a strong preference to have a single binarization scheme as part of the HEVC CD. Based on that conclusion, it was specified that it will be preferable to select a scheme with minor BD-rate implications instead of having a different high throughput scheme in addition to a high efficiency scheme.

One expert asked the BoG members if the improvements presented by the schemes under discussion where achieving the goals that the BoG was set up to investigate. It was the general understanding that both method 2 from JCTVC-H0510 and scheme II from JCTVC-H0554 will satisfy the requirements to substantially increase the throughput compared with the current scheme used on WD5[1]. The members of the BoG identified that method 2 from JCTVC-H0510 will deliver higher throughput than scheme II from JCTVC-H0554.

Based on that conclusion and the fact that Scheme II from JCTVC-H0554 offers a better trade-off between BD-rate as well as improvements on throughput compared with method 2 from JCTVC-H0510, the group recommends adoption pending of satisfactory results after integration with JCTVC-H0130 and JCTVC-H0498 of scheme II from JCTVC-H0554.

# Reviewed Proposals

JCTVC-H0554 Non-CE1: throughput improvement on CABAC coefficients level coding [J. Chen, W.-J. Chien, R. Joshi, J. Sole, M. Karczewicz (Qualcomm)]

This proposal targets improving CABAC throughput by reducing context coded bins for coefficients level coding. In current HM, three flags, significant_coeff_flag, coeff_abs_level_greater1_flag and coeff_abs_level_greater2_flag are coded using contexts before switching to the Golomb-Rice by-pass mode coding for the remaining level. This contribution proposes coding coeff_abs_level_greater1_flag only for a few starting non-zero coefficients and only one coeff_abs_level_greater2_flag in a subset consisting of16 coefficients and an early switch to Golomb-Rice code.

Coding efficiency impact of the first proposed scheme is -0.04% BD-Rate gain on average in common test condition, and -0.10% at low QPs (1, 5, 9 and 13). It is further reported that this scheme reduces the maximum number of context adaptively coded bins by averagely 41% among all configurations, up to 44% in intra configuration at QP being 1, and, by averagely 10% among all configurations, up to 18% in intra configuration at QP being 22.

Coding efficiency impact of the second proposed scheme is -0.00% BD-Rate gain on average in common test condition, and -0.05% at low QPs. It is further reported that this scheme reduces the maximum number of context adaptively coded bins by averagely 32% among all configurations, up to 36% in intra configuration at QP being 1, and, by averagely 8% among all configurations, up to 15% in intra configuration at QP being 22.

The proponent identified scheme II from JCTVC-H0554 as its preference to be further investigated.

BD-Rate performance at common test conditions Qp= [22, 27, 32, 37] and at low QP test conditions Qp= [1, 5, 9, 13]

| Test configuration | Scheme II (Normal QP) |  |  | Scheme II (Low QP) |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate |  |  |  |  |  |
|  | Y | Cb | Cr | Y | Cb | Cr |
| HE_Intra | -0.1% | -0.1% | -0.1% | -0.3% | -0.2% | -0.2% |
| HE_RA | 0.0% | -0.1% | -0.2% | 0.1% | 0.2% | 0.1% |
| HE_LD | 0.0% | 0.1% | -0.1% | 0.1% | 0.2% | 0.2% |
| LC_Intra | -0.1% | -0.1% | -0.1% | -0.3% | -0.2% | -0.2% |
| LC_RA | 0.0% | 0.1% | -0.1% | 0.0% | 0.0% | 0.0% |
| LC_LD | 0.0% | 0.1% | -0.3% | 0.0% | 0.1% | 0.1% |
| HE_RA_10bits | 0.1% | 0.1% | 0.2% | 0.1% | 0.3% | 0.3% |
| Average | 0.00% | 0.02% | -0.06% | -0.05% | 0.06% | 0.03% |

The cross-checker document JCTVC-H0660 was reviewed, this document indicated that: “The BD-rates from our experimental results match exactly those provided by Qualcomm. The encoding time and decoding time are the same to those provided by Qualcomm.”  This indicates that the cross-checker verified the results presented on JCTVC-H0554.

The proponent of JCTVC-H0554 presented to the rest of the BoG members the necessary software changes required to implement its proposal. The software changes to implement JCTVC-H0554 scheme II were reviewed by the experts to have a better understanding of their scheme. It was expressed by some of the experts that the software changes presented on the proposal were easy to understand and easy to implement in the current HM.

No issues where identified by the BoG members in relation to the software presented by the proponent of JCTVC-H0554.

Recommend Adoption.

JCTVC-H0510 Non-CE1: High Throughput Binarization (HTB) method with modified level coding [S.H. Kim, K. Misra, L. Kerofsky, A. Segall (Sharp)]

In this contribution, a new efficient high throughput binarization (HTB) method for CABAC is presented.

The intention of this approach is to reduce the worst-case complexity of CABAC for low complexity use cases, keeping easy compatibility with the existing CABAC. In addition, the proposed method provides the flexibility between coding performance and throughput efficiency by selectively applying HTB mode for each 4x4 coefficient block. It is reported that the proposed method reduces the number of context adaptively coded bins about 24 % under the common test conditions, and about 46 % in the low QP range where the complexity of CABAC is reportedly more problematic. It is further reported that the proposed approach can provide more flexible HTB method by employing selective activation of HTB mode in each sub-block level between coding performance and throughput efficiency.

The proponent identified Method 2 from JCTVC-H0510 as their preference to be further investigated.

H0510 BD-Rate performance at common test conditions[2] Qp= [22, 27, 32, 37]

| Configuration | M2+TH_2 |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| AI-HE | 1.2% | 1.3% | 1.3% |
| RA-HE | 1.2% | 1.0% | 1.0% |
| LB-HE | 1.6% | 0.9% | 0.7% |
| AI-LC | 1.0% | 1.1% | 1.1% |
| RA-LC | 0.6% | 0.6% | 0.5% |
| LB-LC | 0.7% | 0.2% | 0.0% |
| RA-HE10 | 2.0% | 1.4% | 1.6% |

H0510 BD-Rate performance at low QP test conditions Qp= [1, 5, 9, 13]

| Configuration | M2+TH_2 |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| AI-HE | 0.9% | 1.3% | 1.4% |
| RA-HE | 3.1% | 3.5% | 3.3% |
| LB-HE | 1.6% | 0.9% | 0.7% |
| AI-LC | 0.7% | 0.9% | 0.9% |
| RA-LC | 1.8% | 2.1% | 1.9% |
| LB-LC | 2.0% | 2.1% | 2.0% |
| RA-HE10 | 3.7% | 4.2% | 3.9% |

The cross-checker document JCTVC-H0704 indicates that: “This contribution reports results by Nokia’s cross-check on JCTVC-H0510 on a high throughput binarization proposed by Sharp. It is reported that the proposed high throughput mode reduces the average number of context coded bins by 20 % in common conditions range. The effect of the proposed high throughput mode on objective compression efficiency is reported to vary from +1.5 % (RA_LC) to +2.8 % (RA_10) in common conditions simulations.”  Indicating that the cross-checker verified the results presented on JCTVC-H0510.

The software changes to implement JCTVC-H0510 Method 2 were reviewed by the experts to have a better understanding of their scheme. The group acknowledged that the changes to implement this proposal were very simple.

It was identified that JCTVC-H0510 Method 2 did not modify the RDOQ and the lost was smaller on the LC cases than the HE cases. Further optimization of the RDOQ may produce some enhancements on the BD-rate for the HE mode when using this scheme. It was noted that the LC cases have a DB-rate change of 0.6-2.0%.

JCTVC-H0489  Complexity analysis of high throughput CABAC Entropy codecs [Alberto Duenas, Prashant Arora, Oscar Patino, Francisco Javier Roncero (Cavium)]

This contribution presents the importance of defining a method to analyze the complexity of high throughput CABAC Entropy codec schemes. It does so by examining an example analysis of the current entropy codec method part of the HEVC WD5. This contribution suggests that JCT-VC establishes a breakout group (BoG) in order to analyze the complexity of different options for a high throughput CABAC Entropy codec.

This proposal indicated the importance of focusing on the worse case scenarios when defining high throughput CABAC Entropy codec schemes.

Summary of JCTVC-H0510 and JCTVC-H0554 high throughput proposals was presented:

|  | Bjontegaard Delta Rate (Normal QP[2] and Low QP) |  |  | Coments |
| --- | --- | --- | --- | --- |
| Tool | Min | Max | Ave | Proposed modification |
| JCTVC-H0510 Method2: TH=2 | +0.6% +1.3% | +1.6% +2.9% | +1.2% +2.0% | H0510 Method 1 is based on CAVLC Method 2 and 3 based on rice-Golomb coding of non-bypass bin. |
| JCTVC-H0554 Scheme II | - -0.5% -0.3% | +0.1% +0.1% | -0.1% -0.0% | Level bins coded for a few starting non-zero coefficients in each 16 coefficient group. Modified Golomb-Rice parameters update table (right shift by 1 element) |

It was agreed by the BoG to use the number of coded bins in the worst case as the method to perform analysis of different proposals. It was generally agreed that the he worst case scenario is more important than the average case.

The results for the worst case for the scheme under analysis are as follow:

| Proposal | H0510 | H0554 |
| --- | --- | --- |
| Number of Coded Bins per coefficient | Method 2: 1.25 | Scheme II: 1.56 |
| Performance BD-rate change (%) | Method 2: 0.6 to 3.1 | Scheme II: -0.3 to 0.1 |

The results from the analysis performed by the member of the BoG and presented on JCTVC-H0489 were reviewed and verified to be correct. No objections on the accuracy of the results were raised among the members of the BoG.

# Reviewed results after final integration

The members of the BoG were asked to work on the integration of Scheme II JCTVC-H0554 with JCTVC-H0130 and JCTVC-H0498 to be able to analyze the results and to make sure that they were not unexpected interactions.

Results have been obtained by the proponent of JCTVC-H0554 of the integrated software and the results were cross-checked by Samsung (the proponent of JCTVC-H0130) and Mediatek (the original cross-checker of JCTVC-H0554).

The results of the integration are satisfactory and also showing positive synergies between the three contributions.

The results of the integration of JCTVC-H0554 (scheme II + JCTVC-H0130 + JCTVC-H498) are as follow:

| Test configuration | Normal QP |  |  | Low QP |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Bjontegaard Delta Rate |  |  |  |  |  |
|  | Y | Cb | Cr | Y | Cb | Cr |
| HE_Intra | -0.06% | -0.02% | -0.04% | -0.7% | -0.5% | -0.5% |
| HE_RA | -0.02% | -0.10% | -0.09% | 0.1% | 0.2% | 0.1% |
| HE_LD | 0.05% | 0.01% | -0.11% | 0.1% | 0.3% | 0.2% |
| LC_Intra | -0.05% | -0.04% | -0.04% | -0.6% | -0.5% | -0.5% |
| LC_RA | -0.02% | 0.01% | -0.02% | 0.0% | 0.0% | 0.0% |
| LC_LD | -0.01% | 0.11% | -0.13% | 0.0% | 0.1% | 0.1% |
| HE_RA_10bits | 0.16% | 0.12% | 0.15% | 0.2% | 0.4% | 0.3% |
| Average | 0.01% | 0.01% | -0.04% | -0.12% | 0.00% | -0.04% |

Working draft text created by the proponents of JCTVC-H0554, JCTVC-H0130 and JCTVC-H0498 has been attached with this BoG report and it is available for review and integration in the following HEVC Working Draft.

# References

- B. Bross, W.-J. Han, J. R. Ohm, G. J. Sullivan, and T. Wiegand, “WD5: Working Draft 5 of High-Efficiency Video Coding”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC- G1103, 7th Meeting: Geneva, CH, November 2011.

- F. Bossen, “Common conditions and software reference configurations,” in JCTVC-G1200, 7th Meeting: Geneva, CH, November 2011.