| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0729 |
| --- | --- |

| Title: | BoG report on motion compensation operation and interpolation filters |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Takeshi Chujoh 1, Komukai-Toshiba-cho, Saiwai-ku, Kawasaki, 212-8582, Japan | Tel: Email: | +81-44-549-2152 takeshi.chujoh@toshiba.co.jp |
| Source: | BoG |  |  |

_____________________________

# Abstract

In this contribution, the result of break out group on motion compensation operation and interpolation filters is reported. On 5th Feb. 2012, four contributions that are JCTVC-H0180, H0569, H0401, H0546 and their cross-checked documents were presented and discussed.

# Recommendations

The BoG recommends conducting the following actions:

- JCTVC-H0180: No action. The current specification of interpolation filters on HM has been verified.

- JCTVC-H0569: If a profile does not include SAO and ALF, it should be recommended to be discussed again.

- JCTVC-H0401: Further study. It is recommended that its experiment should be conducted.

- JCTVC-H0546: The point of proposal has been understood. There might be some benefits for hardware implementation. Further study.

# Document review notes

### Interpolation filters and MV precision (ref CE3)

JCTVC-H0180 Non-CE3: Phase adjustment of interpolation filter [T.Chujoh, T.Yamakage (Toshiba)]

New interpolation filter coefficients for quarter pixel position for luminance, eighth pixel and three-eighth pixels position for chrominance have been introduced at previous Geneva meeting.  However, the phase shift of those filter coefficients of quarter pixel position for luminance and three-eighth pixels position for chrominance are not accurate fractional pixel positions.  Therefore, in this contribution, two filter coefficients are designed to adjust accurate fractional pixel position and their experimental results are reported.  As experimental results, all average of coding gains are almost zero; however, an average of 1.2% gain has been achieved in the case of low delay P on low complexity condition.  The computational complexities have been reduced about 2% on average and the results of memory bandwidth have been reduced about 1% on average.

The following two filter coefficients sets are changed.

Table 1	 Interpolation filter coefficients on HM5

|  | Fractional position | Filter coefficients | Phase shift (DC) |
| --- | --- | --- | --- |
| Luma | 1/4 | -1, 4, -10, 58, 17, -5, 1, 0 | 15/64 |
|  | 1/2 | -1, 4, -11, 40, 40, -11, 4, -1 | 32/64 |
| Chroma | 1/8 | -2, 58, 10, -2 | 8/64 |
|  | 1/4 | -4, 54, 16, -2 | 16/64 |
|  | 3/8 | -6, 46, 28, -4 | 26/64 |
|  | 1/2 | -4, 36, 36, -4 | 32/64 |

Table 2	 Improved interpolation filter coefficients

|  | Fractional position | Filter coefficients | Phase shift (DC) |
| --- | --- | --- | --- |
| Luma | 1/4 | -1, 4, -10, 57, 18, -5, 1, 0 | 16/64 |
|  | 1/2 | -1, 4, -11, 40, 40, -11, 4, -1 | 32/64 |
| Chroma | 1/8 | -2, 58, 10, -2 | 8/64 |
|  | 1/4 | -4, 54, 16, -2 | 16/64 |
|  | 3/8 | -6, 48, 26, -4 | 24/64 |
|  | 1/2 | -4, 36, 36, -4 | 32/64 |

Regarding coding efficiency, the current specification has no problem. The experimental result showed a verification of current specification for interpolation filters.

The cross-checker explained how to design filter coefficients of HM5.  Since they were designed automatically, the error of phase shift was caused by arithmetic precision of filter coefficients.

Current specification of interpolation filters on HM has been verified. No action.

JCTVC-H0610 Non-CE3: Cross-check for phase adjustment of interpolation filter (JCTVC-H0180) by Samsung [E. Alshina, A. Alshin, J. H. Park (Samsung)] [late]

Test results provided by Toshiba are verified here. Proponent proposed to modify interpolation filter coefficients for Luma and Chroma in order to remove phase shift mismatch appeared due to rounding filter coefficients. Proposed modification doesn’t lead to performance improvement: in average 0.3% (mandatory tests)/ 0.1% (all tests) Luma BD-rate drop. Statistically modified filter reduces hit-ratio of fractional MVs.

| Fig. 1. Frequency responses in amplitude (a) and phase (b) domains for 1/4 7 taps Luma filter. |  |
| --- | --- |

| Fig. 2. Frequency responses in amplitude and phase domain for 3/84 taps Chroma filter. |  |
| --- | --- |

From these graphs we conclude that for both Luma 1/4-pel filter and 3/8 Chroma filter proponent resolves mismatch between theoretical and actual phase shift. From view point of error accumulation or de-noising effect both HM5.0 and filters in verified tests should have almost equal effect (Chroma filter may have higher error accumulation compare to HM5.0, but difference is very small).

The analyses of the filter coefficients are very useful and it should be recorded.

JCTVC-H0569 Non-CE3: An Adaptive Interpolation Filtering Technique for Improved Subjective Quality in the Low Complexity Configuration [F. Kossentini, N. Mahdi, H. Guermazi, M. Horowitz, S. Xu (eBrisk Video Inc.)]

In this contribution, a simplified version of the Adaptive Interpolation Filtering (AIF) technique described in JCTVC-D397, JCTVC-E284, JCTVC-F216 and JCTVC-G057 is proposed for interpolation filtering of the luma and chroma samples of video sequences. For each slice, the encoder first generates two new one-dimensional filters for the luma samples and four new one-dimensional filters for the chroma samples. The new filters are constrained to have identical characteristics to the HM 5.0 default filters (e.g., same number of coefficients, same symmetry, and same bit depths). Then, the encoder decides, for each slice, whether to use the HM 5.0 default filter(s) or corresponding new filter(s) and indicates its choice(s) and, if necessary, sends coefficient values using newly proposed syntax elements.

Compared to HM5.0 LC, the proposed technique yields an average BD-rate reduction of 2.5%, 2.4% and 1.9% in Y, U and V (respectively), while requiring the same average decoding time and a ~3% increase in average encoding time. More importantly, the proposed AIF technique is reported to improve the subjective quality of several common test sequences (Vidyo1, Vidyo3 and BQTerrace) and non-common test video sequences (vidyo1_fade, vidyo3_fade and SpinCalendar).

The default filters are HM5.0 and the filter set is selected slice by slice.

The encoding time is increased 3% and the decoding time is the same as anchor. Pre-analyses of encoding process are introduced.

Formal subjective quality test has been conducted. Several fade scenes were added to test sequences. The results were obviously improved.

There was a concern about computational precision of filter processing; however, proponent said it is possible to solve it and the restriction of filter coefficients is no problem for coding efficiency.

From the view point of hardware implement, there was also a concern about the variable multiple operations.

Proponents said "if would be profile which will not use both SAO and ALF, then adaptive interpolation filter seems to be a good technical solution".

Cross-checker verified the subjective improvement of proposal and found the improvement of fade-scenes especially.

Recommendation: The technical benefits and week points of this contribution have well-understood from past discussions. If a profile does not include SAO and ALF, it is recommended to be discussed again.

JCTVC-H0207 Non-CE3: Crosscheck of JCTVC-H0569 – An Adaptive Interpolation Filtering Technique for Improved Subjective Quality in the Low Complexity Configuration [J. Park, Hendry, B. Jeon (LG)]

There was misunderstanding about the anchor data used in this experiment between proponent and cross-checker. Cross-checker initially assumed that the anchor used in this experiment is generated from HM-5.0 while later it was found out that HM-5.0rc1 should have been used.

JCTVC-H0304 6H7Q luma interpolation filter [Hongbo Zhu] [late][miss]

The document is not uploaded.

### Prediction methods

JCTVC-H0401 Differentiated weighting parameter sets for weighted prediction [J. Xu (Sony), A. Tourapis (Magnum), A. Tabatabai (Sony)]

This proposal introduces differentiated weighting parameter sets for weighted prediction. Two separate weighting parameter sets serving for uni-prediction and bi-prediction respectively are signaled explicitly in weighted prediction syntax. The overhead for weighting parameter sets is small while weighted prediction can be improved.

The proposal is as follow.

Two weighting parameter sets;

One for uni-prediction

The other for bi-prediction

Bi-prediction weighting parameter sets are encoded using prediction from uni-prediction weighting parameter sets.

Chroma weighted prediction;

Add one flag (chroma_offset_pred_flag) to select between two approaches in chroma weighted prediction

E.g. (8-bit)

If chroma_offset_pred_flag == 1

P’ = w × (P -128)+ o+128

else

P	’ = w × P + o

Since this contribution does not include any experimental result, it was suggested that the experimental results should be shown.

Recommendation: Further study should be encouraged.

JCTVC-H0546 On weighted prediction precision [M. Coban, M. Karczewicz (Qualcomm)]

This contribution presents a reduced precision weighted prediction scheme with negligible impact on coding efficiency. In the current design, input to the weighting module of weighted prediction scheme has 16-bit precision. When input precision to the weighting module is decreased by 4 bits there is no drop in performance.  Simulation results show 0.0%, 0.0%, 0.0% and 0.0% BD rate change for RA_HE, RA_LC, LB_HE and LB_LC cases, respectively.

Proposal is as follow.

Uni-prediction:

predSamples[ x, y ] = Clip1H( ( ( ((predSamplesLX [ x, y ]+8)>>4) * wXC + 2logWDC – 1- 4 ) >> (logWDC - 4)) + oXC )

Bi-prediction:

predSamples[ x, y ] = Clip1H( ( (( ((predSamplesL0 [ x, y ]+8)>>4) * w0C +

( ((predSamplesL1 [ x, y ]+8)>>4) * w1C + ( ( o0C + o1C ) << ( logWDC + 1 - 4 ) )+ 2logWDC - 4 ) >> ( logWDC + 1 - 4 ) ) )

The motivation is to reduce input precision of weighted processing for hardware implementation.

By comments from several experts, it is found that there is no benefit for PC software.

If truncation instead of rounding is introduced, proponent said that larger loss of coding efficiency has been gotten.

Cross-checker suggested encouraging further study.

Recommendation: Further study

JCTVC-H0643 Cross-check of weighted prediction precision reduction (JCTVC-H0546) [A. Tanizawa, T. Chujoh, T. Yamakage (Toshiba)] [late]

This contribution reports cross-checking results on weighted prediction precision proposed in JCTVC-H0546. In this method, the input precision to the weighted prediction module is decreased by 4 bits. The verification task has been done successfully and the results match those provided by proponent. The encoding time and the decoding time of this cross-check are also similar to those provided by proponents.

Experimental results show that BD-bitrate gain of the weighted prediction precision reduction is 0.0%/0.0% and 0.0%/0.0% for black/white-fade sequences on RA-LC and LB-LC cases, respectively, and 0.0%/0.0% and 0.0%/0.1% for black/white-fade sequences on RA-HE and LB-HE cases, respectively.

Although cross-checking results match the proponent’s one, there are some concerns about the consistency with current WD test.