| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0730 |
| --- | --- |

| Title: | Report on Quantization Matrices Subjective Viewing |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Teruhiko Suzuki Kazushi Sato Ximin Zhang Rajan Joshi Jianhua Zheng Minhua Zhou | Email: | teruhikos@jp.sony.com Kazushi.Sato@jp.sony.com Ximin.Zhang@mediatek.com rajanj@qualcomm.com zhengjianhua@huawei.com zhou@ti.com |
| Source: | Sony Corp. , Media Tek, HiSilicom, Qualcomm, TI |  |  |

_____________________________

# Abstract

HEVC employs 16x16 and 32x32 as well as 4x4 and 8x8 transforms, therefore more bits would be needed to transmit quantization matrices within a bitstream. To tackle this problem lossy compression of quantization matrices has been studied under CE4 subtest 2. However the impact of error in QM domain on the subjective quality is not known. Therefore subjective viewing has been conducted to compare lossless and lossy QM compression.

This document reports result of this subjective quality evaluation.

# Introduction

Subjective quality evaluation related to quantization matrices was held as follows:

- Flat matrices vs lossless QM compression

- Lossless QM compression vs lossy QM compression

The purpose of the former test is to confirm that there exists difference with subjective quality between with- and without- quantization matrices, and the purpose of the latter is to see how error in QM domain affects visual quality.

Note: This test was conducted to confirm the impact on the visual quality by using lossy coding of quantization matrix. It is not to compare each lossy coding proposal.

# Settings and Test Method

## Test Materials

Assignment:

|  | Bitstream generator |
| --- | --- |
| Lossless coding | Sony (HM-5.1) |
| Lossy coding (2.1) | Sony |
| Lossy coding (2.2) | Media Tek |
| Lossy coding (2.4) | HiSilicon |
| Flat matrix | (HM-5.0) |

With all lossy coding proposals 4x4 and 8x8 matrices are losslessly coded.

The following conditions were tested

- B.Kimono; IntraLC; QP=32

- C.BQMall; IntraLC; QP=32

- C.BQMall; RALC; QP=32

with 3 matrices “symmetry1”, “symmety7” and “assymetry7”.

“Symmetry 1” has been distributed as part of CE4 description. “Symmetry 7” and “Asymmetry 7” will be kindly generated and provided by Rajan Joshi@Qualcomm. On these matrices property of 2Nx2N Transform for Luma and NxN Transform for Chroma is similar.

The value of QP is fixed throughout the test, because if different QPs are used for rate adjustment, it cannot be said that subjective quality difference comes from either with the difference of QM or of QP.

With lossy compression 3 proponents (Hisilicon, MediaTek and Sony) provided the results for evaluation. QM bits and errors in QM domain are shown in Table 2.1.

Table 2.1

| Sequence | Class | Config | Cond | Matrix | Bitrate |
| --- | --- | --- | --- | --- | --- |
| Kimono | B | IntraLC | HM-5.0(flat) |  | 7455.85 |
|  |  |  | 2.1 | Symmetry1 | 10841.9760 |
|  |  |  |  | Symmetry7 | 8772.4088 |
|  |  |  |  | Assymetry7 | 10975.3424 |
|  |  |  | 2.2 | Symmetry1 | 9624.29 |
|  |  |  |  | Symmetry7 | 9142.87 |
|  |  |  |  | Assymetry7 | 11070.71 |
|  |  |  | 2.4 | Symmetry1 | 9933.14 |
|  |  |  |  | Symmetry7 | 8557.85 |
|  |  |  |  | Assymetry7 | 11226.45 |
|  |  |  | HM-5.1(lossless) | Symmetry1 | 10167.7216 |
|  |  |  |  | Symmetry7 | 8680.7456 |
|  |  |  |  | Assymetry7 | 11102.0808 |
| BQMall | C | IntraLC | HM-5.0(flat) |  | 8820.24 |
|  |  |  | 2.1 | Symmetry1 | 9978.0128 |
|  |  |  |  | Symmetry7 | 9522.4496 |
|  |  |  |  | Assymetry7 | 10951.5360 |
|  |  |  | 2.2 | Symmetry1 | 9803.22 |
|  |  |  |  | Symmetry7 | 9813.19 |
|  |  |  |  | Assymetry7 | 11033.58 |
|  |  |  | 2.4 | Symmetry1 | 9822.90 |
|  |  |  |  | Symmetry7 | 9412.01 |
|  |  |  |  | Assymetry7 | 10934.60 |
|  |  |  | HM-5.1(lossless) | Symmetry1 | 9857.5040 |
|  |  |  |  | Symmetry7 | 9461.4752 |
|  |  |  |  | Assymetry7 | 10939.4552 |
| BQMall | C | RALC | HM-5.0(flat) |  | 857.07 |
|  |  |  | 2.1 | Symmetry1 | 1314.6872 |
|  |  |  |  | Symmetry7 | 1185.1272 |
|  |  |  |  | Assymetry7 | 1378.2296 |
|  |  |  | 2.2 | Symmetry1 | 994.97 |
|  |  |  |  | Symmetry7 | 1007.46 |
|  |  |  |  | Assymetry7 | 1056.28 |
|  |  |  | 2.4 | Symmetry1 | 886.13 |
|  |  |  |  | Symmetry7 | 879.61 |
|  |  |  |  | Assymetry7 | 939.26 |
|  |  |  | HM-5.1(lossless) | Symmetry1 | 887.6808 |
|  |  |  |  | Symmetry7 | 881.2112 |
|  |  |  |  | Assymetry7 | 942.6608 |

Table 2.2

|  |  | Symmetry1 | Symmetry7 | Assymetry7 |
| --- | --- | --- | --- | --- |
| Subtest 2.1 (Sony) | QM bit | 1312 | 2150 | 3256 |
|  | MaxError | 39 | 94 | 20 |
|  | AvgError | 2.39 | 5.03 | 2.63 |
| Subtest2.2 (MediaTek) | QM bit | 2026 | 1939 | 2703 |
|  | MaxError | 19 | 103 | 55 |
|  | AvgError | 2.00 | 34.40 | 16.30 |
| Subtest2.4 (Hisilicon) | QM bit | 1231 | 2079 | 3597 |
|  | MaxError | 11 | 25 | 4 |
|  | AvgError | 0.65 | 1.40 | 0.45 |

## Hardware and Software Settings

Hardware for subjective testing:

| System | Dell Studio XPS Desktop 435T/9000 |
| --- | --- |
| CPU | Intel Core i7 975 @3.33GHz |
| RAM | 12.0 GB |
| OS | Windows 7 Ultimate 64 bit |
| Video Disk | OCZ Tech REVODRIVE 3 X2 SERIES SSD 240GB (Blackmagic Design Video Disk Speed Read Test > 600 MB/s) (CrystalDiskMark Sequential read speed > 600 MB/s) |
| Graphic Card | EVGA GEFORCE GTX590 3GB GDDR5 |
| Monitor | Sony professional calibrated 2D monitor ( at least 42 in) Final monitor model is to be determined later. |

Software for subjective testing:

| Video Player | David Flynn’s video player will be used. |
| --- | --- |
| Scripts for displaying test videos | David Flynn’s scripts |

VIEWING CONDITIONS

This “expert viewing” protocol requires the following viewing conditions to be satisfied:

- Quite office with low ambient light (no light directly on the screen, no external windows light))

- six expert seated at the same desk;

- 32” or bigger HD display;

- Viewing distance 2H (H is the height of the active part of the screen)

- Paper sheet vote collection

## Test Method

The test methodology was defined consulted with David Flynn@BBC and Gary Sullivan@Microsoft. As their suggestion, A&B preference test was selected.

A&B Preference TEST

The Test Cell was done of:

- The message “ Reference” on the screen for 1 second (white letter on black background or, better, black letters on mid gray background);

- Play the uncompressed reference video clip;

- The message “A” on the screen for 1 second (white letter on black background or, better, black letters on mid gray background);

- Play the Anchor video clip for a given QP and coding condition;

- The message “B” on the screen for 1 second (white letter on black background or, better, black letters on mid gray background);

- Play the Test video clip for the same QP and coding condition;

- The message “A” on the screen for 1 second (white letter on black background or, better, black letters on mid gray background);

- Play the Anchor video clip for a given QP and coding condition;

- The message “B” on the screen for 1 second (white letter on black background or, better, black letters on mid gray background);

- Play the Test video clip for the same QP and coding condition;

- The message “VOTE” on the screen for 5 second (white letter on black background or, better, black letters on mid gray background);

There are 2 kinds of tests as follow:

Test (A) : Flat matrices vs lossless matrices compression

3 matrices * 3 conditions = 9 tests

Test (B) : Lossless matrices vs lossless

3 proponents * 3 matrices * 3 conditions = 27 tests.

For each test sequences are displayed as shown in Fig 2.1. In the “Vote” period, each user votes either “A” or  “B”. It takes 61sec for each test. So total time for the test is:

61sec * (9 + 27 tests) = 2196sec = 36.6min

With Test (A), “A” and “B” are either “flat matrices” or “lossless matrices.”  With Test (B), “A” and “B” are either “lossless matrices” or “lossy matrices”. The order of tests is shuffled randomly to make fair comparisons.

Fig 2.1

Viewing sessions have been held at “Silicon Valley” room on Feb 4, 2012 with the following schedule:

Session 1: 15:15-15:45

Session 2: 16:00-16:30

Session 3: 16:45-17:15

Session 4: 17:30-18:00

There were 6 non-proponent viewers with each of the session above. So there were totally 24 viewers. The authors would like to thank experts who participated in viewing.

# Test Result

In Fig 3.1 comparison between flat matrices and lossless matrices are shown.

As can be seen there exists subjective quality difference between without- and with- matrices even if DCT coefficients before quantization is same. It should be noted that under this test conditions rate for lossless matrices are higher than the one with flat matrices.

In Fig 3.2 overall result of comparison between lossless and lossy matrices are shown. Results of each proposal are shown in Fig 3.3, 3.4, and 3.5 respectively.

Here differences between proposals or matrices are not important. What is important is that the result is always almost 50%, which means that subjective quality with lossy matrices is comparable with the one with lossless matrices.

It was confirmed that the impact on visual quality to use lossy coding is negligible.

Fig 3.1

Fig3.2

Fig 3.3

Fig 3.4

Fig 3.5

# Conclusion

As a result of the evaluation it is confirmed that

- There exists subjective quality difference between flat matrix and test matrices at tested QP, although rates are also different, and

- the impact on visual quality to use lossy coding is negligible.

Therefore it is recommended that lossy as well as lossless quantization matrices functionality be adopted into HEVC standard.

The authors would like to thank Ohji Nakagami, Masaru Ikeda and Yoshitomo Takahashi @Sony to support viewing activity.

# References

[1] “CE4: Summary report of Core Experiment on Quantization”, JCTVC-H0034, Feb 2012

[2] “BoG Report on Quantization Matrices”, JCTVCV-H0710, Feb 2012