| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1-11 February, 2012 | Document: JCTVC-H0731 |
| --- | --- |

| Title: | CE2: Additional tests on reducing Max chroma RQT depth |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | 1,5Xin Zhao, 1Jicheng An, 1Xun Guo, 1Shawmin Lei North Building 10F, Raycom Infotech Park Tower C, No. 2 Kexueyuan South Rd., Haidian District, Beijing, China 100190 2Liwei Guo, 2Xianglin Wang, 2Marta Karczewicz 5775 Morehouse Drive San Diego, CA 92121, USA 3Akira Minezawa, 3Kazuo Sugimoto, 3Shun-ichi Sekiguchi Information Technology R&D Center Mitsubishi Electric Corporation 5-1-1, Ofuna, Kamakura, JAPAN 4Atsuro Ichigaya, 4Shinichi Sakaida NHK Science & Technology Research Laboratories 1-10-11, Kinuta, Setagaya-ku, Tokyo, JAPAN 5Siwei Ma, 5Wen Gao School of EECS, Peking University, Institute of Digital Media, Room 2641, Science Building #2, No.5, Yiheyuan Rd., Haidian District, Beijing, China, 100871 | Email: | {x.zhao, jijing.an, xun.guo, shawmin.lei}@mediatek.com {liweig, xianglin} @qualcomm.com {Sekiguchi.Shunichi@eb, Sugimoto.Kazuo@ak, Minezawa.Akira@ds}.MitsubishiElectric.co.jp ichigaya.a-go@nhk.or.jp {swma, wgao}@pku.edu.cn |
| Source: | 1MediaTek Inc., 2Qualcomm Inc., 3Mitsubishi Electric Corporation, 4NHK(Japan Broadcasting Corporation), 5Peking University |  |  |

_____________________________

# Abstract

This contribution reports additional test results of the harmonized software in CE2. Various combination of max RQT depth luma and chroma are tested. It is reported that maximum RQT depth for chroma in intra coding set to 2 levels smaller than that of luma, and maximum RQT depth for chroma in inter coding set to 1 level smaller than that of luma provides the best performance in coding efficiency.

# Introduction

During the review on CE2 summary report[1], it was suggested to study further to find a possibility to make the chroma depth fixed relative to luma. It appears that either luma depth minus one (as from H0317) or minus two may make sense, but the latter case was not studied. Further it was not known how the performance would be with luma RQT depth other than 3.

# Simulations and results

The harmonized software used for CE2 test 4.4.0[2] is used for simulations with settings as follows.

| Test ID | MaxDepthLuma Intra | MaxDepthLuma Inter | MaxDepthChroma Intra | MaxDepthChroma Inter |
| --- | --- | --- | --- | --- |
| Test 4.1 done in CE2 | 3 | 3 | 1 | 1 |
| Test 4.4.0 done in CE2 | 3 | 3 | 1 | 2 |
| Test A | 3 | 3 | 2 | 2 |
| Test B | 2 | 2 | 2 | 2 |
| Test C | 2 | 2 | 1 | 1 |

These simulations are performed under common test conditions defined in [3].

| test | Maxdepth |  |  |  | High Efficiency |  |  |  |  |  | Low Complexity |  |  |  |  |  | High 10 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y |  | UV |  | AI |  | RA |  | LB |  | AI |  | RA |  | LB |  | RA |  |
|  | Int ra | Int er | Int ra | Int er | Y | UV | Y | UV | Y | UV | Y | UV | Y | UV | Y | UV | Y | UV |
| 4.1 | 3 | 3 | 1 | 1 | 0.0 | -2.5 | 0.0 | -1.3 | -0.1 | 0.0 | 0.0 | -1.9 | 0.0 | -1.4 | 0.0 | -0.9 | 0.0 | -2.6 |
| 4.4.0 | 3 | 3 | 1 | 2 | 0.0 | -2.5 | 0.0 | -1.6 | 0.0 | -0.9 | 0.0 | -1.9 | 0.0 | -1.4 | -0.0 | -1.1 | 0.0 | -2.9 |
| A | 3 | 3 | 2 | 2 | 0.0 | -0.6 | 0.0 | -0.5 | 0.0 | -0.1 | 0.0 | -0.3 | 0.0 | -0.4 | 0.0 | -0.3 | 0.1 | -1.3 |
| C over B | 2 | 2 | 1 | 1 | 0.0 | -1.3 | 0.0 | -0.3 | -0.1 | 0.8 | 0.0 | -1.0 | 0.0 | -0.6 | 0.0 | 0.1 | -0.6 | -0.8 |

# Observation

When MaxLumaDepth=3:

- By comparing Test A with Test 4.4.0, MaxChromaDepthIntra = MaxLumaDepthIntra – 2 works better than MaxChromaDepthIntra = MaxLumaDepthIntra – 1 when MaxLumaDepthInter = 3.

- By comparing Test 4.1 with Test 4.4.0, MaxChromaDepthInter = MaxLumaDepthInter – 1 works better than MaxChromaDepthInter = MaxLumaDepthInter – 2 when MaxLumaDepthInter = 3.

When MaxLumaDepth=2:

- By comparing Test B with Test C, MaxChromaDepth = MaxLumaDepth – 1 generally works better than MaxChromaDepth = MaxLumaDepth.

Considering these comparison, best performing combination is as follows:

MaxChromaDepthIntra = Max(1, MaxLumaDepthIntra – 2)

MaxChromaDepthInter = Max(1, MaxLumaDepthInter – 1)

# Conclusion

In this contribution, additional test results of the harmonized software in CE2 are reported. Fixing max chroma RQT depth set as luma depth minus 2 for intra, and luma depth minus 1 for inter are recommended to be adopted in the next HM and CD.

# References

[2] K. Sugimoto, X. Guo and X. Wang, “CE2: Summary report of Core Experiment on Chroma RQT depth”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-H0032, 8th Meeting: San Jose, CA, US, 1-11 February, 2012.

[2] X. Zhao, J. An, X. Guo, S. Lei, L. Guo, X. Wang, M. Karczewicz, A. Minezawa, K. Sugimoto, S. Sekiguchi, A. Ichigaya, S. Sakaida, S. Ma and W. Gao, “CE2: Subset 4.4: Test Results on Harmonization of JCTVC-G283, JCTVC-G442 and JCTVC-G980”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-H0308, 8th Meeting: San Jose, CA, US, 1-11 February, 2012.

[3] F. Bossen, “Common test conditions and software reference configurations”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-G1200, 7th Meeting: Geneva, CH, 21-30 November, 2011.

# Patent rights declaration

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Mitsubishi Electric Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

NHK may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Peking University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).