| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San Jose, US, 1-10 February, 2012 | Document: JCTVC-H0735_r1 WG11 Number: m23995 |
| --- | --- |

| Title: | Analysis of differences of various ALF signalling schemes |  |  |
| --- | --- | --- | --- |
| Status: | Input Document of JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tomoo Yamakage |  | tomoo.yamakage@toshiba.co.jp |
| Source: | Toshiba Corporation |  |  |

_____________________________

# Introduction

Various ALF signallingschemes are proposed to support various application domains.

Video conferencing: two-way communication, low latency

Broadcasting: one-way transmission, live or long latency

VOD, DVD, BD, etc.: one-way transmission, long latency

All applications are equally important for HEVC. However, each signalling scheme has pros. and cons.

This contribution analyzes various ALF schemes, mainly in terms of latency point of view,                          in order to provide information to the committee members so that the members can find the best way.

Discussion of syntax part and signal processing part (decoder) should be isolated, and discussion to find the best signaling scheme should be done based on the well understanding of the committee members.

# How ALF encoder works

From R-D point of view, Wiener-based filter design has been adopted in HM. Other filter design schemes might be possible (e.g. optimize for subjective picture quality --- some proprietary scheme will be developed in the future.)

Wiener-based filter is designed by the following steps:

- Gather the statistics (distortion) of a region during encoding the region.

  - A region may be a picture, a slice (sub-frame), or an LCU.

  - Depending on the size of the region, latency will vary.

  - Larger region enables various optimizations, but latency increases.

- Design optimal filter coefficients by solving the correlation matrix.

- Apply the designed filter to a block (LCU or CU) and determine whether the block should be filtered based on distortion (filter on vs off).

Note: This process can be decoupled from signaling.

# Analysis of various ALF signaling schemes

The following ALF signaling schemes are under the study at this moment:

- Current design (WD5 / HM5)

ALF coef. signalling: APS

CU flag signalling: Slice header

- JCTVC-H0066

ALF coef. signalling: LCU data

LCU flag signalling: LCU data

- JCTVC-H0274

ALF coef. signalling: Slice header or APS

CU flag signalling: Slice header

The following figures summarize the differences of each signaling schemes:

Since JCTVC-H0066 is intended for low latency applications and JCTVC-H0274 is intended for typical latency applications, their signalling schemes can provide the straightforward way to encode each application. The following section explains how a signalling scheme for an application domain will realize the encoding scheme for the other application domain.

## How JCTVC-H0066 singalling can achieve picture-based encoding

When JCTVC-H0066 signalling scheme is applied to picture-based encoding, the following steps are performed:

- Gather the statistics of a picture during encoding.

- Design optimal filter coefficients by solving the correlation matrix.

- Apply the designed filter to each block (LCU) and determine whether the block should be filtered based on distortion (filter on vs off).

- Interleave filter coefficients and LCU on/off flag into each LCU data.

  - Interleaving is difficult, since LCU data has been encoded by CABAC.

## How H0274 singalling can achieve LCU-based encoding

When JCTVC-H0274 signalling scheme is applied to picture-based encoding, the following steps are performed:

- Gather the statistics of an LCU during encoding the LCU

- Design optimal filter coefficients by solving the correlation matrix

- Apply the designed filter to the block (LCU or CU) and determine whether  the block should be filtered based on distortion (filter on vs off)

- Store LCU data to a buffer, and filter coefficients and CU on/off flag(s) to the other buffers

  - Number of generated bits for the LCU can be derived exactly, therefore, MTU-size slice scenario can be realized in the same manner as H0066

  - Marginal complexity increase to use several buffers

- When a slice encoding is completed, transmission of the slice header/data can start

## Comparison table

The following table summarizes the comparison of various ALF signalling schemes:

(*) If transmission of a slice will start right after all slice data is ready.

# Conclusion

Various ALF schemes are analyzed from the viewpoint of latency.

- Both H0066 and H0274 can support picture-based encoding

  - H0066 encoder requires high complexity (i.e., interleaving)

- Both H0066 and H0274 can support fixed slice encoding

  - H0274 encoder requires marginal complexity (i.e., buffering)

- Both H0066 and H0274 can support MTU-based slice encoding

  - H0274 encoder requires marginal complexity (i.e., buffering)

The latency becomes slice-level latency in both cases. If transmission of a slice will start right after all slice data is ready.

Considering the provided information, the committee should discuss the best way.

# Patent rights declarations

Toshiba Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).