| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0738 WG11 Document: m24000 |
| --- | --- |

| Title: | BoG Report on Profiles and Levels |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s) | K. McCann (Samsung/ZetaCast) T.K. Tan (NTT DOCOMO) | Email: | ken@zetacast.com ttk@pacific.net.sg |
| Source: | BoG Coordinators |  |  |

_____________________________

# Abstract

This document reports on the break-out group on profiles and levels that met during the 8th JCT-VC meeting, from 08:00 to 13:00 on 9 February.  This Revision 1 of the document also includes modifications to the draft text that were made during discussion in JCT-VC plenary on 9 February.

# Definition of Profiles and Levels

Profiles and Levels are defined as follows in the MPEG document specifying HEVC Requirements [1]:

“A Profile is a set of algorithmic tools, representing a particular trade-off of performance and resource consumption, supporting the requirements of a particular set of applications.  Applications which require similar trade-offs between these parameters should generally use the same Profile.”

“A Level describes performance parameters within each Profile, which describe lower limits on decoder capability.  Performance parameters typically include maximum picture size, macroblocks per second, bitrate, frame rate, buffer sizes and similar parameters.”

# Goals of Break-Out Group

The primary goal of the BoG was agreed to be to draft Annex A text that is appropriate for JCT-VC to approve for inclusion in the HEVC CD.

The key issues to consider when reviewing tools were agreed to be:

- Performance

- Maturity

- Decoder Complexity: Both software and hardware decoder complexity are important.  Results from optimised software implementations are more important than HM decoder time. Some form of indication of hardware complexity is requested from silicon vendors – silicon area, memory bandwidth, etc.

- Encoder Complexity: Encoder complexity is important in communication applications, particularly if it affects the ability to create a compliant bitstream.  However, leaving freedom for future encoders to implement optional tools was also agreed to be useful.

# Presentation of Documents

JCTVC-H0734 “Straw man for Profiles and levels”, M. Horowitz (eBrisk Video), A. Ichigaya (NHK), K. McCann (Samsung/ZetaCast), T. Nishi (Panasonic), S. Sekiguchi (Mitsubishi), T. Suzuki (Sony Corporation), T. K. Tan (NTT DOCOMO), W. Wan (Broadcom), M. Zhou (TI)

This contribution suggested a clustering of tools as shown below, based on indications of decoder complexity, maturity and performance:

| Tool cluster | Cluster 1: LMChroma | Cluster 2: AMP | Cluster 3: NSQT | Cluster 4: SAO | Cluster 5: ALF |
| --- | --- | --- | --- | --- | --- |
| Impact on decoding time JCTVC-H0116 | Negligible | Negligible | Negligible | Low to medium | High |
| Software implementations JCTVC-H0693 JCTVC-G0757 JCTVC-G0988 | Not implemented | Not implemented Not implemented | Not implemented Not implemented Not implemented | 11% - 13% 5 - 10% 5 - 7% | 17% Not implemented Not implemented |
| Stability indicated by number of input documents | Documents with keyword in title: LM (7) | Documents with keyword in title: AMP (1) | Documents with keyword in title: NSQT (11) | Documents with keyword in title: SAO (25) | Documents with keyword in title: ALF (24) |
| WD Editors’ assessment of WD maturity & completeness | OK except for one small issue | Stable since last meeting | Stable since last meeting | The text quality is not stable (but better than ALF) | The text quality is not stable |
| Software coordinators’ assessment of HM maturity & stability | Stable | Stable | Unstable/buggy but might stabilize at this meeting | Has had stability issues, some of these related to slices/tiles | Similar to SAO but with more issues. Amount of code related to ALF is quite large |
| Objective Gains (BD-rate relative to BD-rate LC) | Mostly chroma only 1.9% to 5.6% | 0.6% to 0.7% | 0.2% to 0.6% | 0.5% to 5.4% | 1.7% to 6.4% |
| Subjective improvements reported in JCTVC-H0116 | Not visible | Not visible | Not visible | Visible on some sequences | Not visible |

JCTVC-H0382 “Inclusion of adaptive loop filtering in the HEVC standard”, T. Yamakage (Toshiba), I. S. Chong (Qualcomm), Y.-W. Huang (MediaTek)

This contribution asserts that it would be beneficial to support ALF in the HEVC standard and proposes to include ALF in the HEVC standard.  It proponents suggested that if there is only one profile then ALF should be in that profile; if there are multiple profiles then at least one profile should include ALF.

JCTVC-H0277 reported that the coding efficiency improvement for KTA 720p and 1080p sequences is 4.8% (RA-HE), 4.4% (LDB-HE) and 9.5% (LDP-HE) on average. Some evidence was presented in CE8.f, but it was suggested that more evidence of subjective benefit is needed.  One participant stated that some of the benefit of ALF may be achieved by non-flat quantisation.

JCTVC-H0168 “Proposal of HEVC profile/level definitions”, T. Suzuki (Sony), H. Sasai, T. Nishi (Panasonic)

The contribution suggested various constraints:

LCU size:

-	Max LCU size is equal to 64x64

-	Min LCU size is equal to 16x16

Slice constraints:

-	Min slice granularity is equal to LCU size

-	Each slice starts from LCU boundary

Max number of reference frames:

Max value of num_ref_frame is equal to 4 (may be 5, if necessary)

Number of PS:

-	Max number of SPS is equal to 8

-	Max number of PPS is equal to 16

-	Max number of APS is equal to 8

CABAC constraints

Parallel processing tool constraints

Potential candidate constraint

The contribution suggested that implementation could be simpler if picture size is multiple of LCU. However, it was noted that it has already been decided that picture size is multiple of smallest CU

Level-specific constraints were suggested in the contribution to reduce MC memory bandwidth.  It was stated by one participant that level-specific constraints tend to be less respected by encoders.  Further study will take place in AHG on motion compensation and memory bandwidth.

# Discussions

The conclusions of the discussions were documented in a draft version of the WD text.  This was further modified during discussion in JCT-VC plenary; see Annex A (please note that not all changes since the initial version of this Annex are marked with tracking in this Annex).

It was agreed that the initial profile should be named the “Main Profile”.  This would be the only profile in the CD text, but it was agreed that there was the potential to add further profiles later.

It was agreed that weighted prediction may not be used in all applications.  Some participants suggested that the decision on whether or not use of weighted prediction is constrained in Main profile depends on whether or not there are multiple profiles; if there is another profile that does not allow the use of weighted prediction then no constraint is needed in Main Profile.

Concern was expressed by some participants over the decoder implementation complexity for ALF. The consensus view was that ALF should not be included in the Main profile at this stage, although some participants expressed reservations about this.  It was agreed that this issue should be considered again at the next meeting, if more evidence of the subjective benefit is available.

Recommendation: ALF should not be included in the Main profile at this stage

Concern was expressed by some participants over the decoder implementation complexity for LMChroma.

Recommendation: LMChroma should not be included in the Main profile at this stage

Ypprime_y_zero_transquant_bypass_flag was stated to have minor complexity impact on the decoder.

It was reported that the Tiles BoG had not recommended either requiring or prohibiting the three available parallel techniques. However, some off-line discussion had taken place on tile width issues and there was now consensus on this issue.

Recommendation: Defer decision on which parallel techniques should be used in the Main profile to plenary

Recommendation: It was agreed that if tiles are allowed then they shall be no less than 384 luma samples wide (other than for a single tile picture).

AMP objective gain is 0.6% to 0.7% relative to JM (0.8-1.1% relative to HM).  NSQT objective gain is 0.2% to 0.6% (0.3-1.0% relative to HM).  One participant commented that the gains are approximately additive when the two are used together, i.e. 1.3-2.3% relative to HM when used together.  Another participant expressed concern over the encoder complexity for AMP.  Another participant said that a real encoder would speed up motion search and the encoder impact of AMP would be less. Decoder implementation of AMP was stated to be less complex than that of NSQT.

Recommendation: Neither AMP nor NSQT should be included in the Main profile at this stage

A number of other constraints were agreed:

LCU size: LCU size from 16x16 to 64x64

Max number of SPS 16

Max number of PPS 64

Max number of APS 64

Slices must contain an integer number of LCUs

The names of the various levels were discussed.

Recommendation: Names of levels should be aligned with the nearest equivalent from H.264/AVC

# References

- ISO/IEC JTC1/SC29/WG11/N11872, “Vision, Applications and Requirements for High Efficiency Video Coding (HEVC)”, January 2011

- JCTVC-H0734 “Straw man for Profiles and levels”, M. Horowitz (eBrisk Video), A. Ichigaya (NHK), K. McCann (Samsung/ZetaCast), T. Nishi (Panasonic), S. Sekiguchi (Mitsubishi), T. Suzuki (Sony Corporation), T. K. Tan (NTT DOCOMO), W. Wan (Broadcom), M. Zhou (TI)

- JCTVC-H0382 “Inclusion of adaptive loop filtering in the HEVC standard”, T. Yamakage (Toshiba), I. S. Chong (Qualcomm), Y.-W. Huang (MediaTek)

- JCTVC-H0168 “Proposal of HEVC profile/level definitions”, T. Suzuki (Sony),  H. Sasai, T. Nishi (Panasonic)

- Annex A

Profiles and levels

(This annex forms an integral part of this Recommendation | International Standard)

Profiles and levels specify restrictions on bitstreams and hence limits on the capabilities needed to decode the bitstreams. Profiles and levels may also be used to indicate interoperability points between individual decoder implementations.

NOTE 1 – This Recommendation | International Standard does not include individually selectable "options" at the decoder, as this would increase interoperability difficulties.

Each profile specifies a subset of algorithmic features and limits that shall be supported by all decoders conforming to that profile.

NOTE 2 – Encoders are not required to make use of any particular subset of features supported in a profile.

Each level specifies a set of limits on the values that may be taken by the syntax elements of this Recommendation | International Standard. The same set of level definitions is used with all profiles, but individual implementations may support a different level for each supported profile. For any given profile, levels generally correspond to decoder processing load and memory capability.

The profiles that are specified in subclause A.2 are also referred to as the profiles specified in Annex Error: Reference source not found.

  - Requirements on video decoder capability

Capabilities of video decoders conforming to this Recommendation | International Standard are specified in terms of the ability to decode video streams conforming to the constraints of profiles and levels specified in this annex. For each such profile, the level supported for that profile shall also be expressed.

Specific values are specified in this annex for the syntax elements profile_idc and level_idc. All other values of profile_idc and level_idc are reserved for future use by ITU-T | ISO/IEC.

NOTE – Decoders should not infer that when a reserved value of profile_idc or level_idc falls between the values specified in this Recommendation | International Standard that this indicates intermediate capabilities between the specified profiles or levels, as there are no restrictions on the method to be chosen by ITU-T | ISO/IEC for the use of such future reserved values.

  - Profiles

All constraints for picture parameter sets that are specified in subclauses Error: Reference source not found to Error: Reference source not found are constraints for picture parameter sets that are activated in the bitstream. All constraints for sequence parameter sets that are specified in subclauses Error: Reference source not found to Error: Reference source not found are constraints for sequence parameter sets that are activated in the bitstream.

    - Main profile

Bitstreams conforming to the Main profile shall obey the following constraints:

–	Only I, P, and B slice types may be present.

–	Sequence parameter sets shall have chroma_format_idc equal to 1 only.

–	Sequence parameter sets shall have bit_depth_luma_minus8 equal to 0 only.

–	Sequence parameter sets shall have bit_depth_chroma_minus8 equal to 0 only.

–	Sequence parameter sets shall have adaptive_loop_filter_enabled_flag equal to 0 only.

–	Sequence parameter sets shall have chroma_pred_from_luma_enabled_flag equal to 0 only.

–	Sequence parameter sets shall have inter_4x4_enabled_flag equal to 0 only.

–	Sequence parameter sets shall have asymmetric_motion_partitions_enabled_flag equal to 0 only.

–	Sequence parameter sets shall have non_square_quadtree_enabled_flag equal to 0 only.

–	Sequence parameter sets shall have seq_parameter_set_id in the range of 0 to 15, inclusive.

–	Picture parameter sets shall have pic_parameter_set_id in the range of 0 to 63, inclusive.

–	Adaptation parameter sets shall have aps_id in the range of 0 to 63, inclusive.

–	LCU size shall range from 16 to 64, inclusive. [Ed.  Specify (MaxLCUSize is 64, MinLCUSize is 16) as functions of log2_min_coding_block_size_minus3 and log2_diff_max_min_coding_block_size ?]

–	Slices shall be an integer number of LCU. [Ed. See if this can be expressed as a function of slice granularity in the PPS, it was noted that the slice_granularity should probably be in the SPS]

–	Picture parameter sets / Sequence parameter sets shall have tiles_or_entropy_coding_synchro_idc values in the range of 0 to 1, inclusive.

–	When tiles_or_entropy_coding_synchro_idc ie equal to 1, tiles shall be no smaller than 384 luma samples wide

–	WPP is not allowed in this profile [Ed. already stated above, as tiles_or_entropy_coding_synchro_idc cannot be 2]

–	Entropy slices are not allowed in this profile

–	The level constraints specified for the Main profile in subclause A.3 shall be fulfilled.

Conformance of a bitstream to the Main profile is indicated by profile_idc being equal to 100. Decoders conforming to the Main profile at a specific level shall be capable of decoding all bitstreams in which the profile_idc is equal to 100 and level_idc represents a level less than or equal to the specified level.

  - Levels

The following is specified for expressing the constraints in this annex.

–	Let access unit n be the n-th access unit in decoding order with the first access unit being access unit 0.

–	Let picture n be the primary coded picture or the corresponding decoded picture of access unit n.

Let the variable fR be set to 1  300.

- Table A-1 – Level limits

| Level | Max luma pixel rate MaxLPR (samples /sec) | Max luma frame size MaxFS (samples) | Max bit rate MaxBR (1000 bits/s) | MinCR | Max DPB size MaxDPBFrames (number of Max luma frame size) | Max CPB size (1000 bits) |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 552,960 | 36,864 | 128 | 2 | 6 | 350 |
| 2 | 3,686,400 | 122,880 | 1,000 | 2 | 6 | 1,000 |
| 3 | 13,762,560 | 458,752 | 5,000 | 2 | 6 | 5,000 |
| 3.1 | 33,177,600 | 983,040 | 9,000 | 2 | 6 | 9,000 |
| 4 | 62,668,800 | 2,088,960 | 15,000 | 4 | 6 | 15,000 |
| 4.1 | 62,668,800 | 2,088,960 | 30,000 | 4 | 6 | 30,000 |
| 4.2 | 133,693,440 | 2,228,224 | 30,000 | 4 | 6 | 30,000 |
| 4.3 | 133,693,440 | 2,228,224 | 50,000 | 4 | 6 | 50,000 |
| 5 | 267,386,880 | 8,912,896 | 50,000 | 6 | 6 | 50,000 |
| 5.1 | 267,386,880 | 8,912,896 | 100,000 | 8 | 6 | 100,000 |
| 5.2 | 534,773,760 | 8,912,896 | 150,000 | 8 | 6 | 150,000 |
| 6 | 1,002,700,800 | 33,423,360 | 300,000 | 8 | 6 | 300,000 |
| 6.1 | 2,005,401,600 | 33,423,360 | 500,000 | 8 | 6 | 500,000 |
| 6.2 | 4,010,803,200 | 33,423,360 | 800,000 | 6 | 6 | 800,000 |

Note: MaxDPBFrames includes current picture

Note: Higher level decoders are required to decode lower level bitstreams

Levels with non-integer level numbers in Table A-1 are referred to as "intermediate levels".

NOTE 4 – All levels have the same status, but some applications may choose to use only the integer-numbered levels.

Informative subclause A.3.3 shows the effect of these limits on frame rates for several example picture formats.

    - Level limits

Bitstreams conforming to the Main profile at a specified level shall obey the following constraints:

- The nominal removal time of access unit n (with n > 0) from the CPB as specified in subclause Error: Reference source not found, satisfies the constraint that tr,n( n ) − tr( n − 1 ) is greater than or equal to Max( PicSizeInSamples  MaxLPR , fR ), where MaxLPR  is the value specified in Table A-1 that applies to picture n − 1, and PicSizeInSamples is the number of pic_width_in_luma_samples * pic_height_in_luma_samplesin picture n − 1.

- The difference between consecutive output times of pictures from the DPB as specified in subclause Error: Reference source not found, satisfies the constraint that to,dpb( n ) >= Max( PicSizeInSamples  MaxLPR, fR ), where MaxLPR is the value specified in Table A-1 for picture n, and PicSizeInSamples is the number of pic_width_in_luma_samples * pic_height_in_luma_samplesof picture n, provided that picture n is a picture that is output and is not the last picture of the bitstream that is output.

- pic_width_in_luma_samples * pic_height_in_luma_samples <= MaxFS, where MaxFS is specified in Table A-1

- pic_width_in_luma_samples<= Sqrt( MaxFS * 8 )

- pic_height_in_luma_samples<= Sqrt( MaxFS * 8 )

- max_dec_frame_buffering <= MaxDPBFrames, where MaxDPBFrames is specified in Table A-1.

Table A-1 specifies the limits for each level. A definition of all levels identified in the "Level number" column of Table A-1 is specified for the Main profile. Entries marked "-" in Table A-1 denote the absence of a corresponding limit. The use of the MinCR parameter column of Table A-1 for this profile is specified in subclause A.3.2.

A level to which the bitstream conforms shall be indicated by the syntax element level_idc as follows.

–	level_idc shall be set equal to a value of ten times the level number specified in Table A-1.

    - Profile-specific level limits

- In bitstreams conforming to the Main profile, the removal time of access unit 0 shall satisfy the constraint that the number of slices in picture 0 is less than or equal to ( Max( PicSizeInSamples, fR * MaxLPR ) + MaxLPR * ( tr( 0 ) − tr,n( 0 ) ) ) ÷ SliceRate, where MaxLPR and SliceRate are the values specified in Tables A-1 and A-3, respectively, that apply to picture 0 and PicSizeInSamples is the number of pic_width_in_luma_samples * pic_height_in_luma_samples in picture 0.

- In bitstreams conforming to the Main profile, the difference between consecutive removal time of access units n and n − 1 with n > 0 shall satisfy the constraint that the number of slices in picture n is less than or equal to MaxLPR * ( tr( n ) − tr( n − 1 ) ) ÷ SliceRate, where MaxLPR and SliceRate are the values specified in Tables A-1 and A-3, respectively, that apply to picture n.

- In bitstreams conforming to the Main profile, for the VCL HRD parameters, BitRate[ SchedSelIdx ] <= cpbBrVclFactor * MaxBR and CpbSize[ SchedSelIdx ] <= cpbBrVclFactor * MaxCPB for at least one value of SchedSelIdx, where cpbBrVclFactor is specified in Table A-2 and BitRate[ SchedSelIdx ] and CpbSize[ SchedSelIdx ] are given as follows.

–	If vcl_hrd_parameters_present_flag is equal to 1, BitRate[ SchedSelIdx ] and CpbSize[ SchedSelIdx ] are given by Equations Error: Reference source not found and Error: Reference source not found, respectively, using the syntax elements of the hrd_parameters( ) syntax structure that immediately follows vcl_hrd_parameters_present_flag.

–	Otherwise (vcl_hrd_parameters_present_flag is equal to 0), BitRate[ SchedSelIdx ] and CpbSize[ SchedSelIdx ] are inferred as specified in subclause Error: Reference source not found for VCL HRD parameters.

MaxBR and MaxCPB are specified in Table A-1 in units of cpbBrVclFactor bits/s and cpbBrVclFactor bits, respectively. The bitstream shall satisfy these conditions for at least one value of SchedSelIdx in the range 0 to cpb_cnt_minus1, inclusive.

- In bitstreams conforming to the Main profile, for the NAL HRD parameters, BitRate[ SchedSelIdx ] <= cpbBrNalFactor * MaxBR and CpbSize[ SchedSelIdx ] <= cpbBrNalFactor * MaxCPB for at least one value of SchedSelIdx, where cpbBrNalFactor is specified in Table A-2 and BitRate[ SchedSelIdx ] and CpbSize[ SchedSelIdx ] are given as follows.

–	If nal_hrd_parameters_present_flag is equal to 1, BitRate[ SchedSelIdx ] and CpbSize[ SchedSelIdx ] are given by Equations Error: Reference source not found and Error: Reference source not found, respectively, using the syntax elements of the hrd_parameters( ) syntax structure that immediately follows nal_hrd_parameters_present_flag.

–	Otherwise (nal_hrd_parameters_present_flag is equal to 0), BitRate[ SchedSelIdx ] and CpbSize[ SchedSelIdx ] are inferred as specified in subclause Error: Reference source not found for NAL HRD parameters.

MaxBR and MaxCPB are specified in Table A-1 in units of cpbBrNalFactor bits/s and cpbBrNalFactor bits, respectively. The bitstream shall satisfy these conditions for at least one value of SchedSelIdx in the range 0 to cpb_cnt_minus1, inclusive.

- In bitstreams conforming to the Main profile, the sum of the NumBytesInNALunit variables for access unit 0 is less than or equal to 1.5 * ( Max( PicSizeInSamples, fR * MaxLPR ) + MaxLPR * ( tr( 0 ) − tr,n( 0 ) ) ) ÷ MinCR, where MaxLPR and MinCR are the values specified in Table A-1 that apply to picture 0 and PicSizeInSamples is the number of pic_width_in_luma_samples * pic_height_in_luma_samples in picture 0.

- In bitstreams conforming to the Main profile, the sum of the NumBytesInNALunit variables for access unit n with n > 0 is less than or equal to 1.5 * MaxLPR * ( tr( n ) − tr( n − 1 ) ) ÷ MinCR, where MaxLPR and MinCR are the values specified in Table A-1 that apply to picture n.

- Table A-2 – Specification of cpbBrVclFactor and cpbBrNalFactor

| Profile | cpbBrVclFactor | cpbBrNalFactor |
| --- | --- | --- |
| Main | 1 000 | 1 100 |

      - Level limits of the Main profile

Table A-3 specifies limits for each level that are specific to bitstreams conforming to the Main profile. Entries marked "-" in Table A-3 denote the absence of a corresponding limit.

NOTE – A row of entries marked "-" in Table A-3 does not indicate that the corresponding level is undefined for the Main profile.

- Table A-3 – Main profile level limits

| Level number | SliceRate |
| --- | --- |
| 1 | - |
| 2 | - |
| 3 | 22 |
| 3.1 | 60 |
| 4 | 60 |
| 4.1 | 24 |
| 4.2 | 24 |
| 4.3 | 24 |
| 5 | 24 |
| 5.1 | 24 |
| 5.2 | 24 |
| 6 | 24 |
| 6.1 | 24 |
| 6.2 | 24 |

    - Effect of level limits on frame rate (informative)

This subclause does not form an integral part of this Recommendation | International Standard.

- Table A-4 – Maximum frame rates (frames per second) for some example frame sizes

| Level: |  |  |  | 1 | 2 | 3 | 3.1 | 4, 4.1 | 4.2, 4.3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Max frame size (samples): |  |  |  | 36,864 | 122,880 | 458,752 | 983,040 | 2,088,960 | 2,228,224 |
| Max luma pixel rate (samples/sec) |  |  |  | 552,960 | 3,686,400 | 13,762,560 | 33,177,600 | 62,668,800 | 133,693,440 |
| Format | Luma Width | Luma Height | Frame Size |  |  |  |  |  |  |
| SQCIF | 128 | 96 | 16,384 | 33.7 | 225.0 | 300.0 | 300.0 | 300.0 | 300.0 |
| QCIF | 176 | 144 | 36,864 | 15.0 | 100.0 | 300.0 | 300.0 | 300.0 | 300.0 |
| QVGA | 320 | 240 | 81,920 | - | 45.0 | 168.0 | 300.0 | 300.0 | 300.0 |
| 525 SIF | 352 | 240 | 98,304 | - | 37.5 | 140.0 | 300.0 | 300.0 | 300.0 |
| CIF | 352 | 288 | 122,880 | - | 30.0 | 112.0 | 270.0 | 300.0 | 300.0 |
| 525 HHR | 352 | 480 | 196,608 | - | - | 70.0 | 168.7 | 300.0 | 300.0 |
| 625 HHR | 352 | 576 | 221,184 | - | - | 62.2 | 150.0 | 283.3 | 300.0 |
| VGA | 640 | 480 | 327,680 | - | - | 42.0 | 101.2 | 191.2 | 300.0 |
| 525 4SIF | 704 | 480 | 360,448 | - | - | 38.1 | 92.0 | 173.8 | 300.0 |
| 525 SD | 720 | 480 | 393,216 | - | - | 35.0 | 84.3 | 159.3 | 300.0 |
| 4CIF | 704 | 576 | 405,504 | - | - | 33.9 | 81.8 | 154.5 | 300.0 |
| 625 SD | 720 | 576 | 442,368 | - | - | 31.1 | 75.0 | 141.6 | 300.0 |
| 480p (16:9) | 854 | 480 | 458,752 | - | - | 30.0 | 72.3 | 136.6 | 291.4 |
| SVGA | 800 | 600 | 532,480 | - | - | - | 62.3 | 117.6 | 251.0 |
| QHD | 960 | 544 | 552,960 | - | - | - | 60.0 | 113.3 | 241.7 |
| XGA | 1024 | 768 | 786,432 | - | - | - | 42.1 | 79.6 | 170.0 |
| 720p HD | 1280 | 720 | 983,040 | - | - | - | 33.7 | 63.7 | 136.0 |
| 4VGA | 1280 | 960 | 1,228,800 | - | - | - | - | 51.0 | 108.8 |
| SXGA | 1280 | 1024 | 1,310,720 | - | - | - | - | 47.8 | 102.0 |
| 525 16SIF | 1408 | 960 | 1,351,680 | - | - | - | - | 46.3 | 98.9 |
| 16CIF | 1408 | 1152 | 1,622,016 | - | - | - | - | 38.6 | 82.4 |
| 4SVGA | 1600 | 1200 | 1,945,600 | - | - | - | - | 32.2 | 68.7 |
| 1080 HD | 1920 | 1088 | 2,088,960 | - | - | - | - | 30.0 | 64.0 |
| 2Kx1K | 2048 | 1024 | 2,097,152 | - | - | - | - | - | 63.7 |
| 2Kx1080 | 2048 | 1088 | 2,228,224 | - | - | - | - | - | 60.0 |
| 4XGA | 2048 | 1536 | 3,145,728 | - | - | - | - | - | - |
| 16VGA | 2560 | 1920 | 4,915,200 | - | - | - | - | - | - |
| 3616x1536 (2.35:1) | 3616 | 1536 | 5,603,328 | - | - | - | - | - | - |
| 3672x1536 (2.39:1) | 3680 | 1536 | 5,701,632 | - | - | - | - | - | - |
| 4Kx2K | 4096 | 2048 | 8,388,608 | - | - | - | - | - | - |
| 4096x2160 | 4096 | 2160 | 8,912,896 | - | - | - | - | - | - |
| 4096x2304 (16:9) | 4096 | 2304 | 9,437,184 | - | - | - | - | - | - |
| 7680x4320 | 7680 | 4320 | 33,423,360 | - | - | - | - | - | - |

- Table A-4 (continued) – Maximum frame rates (frames per second) for some example frame sizes

| Level: |  |  |  | 5, 5.1 | 5.2 | 6 | 6.1 | 6.2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Max frame size (samples): |  |  |  | 8,912,896 | 8,912,896 | 33,423,360 | 33,423,360 | 33,423,360 |
| Max luma pixel rate (samples/sec) |  |  |  | 267,386,880 | 534,773,760 | 1,002,700,800 | 2,005,401,600 | 4,010,803,200 |
| Format | Luma Width | Luma Height | Frame Size |  |  |  |  |  |
| SQCIF | 128 | 96 | 16,384 | 300.0 | 300.0 | 300.0 | 300.0 | 300.0 |
| QCIF | 176 | 144 | 36,864 | 300.0 | 300.0 | 300.0 | 300.0 | 300.0 |
| QVGA | 320 | 240 | 81,920 | 300.0 | 300.0 | 300.0 | 300.0 | 300.0 |
| 525 SIF | 352 | 240 | 98,304 | 300.0 | 300.0 | 300.0 | 300.0 | 300.0 |
| CIF | 352 | 288 | 122,880 | 300.0 | 300.0 | 300.0 | 300.0 | 300.0 |
| 525 HHR | 352 | 480 | 196,608 | 300.0 | 300.0 | 300.0 | 300.0 | 300.0 |
| 625 HHR | 352 | 576 | 221,184 | 300.0 | 300.0 | 300.0 | 300.0 | 300.0 |
| VGA | 640 | 480 | 327,680 | 300.0 | 300.0 | 300.0 | 300.0 | 300.0 |
| 525 4SIF | 704 | 480 | 360,448 | 300.0 | 300.0 | 300.0 | 300.0 | 300.0 |
| 525 SD | 720 | 480 | 393,216 | 300.0 | 300.0 | 300.0 | 300.0 | 300.0 |
| 4CIF | 704 | 576 | 405,504 | 300.0 | 300.0 | 300.0 | 300.0 | 300.0 |
| 625 SD | 720 | 576 | 442,368 | 300.0 | 300.0 | 300.0 | 300.0 | 300.0 |
| 480p (16:9) | 854 | 480 | 458,752 | 300.0 | 300.0 | 300.0 | 300.0 | 300.0 |
| SVGA | 800 | 600 | 532,480 | 300.0 | 300.0 | 300.0 | 300.0 | 300.0 |
| QHD | 960 | 544 | 552,960 | 300.0 | 300.0 | 300.0 | 300.0 | 300.0 |
| XGA | 1024 | 768 | 786,432 | 300.0 | 300.0 | 300.0 | 300.0 | 300.0 |
| 720p HD | 1280 | 720 | 983,040 | 272.0 | 300.0 | 300.0 | 300.0 | 300.0 |
| 4VGA | 1280 | 960 | 1,228,800 | 217.6 | 300.0 | 300.0 | 300.0 | 300.0 |
| SXGA | 1280 | 1024 | 1,310,720 | 204.0 | 300.0 | 300.0 | 300.0 | 300.0 |
| 525 16SIF | 1408 | 960 | 1,351,680 | 197.8 | 300.0 | 300.0 | 300.0 | 300.0 |
| 16CIF | 1408 | 1152 | 1,622,016 | 164.8 | 300.0 | 300.0 | 300.0 | 300.0 |
| 4SVGA | 1600 | 1200 | 1,945,600 | 137.4 | 274.8 | 300.0 | 300.0 | 300.0 |
| 1080 HD | 1920 | 1088 | 2,088,960 | 128.0 | 256.0 | 300.0 | 300.0 | 300.0 |
| 2Kx1K | 2048 | 1024 | 2,097,152 | 127.5 | 255.0 | 300.0 | 300.0 | 300.0 |
| 2Kx1080 | 2048 | 1088 | 2,228,224 | 120.0 | 240.0 | 300.0 | 300.0 | 300.0 |
| 4XGA | 2048 | 1536 | 3,145,728 | 85.0 | 170.0 | 300.0 | 300.0 | 300.0 |
| 16VGA | 2560 | 1920 | 4,915,200 | 54.4 | 108.8 | 204.0 | 300.0 | 300.0 |
| 3616x1536 (2.35:1) | 3616 | 1536 | 5,603,328 | 47.7 | 95.4 | 178.9 | 300.0 | 300.0 |
| 3672x1536 (2.39:1) | 3680 | 1536 | 5,701,632 | 46.8 | 93.7 | 175.8 | 300.0 | 300.0 |
| 4Kx2K | 4096 | 2048 | 8,388,608 | 31.8 | 63.7 | 119.5 | 239.0 | 300.0 |
| 4096x2160 | 4096 | 2160 | 8,912,896 | 30.0 | 60.0 | 112.5 | 225.0 | 300.0 |
| 4096x2304 (16:9) | 4096 | 2304 | 9,437,184 | - | - | 106.2 | 212.5 | 300.0 |
| 7680x4320 | 7680 | 4320 | 33,423,360 | - | - | 30.0 | 60.0 | 120.0 |

The following should be noted:

–	This Recommendation | International Standard is a variable-frame-size specification. The specific frame sizes in Table A-4 are illustrative examples only.

–	As used in Table A-4, "525" refers to typical use for environments using 525 analogue scan lines (of which approximately 480 lines contain the visible picture region), and "625" refers to environments using 625 analogue scan lines (of which approximately 576 lines contain the visible picture region).

–	XGA is also known as (aka) XVGA, 4SVGA aka UXGA, 16XGA aka 4Kx3K, CIF aka 625 SIF, 625 HHR aka 2CIF aka half 625 D-1, aka half 625 ITU-R BT.601, 525 SD aka 525 D-1 aka 525 ITU-R BT.601, 625 SD aka 625 D-1 aka 625 ITU-R BT.601.

–	Frame rates given are correct for progressive scan modes. The frame rates are also correct for interlaced video coding for the cases of frame height divisible by 32.