| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H1004 |
| --- | --- |

| Title: | Report on preliminary subjective testing of HEVC compression capability |
| --- | --- |
| Status: | Output Document Approved by JCT-VC |
| Purpose: | Report |
| Author(s) or Contact(s): | Vittorio Baroncini, Jens-Rainer Ohm, Gary Sullivan |
| Source: | JCT-VC |

_____________________________

# Abstract

This document reports on the HM subjective quality investigation that was performed prior to the Feb. 2012 San Jose meeting. The goal was to quantify, to the extent feasible, the rate savings for which similar subjective quality is achieved for the current HEVC model (HM5), compared to a similarly-configured JM encoder/decoder.

# Method of Investigation

It was assessed to be important to conduct the subjective tests with non-expert viewers. Due to the limited resources that were available for this purpose, tests were restricted to the random-access high-efficiency (RA-HE) settings of the HM (ref JCTVC-G1200) and to sequence test sequence Classes B and C (1920x1080 and 832x480, respectively). A somewhat-enhanced version of the JM encoder was used in the test, which included some non-normative improvements as had been described previously in contribution JCTVC-G399 and updated in JCTVC-H0360.

Instead of testing pre-defined bit rate points, the encodings for the two designs were determined by finding QP value points covering a roughly similar range of subjective quality, and running both codecs with fixed QP. This was done so that both designs were operated in as similar a manner as possible while maintaining the visual quality within a range spanning reasonable and measurable levels of quality. In terms of the subjective quality that would provide reasonable results when assessed by non-expert viewers, QP settings 27, 30, 33 and 36 were selected for the JM encoder, with confirmation by the test coordinator. Several experts involved in the preparation performed pre-assessments by informally comparing the JM versus the HM visual quality, and the overall opinion unveiled that with same QP settings, the HM would provide substantially better visual quality than the JM, which in fact can also be confirmed by PSNR numbers. As a result of these informal pre-assessments, it was decided to use QPHM= QPJM+4 settings for the HM encoded cases. Though the HM PSNR numbers are slightly lower for this case, the experts involved in pre-viewing came to the opinion that this gave a quality which was approximately equal to the JM quality in most cases. It was therefore decided to run the tests with QP settings of 31, 34, 37 and 40 for the HM (and QP settings of 27, 30, 33 and 36 for the JM).

# HM Configuration used for Bitstream Generation

The HM bitstreams were generated using the HM reference software from

https://hevc.hhi.fraunhofer.de/svn/svn_HEVCSoftware/tags/HM-5.0/.

The simulations were run using the configuration file cfg/encoder_randomaccess.cfg from that version of the reference software. The HM encoder was run with this configuration file and command line parameters adapted for each sequence. An example for encoding the sequence BQMall_832x480_60.yuv is shown below:

bin/TAppEncoderStatic -c cfg/encoder_randomaccess.cfg --FrameRate=60 \\
--FramesToBeEncoded=600 --SourceHeight=480 --SourceWidth=832 \\
--InputFile=orig/BQMall_832x480_60.yuv --IntraPeriod=64 --QP=40 --SEIpictureDigest=0 \\
--BitstreamFile=str/BQMall_832x480_60_40.hvc --ReconFile=tmp/BQMall_832x480_60_40.yuv

The sequences of Class B and Class C were encoded. The command line parameters used in the simulations are listed in the table below. The simulations were run for QP={31, 34, 37, 40} for the HM.

| InputFile | SourceWidth | SourceHeight | FrameRate | FramesToBeEncoded | IntraPeriod |
| --- | --- | --- | --- | --- | --- |
| Class B |  |  |  |  |  |
| BasketballDrive_1920x1080_50.yuv | 1920 | 1080 | 50 | 500 | 48 |
| BQTerrace_1920x1080_60.yuv | 1920 | 1080 | 60 | 600 | 64 |
| Cactus_1920x1080_50.yuv | 1920 | 1080 | 50 | 500 | 48 |
| Kimono_1920x1080_24.yuv | 1920 | 1080 | 24 | 240 | 24 |
| ParkScene_1920x1080_24.yuv | 1920 | 1080 | 24 | 240 | 24 |
| Class C |  |  |  |  |  |
| BasketballDrill_832x480_50.yuv | 832 | 480 | 50 | 500 | 48 |
| BQMall_832x480_60.yuv | 832 | 480 | 60 | 600 | 64 |
| PartyScene_832x480_50.yuv | 832 | 480 | 50 | 500 | 48 |
| RaceHorses_832x480_30.yuv | 832 | 480 | 30 | 300 | 32 |

Table 1 – Configurations used for the test

# Subjective Test

The tests were designed by the Test Coordination (Vittorio Baroncini), and performed at FUB-SICOM Laboratories (Rome, Italy) with the support of the test managers of ISCOM (G. Gaudino) and FUB (C. Delogu).

The test site followed the specification of Recommendation ITU-R BT 500-11, and a 32” HDTV professional monitor was used (EIZO CG301W), calibrated with the Eye One Display 2.

Ambient light was low and was illuminating the wall behind the monitors, with a fluorescent light source at 6500 K°.

The walls, ceiling and floor of the viewing area were non reflecting and mid gray coloured.

Two subjects at a time were seated in front of the monitor at a viewing distance of 3H (for Class B) and 4H (for Class C).

## Test method

The test method applied in individual test sessions was the Single Stimulus test method with an 11 level (0 to 10) quality scale methodology.

To minimize the “contextual effect” of the Single Stimulus test method, the video clips were shown to each group of two subjects in a different random order.

All mean opinion score (MOS) data points in the figures below show a 95% confidence interval (CI) in addition to the mean MOS value.

## Test design

The SS test method was selected to enable a more quick test execution.

As reported in Table 1, five test sequences were considered for Class B and four for Class C. For each test sequence, four coding conditions were considered, for both the JM and HM cases.

This led to evaluation of a total of 45 video clips for Class B and 36 video clips for Class C (including the non-compressed references).

Two test sessions were run: one for Class B and one for Class C, running for 12 and 10 minutes, respectively. 18 subjects participated to the test; all subjects passed the pre-test screening (Snellen and Ishihara acuity and colour vision test charts) and the post screening (raw data analysis) tests.

Post-screening was done including checking the completeness of the data for each viewer (wherein a viewer's scores would need to be discarded if there was more than one vote missing in a session) and verifying each subject for unstable scores and/or extreme scores (i.e. outliers). No subjects were discarded as a result of this post screening check.

Before running the test sessions, an accurate training of the subjects was done; this included an accurate explanation of the meaning of the voting scale, of the voting procedure (how to fill in the forms and when to vote) a detailed explanation of the scoring scale; then all the subjects went through a short training session made of 8 coded examples (using video clips different from than those used in the test).

Scores were collected on paper scoring sheets (see Figure 1 for a “training session” scoring sheet sample).

Figure 1 - Example of “training session” scoring sheet.

Viewing subjects were asked to write into each scoring box a number from 0 (lowest quality) to 10 (best quality) to indicate their “opinion score” on the video clip just seen. This new style of score sheet was used in order to present a simple form to the viewers; reactions from viewers, collected by the test manager after the test, confirmed that the new scoring sheet was considered more practical and easy to be used by all the test subjects. (All of them had participated in the past in other subjective assessment tests using “classic” scoring forms.)

# Results and Evaluation

Results of the test (MOS, standard deviation and confidence intervals) are available from the attached Excel sheet. As the subjective tests did not unveil exactly the same quality of HM and JM for the different test cases, linear interpolation was performed on RD graphs (MOS versus rate). Such an approach appears justified since the QP differences between the various test points are relatively small, such that a fairly precise estimate of the rate difference between the two codecs at same quality could be achieved. Rate differences have been computed for all cases where an MOS point for one of the codecs has a matching point (i.e. in the range where the two plots overlap on the MOS axis). The detailed plots are shown in the subsequent Figures.

# Conclusions

From the results, an approximate average bit rate reduction for equivalent subjective quality can be observed of 67% for the Class B sequences, 49% for Class C sequences, and 58% overall. The major goal of the HEVC project is to develop the next generation video coding standard that could achieve the same level of video quality with a substantial savings (e.g. reduction by half) relative to the bit rate required by AVC. The initial measurements as performed at this stage indicate that, in terms of subjective quality, the HEVC performance is roughly meeting this goal. Even though the test cases were limited, the fact that the same tendency can be observed for all the different sequences is a promising indicator of the substantial improvement in compression efficiency that is delivered by HEVC. Nevertheless, it should be noted that these results are only giving a preliminary indication, based on an intermediate snapshot of standard reference software development in advance of the finalization of the design of the HEVC coding standard, whereas more rigorous verification tests with a larger variety of test material will be necessary after finalization of the video coding standard.

Acknowledgements

FUB and ISCOM are thanked for their support in the execution of the test.