| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San Jose, CA, US, 1-10 February, 2012 | Document: JCTVC-H1101 WG11 Number: m24014 |
| --- | --- |

| Title: | Description of Core Experiment 1 (CE1): Sample adaptive offset filtering |  |  |
| --- | --- | --- | --- |
| Status: | Output Document of JCT-VC |  |  |
| Purpose: | CE description |  |  |
| Author(s) or Contact(s): | Yu-Wen Huang (Primary) Elena Alshina In Suk Chong Wade Wan Minhua Zhou |  | yuwen.huang@mediatek.com elena_a.alshina@samsung.com ichong@qualcomm.com wwan@broadcom.com zhou@ti.com |
| Source: | CE coordinators |  |  |

_____________________________

# Introduction and general rules

CE1 focuses on sample adaptive offset (SAO) filtering including band offset (BO) and edge offset (EO). Related proposals are as follows.

JCTVC-H0067 Non-CE8.a.2: SAO with LCU-based syntax [A. Fuldseth, G. Bøntegaard (Cisco)]

JCTVC-H0114 Non-CE8: Sample adaptive offset with constrained ranges of edge offset [E. Alshina, A. Alshin, J. H. Park (Samsung), I. S. Chong, M. Karczewicz (Qualcomm), C.-M. Fu, C.-Y. Chen, C.-Y. Tsai, Y.-W. Huang, S. Lei (MediaTek)]

JCTVC-H0174 CE8: Results related to section e.2 [G. Laroche, T. Poirier, P. Onno (Canon)]

JCTVC-H0201 Non-CE8: Method of removing boundary artifacts for SAO [T.Matsunobu, H. Sasai, T. Nishi (Panasonic)]

JCTVC-H0273 CE8.a.3: SAO with LCU-based syntax [C.-M. Fu, C.-Y. Chen, C.-Y. Tsai, Y.-W. Huang, S. Lei (MediaTek), I. S. Chong, M. Karczewicz (Qualcomm), E. Alshina, A. Alshin (Samsung)]

JCTVC-H0406 Flexible band offset mode in SAO [E. Maani, O. Nakagami (Sony)]

JCTVC-H0552 Non-CE8: Modified EO offset operation and offset quantization [K. Minoo, D. Baylon (Motorola Mobility)]

JCTVC-H0553 Non-CE8: Harmonization of LCU-based SAO [K. Minoo, Y. Yu, D. Baylon, L. Wang (Motorola Mobility)]

JCTVC-H0557 CE8.e.4: Selection and coding of SAO parameters [Koohyar Minoo, David Baylon (Motorola)]

These proposals will be further studied on top of HM-6.0 under the common test conditions defined in JCTVC-H1100. For all proposals that will affect the adaptation parameter set (APS) mode of SAO, picture-based optimization shall be tested. This includes HE10-RA and Main-RA conditions with only one LCU size (i.e. 64x64) if additional conditions are not specified in Section 4. For all proposals that will affect the interleaving mode of SAO, LCU-based optimization shall be tested. This includes HE10-LDB and Main-LDB conditions with three different LCU sizes (i.e. 64x64, 32x32, and 16x16) if additional conditions are not specified in Section 4. Moreover, subjective viewing will be conducted for JCTVC-H0201 to verify visual quality improvements.

In the SAO of HM-6.0, as adopted from JCTVC-H0273 software, when LCU-based optimization is used, the encoder does not use any pixel from the bottom three luma pixel rows and the bottom one chroma pixel row in each LCU during parameter estimation because in practical LCU-based pipelining systems these pixels are not yet processed by deblocking filter (DF). However, the rightmost pixel columns in each LCU are still involved in parameter estimation. This means deriving parameters for one LCU is dependent on one future LCU, which is not desirable for practical LCU-based pipelining systems either. Therefore, the following rule is requested. In this CE, all anchors and proposals for the interleaving mode shall avoid using the bottom three luma pixel rows, the bottom one chroma pixel row, the rightmost four luma pixel columns, and the rightmost two chroma pixel columns during parameter estimation on the encoder side.

In addition, in the SAO of HM-6.0, when LCU-based optimization is used, three non-normative encoder-only chroma constraints are enabled to reduce run time. One is to disallow chroma BO, another is to limit chroma EO ranges, and the other is to disable chroma SAO for the entire slice according to the first LCU. However, these chroma constraints were not tested in combination with other SAO adoptions including JCTVC-H0174, JCTVC-H0406, and JCTVC-H0434 at the last meeting. In this CE, all anchors and proposals for the interleaving mode shall remove the encoder-only chroma constraints as follows.
In TypeDef.h, at lines #117-119
#define REMOVE_SAO_LCU_ENC_CONSTRAINTS_1    1
#define REMOVE_SAO_LCU_ENC_CONSTRAINTS_2    1
#define REMOVE_SAO_LCU_ENC_CONSTRAINTS_3    1

The anchor software will be released by MediaTek to all CE1 participants.

# Test descriptions

HM-6.0

| Test | Description | Purpose |
| --- | --- | --- |
| 0.1 | Enable the APS mode of SAO for HE10-AI, HE10-RA, HE10-LDB, HE10-LDP, Main-AI, Main-RA, Main-LDB, and Main-LDP with LCU size = 64x64 | Information |
| 0.2 | Enable the APS mode of SAO for HE10-AI, HE10-RA, HE10-LDB, HE10-LDP, Main-AI, Main-RA, Main-LDB, and Main-LDP with LCU size = 32x32 | Information |
| 0.3 | Enable the APS mode of SAO for HE10-AI, HE10-RA, HE10-LDB, HE10-LDP, Main-AI, Main-RA, Main-LDB, and Main-LDP with LCU size = 16x16 | Information |
| 0.4 | Enable the interleaving mode of SAO for HE10-AI, HE10-RA, HE10-LDB, HE10-LDP, Main-AI, Main-RA, Main-LDB, and Main-LDP with LCU size = 64x64 | Information |
| 0.5 | Enable the interleaving mode of SAO for HE10-AI, HE10-RA, HE10-LDB, HE10-LDP, Main-AI, Main-RA, Main-LDB, and Main-LDP with LCU size = 32x32 | Information |
| 0.6 | Enable the interleaving mode of SAO for HE10-AI, HE10-RA, HE10-LDB, HE10-LDP, Main-AI, Main-RA, Main-LDB, and Main-LDP with LCU size = 16x16 | Information |

JCTVC-H0067 Non-CE8.a.2: SAO with LCU-based syntax [A. Fuldseth, G. Bøntegaard (Cisco)]

| Test | Description | Purpose | Picture-based optimization (LCU size) | LCU-based optimization (LCU size) |
| --- | --- | --- | --- | --- |
| 1.1 | Disallow BO for the interleaving mode | Objective coding gain and complexity reduction | No | Yes (64/32/16) |
| 1.2 | For the interleaving mode, use one implicit flag to switch between explicit parameters and implicit parameters, where the implicit parameters are as follows: -1 for the peak and slope down categories and, +1 for the valley and slope up categories, and same EdgeIdx for chroma and luma. The implicit flag will be signaled or derived from the LCU size. | Objective coding gain | No | Yes (64/32/16) |
| 1.3 | Add a pre-check before the pixel classification in EO x: current pixel x1: one of the two neighbors x2: one of the two neighbors if (abs(x1+x2-x-x) > 1) do pixel classification and add an offset else do nothing (offset=0) | Objective coding gain | Yes (64) | Yes (64/32/16) |
| 1.4 | Combine Test1.2 and Test1.3 for the interleaving mode | Objective coding gain | No | Yes (64/32/16) |
| 1.5 | Share information between Cb and Cr for the interleaving mode | Objective coding gain | No | Yes (64/32/16) |

JCTVC-H0114 Non-CE8: Sample adaptive offset with constrained ranges of edge offset [E. Alshina, A. Alshin, J. H. Park (Samsung), I. S. Chong, M. Karczewicz (Qualcomm), C.-M. Fu, C.-I. Chen, C.-Y. Tsai, Y.-W. Huang, S. Lei (MediaTek)]

JCTVC-H0174 CE8: Results related to section e.2 [G. Laroche, T. Poirier, P. Onno (Canon)]

JCTVC-H0406 Flexible band offset mode in SAO [E. Maani, O. Nakagami (Sony)]

| Test | Description | Purpose | Picture-based optimization (LCU size) | LCU-based optimization (LCU size) |
| --- | --- | --- | --- | --- |
| 2.1 | This test mainly comes from JCTVC-H0174 and JCTVC-H0406. Send N offsets in BO for the interleaving mode, where N=1, 2, or 4, N can be always fixed, explicitly signaled, or dependent on LCU size; proponents will decide only one of the variants for crosscheck. | Feasibility study and objective coding gain | No | Yes (64/32/16) |
| 2.2 | This test mainly comes from JCTVC-H0114. Send M offsets in EO for the interleaving mode, where M=1, 2, or 4, M can be always fixed, explicitly signaled, or dependent on LCU size; use the proposed offset ranges; use the offset coding method with a most probable offset flag and fixed length representation of remaining offsets for transmitted offsets; when M<4, use predefined values or transmitted offsets to derive the (4-M) offsets, e.g., send EO(0), derive the other three offsets as follows: EO(1) = EO(0) / 2; EO(2) = -EO(0) / 2; EO(3) = -EO(0); proponents will decide only one of the variants for crosscheck. | Feasibility study and objective coding gain | No | Yes (64/32/16) |
| 2.3 | This test mainly comes from the ALF codebook design in JCTVC-H0066. Allow sending SAO codebook index for the interleaving mode, i.e., use a fixed-size codebook that stores previously coded LCU-level SAO parameters for future LCUs of the same slice to reference. If current LCU's offsets are the same as one of stored codes, the index is sent to the decoder. Otherwise new offsets are sent. Its intention is harmonization with the codebook idea in H0066. | Feasibility study and objective coding gain | No | Yes (64/32/16) |

JCTVC-H0201 Non-CE8: Method of removing boundary artifacts for SAO [T.Matsunobu, H. Sasai, T. Nishi (Panasonic)]

| Test | Description | Purpose | Picture-based optimization (LCU size) | LCU-based optimization (LCU size) |
| --- | --- | --- | --- | --- |
| 3.1 | Reduce the offset magnitude by half for BO | Subjective quality gain (non-normative) | Yes (64) | Yes (64/32/16) |
| 3.2 | Weakening offset values near LCU boundaries for BO | Subjective quality gain | Yes (64) | Yes (64/32/16) |
| 3.3 | Weakening offset values near category boundaries for BO | Subjective quality gain | Yes (64) | Yes (64/32/16) |
| 3.4 | Weakening offset values near both LCU and category boundaries for BO | Subjective quality gain | Yes (64) | Yes (64/32/16) |

JCTVC-H0273 CE8.a.3: SAO with LCU-based syntax [C.-M. Fu, C.-Y. Chen, C.-Y. Tsai, Y.-W. Huang, S. Lei (MediaTek), I. S. Chong, M. Karczewicz (Qualcomm), E. Alshina, A. Alshin (Samsung)]

| Test | Description | Purpose | Picture-based optimization (LCU size) | LCU-based optimization (LCU size) |
| --- | --- | --- | --- | --- |
| 4.1 | Remove run prediction for the APS mode | Complexity reduction | Yes (64) | No |
| 4.2 | Change the APS mode syntax from LCU-based to region-based, where each region contains MxN pixels, M and N must be multiples of LCU size and are signaled in APS. SAO parameters are adapted at the region level. Minimum region size X which is Kx64x64 (where K is power of 2) will be clearly defined for different levels in order to achieve a good trade-off between decoder memory size and the coding efficiency. A compliant bitstream should satisfy M x N >=X. In the APS, (M, N) is signaled by using (log2_M_minus_n, log2_N_minus_n) with ue(v), where M = (1<< (log2_M_minus_n + n)) N = (1<< (log2_N_minus_n + n)) and (1<<n)*(1<<n) (n = 4, 5, or 6) is LCU size signaled in the bitstream | Complexity reduction | Yes (64) | No |
| 4.3 | Remove the pixel access constraint for the interleaving mode | Information | No | Yes (64/32/16) |
| 4.4 | Remove sao_merge_up_flag for the interleaving mode | Complexity reduction | No | Yes (64/32/16) |
| 4.5 | Remove sao_merge_up_flag and sao_merge_left_flag for the interleaving mode | Complexity reduction | No | Yes (64/32/16) |

JCTVC-H0552 Non-CE8: Modified EO offset operation and offset quantization [K. Minoo, D. Baylon (Motorola Mobility)]

JCTVC-H0557 CE8.e.4: Selection and coding of SAO parameters [Koohyar Minoo, David Baylon (Motorola)]

| Test | Description | Purpose | Picture-based optimization (LCU size) | LCU-based optimization (LCU size) |
| --- | --- | --- | --- | --- |
| 5.1 | Modify the offset application for EO as described in Section 2 of JCTVC-H0552 which allows to reduce the number of offset classes from 4 to 2 and 1 by merging existing 4 offset classes as done in Section 4 of JCTVC-H0552 C: current pixel before applying EO L: one of the two neighbors R: one of the two neighbors O: offset V: current pixel after applying EO If 2C>(L+R), I=(L+R)>>1 If 2C<(L+R), I=(L+R+1)>>1 If 2C=(L+R), I=C When C!=I If O applied to C brings the pixel value away from I, V=C+O If O applied to C brings the pixel value towards I but does not pass I, V=C+O If O applied to C brings the pixel value towards I and it passes I, V=I When C=I V=C+O | Objective coding gain | Yes (64) | Yes (64/32/16) |
| 5.2 | Use quantization and coding of offset values as described in Section 3 of JCTVC-H0552 and the parent document JCTVC-H0557 This would require: When the number of EO offsets is two: swapping the code word for offset index = 0 and offset index = 1 (for peak (valley) class only) When the number of EO offsets is one: skipping the offset zero (therefore offset index = 0 corresponds to non-zero value). | Objective coding gain | Yes(64) | Yes (64/32/16) |

JCTVC-H0553 Non-CE8: Harmonization of LCU-based SAO [K. Minoo, Y. Yu, D. Baylon, L. Wang (Motorola Mobility)]

| Test | Description | Purpose | Picture-based optimization (LCU size) | LCU-based optimization (LCU size) |
| --- | --- | --- | --- | --- |
| 6.1 | Send N offsets in BO (N=2 for chroma and N=4 for luma) where each offset applies to a set of pixels values (offset bands). The N offset bands are derived from the statistics of a subset of pixels inside the LCU (these pixels are not affected by de-blocking operation) | Objective coding gain | Yes(64) | Yes (64/32/16) |
| 6.2 | Modify the entropy coding of the SAO types as described in the Section of 5 of the parent document JCTVC-H0557 if(SAO off) { code(1); } else { code(0); if(EO Type) { code(0); if (EO Type == x) code(x); } else { code(1); } } | Objective coding gain | Yes(64) | Yes (64/32/16) |

Harmonization

| Test | Description | Purpose | Picture-based optimization (LCU size) | LCU-based optimization (LCU size) |
| --- | --- | --- | --- | --- |
| 7.1 | Combine above tests that show promising results | Integration | Yes (64/32/16) | Yes (64/32/16) |
| 7.2 | On top of Test7.1, use bypass bins as many as possible without sacrificing coding efficiency for the interleaving mode | Complexity reduction | No | Yes (64/32/16) |

# Timeline

T1:		2012-Mar-06:	Release HM-6.0

T2:		2012-Mar-09:	Upload finalized CE description and release anchor software

T3:		2012-Mar-29:	Release individual software packages to all CE participants for crosscheck

T4:		2012-Apr-02:	Report test results to coordinators

T5:		2012-Apr-05:	Reach consensus on Test7.1 according to test results

T6:		2012-Apr-13:	Release Test7.1 and Test7.2 software packages for crosscheck

T7:		2012-Apr-16:	Upload contributions and report document numbers to all CE participants

T8: 	2012-Apr-25:	Upload CE summary report

# Responsibilities

| Test | Proponent | Crosschecker | Picture-based optimization (default conditions if not specified: HE10-RA & Main-RA) | LCU-based optimization (default conditions if not specified: HE10-LDB & Main-LDB) |
| --- | --- | --- | --- | --- |
| 0.1 | MediaTek | N/A | LCU=64 HE10-AI/RA/LDB/LDP Main-AI/RA/LDB/LDP | N/A |
| 0.2 | MediaTek | N/A | LCU=32 HE10-AI/RA/LDB/LDP Main-AI/RA/LDB/LDP | N/A |
| 0.3 | Samsung | N/A | LCU=16 HE10-AI/RA/LDB/LDP Main-AI/RA/LDB/LDP | N/A |
| 0.4 | Samsung | N/A | N/A | LCU=64 HE10-AI/RA/LDB/LDP Main-AI/RA/LDB/LDP |
| 0.5 | Qualcomm | N/A | N/A | LCU=32 HE10-AI/RA/LDB/LDP Main-AI/RA/LDB/LDP |
| 0.6 | Qualcomm | N/A | N/A | LCU=16 HE10-AI/RA/LDB/LDP Main-AI/RA/LDB/LDP |
| 1.1 | Cisco | Qualcomm | N/A | LCU=64/32/16 |
| 1.2 | Cisco | Qualcomm | N/A | LCU=64/32/16 |
| 1.3 | Cisco | Samsung | LCU=64 | LCU=64/32/16 |
| 1.4 | Cisco | Samsung | N/A | LCU=64/32/16 |
| 1.5 | MediaTek | TI | N/A | LCU=64/32/16 |
| 2.1 | MediaTek | Canon | N/A | LCU=64/32/16 |
| 2.2 | Samsung | Cisco | N/A | LCU=64/32/16 |
| 2.3 | Qualcomm | Panasonic | N/A | LCU=64/32/16 |
| 3.1 | Panasonic | Samsung | LCU=64 | LCU=64/32/16 |
| 3.2 | Panasonic | Samsung | LCU=64 | LCU=64/32/16 |
| 3.3 | Panasonic | Samsung | LCU=64 | LCU=64/32/16 |
| 3.4 | Panasonic | Samsung | LCU=64 | LCU=64/32/16 |
| 4.1 | MediaTek | Panasonic | LCU=64 | N/A |
| 4.2 | MediaTek | TI | LCU=64 | N/A |
| 4.3 | TI | N/A | N/A | LCU=64/32/16 |
| 4.4 | TI | Panasonic | N/A | LCU=64/32/16 |
| 4.5 | TI | Broadcom | N/A | LCU=64/32/16 |
| 5.1 | Motorola Mobility, Samsung | TI | LCU=64 | LCU=64/32/16 |
| 5.2 | Motorola Mobility, Samsung | Ericsson | LCU=64 | LCU=64/32/16 |
| 6.1 | Motorola Mobility, Samsung | Sony | LCU=64 | LCU=64/32/16 |
| 6.2 | Motorola Mobility, Samsung | Cisco | LCU=64 | LCU=64/32/16 |
| 7.1 | To be decided by T5 | To be decided by T5 | LCU=64/32/16 HE10-AI/RA/LDB/LDP Main-AI/RA/LDB/LDP | LCU=64/32/16 HE10-AI/RA/LDB/LDP Main-AI/RA/LDB/LDP |
| 7.2 | To be decided by T5 | To be decided by T5 | N/A | LCU=64/32/16 HE10-AI/RA/LDB/LDP Main-AI/RA/LDB/LDP |

Remark: All anchors and proposals shall follow the common test conditions in JCTVC-H1100 except LCU size and SAO on/off.

Remark: The LCU size of a corresponding anchor shall be the same as that of a proposal under testing.

Remark: When proposals are tested with LCU size equal to 64, BD-rates and run time percentages shall be accurate.

Remark: When proposals are tested with LCU size equal to 32 or 16, BD-rates shall be accurate, but run time percentages can be inaccurate for Test1-6 series. The corresponding anchor data of Test1-6 series can be found in Test0.5 or Test0.6.

Remark: Test0 and Test7 series use SAO-off as anchors, while all the other tests use SAO-on as anchors.

Remark: General rules for anchors and proposals are described in Section 1 and shall be strictly obeyed.

Remark: Crosscheckers of Test3.1, Test3.2, Test3.3, and Test3.4 are required to evaluate the subjective quality carefully before T4.

Remark: MediaTek volunteers to implement Test7.1 and Test7.2.

# Participants

| Company | Name | Email |
| --- | --- | --- |
| Broadcom | Wade Wan Peisong Chen Tim Hellman Yong Yu | wwan@broadcom.com peisongc@broadcom.com timhellman@broadcom.com yongyu@broadcom.com |
| Canon | Edouard Francois Patrice Onno Guillaume Laroche Christophe Gisquet | edouard.francois@crf.canon.fr patrice.onno@crf.canon.fr guillaume.laroche@crf.canon.fr Christophe.Gisquet@crf.canon.fr |
| Cisco | Arild Fuldseth Thomas Davies | arilfuld@cisco.com thdavies@cisco.com |
| eBrisk Video | Faouzi Kossentini Michael Horowitz Hsan Guermazi | faouzi@ebriskvideo.com michael@ebriskvideo.com hsan@ebriskvideo.com |
| Ericsson | Andrey Norkin Kenneth Andersson | andrey.norkin@ericsson.com kenneth.r.andersson@ericsson.com |
| ETRI | Jinho Lee Hui Yong Kim Sung-Chang Lim | jinosoul@etri.re.kr hykim5@etri.re.kr sclim@etri.re.kr |
| Fraunhofer HHI | Benjamin Bross | benjamin.bross@hhi.fraunhofer.de |
| Huawei | Yongbing Lin Wen Gao Xiaozhen Zheng | linyongbing@huawei.com wen.gao@huawei.com zhengxiaozhen@huawei.com |
| Institute for Infocomm Research (I2R) | Chuohao Yeo Yih Han Tan | chyeo@i2r.a-star.edu.sg yhtan@i2r.a-star.edu.sg |
| Intel | Yi-Jen Chiu Lidong Xu Wenhao Zhang Han Yu | Yi-jen.chiu@intel.com Lidong.xu@intel.com Wenhao.zhang@intel.com yu.han@intel.com |
| LG | Seungwook Park Yong-Joon Jeon Joonyoung Park Jungsun Kim | Seungwook.park@lge.com yongjoon.jeon@lge.com jy.park@lge.com jungsun.kim@lge.com |
| MediaTek | Chia-Yang Tsai Chih-Ming Fu Ching-Yeh Chen Chih-Wei Hsu Shawmin Lei Xun Guo Yu-Wen Huang | chiayang.tsai@mediatek.com chihming.fu@mediatek.com chingyeh.chen@mediatek.com cw.hsu@mediatek.com shawmin.lei@mediatek.com xun.guo@mediatek.com yuwen.huang@mediatek.com |
| Mitsubishi | Shun-ichi Sekiguchi Kazuo Sugimoto | Sekiguchi.Shunichi@eb.MitsubishiElectric.co.jp Sugimoto.Kazuo@ak.MitsubishiElectric.co.jp |
| Motorola Mobility | David Baylon Koohyar Minoo Yue Yu | david.baylon@motorola.com kminoo@motorola.com yyu@motorola.com |
| NTT | Masaaki Matsumura | matsumura.masaaki@lab.ntt.co.jp |
| NTT DOCOMO | Frank Bossen TK Tan | bossen@docomoinnovations.com ttk@pacific.net.sg |
| Nokia | Kemal Ugur Oguz Bici Jani Lainema | Kemal.Ugur@nokia.com ext-oguz.bici@nokia.com jani.lainema@nokia.com |
| Panasonic | Matthias Narroschke Thomas Wedi Semih Esenlik Toru Matsunobu Toshiyasu Sugio Hisao Sasai Takahiro Nishi | Matthias.Narroschke@panasonic.de Thomas.Wedi@eu.panasonic.com Semih.Esenlik@eu.panasonic.com matsunobu.toru@jp.panasonic.com sugio.toshiyasu@jp.panasonic.com sasai.hisao@jp.panasonic.com nishi.takahiro@jp.panasonic.com |
| Peking University | Siwei Ma | swma@jdl.ac.cn |
| Qualcomm | Marta Karczewicz In Suk Chong Wei-Jung Chien Sanjeev Kumar | martak@qualcomm.com ichong@qualcomm.com wchien@qualcomm.com sanjeevk@qualcomm.com |
| Samsung | PoLin Lai Felix C. Fernandes Elena Alshina Elena Alshina Il-Koo Kim Ankur Saxena | wlai@sta.samsung.com ffernandes@sta.samsung.com elena_a.alshina@samsung.com elena.alshina@gmail.com ilkoo.kim@samsung.com asaxena@sta.samsung.com |
| Santa Clara University | Guichun Li Rafael Kaliski | g1li@scu.edu RKaliski@scu.edu |
| Sejong University | Ju Ock Lee | jolee@sju.ac.kr |
| Sharp | Tomohiro Ikai Tomoyuki Yamamoto Yukinobu Yasugi Andrew Segall Jane Zhao Kiran Misra | ikai.tomohiro@sharp.co.jp yamamoto.tomoyuki@sharp.co.jp yasugi.yukinobu@sharp.co.jp asegall@sharplabs.com jzhao@sharplabs.com misrak@sharplabs.com |
| SK Telecom | Joo-Hee Moon Jeongyoen Lim | jhmoon@sejong.ac.kr jeongyeon@sk.com |
| Sony | Ali Tabatabai Ehsan Maani Masaru Ikeda | Ali.Tabatabai@am.sony.com Ehsan.Maani@am.sony.com Masaru.Ikeda@jp.sony.com |
| Texas Instruments (TI) | Madhukar Budagavi Minhua Zhou Woo-Shik Kim Do-Kyoung Kwon | madhukar@ti.com zhou@ti.com wskim@ti.com d-kwon@ti.com |
| Toshiba | Takeshi Chujoh Tomoo Yamakage | takeshi.chujoh@toshiba.co.jp tomoo.yamakage@toshiba.co.jp |
| Zenverge | Dzung Hoang | Dzung.Hoang@Zenverge.com |