| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San Jose, CA, US, 1-10 February, 2012 | Document: JCTVC-H1102_r1 WG11 Number: m24012 |
| --- | --- |

| Title: | Description of Core Experiment 2 (CE2): Adaptive loop filtering |  |  |
| --- | --- | --- | --- |
| Status: | Output Document of JCT-VC |  |  |
| Purpose: | CE description |  |  |
| Author(s) or Contact(s): | Tomoo Yamakage (Primary) In Suk Chong Yu-Wen Huang Matthias Narroschke Madhukar Budagavi |  | tomoo.yamakage@toshiba.co.jp ichong@qualcomm.com yuwen.huang@mediatek.com Matthias.Narroschke@eu.panasonic.com madhukar@ti.com |
| Source: | CE Coordinators |  |  |

_____________________________

# Introduction

The goal of this Core Experiment (CE) is to investigate simplification of Adaptive Loop Filtering (ALF).

In CE2, there are 9 subtests. (a) To investigate an appropriate signaling mode (b) To investigate simplification of ALF signaling (c) To investigate an appropriate number of classes in BA mode. (d) To investigate an appropriate of regions in RA mode (e) To investigate an appropriate depth of LCU division for filtering control. (f) To investigate double padding issue on tile boundary. (g) To investigate encoder complexity reduction for LCU-based optimization (h) To investigate reusing luma filter coefficients in chroma filtering for LCU-based optimization.

In addition, (i) combination of all subtests will be investigated to provide the overall performance.

Techniques are evaluated according to BD-rate gain, complexity and/or subjective picture quality. The following items are categorized as subtests:

- Signalling mode change from "switch between APS mode and slice header mode" to "switch between APS mode and interleaving mode"

JCTVC-H0066/H0274 (Qualcomm, MediaTek, Toshiba, Cisco)

JCTVC-H0066/H0274/JCTVC-C082 (Qualcomm, MediaTek, Toshiba, Cisco)

- Simplification of ALF signalling

JCTVC-H0066/H0274 (MediaTek)

JCTVC-H0274 (Qualcomm, MediaTek, Toshiba)

Necessity of alf_pred_flag (Toshiba, Nokia)

Necessity of alf_nb_pred_luma_flag (Toshiba, Nokia)

Qp dependent quantization of ALF coefficients JCTVC-H0274/H0066/HM5 (Toshiba, Broadcom)

- Number of classes in BA mode

JCTVC-H0066/H0415/H0409 (Cisco, Samsung, Sony, Sharp)

JCTVC-H0094 (NTT)

- Number of regions in RA mode

JCTVC-H0274/HM5 (Toshiba, MediaTek, Qualcomm, TI, Broadcom)

- Depth of LCU division for filtering control

JCTVC-H0066/H0274 (Cisco, Samsung, Qualcomm, MediaTek, Toshiba)

- Line memory reduction fix at independent slice and tile boundaries

JCTVC-H0372 (Panasonic)

- Encoder complexity reduction for LCU-based optimization

JCTVC-H0274 (MediaTek)

- Reuse luma filter coefficients in chroma filtering for LCU-based optimization

JCTVC-H0066 (Cisco, Qualcomm, MediaTek, Toshiba)

- Combination subtests

JCTVC-H0066 (Cisco)

Subjective viewing will be conducted.

The software basis for this CE is HM 6.0, and the experiments are conducted based on the common test conditions in JCTVC-H1100.

# Participants

| Nr. | Name | Company | Email | Remarks |
| --- | --- | --- | --- | --- |
| 1 2 3 4 5 6 7 8 9 10 11 12 13 14 15 16 17 18 19 20 21 22 23 24 25 26 27 28 29 30 31 32 33 34 35 36 37 38 39 40 41 42 43 44 45 46 47 48 49 50 51 52 53 54 55 56 57 58 59 | Chia-Yang Tsai Chih-Ming Fu Ching-Yeh Chen Shawmin Lei Xun Guo Yu-Wen Huang Matthias Narroschke (coordinator) Thomas Wedi Semih Esenlik Toru Matsunobu Marta Karczewicz In Suk Chong (coordinator) Wei-Jung Chien Wang (PoLin) Lai Felix C. Fernandes Elena Alshina Il-Koo Kim Ankur Saxena Takeshi Chujoh Tomoo Yamakage (Primary coordinator) Ali Tabatabai Ehsan Maani Masaru Ikeda Arild Fuldseth Thomas Davies Masaaki Matsumura Seishi Takamura Kenneth Andersson Andrey Norkin B. Bross T. K. Tan Frank Bossen Tomohiro Ikai Tomoyuki Yamamoto Yukinobu Yasugi Andrew Segall Jie Zhao Kiran Misra Yeo Chuo Hao Madhukar Budagavi (coordinator) Woo-Shik Kim Minhua Zhou Wen Gao Peisong Chen Wade Wan Tim Hellman Yong (Andover) Yu Kemal Ugur Kazuo Sugimoto Dzung Hoang John Sievers Yi-Jen Chiu Lidong Xu Wenhao Zhang Jung-Sun Kim Masaaki Matsumura Guichun Li Rafael Kaliski Xiaozhen Zheng | MediaTek MediaTek MediaTek MediaTek MediaTek MediaTek Panasonic Panasonic Panasonic Panasonic Qualcomm Qualcomm Qualcomm Samsung Samsung Samsung Samsung Samsung Toshiba Toshiba Sony Sony Sony Cisco Cisco NTT NTT Ericsson Ericsson HHI DOCOMO NTT Docomo Sharp Sharp Sharp Sharp Sharp Sharp I2R TI TI TI Huawei Broadcom Broadcom Broadcom Broadcom Nokia Mitsubishi Zenverge LifeSize Com. Intel Intel Intel LGE NTT Santa Clara Univ. Santa Clara Univ. Huawei | chiayang.tsai@mediatek.com chihming.fu@mediatek.com chingyeh.chen@mediatek.com shawmin.lei@mediatek.com xun.guo@mediatek.com yuwen.huang@mediatek.com Matthias.Narroschke@panasonic.de Thomas.Wedi@eu.panasonic.com Semih.Esenlik@eu.panasonic.com matsunobu.toru@jp.panasonic.com martak@qualcomm.com ichong@qualcomm.com wchien@qualcomm.com wlai@sta.samsung.com ffernandes@sta.samsung.com elena_a.alshina@samsung.com ilkoo.kim@samsung.com asaxena@sta.samsung.com takeshi.chujoh@toshiba.co.jp tomoo.yamakage@toshiba.co.jp Ali.Tabatabai@am.sony.com Ehsan.Maani@am.sony.com Masaru.Ikeda@jp.sony.com arilfuld@cisco.com kenneth.r.andersson@ericsson.com matsumura.masaaki@lab.ntt.co.jp takamura.seishi@lab.ntt.co.jp thdavies@cisco.com andrey.norkin@ericsson.com benjamin.bross@hhi.fraunhofer.de ttk@pacific.net.sg bossen@docomoinnovations.com ikai.tomohiro@sharp.co.jp yamamoto.tomoyuki@sharp.co.jp yasugi.yukinobu@sharp.co.jp asegall@sharplabs.com jzhao@sharplabs.com misrak@sharplabs.com chyeo@i2r.a-star.edu.sg madhukar@ti.com wskim@ti.com zhou@ti.com wen.gao@huawei.com peisongc@broadcom.com wwan@broadcom.com timhellman@broadcom.com yongyu@broadcom.com Kemal.Ugur@nokia.com Sugimoto.Kazuo@ak.mitsubishielectric.co.jp Dzung.Hoang@Zenverge.com jsievers77@gmail.com yi-jen.chiu@intel.com lidong.xu@intel.com wenhao.zhang@intel.com jungsun.kim@lge.com jctvc-te@lab.ntt.co.jp g1li@scu.edu RKaliski@scu.edu zhengxiaozhen@huawei.com | Proponent Proponent Proponent Proponent Proponent Proponent Proponent Proponent Proponent Proponent Proponent Proponent Proponent Proponent Proponent Proponent Proponent Proponent Proponent Proponent Proponent Proponent Proponent Proponent Proponent Proponent Proponent |

# Experimental Conditions

## Software

HM 6.0 shall be used, which will be released three weeks after the 8th JCT-VC meeting.

In addition, the patch file for ticket #400 for HM shall be applied to both anchor/reference and test for picture-based encoding. (Note: The patch file does not affect results for LCU-based encoding.)

## Test Sequences, Bit Rates and Coding Conditions

All test sequences under the configurations described in JCTVC-H1100 shall be tested.

For HM6 software, all coding structures are evaluated for both picture-based encoding and LCU-based encoding with 8-bits (Main (w/o ALF)) and 10-bits (HE10 (w/ ALF)) as anchor/reference. Test results shall be reported by comparing the corresponding anchor/reference. (“Anchor” is defined in the common test condition, and “reference” is not defined in the common test condition, but will be used to obtain BD-rate gain/loss and runtime difference.) In addition, 64x64 LCU size shall be tested, and testing other LCU size(s) are recommended.

For picture-based encoding, both SAO and ALF shall use picture-based encoding. For LCU-based encoding, both SAO and ALF shall use LCU-based encoding.

## Evaluation of CE Results

Results of the CE will be evaluated on the basis of BD-measures, computational and implementation complexity and/or functionality.

BD-rate gain/loss and runtime numbers shall be provided according to JCTVC-H1100.

For the complexity measurement, it is recommended that the HM 6.0 software with the anchor configuration and the proposal implemented on the HM 6.0 software will be executed on the same (or identically configured) machine(s), and the computational time will be measured for each test sequence and test case. Relative computational time is calculated against the anchor. Furthermore, if a proposal modifies complexity assessment criteria from CD such as number of pixels to be used for ALF filter classification and area for the classification, those numbers shall be reported in comparison with CD. If a proposal will claim the benefit (and assumption) that cannot be assessed by the above metrics, it shall be explained in the contribution document so that all committee members can study in advance to the discussion during the Geneva meeting.

In addition, subjective quality evaluation will be performed.

It is requested that every proposal shall avoid using the bottom four luma pixel rows, the bottom two chroma pixel rows, the rightmost five luma pixel columns, and the rightmost three chroma pixel columns in each LCU during parameter estimation on the encoder side for LCU-based optimization (i.e. slice header or interleaving mode). This encoder restriction applies to both anchor/reference and test. A patch file contained in the zip file shall be used.

# Subtest a: Signalling mode change from "switch between APS mode and slice header mode" to "switch between APS mode and interleaving mode"

## Functionality Addressed

- Complexity reduction of decoder and encoder

- Appropriate signalling scheme for ALF coefficients and control flag

## Expected Gains

- Determine appropriate signalling scheme for ALF coefficients and control flag, possibly converge into a single signaling scheme

## Evaluation Criteria

- Coding efficiency: BD-rate

- Complexity – required buffer to store ALF coefficients and control flag

## Tool Description

### CE2.a.1 Signalling mode change from "switch between APS mode and slice header mode" to "switch between APS mode and interleaving mode" (JCTVC-H0066/H0274, Qualcomm, MediaTek, Toshiba, Cisco)

To study the “real” LCU-based signalling mode based on the syntax in JCTVC-H0066. This test will not modify the signal processing part in WD6/HM6, and will modify only signalling position of ALF coefficients and control flag to be interleaved in each LCU. The interleaved information will be coded by CABAC. In summary new signaling scheme is as follows.

SPS: Transmit ALF Mode flag

If(ALF Mode == FRAME based ALF)

Same as HM6.0

else (ALF Mode == LCU based ALF)

APS: Transmit nothing.

Slice header: Transmit nothing.

LCU: Use CABAC. Interleave following with LCU data

alf_lcu_enable_flag

alf_new_filter_set_flag

alf_stored_filter_set_idx or one ALF filter set (up to 16 luma filters + 2 chroma filters)

This test will use LCU-based encoding, however, coding results will also be compared with picture-based encoding in WD6/HM6. Also two options will be tested for LCU-based encoding: allowing multiple filters per LCU (up to 16 as in HM6.0) vs. forcing 1 filter per LCU. Even though current HM6.0’s RDO mostly chooses to use 1 filter per LCU, in order to see whether there is any difference in performance between current RDO based filter selection vs. forcing 1 filter per LCU.

Software will be provided by Qualcomm.

This test will use LCU-based 10-bit encoding for all coding structure. Additionally, 8-bit encoding results are recommended.

|  | Tester 1 | Tester 2 |
| --- | --- | --- |
| LCU-based signalling | Qualcomm | Broadcom |

### CE2.a.2 Additional signalling of picture-level ALF coefficients on top of CE2.a.1 (JCTVC-H0066/H0274/JCTVC-C082, Toshiba, MediaTek, Qualcomm)

To study the coding efficiency improvement by additional signalling of picture-level ALF coefficients on top of CE2.a.1. The picture-level ALF coefficients will be derived by the previous pictures in the same manner as in JCTVC-C082 in order to avoid additional frame memory scan to design ALF coefficients for the picture and provide low latency capability, where the coefficients are expected to be sub-optimal coefficients for the current picture. In each LCU, ALF control flag is signalled. If the flag is on, ALF coefficients that have already been signalled may be re-used. Otherwise, ALF coefficients will be newly signalled for the LCU.

Three options will be tested for LCU-based encoding: (1) allowing multiple filters per APS and LCU (up to 16 as in HM6.0), (2) allowing multiple filters per APS and one filter per LCU, and (3) allowing one filter per APS and LCU.

Software will be provided by Toshiba.

This test will use LCU-based 10-bit encoding for all coding structure. Additionally, 8-bit encoding results are recommended.

|  | Tester 1 | Tester 2 |
| --- | --- | --- |
| CE2.a.1 + APS signalling | Toshiba | TI |

## Participants

| Participant | Contact |
| --- | --- |
| Arild Fuldseth | arilfuld@cisco.com |
| Tomoo Yamakage | tomoo.yamakage@toshiba.co.jp |
| In Suk Chong | ichong@qualcomm.com |
| Yu-Wen Huang | yuwen.huang@mediatek.com |
| Minhua Zhou | zhou@ti.com |
| Wade Wan | wwan@broadcom.com |

# Subtest b: Simplification of ALF signalling

## Functionality Addressed

- Simplification of ALF signalling such as run prediction and coefficient prediction

## Expected Gains

- Simplification in syntax/signalling

## Evaluation Criteria

- Coding efficiency: BD-rate

## Tool Description

### CE2.b.1 Remove run prediction for APS mode (JCTVC-H0066/H0274,  MediaTek)

To study the coding efficiency loss when picture-based encoder does not use run prediction.

Software will be provided by MediaTek.

This test will use picture-based 10-bit encoding for all coding structure. Additionally, 8-bit encoding results are recommended.

|  | Tester 1 | Tester 2 |
| --- | --- | --- |
| Remove run prediction | MediaTek | Cisco |

### CE2.b.2 Removal of explicit signaling of K-table JCTVC-H0274 (Qualcomm, MediaTek, Toshiba)

To study the removal of explicit signaling of K-table (i.e., trying to test removal of alf_min_kstart_minus1 && alf_golomb_index_flag).

Software will be provided by Qualcomm.

This test will use picture-based 10-bit encoding and LCU-based 10-bit encoding for all coding structure. Additionally, 8-bit encoding results are recommended.

|  | Tester 1 | Tester 2 |
| --- | --- | --- |
| Removal of explicit signalling of K-table | Qualcomm | Cisco |

### CE2.b.3 Necessity of alf_pred_flag (Toshiba, Nokia)

To study the necessity of alf_pred_flag. This test will compare the current design with restricting the value of alf_pred_flag to 0 or 1 (alf_pred_flag will not be signalled).

Software will be provided by Toshiba.

This test will use picture-based 10-bit encoding and LCU-based 10-bit encoding for all coding structure. Additionally, 8-bit encoding results are recommended.

|  | Tester 1 | Tester 2 |
| --- | --- | --- |
| Necessity of alf_pred_flag | Toshiba | Nokia |

### CE2.b.4 Necessity of alf_nb_pred_luma_flag (Toshiba, Nokia)

To study the necessity of alf_nb_pred_luma_flag. This test will compare the current design with restricting the value of alf_nb_pred_luma_flag to 0 or 1 (alf_nb_pred_luma_flag will not be signalled).

Software will be provided by Toshiba.

This test will use picture-based 10-bit encoding and LCU-based 10-bit encoding for all coding structure. Additionally, 8-bit encoding results are recommended.

|  | Tester 1 | Tester 2 |
| --- | --- | --- |
| Necessity of alf_nb_pred_luma_flag | Toshiba | Nokia |

### CE2.b.5 Qp dependent quantization of ALF coefficients JCTVC-H0274/H0066/HM5 (Toshiba, Broadcom)

To study the necessity of Qp dependent quantization of ALF coefficients introduced in WD6/HM6. Instead of using Qp dependent quantization of ALF coefficients, HM5-like quantization is tested.

Software will be provided by Toshiba.

This test will use picture-based 10-bit encoding and LCU-based 10-bit encoding for all coding structure. Additionally, 8-bit encoding results are recommended.

|  | Tester 1 | Tester 2 |
| --- | --- | --- |
| Necessity of alf_nb_pred_luma_flag | Toshiba | Broadcom |

## Participants

| Participant | Contact |
| --- | --- |
| Yu-Wen Huang | yuwen.huang@mediatek.com |
| Arild Fuldseth | arilfuld@cisco.com |
| In Suk Chong | ichong@qualcomm.com |
| Tomoo Yamakage | tomoo.yamakage@toshiba.co.jp |
| Kemal Ugur | Kemal.Ugur@nokia.com |
| Wade Wan | wwan@broadcom.com |

# Subtest c: Number of classes in BA mode

## Functionality Addressed

- Confirm an appropriate number of BA filter classes, including no BA mode

## Expected Gains

- The best trade-off between complexity and coding efficiency/subjective picture quality

## Evaluation Criteria

- Coding efficiency: BD-rate

- Encoding and decoding time

- Subjective picture quality

- Required memory size to store ALF coefficients

## Tool Description

### CE2.c.1 Reduction of number of filter classes in BA mode, including removal of BA mode (JCTVC-H0066, JCTV-H0415/H0409, Cisco, Samsung/Sony)

To study an appropriate number of filter classes, including removal of BA mode, considering the trade-off between complexity and objective/subjective gain.

Current WD6 includes 16 filter classes with classified by BADIR feature. This subtest investigates reduction of number of filter classes from 10, 7, 4, or 1.

In addition, reduction of number of operations for BADIR calculation is investigated based on the description in JCTVC-H0415.

Software will be provided by Qualcomm.

BA computation follows HM6, only the maximum number of filter sets is limited.

This test will use picture-based 10-bit encoding for all coding structure. Additionally, 8-bit encoding results are recommended.

| Number of filter classes | Tester 1 | Tester 2 |
| --- | --- | --- |
| 10 | Toshiba | Sony |
| 7 | Sony | Sharp |
| 4 | Samsung | Cisco |
| 1 | Cisco | Samsung |

Software will be provided by Samsung.

BA computation introduces H0415 and the maximum number of filter sets is limited.

This test will use picture-based 10-bit encoding for all coding structure. Additionally, 8-bit encoding results are recommended.

| Number of filter classes | Tester 1 | Tester 2 |
| --- | --- | --- |
| 7, modify BA calculation | Samsung | Sharp |
| 4, modify BA calculation | Samsung | Qualcomm |

### CE2.c.2 Reduction of number of filter classes in BA mode with having flexible directional features (JCTVC-H0094, NTT)

To study an appropriate number of filter classes with having flexible directional features as described in JCTVC-H0094, considering the trade-off between complexity and objective/subjective gain.

Software will be provided by NTT.

This test will use picture-based encoding for all coding structure.

|  | Tester 1 | Tester 2 |
| --- | --- | --- |
| H0094 | NTT | Mitsubishi |

## Participants

| Participant | Contact |
| --- | --- |
| Arild Fuldseth | arilfuld@cisco.com |
| PoLin Lai | wlai@sta.samsung.com |
| Ali Tabatabai | Ali.Tabatabai@am.sony.com |
| Masaaki Matsumura | matsumura.masaaki@lab.ntt.co.jp |
| Tomoo Yamakage | tomoo.yamakage@toshiba.co.jp |
| Tomohiro Ikai | ikai.tomohiro@sharp.co.jp |
| Kazuo Sugimoto | Sugimoto.Kazuo@ak.MitsubishiElectric.co.jp |

# Subtest d: Number of regions in RA mode

## Functionality Addressed

- Effectiveness and necessity of RA mode in picture-based encoding

## Expected Gains

- The best trade-off between complexity and coding efficiency/subjective picture quality by comparing RA and BA results

## Evaluation Criteria

- Coding efficiency: BD-rate

- Encoding and decoding time

- Subjective picture quality

- Required memory size to store ALF coefficients

## Tool Description

### CE2.d.1 Number of regions in RA mode (JCTVC-H0274/HM5, Toshiba, MediaTek, Qualcomm, TI, Broadcom)

To study the effectiveness of RA mode by dividing a picture up to 1/16 size or 1/64 size. This test will use picture-based encoding. Coding results will be compared with the anchor that adopts BA mode.

Software will be provided by Toshiba.

This test will use picture-based 10-bit encoding for all coding structure. Additionally, 8-bit encoding results are recommended.

| RA minimum region | Tester 1 | Tester 2 |
| --- | --- | --- |
| 1/16 of picture size | Toshiba | TI |
| 1/64 of picture size | Toshiba | Broadcom |

## Participants

| Participant | Contact |
| --- | --- |
| Tomoo Yamakage | tomoo.yamakage@toshiba.co.jp |
| In Suk Chong | ichong@qualcomm.com |
| Yu-Wen Huang | yuwen.huang@mediatek.com |
| Minhua Zhou | zhou@ti.com |
| Wade Wan | wwan@broadcom.com |

# Subtest e: Depth of LCU division for filtering control

## Functionality Addressed

- Confirm an appropriate depth of LCU division for filtering control, including only LCU based filtering control

## Expected Gains

- The best trade-off between complexity and coding efficiency/subjective picture quality

## Evaluation Criteria

- Coding efficiency: BD-rate

- Required memory size

## Tool Description

### CE2.e.1 Reduction of depth of LCU division for filtering control, including only LCU based filtering control (JCTVC-H0066, Cisco, Samsung/Qualcomm/MediaTek/Toshiba)

To study an appropriate depth divide the depth of LCU division for filtering controlling, including only LCU based filtering control.

Current WD6 includes LCU division for filtering control up to xx depth. This subtest investigates reduction of max depth of 2 or 0.

Software will be provided by Samsung (or, we only need to modify config file).

This test will use picture-based 10-bit encoding for all coding structure. Additionally, 8-bit encoding results are recommended.

| Maximum depth of LCU division | Tester 1 | Tester 2 |
| --- | --- | --- |
| 2 | Samsung | Toshiba |
| 0 | Cisco | Qualcomm |

## Participants

| Participant | Contact |
| --- | --- |
| Arild Fuldseth | arilfuld@cisco.com |
| PoLin Lai | wlai@sta.samsung.com |
| Tomoo Yamakage | tomoo.yamakage@toshiba.co.jp |
| In Suk Chong | ichong@qualcomm.com |
| Yu-Wen Huang | yuwen.huang@mediatek.com |

# Subtest f: Line memory reduction modification at independent slice and tile boundaries

## Functionality Addressed

- Solve subjective picture quality degradation caused by double padding processing at independent slice and tile boundaries

## Expected Gains

- Subjective picture quality improvement

## Evaluation Criteria

- Coding efficiency: BD-rate

- Required memory size

## Tool Description

### CE2.f.1 Line memory reduction modification at independent slice and tile boundaries (Panasonic, JCTVC-H0372)

In the current HM Virtual Boundary (VB) padding operation is applied at the bottom of each LCU in order to eliminate the line memory requirement for ALF. The current contribution is intended to improve the line memory reduction method for ALF by switching VB padding off at tile and independent slice boundaries.

Figure below describes the ALF operation at tile or independent slice boundaries. VB padding and regular boundary padding are applied in the same LCU erroneously causing double padding at tile/slice boundaries.

It is proposed to switch VB padding off at tile and independent slice boundaries (where no line memory is required) in order to avoid visual artifacts and coding loss. Additional control information might be required to be signalled in the bitstream.

Figure 1: VB padding and slice boundary padding at independent slice boundaries

Software will be provided by Panasonic.

Two tests are going to be performed. Tickets #299 and #400 are going to be implemented.

Test 1 (1500 Byte slices operation mode)

- Deblocking Filter and SAO will be enabled and ALF will be disabled across slices.

Test 2 (Multiple tiles per frame operation mode):

- 8 tiles per frame (for class D sequences) and 16 tiles per frame (for classes A, B, C, E and F) are going to be used.

- Deblocking Filter and SAO will be enabled and ALF will be disabled across tiles.

- Picture-based ALF encoding is going to be used for all test cases (since LCU-based ALF reportedly has a bug in multiple tiles mode).

This test will use picture-based 10-bit encoding and LCU-based 10-bit  for all coding structure. Additionally, 8-bit encoding results are recommended.

|  | Tester 1 | Tester 2 |
| --- | --- | --- |
| H0372 | Panasonic | TI |

## Participants

| Participant | Contact |
| --- | --- |
| Semih Esenlik | Semih.Esenlik@eu.panasonic.com |
| Yu-Wen Huang | yuwen.huang@mediatek.com |

# Subtest g: Encoder complexity reduction in LCU-based encoding

## Functionality Addressed

- Reduction of complexity in LCU-based encoding

## Expected Gains

- Obtain the coding efficiency loss when disallowing to use the last four pixel rows and the rightmost four pixel columns in each LCU during parameter estimation for the slice header mode

## Evaluation Criteria

- Coding efficiency: BD-rate

## Tool Description

### CE2.g.1 Disallow using the last four pixel rows and the rightmost four pixel columns (JCTVC-H0274, MediaTek)

To study the coding efficiency loss when LCU-based encoder disallows using the bottom four luma pixel rows, the rightmost four luma pixel columns, the bottom two chroma pixel rows, and the rightmost two chroma pixel columns in each LCU during parameter estimation for the slice header mode (LCU-based optimization).

Software will be provided by MediaTek.

This test will use picture-based 10-bit encoding and LCU-based 10-bit encoding for all coding structure. Additionally, 8-bit encoding results are recommended.

|  | Tester 1 | Tester 2 |
| --- | --- | --- |
| H0274 | MediaTek | Qualcomm |

## Participants

| Participant | Contact |
| --- | --- |
| In Suk Chong | ichong@qualcomm.com |
| Yu-Wen Huang | yuwen.huang@mediatek.com |

# Subtest h: Reuse luma filter coefficients in chroma filtering for the slice header mode

## Functionality Addressed

- Overhead reduction to signal ALF coefficients in LCU-based encoding

## Expected Gains

- Improve coding efficiency

## Evaluation Criteria

- Coding efficiency: BD-rate

## Tool Description

### CE2.h.1 Reuse luma filter coefficients in chroma filtering for the slice header mode (JCTVC-H0066, Cisco, MediaTek)

To study the coding efficiency gain when LCU-based encoder reuses luma filter coefficients to chroma as in JCTVC-H0066.

Software will be provided by MediaTek.

This test will use LCU-based 10-bit encoding for all coding structure. Additionally, 8-bit encoding results are recommended.

|  | Tester 1 | Tester 2 |
| --- | --- | --- |
| H0066 | MediaTek | Cisco |

## Participants

| Participant | Contact |
| --- | --- |
| Yu-Wen Huang | yuwen.huang@mediatek.com |
| Arild Fuldseth | arilfuld@cisco.com |

# Subtest i: Combination test of all subtests about ALF (ALF)

## Functionality Addressed

- Confirm overall performance of combination of tools in subtests 8.b to 8.d

## Expected Gains

- Better understanding of the integrated results

## Evaluation Criteria

- Coding efficiency: BD-rate

- Subjective picture quality

## Tool Description

### CE2.i.1 Combination test of CE2.a.1, CE2.b.1-4, CE2.c.1, CE2.e.1 and CE2.h.1 (JCTVC-H0066, Cisco)

All signalling and signal processing modification in JCTVC-H0066 are tested. These modifications include combination of CE2.a.1, CE2.b.1-4, CE2.c.1, CE2.e.1 and CE2.h.1 conceptually.

The software will be provided by Cisco based on JCTVC-H0066 code.

This test will use LCU-based 10-bit encoding for all coding structure. Additionally, 8-bit encoding results are recommended.

| Combination | Tester 1 | Tester 2 |
| --- | --- | --- |
| 1 | Cisco | Toshiba |

## Participants

| Participant | Contact |
| --- | --- |
| Arild Fuldseth | arilfuld@cisco.com |
| Tomoo Yamakage | tomoo.yamakage@toshiba.co.jp |

# Time-line and Responsibilities

T1:		2012-Mar-2:	Finalize CE description and upload two weeks after the meeting

T2:		2012-Mar-6:	HM 6.0 available (expected 3 weeks after the meeting)

(First Stage CE)

T3:		2012-Mar–27:	Software available to all CE participants and start cross-checking (three weeks after the release of HM 6.0 software) (Except CE2.f.1)

T4:		2012-Apr-3:	Report proponents’ results to Coordinators (one week after T3) (Except CE2.f.1)

T3.1:		2012-Mar–30:	Software available to all CE participants and start cross-checking (three weeks after the release of HM 6.0 software) (Only CE2.f.1)

T4.1:		2012-Apr-6:	Report proponents’ results to Coordinators (one week after T3) (Only CE2.f.1)

(Second Stage CE)

T5:		2012-Apr-14:	Contribution documents upload, and report the document number and title to coordinators

T6: 	2012-Apr-21:	Summary report upload

Note: Time-line may be changed to reflect the release date of HM 6.0.

# References

[1]	“Common HM test conditions and software reference configurations”, JCTVC-H1100, San Jose, CA, US, February 2012.

[2]	“High Efficiency Video Coding (HEVC) Test Model 5 (HM 6) Encoder Description”, JCTVC-H1002, San Jose, CA, US, February 2012.

[3]	“High Efficiency Video Coding (HEVC) text specification draft 6”, JCTVC-H1003, San Jose, CA, US, February 2012.