| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, USA, 1-10 February, 2012 | Document: JCTVC-H1103 |
| --- | --- |

| Title: | Description of Core Experiment (CE3): Coefficient scanning and coding |  |  |
| --- | --- | --- | --- |
| Status: | Output document |  |  |
| Purpose: | Core Experiment Description |  |  |
| Author(s) or Contact(s): | Vivienne Sze (primary) Tung Nguyen Yinji Piao Joel Sole Krit Panusopone | Tel: Email: Tel: Email: Tel: Email: Tel: Email: Tel: Email: | +1-214-567-4005 sze@ti.com +49-30-31002-230 tung.nguyen@hhi.fraunhofer.de +82-10-6296-4261. yj1003.piao@samsung.com +1-858-845-2429 joels@qualcomm.com +1-858-404-3618 krit@motorola.com |
| Source: | CE coordinators |  |  |

_____________________________

# Abstract

This document defines Core Experiment 3 (CE3) on coefficient scanning and coding, to be performed for the 9th JCT-VC meeting. The CE consists of proposals related modifications to significance map and coefficient level coding to increase throughput and/or improve coding efficiency.

# Introduction

Various methods have been proposed for coding the transform coefficients of the residual signal to reduce complexity, increase throughput and/or improve the coding efficiency. This core experiment evaluates the impact of:

- context modeling/selection for syntax elements related to transform coefficients

- scanning of coefficients

The objectives of this CE include:

- Investigation of the impact of context selection and signalling schemes for transform coefficient coding on the coding efficiency and complexity

- Coding efficiency and complexity measurement of transform coefficient scanning order methods

Three subtests are defined in this core experiment.

  - Subtest 1 will investigate parallel context derivation for significance map (significant_coeff_flag) to enable 3 and 4 bin parallelism.

  - Subtest 2 will investigate parallel context derivation of coefficient level (coeff_abs_level_greater1_flag).

  - Subtest 3 will investigate transform coefficient coding as presented in H0228.

Complexity measures are described in Section 7.2 will be reported for all tools.

# Participants List

| Company | Participants | P | C |
| --- | --- | --- | --- |
| TI | Vivienne Sze, sze@ti.com Minhua Zhou, zhou@ti.com | X |  |
| Qualcomm | Joel Sole, joels@qualcomm.com Wei-Jung Chien, wchien@qualcomm.com Jianle Chen, cjianle@qualcomm.com Vadim Seregin, vseregin@qualcomm.com Rajan Joshi, rajanj@qualcomm.com Marta Karczewicz, martak@qualcomm.com | X |  |
| Bejing Univ. Tech | Wenpeng Ding, wpding@gmail.com |  |  |
| Microsoft | Jizheng Xu, jzxu@microsoft.com |  |  |
| Motorola Mobility | Jian Lou, lou@motorola.com Shuy Fang, x.fang@motorola.com Yue Yu, yyu@motorola.com Wei-Ying Kung, Vivian.Kung@motorola.com | X |  |
| Huawei | Jin Song, andy.songjin@huawei.com Xiaozhen Zheng, zhengxiaozhen@huawei.com Haoping Yu, haoping.yu@huawei.com |  |  |
| Canon | Chris Rosewarne, Chris.Rosewarne@cisra.canon.com.au |  |  |
| MediaTek | Xun Guo, xun.guo@mediatek.com Shih-Ta Hsiang, shih-ta.hsiang@mediatek.com Chih-Wei Hsu, cw.hsu@mediatek.com T-D. Chuang, peter.chuang@mediatek.com YW Huang, yuwen.huang@mediatek.com, Shawmin Lei, shawmin.lei@mediatek.com | X |  |
| Samsung | Ankur Saxena, asaxena@sta.samsung.com Yinji Piao, yj1003.piao@samsung.com Zhan Ma, zhan.ma@sta.samsung.com |  |  |
| Mitsubishi | Kazuo Sugimoto, Sugimoto.Kazuo@ak.MitsubishiElectric.co.jp Akira Minezawa, Minezawa.Akira@ds.MitsubishiElectric.co.jp | X |  |
| I2R | Yeo Chuo Hao, chyeo@i2r.a-star.edu.sg |  | X |
| Nokia | Jewon Kang, ext-jewon.kang@nokia.com |  |  |
| Orange | Felix Henry, felix.henry@orange.com Joel Jung, joelb.jung@orange.com Gordon Clare, gordon.clare@orange.com |  |  |
| Panasonic | Hisao Sasai, sasai.hisao@jp.panasonic.com Terada Kengo, terada.kengo@jp.panasonic.com |  |  |
| Sony | Cheung Ayeung, Cheung.Auyeung@am.sony.com Jun Xu, Jun.Xu@am.sony.com |  |  |
| LGE | Jungsun Kim, jungsun.kim@lge.com Seungwook Park, Seungwook.park@lge.com |  |  |
| RIM | Gergely Korodi, gkorodi@rim.com Dake He, dhe@rim.com Gaëlle Martin-Cocher, gmartincocher@rim.com Tianying Ji, tiji@rim.com Nguyen Nguyen, nnguyen@rim.com Jing Wang, wangj@rim.com |  | X |
| Cisco | Thomas Davies, thdavies@cisco.com |  |  |
| Sharp | Tomohiro Ikai, ikai.tomohiro@sharp.co.jp Tsukuba Takeshi, tsukuba.takeshi@sharp.co.jp Yamamoto Tomoyuki, yamamoto.tomoyuki@sharp.co.jp Seung-Hwan Kim, kimse@sharplabs.com Andrew Seagall, asegall@sharplabs.com Louis Kerofsky, lkerofsky@sharplabs.com Kiram Misra misrak@sharplabs.com |  |  |
| NEC | Keiichi Chono, chono@ct.jp.nec.com |  |  |
| BBC | Marta Mrak, Marta.Mrak@bbc.co.uk |  |  |
| Ericsson | Andrey Norkin, andrey.norkin@ericsson.com |  |  |
| HHI | Tung Nguyen, tung.nguyen@hhi.fraunhofer.de Heiner Kirchoffer, heiner.kirchhoffer@hhi.fraunhofer.de | X |  |
| JVC Kenwood | Shigeru Fukushima fukushima.shigeru@jvckenwood.com Toru Kumakura kumakura.toru@jvckenwood.com |  |  |

# Tools under Test

Summary of tools being considered (Note: Tools in throughput and parallelization should report complexity measures describe in Section 7.2)

| Topic | Proponent |  | Doc # |  | Tool description |  | Cross-checker |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Throughput and Parallelization (Subtest 1) | Mitsubishi |  | H0253 |  | Support modification for 4-bin parallel calculation of significant map contexts (This will be tested as a part of sub test 1.3.) |  | N/A |  |
| Throughput and Parallelization (Subtest 1) | MediaTek |  | H0286 |  | Parallel context assignment for significance map coding |  | Canon, Samsung, Qualcomm |  |
| Throughput and Parallelization (Subtest 1) | TI |  | H0352 |  | Parallel context selection for significance map |  | Qualcomm |  |
| Throughput and Parallelization (Subtest 1) | Qualcomm |  | H0427 |  | Support modification for parallel calculation of significant map contexts |  | I2R (Method2 – 4 bins) |  |
| Throughput and Parallelization (Subtest 2) | Qualcomm |  | H0550 |  | Context assignment for parallel coefficient level coding |  | TI |  |
| Coding Efficiency (Subtest 3) | HHI |  | H0228 |  | This experiment tests the performance of the integration of H0228 in HM6.0. Initialization values of not modified syntax elements will not be modified. |  | I2R, Samsung |  |
| Throughput (Subtest 3) | HHI |  | H0228 |  | This experiment investigates the impact on the proposed scheme when a separation into several scan passes is made according to HM6 and therefore the case when the complete information on the transform coefficients is not available. This investigation includes further adjustment to the context modeling stage and the Rice parameter derivation. Separated results will be provided for: - the regular bins part modifications and for the bypass bins modifications - Using only the significance map of H0228 in HM6 - Using H0228 with the rice Golomb of HM6 |  | Qualcomm |  |
| Throughput and Parallelization (Subtest 3) | HHI |  | H0228 |  | In this experiment, the HM 6 template is applied and is adjusted in case of horizontal and vertical scan orders. A first set of results will be provided for that aspect. This investigation includesfurther adjustment to the context modeling stage and the Rice parameter derivation. |  | RIM |  |
| Coding Efficiency (Subtest 3) | HHI |  | H0228 |  | In this experiment, investigation the impact of using context models for all transform block sizes or shapes is investigated. Goal of this investigation is to find a trade-off on sharing the context models along different block sizes and shapes. This investigation includes further adjustment to the context modeling stage and the Rice parameter derivation. |  | Samsung |  |

## Contribution and Experiment Description

## Subtest 1: Parallel Context Derivation for Significance Map

### JCTVC-H0253: “Improvement on parallelism for context derivation process of significant_coeff_flag”

JCTVC-H0253: This sub test investigates 4-bin parallel context derivation for significance map coding. In HM6.0, two different supports are used to enable 2-bin parallel context derivation. The proposed scheme modifies the supports for 4-bin parallel processing. This scheme is exactly the same technique as “Method2 4bins” of JCTVC-H0427.

### JCTVC-H0286: “Parallel context assignment for significance map coding”

JCTVC-H0286: In this sub test, a 3-bin parallel and a 4-bin parallel context formation patterns will be tested. For each sub-block, only one context formation pattern will be used. No additional condition/position check is required in a sub-block. The 3-bin parallel context formation pattern removes the dependency of two previously decoded bins while the 4-bin parallel context formation pattern removes the dependency of three previously decoded bins. The context initial values will be adjusted for less bit rate loss. The complexity of proposed method will be evaluated.

### JCTVC-H0352: “Parallel context selection for significance map”

JCTVC-H0352: This sub test investigates removing neighboring dependencies for context selection of significant_coeff_flag in order to enable parallel bin processing.  Specifically, dependency on recently processed bins is removed to enable 3-bin parallel and 4-bin parallel context selection. This dependency occurs particularly near the corners of the sub-block.  Context index calculation may be modified to account for removed neighbors and coding efficiency.  Additional dependencies may be removed to enable further simplification of context selection logic. The context initial values may be adjusted to account for change in context selection. Harmonization with the newly adopted tools in HM-6.0 might be necessary.

### JCTVC-H0427: “Support modification for parallel calculation of significant map contexts”

Significance map context calculation is dependent on the number of non-zero coefficients in a neighbourhood support. In HM6.0, there is no dependency of the context with respect to the previous coefficient, enabling 2-bin parallel processing. In this experiment, the 3 methods of JCTVC-H0427 to enable 3 and 4 bin parallelization are analyzed. Method 1 extends the current method in HM6.0 to 3 and 4 bins. Method 2 groups coefficients to allow for parallel processing of each group. Finally, method 3 modifies the neighborhood support removing the shorter path dependencies. The context initial values could be adjusted in this test.

## Subtest 2: Parallel Context Derivation for Coefficient Level

    - JCTVC-H0550: “Context assignment for parallel coefficient level coding”

JCTVC-H0550 proposes to remove the dependency of previously coded value of coeff_abs_level_greater1_flag for context modeling. Instead, the number of previously coded coeff_abs_level_greater1_flag is used. The number of contexts is modified.

## Subtest 3: Transform Coefficient Coding in H0228

### JCTVC-H0228: “Proposed Cleanup for Transform Coefficient Coding”

H0228 proposes to modify the context modeling for the coding of transform coefficients. In particular, the coefficient levels are coded completely instead of separating into several bin planes as currently done in the design of HEVC. This modification allows the evaluation of the absolute levels covered by the local template. Hence, all context model indices and the Rice parameter can be derived from the evaluation of the local template. This concept is used for all transform block sizes, and no distinction or additional offset for different transform block size or shape is made.

# Feasibility Study

This section contains tools that have requested to be considered for CE (but not in meeting notes).  Their inclusion into the CE report will depend on software availability and results.

| Topic | Proponent | Doc # | Tool description | Cross-checker |
| --- | --- | --- | --- | --- |
| Throughput | Qualcomm | H0554 | H0554 that improves throughput has been adopted in HM6.0. H0228 has impact on the worst-case number of coded regular bins. This experiment investigates this impact on the worst-case throughput of H0228 when harmonized with H0554 scheme. |  |
| Throughput and Parallelization | Qualcomm | H0427 | In this experiment, the template that is defined in the WD for 2 bins parallelization and the methods defined in H0427 for 3 and 4 bins are applied to H0228 to improve parallelization. |  |
| Throughput | Sharp | H0510 | In this experiment, combination of H0510 with H0288 is investigated. The experiments will measure the improvement in throughput efficiency and coding efficiency performance. |  |

### JCTVC-H0554: “Throughput improvement on  coefficient level coding”

H0554 proposes to switch off context-coded coeff_abs_level_greater1_flag and coeff_abs_level_greater2_flag for partial coefficients in a chunk, and it was adopted in HM6.0. This experiment will test the combination of the H0554 methods and other CE tests which can have impact on throughput, such as H0228. The experiment will also test the case of using separated scan passes instead of coefficient-by-coefficient coding in H0228 method.

### JCTVC-H0510: “High Throughput Binarization (HTB) method with modified level coding”

H0510 proposes to improve CABAC throughput by selectively employing the high throughput mode for level coding based on the significance map information. The experiment investigates combination of H0510 with H0288.  The test will measure the improvement in throughput efficiency and coding efficiency performance.

- Software and Configuration

## Software

The CE will use the HM-6.0 software that is recommended in JCTVC-H1100. Proponents are requested to provide software that can be compiled under Windows and Linux platforms. Also, the proponents are requested to provide software that fulfils the HEVC software guidelines (JCTVC-H1001).

## Test Sequences, Bit Rates and Coding Conditions

The CE will use the test sequences, configuration and conditions that are recommended by the Software Ad Hoc group.

# Timeline and Responsibilities

HM-6.0 software available 3 weeks after the meeting (March 2, 2012)

| T1 | March 2, 2012 (three weeks after meeting) | Final CE-description |
| --- | --- | --- |
| T2 | One week after HM-6.0 software release (March 9, 2012) | Completion of original H0228 integration; release software. |
| T3 | Two weeks after HM-6.0 software release (March 16 , 2012) | Completion of tool integration; release software & results for cross-check |
| T4 | Two weeks before start of meeting (April 16, 2012) | Complete cross-check; input document upload |

# Definition of Performance Measurement Criteria

The performance measurements are evaluated by switching on and off individual tools to identify their relative performance. The following measurements are considered to be used in this core experiment.

## Coding Performance Measurements

The objective rate-distortion measurements using 4-point BD-PSNR and BD-Rate for all tools in section 3 are used under common conditions. All tools should also be tested with low QP (12, 17, 22, 27) based on the recommendation of the BoG on context reduction at 6th JCT-VC Meeting in Turin (F746).

## Complexity Measurements

For the complexity measurement, the anchor, the reference software and the reference software with the proposed method implemented will be executed on the same machine and the computational time will be measured. A time ratio will then be calculated between the reference software and the reference software with the proposed method implemented.

However, the complexity analysis does not solely rely on the encoding and decoding time measurements of a particular implementation (reference software) of the working draft (e.g. should describe need for LUT, additional logic, memory access, memory storage, etc.)

Core experiment participants who investigate parallelization aspects are requested to provide a description showing how the parallelization is improved and which architecture(s) would see the benefit of the parallelization. Average and worst case throughput gain from parallelized methods should be considered.

Parallelism will be assessed based on the number of context derivations that can be done independently from other bins.  In addition to parallelism, throughput will be assessed based on context selection/binarization logic complexity, number of bypass bins and total number of bins.

Core experiment participants who investigate context reduction and bin reduction aspects are requested to report the number of contexts removed and any changes to context selection.