| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0734 WG11 Document: m23991 |
| --- | --- |

| Title: | Straw man for Profiles and levels |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s) | M. Horowitz (eBrisk Video) A. Ichigaya (NHK) K. McCann (Samsung/ZetaCast) T. Nishi (Panasonic Corporation) S. Sekiguchi (Mitsubishi Electric) T. Suzuki (Sony Corporation) T. K. Tan (NTT DOCOMO) W. Wan (Broadcom) M. Zhou (TI) | Email: | michael@ebriskvideo.com ichigaya.a-go@nhk.or.jp ken@zetacast.com nishi.takahiro@jp.panasonic.com Sekiguchi.Shunichi@eb.MitsubishiElectric.co.jp teruhikos@jp.sony.com ttk@pacific.net.sg wwan@broadcom.com zhou@ti.com |
| Source: | Broadcom, eBrisk Video, Mitsubishi Electric, NHK, NTT DOCOMO, Panasonic, Samsung, Sony, Texas Instruments |  |  |

_____________________________

# Abstract

This document represents a “straw man” that is intended to facilitate the definition of profiles and levels.  It is the result of an analysis of documents relating to profile and level by an informal group of interested participants, following the presentation of the relevant documents during the 8th JCT-VC meeting.

# Profiles and Levels

Profiles and Levels have been used in video coding standards since MPEG-2 to facilitate interoperability by defining specific operating points.  Profiles and Levels are defined as follows in the MPEG document specifying HEVC Requirements [1]:

“A Profile is a set of algorithmic tools, representing a particular trade-off of performance and resource consumption, supporting the requirements of a particular set of applications.  Applications which require similar trade-offs between these parameters should generally use the same Profile.”

“A Level describes performance parameters within each Profile, which describe lower limits on decoder capability.  Performance parameters typically include maximum picture size, macroblocks per second, bitrate, frame rate, buffer sizes and similar parameters.”

# Recommendations of AHG on Profile and Level Definitions

The recommendations of the AHG on Profile and level definitions are:

- The profile architecture for HEVC should follow an “onion ring” approach of strict sub-sets and super-sets, unless there are good reasons to do otherwise

- JCT-VC should seek to define some form of “Main” profile that meets the majority of industry requirements for complexity/compression efficiency trade-off in a single profile

- The inclusion or exclusion of specific coding tools from the “Main” profile should be based on the result of an evaluation in terms of coding efficiency vs. complexity trade-off for the majority of applications

- Such an evaluation process should be based on subjective assessment

- JCT-VC should review all relevant contributions and should seek to define levels of HEVC with a reasonable degree of granularity

# Suggested levels

Initial ideas on a potential set of levels were proposed by JCTVC-H0168 [6] JCTVC-H637 [9] and JCTVC-H0668 [10].

Following an analysis of these documents, the set of levels defined in Table 1 below is suggested to provide a reasonable degree of granularity for applications using 8 bit 4:2:0 video:

| Level | Typical size | Max luma pixel rate (pel/sec) | Max luma frame size (pel) | Max bit rate (1000 bit/sec) | MinCR | Max DPB size (number of Max luma frame size) | Max CPB size (1000 bits) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 720x480@30p, 720x576@25p, 854x480@30p, | 13,762,560 | 458,752 | 5,000 | 2 | 6 | 5,000 |
| 2 | 960x544@60p 1280x720@30p | 33,177,600 | 983,040 | 9,000 | 2 | 6 | 9,000 |
| 3 | 1280x720@60p, 1920x1088@30p, | 62,668,800 | 2,088,960 | 15,000 | 4 | 5 | 15,000 |
| 3H | 1280x720@60p, 1920x1088@30p, | 62,668,800 | 2,088,960 | 30,000 | 4 | 5 | 30,000 |
| 4 | 1920x1088@60p, 2048x1088@60p | 133,693,440 | 2,228,224 | 30,000 | 4 | 5 | 30,000 |
| 4H | 1920x1088@60p, 2048x1088@60p | 133,693,440 | 2,228,224 | 50,000 | 4 | 5 | 50,000 |
| 5 | 3840x2160@30p, 4096x2160@30p | 267,386,880 | 8,912,896 | 50,000 | 8 | 4 | 50,000 |
| 5H | 3840x2160@30p, 4096x2160@30p | 267,386,880 | 8,912,896 | 100,000 | 8 | 4 | 100,000 |
| 6 | 3840x2160@60p, 4096x2160@60p | 534,773,760 | 8,912,896 | 150,000 | 8 | 4 | 150,000 |
| 7 | 7680x4320@30p | 1,002,700,800 | 33,423,360 | 300,000 | 8 | 4 | 300,000 |
| 8 | 7680x4320@60p | 2,005,401,600 | 33,423,360 | 500,000 | 8 | 4 | 500,000 |
| 9 | 7680x4320@120p | 4,010,803,200 | 33,423,360 | 800,000 | 6 | 4 | 800,000 |

Table 1: Suggested levels

Note: An alternative approach to defining 3/3H, 4/4H and 5/5H pairs of levels would be to define some form of constraint flag for bitrate (e.g. in SPS)

# Applications for Initial Profile

The AHG 8 report [4] had included the following suggestion from the AHG chairs on a strategy to identify a potential clustering of coding tools and constraints in terms of coding efficiency vs. complexity trade-off for the majority of applications:

- Identify the set of “mainstream” applications that we seek to cover by the "Main" profile (or whatever it will be called)

- Characterise the requirements of these applications

- Identify a set of tools that could form a profile that would fulfil all these requirements

- Within this set, identify those tools whose performance / complexity trade-off may be borderline for this profile

- Further evaluate those tools identified during the above step

The initial HEVC profile is suggested to be targeted at the following applications, which were listed in the MPEG HEVC Requirements [1]

- Broadcast

- Video On Demand

- Home cinema

- Surveillance

- Realtime communications, video chat, video conferencing

- Telepresence

- Mobile streaming, broadcast and communications

- Storage and playback of video

- Camcorders

- Internet streaming, download and play

- Remote video presentation and remote computer graphics display

The following applications, also listed in the MPEG HEVC Requirements [1], are not specifically targeted by this initial profile:

- Digital Cinema

- 3D video

- Content production and distribution

- Medical imaging

# Performance / Complexity Trade-Off

Contribution JCTVC-H0116 [5] reported objective and subjective performance measures of HM5.0.  The clustering of tools described in Table 2 below is implied by this and other forms of indication of decoder complexity, maturity and performance :

| Tool cluster | Cluster 1: LMChroma | Cluster 2: AMP | Cluster 3: NSQT | Cluster 4: SAO | Cluster 5: ALF |
| --- | --- | --- | --- | --- | --- |
| Impact on decoding time JCTVC-H0116 | Negligible | Negligible | Negligible | Low to medium | High |
| Software implementations JCTVC-H0693 JCTVC-G0757 JCTVC-G0988 | Not implemented | Not implemented Not implemented | Not implemented Not implemented Not implemented | 11% - 13% 5 - 10% 5 - 7% | 17% Not implemented Not implemented |
| Stability indicated by number of input documents | Documents with keyword in title: LM (7) | Documents with keyword in title: AMP (1) | Documents with keyword in title: NSQT (11) | Documents with keyword in title: SAO (25) | Documents with keyword in title: ALF (24) |
| WD Editors’ assessment of WD maturity & completeness | OK except for one small issue | Stable since last meeting | Stable since last meeting | The text quality is not stable (but better than ALF) | The text quality is not stable |
| Software coordinators’ assessment of HM maturity & stability | Stable | Stable | Unstable/buggy but might stabilize at this meeting | Has had stability issues, some of these related to slices/tiles | Similar to SAO but with more issues. Amount of code related to ALF is quite large |
| Objective Gains (BD-rate relative to BD-rate LC) | Mostly chroma only 1.9% to 5.6% | 0.6% to 0.7% | 0.2% to 0.6% | 0.5% to 5.4% | 1.7% to 6.4% |
| Subjective improvements reported in JCTVC-H0116 | Not visible | Not visible | Not visible | Visible on some sequences | Not visible |

Table 2: Clustering of Tools

# Suggested Approach for Profile Creation

We recommend starting the work of actual profile creation from a set of minimum effective tools for applications using 8 bit 4:2:0 video. Based on the evidence shown in this contribution, we suggest that JCT-VC should create an initial profile by starting from LC tools plus SAO and then considering what additional tools it may be beneficial to add to give the optimum trade-off between decoder complexity and compression performance for the target applications.

In addition, consideration should be given to defining an appropriate set of constraints to the use of parallel processing tools (Tiles and/or Wavefronts).

# References

- ISO/IEC JTC1/SC29/WG11/N11872, “Vision, Applications and Requirements for High Efficiency Video Coding (HEVC)”, January 2011

- JCTVC-G0757 [F. Bossen (DOCOMO Innovations)], “On software complexity”, November 2011

- JCTVC-G0988 [K. McCann, J. Choi, J. H. Park (Samsung)], “HEVC software player demonstration on mobile devices”, November 2011

- JCTVC-H0008 [M. Horowitz, K. McCann, T. Suzuki, T. K. Tan, W. Wan, Y. -K. Wang], “JCT-VC AHG report: Profile and level definitions (AHG 8)”, February 2012

- JCTVC-H0116 [TK Tan, A Fujibayashi, Y Suzuki, J Takiue (NTT DOCOMO)] “[AHG8] Objective and subjective evaluation of HM5.0”, February 2012

- JCTVC-H0168 [T. Suzuki (Sony), H. Sasai, T. Nishi (Panasonic)] “AHG8: Proposal of HEVC profile/level definitions”, February 2012

- JCTVC-H0353 [M. Zhou, V. Sze, H. Tamama (TI)] “AHG8: Comments on HEVC version 1 Profile and Level definition”, February 2012

- JCTVC-H0463 [W. Wan, T. Hellman (Broadcom)] “Comments on specifying tile support in HEVC Profiles and Levels”, February 2012

- JCTVC-H0637 [Ajay Luthra (Motorola Mobility)] “Profiles and Levels for HEVC”, February 2012

- JCTVC-H0668 [A. Wells, C. Fogg (Ambarella)] “Profiles and Level considerations”, February 2012

- JCTVC-H0693 [K. Veera, R. Ganguly, B. Zhou, N. Kamath, S. Chowdary, J. Du, I. S. Chong, M. Coban] “A real-time ARM HEVC decoder implementation”, February 2012