| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 |  |  | Document: JCTVC-I0005 |  |
| --- | --- | --- | --- | --- |
| Title: | JCT-VC AHG Report: Unified Coefficient Coding (AHG5) |  |  |  |
| Status: | Input Document |  |  |  |
| Purpose: | Report |  |  |  |
| Author(s) or Contact(s): | Vivienne Sze Cheung Auyeung Gaelle Martin-Cocher Tung Nguyen Joel Sole | Tel: Email: Tel: Email: Tel: Email: Tel: Email: Tel: Email: |  | +1-214-567-4005 sze@ti.com 1-408-352-4725 cheung.auyeung@am.sony.com gmartincocher@rim.com +49-30-31002-230 tung.nguyen@hhi.fraunhofer.de +1-858-845-2429 joels@qualcomm.com |
| Source: | AHG |  |  |  |

_____________________________

# Abstract

This report summarizes the unified coefficient coding ad-hoc activities between the 8th JCT-VC meeting in San Jose, US (1 to 10 February, 2012) and the current 9th JCT-VC meeting in Geneva, CH (27 April to 7 May, 2012).

# Mandates

| AHG Title and Email Reflector | Chairs |
| --- | --- |
| Unified Coefficient Coding (AHG5) (jct-vc@lists.rwth-aachen.de) - Study and investigate unified context selection and coefficient scanning for all TU sizes (including SQT/NSQT). - Work on solutions that would unify coefficient coding for all TU sizes while considering throughput/complexity requirements. - Report the results of these studies, discussions and experiments to the JCT-VC, also in relation with CE3. - Recommend a unified solution if available. | V. Sze (chair), C. Auyeung, G. Martin-Cocher, T. Nguyen, J. Sole (vice-chairs) |

# Summary of Activity

AHG5 used the main JCT-VC reflector: jct-vc@lists.rwth-aachen.de.

A kick-off message was sent out by the chairs on March 4, 2012.  It was encouraged that software be made available early for review.  As of April 24, 2012, there were 18 email exchanges on the reflector relating to the work of AHG5.  A document prepared by J. Sole containing a categorized list of the proposals related to coefficient coding was circulated on the reflector on April 20, 2012 to facilitate discussion; comparison tables were added to this document and are included in this AHG report.

Regarding Mandates 1 and 2

The AHG had an active discussion on the email reflector regarding unification of context assignment table for 4x4 and 8x8 TU for luma and chroma.  There are five proposals on this topic (shown in the table in Section 3.1.1). To compare these proposals, it was suggested by experts to evaluate the coding efficiency, context reduction and complexity of each proposal.

Additional results were requested for I0296 which removes the use of a template from all TU sizes.  These results have been include in a table in Section 3.1.3.

# List of AHG related input documents

List of the 46 contributions related to coefficient coding in this meeting split by topic (cross-checks not included). Proposals with software will be noted with “SW” and proposals with text will be noted with “TEXT”.  Tables with coding efficiency results are also provided for proposals in each section.  Unless otherwise noted, the coding efficiency is measured under common conditions.

Significance Coding (23)

### Context tables for 4×4 and 8×8 (5)

- JCTVC-I0031, AHG5: Coefficient coding – significance map clean-up, G. Korodi, D. He, G. Martin-Cocher (RIM) [SW & TEXT]

- JCTVC-I0155, AHG5: Unified context selection for significance map coding of 4x4 and 8x8 Tus, C.-W. Hsu, T.-D. Chuang, Y.-W. Huang, S. Lei (MediaTek) [TEXT]

  - JCTVC-I0368, AHG5: Cross-check on Method 2 of Unified context selection for significance map coding of 4x4 and 8x8 TUs (JCTVC-I0155), T. Tsukuba (Sharp)

- JCTVC-I0271, Harmonization and complexity reduction of context selection of 4x4 and 8x8 significant_coeff_flag,  C. Auyeung (Sony) [TEXT]

  - JCTVC-I0434, Non-CE3: Cross-check result for I0271, Y. Piao, E. Alshina, J. H. Park (Samsung)

- JCTVC-I0294, AHG5: Unified 4x4 and 8x8 significance map coding using one logic, T. Tsukuba, T. Ikai (Sharp) [TEXT]

  - JCTVC-I0201, AHG5: Crosscheck of unified 4x4 and 8x8 significance map coding using one logic in JCTVC-I0294, C.-W. Hsu, Y.-W. Huang (MediaTek)

- JCTVC-I0373, AHG5: context derivation clean-up for significance map, J. Chen, J. Sole, and W.-J. Chien, V. Seregin, M. Karczewicz (Qualcomm) [SW & TEXT]

  - JCTVC-I0541, AHG5: Cross-verification of Context derivation clean-up for significance map (JCTVC-I0373), G. Korodi, D. He (RIM)

|  | Summary of the proposals | BD rate normal QP | BD rate low QP | Nb of ctxts vs HM6 | Complexity expressed in logic, table and branches | Observations |
| --- | --- | --- | --- | --- | --- | --- |
| I0031 Source code provided. | A single table (HM6 4x4 luma) is used for 4x4 and 8x8 luma and chroma. 2 additional contexts for 8x8 DC position. | Intra: 0.02 0.02 RA: 0.01 0.01 LD: 0.03 -0.02 | Intra: 0.02 0.02 RA: 0.01 0.01 LD: 0.02 0.01 | + 1 | Table: 1 Logic: 4 bits Branches: max depth for worst case=3 |  |
| I0155 method 1 | A single table (HM6 4x4 luma) is used for 4x4 and 8x8 luma and chroma. Without separated contexts for 8x8 DC position. | Intra: 0.03 0.02 RA: 0.03 0.03 LD: 0.03 0.00 |  | -1 | Table: 1 Logic: 4 bits Branches: max depth for worst case=3 |  |
| I0155 method 2 | Method 1 with the same context set being shared between 4x4 and 8x8. | Intra: 0.09 0.09 RA: 0.11 0.09 LD: 0.13 0.04 |  | -19 | Table: 1 Logic: 4 bits Branches: max depth for worst case=3 | 0.2% loss average for class D in RA and LD. |
| I0271 C38 | A single mapping table for 4x4 and 8x8 with separated context for DC | Intra: 0.01 0.01 RA: 0.03 0.01 LD: 0.06 0.03 |  | +1 | Table: 1 Logic: 4 bits Branches: max depth for worst case=3 |  |
| I0271 C36 | Same as C38 but without separated context for DC | Intra: 0.03 0.03 RA: 0.04 0.02 LD: 0.06 -0.01 |  | -1 | Table: 1 Logic: 4 bits Branches: max depth for worst case=3 |  |
| I0271 C34 | A single mapping table for 4x4 and 8x8 with separated context for DC | Intra: 0.02 0.02 RA: 0.03 -0.00 LD: 0.03 -0.03 |  | -3 | Table: 1 Logic: 3 bits Branches: max depth for worst case=3 |  |
| I0271 C32 | Same as C34 but without separated context for DC | Intra: 0.03 0.03 RA: 0.02 0.02 LD: 0.05 -0.01 |  | -5 | Table: 1 Logic: 3 bits Branches: max depth for worst case=3 |  |
| I0271 C30 | A single mapping table for 4x4 and 8x8 with separated context for DC | Intra: 0.03 0.03 RA: 0.05 -0.01 LD: 0.05 -0.00 |  | -7 | Table: 1 Logic: 3 bits Branches: max depth for worst case=3 |  |
| I0271 C28 | Same as C30 but without separated context for DC | Intra: 0.04 0.04 RA: 0.04 0.01 LD: 0.07 0.00 |  | -9 | Table: 1 Logic: 3 bits Branches: max depth for worst case=3 |  |
| I0294 | Same method than I0237 C28 | Intra: 0.04 0.04 RA: 0.04 0.01 LD: 0.07 0.00 | Intra: 0.05 0.04 RA: 0.01 0.00 LD: 0.04 -0.01 | -9 | Table: 1 Logic: 3 bits Branches: max depth for worst case=3 |  |
| I0373 Source code provided. | A single table (HM6 4x4 luma), Sharing context of DC among all block sizes respectively for Luma and chroma | Intra: 0.02 0.02 RA: 0.00 0.01 LD: 0.01 0.00 | Intra: 0.01 0.00 RA: -0.01 0.00 LD: 0.00 -0.01 | -4 | Table: 1 Logic: 4 bits Branches: max depth for worst case=3 |  |

Parallelization of the template (3)

- JCTVC-I0174, CE3.1.2: Parallel context assignment for significance map coding, T.-D. Chuang, C.-W. Hsu, C.-Y. Chen, Y.-W. Huang, S. Lei (MediaTek) [SW CE mailing list & TEXT]

- JCTVC-I0203, CE3: Test results for Subtest 3.1.3. (H0352), V. Sze (TI) [SW CE mailing list & TEXT]

- JCTVC-I0354, CE3: Three and four bins parallel calculation for significance map contexts, V. Seregin, J. Sole, M. Karczewicz (Qualcomm) [SW CE mailing list & TEXT]

- JCTVC-I0296, Non-CE3: Simplified context derivation for significance map, T. Kumakura, S. Fukushima (JVC Kenwood) [SW & TEXT]

|  | Nb of // bins | Summary of the proposals | AI-Main | AI-HE10 | RA-Main | RA-HE10 | LB-Main | LB-HE10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| I0174 | 3 |  | 0.13 | 0.12 | 0.15 | 0.14 | 0.18 | 0.15 |
| I0174 | 4 |  | 0.20 | 0.18 | 0.21 | 0.19 | 0.23 | 0.17 |
| I0203 | 3 | Conditions apply to usage of F and I in HM6 template | 0.10 | 0.08 | 0.10 | 0.09 | 0.09 | 0.06 |
| I0203 | 4 | Conditions apply to usage of F, I and B in HM6 template | 0.16 | 0.14 | 0.13 | 0.14 | 0.12 | 0.11 |
| I0354 M 1 | 3 | Remove dependencies in HM6 template, 3 masks | 0.08 | 0.08 | 0.07 | 0.06 | 0.08 | 0.05 |
| I0354 M 1 | 4 | Remove dependencies in HM6 template, 5 masks | 0.22 | 0.21 | 0.19 | 0.19 | 0.17 | 0.15 |
| I0354 M 2 | 3 | Coefficients are split in groups of 3 or 4, 2 masks | 0.01 | 0.01 | 0.03 | 0.02 | 0.02 | 0.00 |
| I0354 M 2 | 4 | Coefficients are split in groups of 3 or 4, 5 masks | 0.12 | 0.11 | 0.10 | 0.09 | 0.11 | 0.07 |
| I0354 M 3 | 3 |  | 0.09 | 0.08 | 0.09 | 0.10 | 0.10 | 0.10 |
| I0354 M 3 | 4 |  | 0.15 | 0.14 | 0.16 | 0.17 | 0.17 | 0.19 |
| I0296 | 16 | Position based | 0.15 | 0.13 | 0.14 | 0.13 | 0.13 | 0.14 |

### Increase of position-based context derivation (2)

- JCTVC-I0296, Non-CE3: Simplified context derivation for significance map, T. Kumakura, S. Fukushima (JVC Kenwood) [SW & TEXT] (also in previous section)

- JCTVC-I0325, AhG5: Modified thresholds for NSQT significance map, J. Sole, W.-J. Chien, J. Chen, M. Karczewicz (Qualcomm) [SW & TEXT]

|  | Main |  |  | HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | AI | RA | LB | AI | RA | LB |
| JCTVC-I0296 (low QP) | 0.06 | 0.05 | 0.02 | 0.04 | 0.07 | 0.06 |
| JCTVC-I0296 (RDOQ off) | 0.14 | 0.07 | 0.03 | 0.13 | 0.10 | 0.13 |
| JCTVC-I0296 (SIGMAP_CONST_AT_HIGH_FREQUENCY off) | 0.12 | 0.09 | 0.06 | 0.11 | 0.11 | 0.11 |
| JCTVC-I0325 |  |  |  |  | 0.02 | 0.00 |

### Template context derivation (6)

- JCTVC-I0060, AhG5: Unified neighbour-based context selection, C. Rosewarne, M. Maeda (Canon) [SW]

- JCTVC-I0273, A proposal to remove redundancy in the context selection of significant_coeff_flag, C. Auyeung (Sony) [TEXT]

- JCTVC-I0274, A proposal to reduce data dependency in the context selection of significant_coeff_flag., C. Auyeung (Sony) [TEXT]

- JCTVC-I0372, Zone Partition for Significance Map Coding of Large TU, V. Kung, K. Panusopone, L. Wang (Motorola Mobility)

- JCTVC-I0375,  Simplification of Significance Map Coding Process for Small TU, V. Kung, K. Panusopone, L. Wang (Motorola Mobility)

- JCTVC-I0383, Template-based context modeling for coefficient coding , W. -J. Chien, J. Chen, J. Sole, M. Karczewicz (Qualcomm)

|  | Main |  |  | HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | AI | RA | LB | AI | RA | LB |
| JCTVC-I0060 | 0.09 | 0.11 | 0.27 | 0.03 | 0.17 | 0.43 |
| JCTVC-I0273 | 0.0 |  |  | 0.0 |  |  |
| JCTVC-I0274 | 0.0 |  |  | 0.0 |  |  |
| JCTVC-I0372 | 0.06 | 0.07 | 0.04 | 0.06 | 0.07 | -0.02 |
| JCTVC-I0375 | 0.11 | -0.02 | -0.05 | 0.08 | -0.04 | -0.08 |
| JCTVC-I0383 | -0.14 | -0.06 | 0.07 | -0.16 | -0.05 | 0.06 |

Syntax elements interleaving (7)

- JCTVC-I0114, Non-CE3: Sub-block base CABAC context selection of coefficient coding, K. Terada, H. Sasai, T. Nishi (Panasonic) [SW]

- JCTVC-I0304, Context Reduction for Significance Map Coding and Level Coding (H0228), Y. Yu, K. Panusopone, L. Wang (Motorola Mobility)

- JCTVC-I0374, 4 bin parallel scheme for significance map coding and level coding (H0228), V. Kung, K. Panusopone, L. Wang (Motorola Mobility)

- JCTVC-I0376, Sub-block Based Zoning for Significance Map Coding and Level Coding (H0228), V. Kung, K. Panusopone, L. Wang (Motorola Mobility)

- JCTVC-I0397, AHG5: Unified coefficient scan for JCTVC-H0228, Heiner Kirchhoffer, Mischa Siekmann, Christian Bartnik, Detlev Marpe, Tung Nguyen, Thomas Wiegand (Fraunhofer HHI)

- JCTVC-I0403, Context derivation by using capped coefficients level, J. Chen, W.-J. Chien, J. Sole, M. Karczewicz (Qualcomm)

- JCTVC-I0406, CE3: Report for CE3 Subtest 3, T. Nguyen, H. Kirchhoffer, C. Bartnik, D. Marpe (Fraunhofer HHI) [SW CE mailing list]

|  | Main |  |  | HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | AI | RA | LB | AI | RA | LB |
| JCTVC-I0114 (pattern 1) vs H0228 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| JCTVC-I0114 (pattern 2) vs H0228 | 0.1 | 0.1 | 0.0 | 0.1 | 0.0 | 0.0 |
| JCTVC-I0304 | 0.0 |  |  | 0.0 |  |  |
| JCTVC-I0374 |  |  |  |  |  |  |
| JCTVC-I0376 |  |  |  |  |  |  |
| JCTVC-I0397 |  |  |  |  |  |  |
| JCTVC-I0403 | -0.6 | -0.4 | -0.2 | -0.6 | -0.4 | -0.2 |
| JCTVC-I0406(direct port to HM-6.0) | -0.8 | -0.5 | -0.2 | -0.8 | -0.4 | -0.2 |
| JCTVC-I0406(grouping bypass together) | -0.7 | -0.5 | -0.2 | -0.7 | -0.4 | -0.2 |
| JCTVC-I0406(remove dependency on previous neighbor for horizontal and vertical) | -0.3 | -0.2 | -0.1 | -0.4 | -0.2 | -0.1 |
| JCTVC-I0406(separate contexts for 4x4 TU) | -0.8 | -0.5 | -0.3 | -0.8 | -0.5 | -0.3 |

## Greater than one and two (7)

### Throughput (4)

- JCTVC-I0269, High Throughput Level Coding with Context Reduction, X. Yu, J. Wang, D. He, G. Martin-Cocher (RIM) [TEXT]

- JCTVC-I0281, Adaptive Thresholds for Greater-than-1 and Greater-than-2 Flags, N. Nguyen, T. Ji, D. He, G. Martin-Cocher (RIM) [TEXT]

- JCTVC-I0337, Improved high throughput coding method based on last position information, S.H. Kim, A. Segall (Sharp)

- JCTVC-I0359, TU level threshold for Greater-than-1 Flags, W. -J. Chien, J. Chen, J. Sole, M. Karczewicz (Qualcomm)

|  | Summary of the proposals | BD rate normal QP | BD rate low QP (QP= 1,5,9,13) | Regular bins per coefficient (worst-case) | Average context coded bins reduction (Normal QP ¦ Low QP) | Complexity expressed in memory, Switch |
| --- | --- | --- | --- | --- | --- | --- |
| I0269 method 1 | After the first lgr1_flag being 1, use bypass mode to code lgr1_flag | Intra: -0.02 -0.00 RA: 0.07 0.08 LD: 0.05 0.08 | Intra: -0.09 -0.02 RA: 0.19 0.36 LD: 0.31 0.49 | Same as HM6, worst-case is not the purpose of the contribution | Intra: 7.0 6.7 ¦ 15.0 14.6 RA: 6.8 6.4 ¦ 13.9 13.4 LD: 2.3 2.0 ¦ 13.2 12.7 | # of contexts saved: 6 over 24 Memory requirement: 0 bits per TU Max # of lgr1_flag per CG: 8 Max # of lgr2_flag per CG: 1 Bypass bins are interleaved with regular bins (but can be de-interleaved.) Extra logic : 7 comparisons per CG (worst case) |
| I0269 method 2 | For CG with odd ctxSet, use bypass mode to code lgr1_flag after the first lgr1_flag being 1, | Intra: -0.05 -0.04 RA: 0.01 0.02 LD: 0.03 -0.01 | Intra: -0.16 -0.11 RA: 0.03 0.14 LD: 0.12 0.23 | Same as HM6, worst-case is not the purpose of the contribution | Intra: 5.0 4.7 ¦ 12.4 11.0 RA: 5.4 5.2 ¦ 11.4 10.5 LD: 1.0 0.9 ¦ 10.5 9.4 | # of contexts saved: 3 over 24 Memory requirement: 0 bits per TU Max # of lgr1_flag per CG: 8 Max # of lgr2_flag per CG: 1 Bypass bins are interleaved with regular bins (but can be de-interleaved) Extra logic : 7 comparisons per CG (worst case) |
| I0281 Method A | Adaptive threshold for the number of lgr1_flag and lgr2_flag depending on neighboring CG | Intra: 0.01 0.01 RA: 0.01 0.01 LD: 0.03 0.01 | Intra: 0.02 0.03 RA: 0.18 0.27 LD: 0.17 0.28 | Same as HM6, worst-case is not the purpose of the contribution | Intra: 2.5 2.6 ¦ 4.5 7.6 RA: 2.4 2.5 ¦ 5.5 6.8 LD: 0.3 0.4 ¦ 4.3 5.9 | Memory requirement: 126 bits per TU Max # of lgr1_flag per CG: 8 or 2 Max # of lgr2_flag per CG: 1 or 0 Extra logic : 1 addition and 2 comparisons per CG |
| I0281 Method B | Adaptive threshold for the number of lgr1_flag and lgr2_flag depending on previous CG | Intra: 0.01 0.01 RA: 0.01 0.01 LD: 0.02 -0.02 | NA | Same as HM6, worst-case is not the purpose of the contribution | Intra: 1.8 1.9 ¦ NA RA: 1.9 2.0 ¦ NA LD: 0.1 0.2 ¦ NA | Memory requirement: 0 bits per TU Max # of lgr1_flag per CG: 8 or 2 Max # of lgr2_flag per CG: 1 or 0 Extra logic : 2 comparisons per CG |
| I0337 | Limit total bins coded in regular mode to 22 for significant map and lgr1_flag combined. | Intra: 0.06 0.06 RA: 0.10 0.09 LD: 0.11 0.05 | Intra: 0.05 0.08 RA: 0.20 0.26 LD: 0.20 0.29 | 1.4375 | NA | Memory requirement: 0 bits per TU Max # of lgr1_flag per CG: from 6 to 11 Max # of lgr2_flag per CG: 1 Extra logic : 1 addition per CG |
| I0359 – M1 | Changing # of lgr1_flag from 8 to 6 | Intra: 0.02 0.03 RA: 0.07 0.09 LD: 0.10 0.11 | Intra: -0.10 0.00 RA: 0.21 0.31 LD: 0.25 0.39 | 1.4375 | Intra: 2.7 2.9 ¦ 5.6 5.6 RA: 2.0 2.2 ¦ 5.1 5.3 LD: 1.0 1.2 ¦ 4.8 5.0 | Memory requirement: 0 bits per TU Max # of lgr1_flag per CG: 6 Max # of lgr2_flag per CG: 1 No extra logic |
| I0359 – M2 | Changing # of lgr1_flag from 8 to 6 Limit # of lgr1_flag per TU to 6, 18, 72, 144 for TU size 4×4, 8×8, 16×16, 32×32 | Intra: 0.03 0.03 RA: 0.10 0.10 LD: 0.10 0.14 | Intra: -0.13 -0.07 RA: 0.30 0.45 LD: 0.37 0.60 | 1.4375 | Intra: 3.0 3.2 ¦ 7.3 8.8 RA: 2.1 2.4 ¦ 7.2 7.8 LD: 1.0 1.4 ¦ 6.4 7.2 | Memory requirement: 9 bits per TU Max # of lgr1_flag per CG: from 0 to 6 Max # of lgr2_flag per CG: 1 Extra logic : 1 addition and 1 comparison per CG |

### Others (3)

- JCTVC-I0049, Non-CE3: Context reduction for coeff_abs_level_greater2_flag, C. Rosewarne, M. Maeda (Canon) [TEXT]

- JCTVC-I0176, Non-CE3: Modified method for coding transform coefficient level, S.-T. Hsiang, S. Lei (MediaTek)

- JCTVC-I0303, CE3 subtest 2.1: context assignment for parallel coefficient level coding, W. -J. Chien, J. Chen, J. Sole, M. Karczewicz (Qualcomm)

|  | Main |  |  | HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | AI | RA | LB | AI | RA | LB |
| JCTVC-I0049 (method 1) | 0.01 | 0.03 | 0.05 | 0.01 | 0.02 | 0.01 |
| JCTVC-I0049 (method 2) | 0.01 | 0.04 | 0.03 | 0.01 | 0.03 | 0.03 |
| JCTVC-I0176 | 0.1 | 0.2 | 0.1 | 0.1 | 0.1 | 0.1 |
| JCTVC-I0303 | 0.2 | 0.0 | 0.1 | 0.2 | 0.1 | 0.0 |

## Golomb-Rice (5)

- JCTVC-I0050, Non-CE3: Rice parameter initialization and update modifications, C. Rosewarne, V. Kolesnikov, M. Maeda (Canon)

- JCTVC-I0124, Simplification of Golomb-Rice Parameter Update, C. Kim, J. Kim, J. H. Park (Samsung)

- JCTVC-I0343, Simplified absolute-3 coding method, S.-H. Kim, L. Kerofsky, A. Segall (Sharp)

- JCTVC-I0358, Truncated EG0 coding method, S.-H. Kim, L. Kerofsky, A. Segall (Sharp)

- JCTVC-I0487, On coefficient level remaining coding, W.J. Chien, M. Karczewicz, J. Chien (Qualcomm) [SW]

|  | Main |  |  | HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | AI | RA | LB | AI | RA | LB |
| JCTVC-I0050 (k param) | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| JCTVC-I0050 (k param;low QP 1, 5, 9, 13) | -0.3 | -0.1 | 0.0 | -0.3 | -0.1 | -0.1 |
| JCTVC-I0050(restricted k param) | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| JCTVC-I0124 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| JCTVC-I0343 (low QP 1, 5, 9, 13) | -0.03 | 0.02 | 0.01 | 0.04 | 0.02 | 0.01 |
| JCTVC-I0358 (low QP 1, 5, 9, 13) | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| JCTVC-I0487 (Unary+VLC, low QP 1, 5, 9, 13) | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |

## Sign Data Hiding (3)

- JCTVC-I0156, Simplification fo multiple sign bit hiding, X. Yu, J. Wang, D. He, G. Martin-Cocher (RIM) [TEXT]

- JCTVC-I0291, Simplified decision for sign hiding, T. Ikai (Sharp) [TEXT]

- JCTVC-I0326, Sign data hiding simplification and enhancement, J. Sole, J. Chen, W.-J. Chien, M. Karczewicz (Qualcomm) [SW & TEXT]

|  | Main |  |  | HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | AI | RA | LB | AI | RA | LB |
| JCTVC-I0156 (fixed threshold 4) | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| JCTVC-I0291 | 0.1 | 0.1 | 0.0 | 0.1 | 0.0 | 0.0 |
| JCTVC-I0326 (method 1) | -0.08 | -0.05 | -0.03 | 0.02 | 0.05 | 0.07 |
| JCTVC-I0326 (method 2) | -0.13 | -0.07 | -0.15 | -0.03 | -0.03 | -0.02 |

## Scans (2)

- JCTVC-I0103, Non-CE3: On scanIdxC derivation for chroma, H. Y. Kim (ETRI), K. Y. Kim, G. H. Park (KHU), S.-C. Lim, J. Lee, J. S. Choi (ETRI) [SW & TEXT]

- JCTVC-I0264, AHG5: Harmonized diagonal scans for coefficient coding, J. Xu, A. Tabatabai (Sony) [TEXT]

|  | Main |  |  | HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | AI | RA | LB | AI | RA | LB |
| JCTVC-I0103 |  |  |  | 0.00 (Y) -0.13 (U) -0.11 (V) |  |  |
| JCTVC-I0264 | 0.03 | 0.03 | 0.06 | 0.03 | 0.03 | 0.04 |

## Last Coefficient Coding (4)

- JCTVC-I0177, Table size reduction and CABAC context reduction for last significant position coding, F. Zou, O. C. Au, J. Dai, X. Zhang, C. Pang (HKUST)

  - JCTVC-I0423, Cross-verification of JCTVC-I0177: Table size reduction and CABAC context reduction for last significant position coding, J. Dong, Y. Ye (InterDigital)

- JCTVC-I0331, Table removal in contexts assignment of last significant position (LAST) coding, L. Guo, M. Karczewicz, W.-J. Chien (Qualcomm) [SW & TEXT]

  - JCTVC-I0474, Non-CE3: Cross-check result for I0331, Y. Piao, E. Alshina, J.H. Park (Samsung)

- JCTVC-I0339, Reduced contexts for last position coding, S.H. Kim, L. Kerofsky, A. Segall (Sharp)

- JCTVC-I0371, Luma Context reduction for Last Coefficient Position Coding in HEVC, X. Fang, K. Panusopone, L. Wang (Motorola Mobility)

  - JCTVC-I0486, Cross-verification of JCTVC-I0371 Luma Context Reduction for LAST coding, L. Guo (Qualcomm)

|  | Summary of the proposals | BD rate normal QP | BD rate low QP | Nb of ctxts vs HM6.0 | Complexity expressed in table and memory access | Observations |
| --- | --- | --- | --- | --- | --- | --- |
| I0177 | 4 unused elements in context mapping table are removed. Contexts of some neighboring bins in 32x32 TU are merged. |  |  | -2 | Table: 1 of size 24 |  |
| I0331 Setting A Source codes WD text provided | Context mapping table is replaced by formula. Context derivations for Luma and Chroma are unified. | Intra:0.00% RA:0.01% LD:-0.01% | Intra:0.00% RA:-0.01% LD:-0.02% | 0 | Table: None Contexts fetching: Consecutive and sequential. | Initialization values are changed. |
| I0339 | Contexts of 32x32 TU are shared between LAST_X and LAST_Y. | Intra:0.03% RA:0.04% LD:0.03% |  | -5 | Table: 1 of size 28 (like HM6.0) |  |
| I0371 method 1 | Contexts are shared between 32x32 TU and 16x16 TU. | Intra:0.01% RA: 0.01% LD: 0.01% |  | -10 | Table: 1 of size 28 |  |

## Information documents (2)

- JCTVC-I0205, AHG5: A throughput study of transform coefficient coding proposals, V. Sze, M. Budagavi (TI)

- JCTVC-I0324, CE3 test 3: parallelization with bit-plane coding (H0228), J. Sole, V. Seregin, W.-J. Chien, J. Chen, M. Karczewicz (Qualcomm) [SW CE mailing list]

# Recommendations

To study all the input contributions.