| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0007 |
| --- | --- |

| Title: | JCTVC AHG Report: Memory bandwidth restrictions in motion compensation (AHG7) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Teruhiko Suzuki 2-10-1, Osaki, Shinagawa-ku , Tokyo, 141-8610 JAPAN | Tel: Email: | +81-50-3750-2740 teruhikos@jp.sony.com |
| Source: | AHG on Memory bandwidth restrictions in motion compensation (AHG7) |  |  |

_____________________________

# Introduction

This AHG was established in Geneva to study memory bandwidth issues in HEVC.

The mandates of this AHG are,

- Study memory bandwidth in motion compensation.

- Study restrictions to reduce the average and worst case of memory bandwidth.

- Study level specific restrictions, for example restrict usage of small PUs, restrict usage of 2D interpolation, restrict subpel accuracy etc, to reduce memory bandwidth in motion compensation.

- Evaluate the impact of such restrictions on coding efficiency.

# Activities

## Reflector

The e-mail reflector is [jct-vc@lists.rwth-aachen.de].  To receive email, please subscribe to the e-mail reflector: http://mailman.rwth-aachen.de/mailman/listinfo/jct-vc. For e-mail exchange, it is recommended to put [AHG7] in the subject line for easy grouping.

# Related contributions

There is 11 contributions on common test condition were submitted.

JCTVC-I0075 [T.Chujoh (Toshiba)] AHG7: A restriction of motion vector for small PU size

JCTVC-I0106 [K. Kondo, T. Suzuki (Sony)] AHG7: Level definition to limit memory bandwidth of MC

JCTVC-I0107 [K. Kondo, T. Suzuki (Sony), T. Yamamoto (Sharp)] AHG7: Modification of merge candidate derivation to reduce MC memory bandwidth

JCTVC-I0120 [M. Zhou (TI)] AHG7: Cross-verification of JCTVC-I0216 entitled reducing HEVC worst-case memory bandwidth by restricting bi-directional 4x8 and 8x4 prediction units

JCTVC-I0121 [M. Zhou (TI)] AHG7: Cross-verification of JCTVC-I0107 entitled modification of merge candidate derivation to reduce MC memory bandwidth

JCTVC-I0216 [T. Hellman, W. Wan (Broadcom)] AHG7: Reducing HEVC worst-case memory bandwidth by restricting bi-directional 4x8 and 8x4 prediction units

JCTVC-I0297 [S. Fukushima, M. Ueda, H. Takehara (JVC Kenwood)] AHG7: Bi-pred restriction for small PUs

JCTVC-I0351 [V. Seregin, X. Wang, J. Chen, M. Karczewicz (Qualcomm)] AHG7: Motion vector rounding for the worst case bandwidth reduction

JCTVC-I0425 [M. Zhou (TI)] AHG7: A combined study on JCTVC-I0216 and JCTVC-I0107

JCTVC-I0432 [T. Lee, J. Park (Samsung)] Cross verification of Motion vector rounding for the worst case bandwidth reduction (JCTVC-I0351)

JCTVC-I0438 [T. Sugio (Panasonic)] AHG7: Cross-check report of A combined study on JCTVC-I0216 and JCTVC-I0107 (JCTVC-I0425)

JCTVC-I0449 [K. Kondo, T. Suzuki (Sony)] AHG7: Cross verification of “Bi-pred restriction for small PUs” (JCTVC-I0297)

JCTVC-I0558 [M. Ueda, S. Fukushima (JVC Kenwood), K. Kondo, T. Suzuki (Sony)] AHG7: High-level syntax for explicit memory bandwidth restriction [late]

Those should be reviewed during Geneva meeting.

# Proposals

## Summary

The proposals can be classified into two categories. One is on the level specific constraints, and the other is on modifications of decoding process. The following is the summary of the contributions.

- Level specific constraints

- A1: proposals on the ways of constraints

  - Disable small PU block when block size is small (depending on levels) (JCTVC-I0106)

  - Limitation of number of MVs in one LCU (and combination with PU size restriction)  (JCTVC-I0106)

  - Disable 4x8, 8x4 bi-pred (JCTVC-I0216)

- A2: proposals on which levels the constraints should be applied

  - Apply proposed constraint to Level 3 and above (JCTVC-I0075)

  - Table of disabled PU size, bi-/single pred, depending on levels (JCTVC-I0106)

- Changes of decoding process and syntax

- B1: Removal of Inter 4x4 (and remove inter_4x4_enable_flag)

  - JCTVC-I0106, JCTVC-I0216, JCTVC-I0425

- B2: Changes to derive merge candidates

  - Replace bi-pred of merge candidate with uni-pred when block size is small (e.g. 4x4, 4x8, 8x4 and 8x8) (JCTV-I0107, JCTVC-I0297, JCTVC-I0425, JCTVC-I0558)

- B3: Syntax changes other than merge

  - Dsiable bi-pred of 4x8, 8x4 (JCTVC-I0216, JCTVC-I0425)

  - Removal of inter_pred_flag (JCTVC-I0216, JCTVC-I0425)

- B4: MV constraints

  - Restrict the value of MV to integer pel for 4x8 and 8x4 for both chroma and luma at level 3 and above (encoder only change) (JCTVC-I0075)

  - Vertical MV is rounded to integer position in MC decoding process (JCTVC-I0351)

  - Both horizontal and vertical MV is rounded to integer position in MC process (JCTVC-I0351)

## Comparisons

The impact on coding efficiency and complexity should be considered as well as reduction level of MC memory bandwidth. Good trade off should be identified.

The comparison of the proposed methods is summarized in the attached Excel sheet.

# Recommendations

The AHG recommends

- to review all related contributions

- to evaluate all proposals

- to identified appropriate method to reduce the memory bandwidth

- to include appropriate method into DIS and HM