| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0008 |
| --- | --- |

| Title: | JCT-VC AHG report: Profile and level definitions (AHG8) |  |  |
| --- | --- | --- | --- |
| Status: | AHG report input to JCT-VC |  |  |
| Purpose: | AHG Report |  |  |
| Author(s) or Contact(s): | M. Horowitz (eBrisk Video) K. McCann (Samsung/ZetaCast) T. Suzuki (Sony) T. K. Tan (NTT DOCOMO) W. Wan (Broadcom) Y. -K. Wang (Qualcomm) T. Yamakage (Toshiba) | Email: | michael@ebriskvideo.com ken@zetacast.com teruhikos@jp.sony.com ttk@pacific.net.sg wwan@broadcom.com yekuiw@qualcomm.com tomoo.yamakage@toshiba.co.jp |
| Source: | AHG |  |  |

_____________________________

# Abstract

This document reports on the work of the JCT-VC ad hoc group on Profile and level definitions (AHG8) between the 8th JCT-VC meeting in San Jose (1–10 February, 2012) and the 9th JCT-VC meeting in Geneva (27 April – 7 May 2012).

# Mandate

| Profile and level definitions (AHG8) (jct-vc@lists.rwth-aachen.de) - Review the set of levels defined in the CD to confirm that they span the target range of 8-bit 4:2:0 video applications with a reasonable degree of granularity. - Review the Main Profile defined in the CD to confirm that it meets the requirements for complexity / compression efficiency trade-off for the target range of 8-bit 4:2:0 video applications. - Evaluate the performance, functionality and complexity of coding tools that are identified as potential candidates for either inclusion or exclusion in the Main Profile. - Study the extensibility of the current concept of profile/level definition towards possible architectures for future profiles that could provide additional trade-off points between complexity and functionality, while facilitating interoperability. | M. Horowitz, K. McCann (co-chairs), T. Suzuki, T. K. Tan, W. Wan, Y.-K. Wang, T. Yamakage (vice-chairs) | N |
| --- | --- | --- |

# Email Activity

AHG8 used the main JCT-VC reflector: jct-vc@lists.rwth-aachen.de.

A kick-off message was sent by the chairs on 23 February 2012.  Subsequently 45 messages related to AHG8 were received on the JCT-VC email reflector between 6 March 2012 and 20 April 2012.  Excerpts from the messages have been organized into three general categories below.

- Messages related to specific coding tools

  - Several messages were posted regarding AMP and NSQT.  The general tone of the messages seemed to lean toward including these tools.  Relevant excerpts from messages on the topic are listed below.

    - “Preference for AMP and NSQT tools to be included in Main profile”

    - “I would also be likely to support AMP & NSQT in the Main profile”

    - “Regarding the software design, NSQT has been significantly cleaned-up (following the adoption of H0492). In particular, the design has been unified for square and non-square transformation.”

    - “In the last meeting, AhG16 provided a solution which adds 2NxN and Nx2N partition types to Intra prediction and applies NSQT to these rectangular Intra PUs … I wonder if NSQT [plus 2 additional partitions] is included in the design for Inter, why not also utilize it for Intra, such that we get 1.4%, 0.7%, 0.3% coding gain.”

  - An AHG8 co-chair posted the following message to stimulate discussion regarding ALF and the Main Profile. The CE2 coordinator and AHG6 chair reported further progress on ALF simplification and coding efficiency to the JCT-VC reflector. No additional messages were posted specifically to AHG 8 on the topic, however.

    - “Since Yamakage-san has distributed the CE2 (ALF) test results, I suggest that we should now broaden the discussion on our third mandate to include ALF.”

- Several messages were posted regarding modifications of specific level parameters in Table A-1 of the HEVC CD. Relevant excerpts from messages on the topic are listed below.

  - “… suggest considering increasing the Max Luma Size and Max Luma Pixel rate in Table A-1 [level 3] a little bit to also include QHDx30”

  - “Suggestion is to lower the max bit rate of Level 4.2 to 20 Mbps”

  - “Level 5.2: This includes 4kx2kx60p - a targeted area for HEVC for SHDTV. It has the max bit rate of 150Mbps. … Suggestion is to lower it - at least lower than around 70Mbps (looking at some of the channel capacity), may be even as low as 50 Mbps.”

  - “MV range limit column in A-1 is missing. Is that now not perceived to be needed?”

    - The above comment received the following response: “This, and the related constraint in AVC about partition sizes for bi-direct, are very important for real time memory performance.”

  - “How about equivalent of "Max number of motion vectors per two MBs" in AVC?”

    - The above comment received the following response: “This may not be so important, unless the bound can be made quite small.”

  - “Looking at the level table and wondering if we might want to bump the Max bit rate MaxBR (1000 bits/s) and Max CPB size (1000 bits) for level 2 up to 1500 instead of 1000”

- The following message and responses were posted regarding the number of profiles that should be defined in version one of the HEVC standard.  It should be noted that no messages asserted that the Main Profile, as defined in the CD text, did not meet the requirements for complexity / compression efficiency trade-off for the target range of 8-bit 4:2:0 video applications.

  - “We can create three categories of the Profiles – Low complexity … Medium complexity with very good efficiency and Highest Efficiency (the kitchen sink). [10 total profiles + sub-profiles were suggested].” This comment received the following responses.

    - “If HEVC application providers agree with this, I support [the idea of having 3 categories of profiles].”

    - “With 10 profiles here, plus the 10 bit versions, at different levels, that feels like interoperability becomes a significant issue.”

    - “If you _did_ want to cover all the bases, having a profile without B pictures would be attractive for low delay use cases”

    - “I agree with Rik's comment that 10 sounds too many [Profiles]. However, Ajay's suggestion of around three profiles does not sound unreasonable.”

    - “Just because we could define large numbers of profiles, it doesn’t mean that we should. We know that we will need to add further HEVC profiles at a later stage to cover additional functionality”

    - “Bottom Line: I support your 3 tiers.  It is the further fragmentation that is in question.”

    - “A larger number of profiles creates problems is a myth and knee-jerk reaction.”

    - “I tend to agree; merging conceptual profiles in order to create fewer profiles is a mistake; it just leads to either a lack of adoption, or externally-constrained profiles.”

    - “As I mentioned at the San Jose meeting, the other extreme of offering the a la carte approach to interoperability as was done H.263+ also does not work.”

# Levels

There are a few suggestions for specific improvements in the definition of the levels.

Contribution JCT-VC I0238 [6] recommends reducing the maximum number of slices allowed per picture from that currently specified.  It asserts that supporting the worst-case number of slices is known to be problematic and is rarely used in practice in the equivalent function for AVC and suggests that modifying the SliceRate parameter would achieve a better cost/functionality trade-off.

Contribution JCT-VC I0255 [7] proposes to introduce level constraints on the maximum tile width as a solution to reduce the size of line-buffers. This reportedly decreases the amount of on-chip line memory that is required for in-loop filtering operations.

Contribution JCT-VC I0391 [9] recommends that the minimum number of uniform tiles and/or slices for Main Profile @ Level 5.2 (4K @ 60 fps) be set to 4, subject to confirmation that no visible boundaries are seen across tiles and slices with deblocking and SAO enabled for QP’s in the range 1 to 35.  Otherwise, the authors suggest that all level 5.2 bitstreams should be decodable with a single core.

Contribution JCT-VC I0455 [10] suggests creating multiple level tables to address different application areas: distribution, contribution and storage.

Contribution JCT-VC I0472 [12] suggests that level definitions, especially at higher levels, need to be investigated and aligned with classes of applications.

Contribution JCT-VC I0474 [13] proposes that, instead of defining two levels with different bitrate for the same picture size, a flag at SPS level is added to indicate that the bit rate is higher/lower than that specified in the level table.

# Main Profile

There have been no suggestions that the Main Profile defined in the CD fails to meet the complexity / compression efficiency trade-off requirements for the target range of 8-bit 4:2:0 video applications.  These applications are:

- Broadcast

- Video On Demand

- Home cinema

- Surveillance

- Real-time communications, video chat, video conferencing

- Telepresence

- Mobile streaming, broadcast and communications

- Storage and playback of video

- Camcorders

- Internet streaming, download and play

- Remote video presentation and remote computer graphics display

However, some contributions suggest that the Main Profile definition could be improved to meet the requirements even better.

# Evaluation of Coding Efficiency Tools

The evaluation of coding efficiency tools largely focussed on those tools that are currently excluded from the Main Profile, but which had been identified as potential candidates for inclusion in the Main Profile at the San Jose meeting: AMP, NSQT and ALF.

Contribution JCT-VC I0063 [1] suggests:

- AMP appears as a straight-forward extension of the current PU split design with some coding efficiency gains (-0.7 % / -1.0 % for RA-Main and LB-Main, respectively) and so adding that tool to the Main profile looks feasible.

- NSQT appears to complicate the transform decoding design by introducing new logic in TU splitting, coefficient scanning and transform selection. Although it provides some coding efficiency gains when applied on top of AMP (-0.4 % / -1.1 % for RA-Main and LB-Main, respectively) it looks not worth complicating the design with the tool.

- ALF has to be significantly simplified with respect to the CD design in order to be included to the profile

Contribution JCT-VC I0305 [14] reports on test results for the combination of AMP and NSQT. It reports that according to the common test conditions, the combination of AMP and NSQT provides 1.1% to 2.1% objective gain with about 15% encoder runtime increase

Contribution JCT-VC I0157 [3] proposes three options for simplifying ALF.  It suggests that among the three options, option 2 has the best trade-off between coding efficiency and complexity. In option 2, one filter set is coded in APS, and on/off flags are coded at LCU level. The APS filter set can be always BA or RA and is trained by using a previous picture. Each LCU can choose the APS filter set or off. The authors suggest that simplified ALF option 2 (either with BA or RA) should be adopted into Main Profile.

Contribution JCT-VC I0363 [8] provides an analysis on the difference between region adaptive (RA) and block adaptive (BA) ALF. It suggests that the complexity difference between the two modes is minor and asserts that BA mode has desirable properties such as better coding gain and visual quality.

# Evaluation of Parallel Processing Tools

A number of documents comment on parallel processing tools.

Contribution JCT-VC I0063 [1] suggests:

- Should have only one tool for picture level parallel processing unless significant benefits for having multiple tools are demonstrated

- Should consider mandating maximum tile size for bitstreams in order to guarantee possibility to parallelize the decoder operation (e.g. tile should not contain more than 2048x1024 pixels)

Contribution JCT-VC I0153 [2] provides an analysis of the key advantages and disadvantages of tiles and Wavefront Parallel Processing (WPP). The key issues focussed on are Motion Estimation bandwidth, parallel processing efficiency, compression efficiency, delay issues, profile specification, and parallel decoder implementation. Based on the analysis, the authors argue that WPP has a number of significant advantages over tiles advantages in terms of compression efficiency, flexibility, and profiling simplicity.  They contend that it makes a stronger candidate than tiles for inclusion in the Main Profile of HEVC.

Contribution JCT-VC I0198 [4] proposes a preliminary assessment of Tiles and Wavefront tools for parallelism. It suggests that while WPP does not present any noticeable subjective issue, some visual artifacts have been reported for Tiles in Chroma components and Tiles boundaries. From these observations, it suggests that WPP is expected to be a part of HEVC Main profile.

Contribution JCT-VC I0228 [5] proposes that parallel tools with a minimal level of potential decoding parallelism become mandatory at the encoder side for each given profile/level (or at least for the levels that include HDTV resolution and beyond).

Contribution JCT-VC I0462 [11] proposes some restrictions on tile sizes, which are intended to allow parallel decoding by as wide a range of applications as possible.

Contribution JCT-VC I0472 [12] suggests that bitstreams should be decodable by both multi-core and single core devices.

# Extensibility of Profile Architecture

Contribution JCT-VC I0063 [1] proposes to keep the number of profiles as small as feasible in order to maximize interoperability of devices and services implementing the standard. It proposes to define two profiles for the version 1 of HEVC; Main profile for generic video use cases and a Still picture profile for still picture coding.

Contribution JCT-VC I0391 [9] suggests that the Main Profile should assume progressive-only output, possibly with a separate profile or display conformance point for interlaced content.

Contribution JCT-VC I0455 [10] suggests creating four profiles:

- Profile 1 (Baseline) – Low Complexity

- Profile 2 – Medium Complexity (progressive-only)

- Profile 2a – Field pictures

- Profile 3 – Highest coding efficiency

Contribution JCT-VC I0472 [12] summarises use cases and offers requirements for consideration in the creation of HEVC profiles that can be regarded as relevant to the applications and services provided by Multiple Systems Operators (MSOs).

# Recommendations

The recommendations of the AHG on Profile and level definitions are:

- JCT-VC should review the parameters defined for Levels to confirm that they do not require any fine tuning

- JCT-VC should maintain the general principles used to define the Main Profile

- JCT-VC should review the performance, functionality and complexity of coding efficiency tools that have been identified as potential candidates for inclusion in the Main Profile, particularly AMP, NSQT and ALF

- JCT-VC should review the performance, functionality, applicability, and complexity of parallel processing schemes to be included in the Main Profile

- JCT-VC should ensure that the HEVC standard allows for the addition of possible future profiles that could provide additional trade-off points between complexity and functionality, while facilitating interoperability

# List of Related Input Documents

The following input documents have been identified as relevant to AHG8.

- JCTVC-I0063 [J. Lainema, K. Ugur, M. Hannuksela, J. Ridge (Nokia)] “On HEVC profiles”

- JCTVC-I0153 [S. Worrall (Aspex)] “AHG8: Inclusion Of Parallel Processing Schemes In The Main Profile”

- JCTVC-I0157 [MediaTek, TOSHIBA, Qualcomm] “AHG6: Baseline options for ALF”

- JCTVC-I0198 [J.-M. Thiesse, J. Viéron (Ateme)] “On Tiles and Wavefront tools for parallelism”

- JCTVC-I0228 [M. Raulet (IETR/INSA), M. Mattavelli (EPFL), P.-L. Lagalaye (Modae Tech.)] “Parallelism tools mandatory at a given profile/level”

- JCTVC-I0238 [W. Wan, T. Hellman, B. Heng (Broadcom)] “Specifying a maximum bound on slices per picture”

- JCTVC-I0255 [A. Norkin, R. Sjöberg (Ericsson)] “Level restrictions on tiles for line buffer reduction”

- JCTVC-I0363 [I. S. Chong, K. Sanjeev, M. Karczewicz (Qualcomm), T. Ikai (Sharp), E. Maani, T. Suzuki (SONY)] “AHG6: Comparison of block adaptive (BA) and region adaptive (RA) ALF”

- JCTVC-I0391 [C. Fogg (Harmonic), A. Wells (Ambarella), O. Bar-Nir (Harmonic)] “Profiles & Levels: multicore, Max DPB, field sequences”

- JCTVC-I0455 [A. Luthra (Motorola Mobility)] “Profiles and Levels in HEVC”

- JCTVC-I0462 [S. Worrall (Aspex)] “AHG4: Profile Requirements For Facilitation Of Parallel Tile Decoding”

- JCTVC-I0472 [M. Kar, A. Hinds (CableLabs), Y. Syed (Comcast Cable)] “Considerations for the creation of HEVC profiles and levels”

- JCTVC-I0475 [T. Suzuki (Sony)] “Considerations for level definitions”

- JCTVC-I0305 [Xiaozhen Zheng (HiSilicon), Lingzhi Liu (HiSilicon), Haoping Yu (Huawei), Il-Koo Kim (Samsung) "Test results on AMP and NSQT"