| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0015 m25124 |
| --- | --- |

| Title: | JCT-VC AHG report: Reference picture buffering and list construction (AHG15) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Rickard Sjöberg KI/EAB/TVV/V 164 80 Stockholm Sweden Ying Chen Ye-Kui Wang 5775 Morehouse Drive San Diego, CA 92121 USA Hendry #221 Yangjae-dong, Seocho-gu Seoul 137-130 Korea TK Tan 3-6 Hikarinooka, Yokosuka-shi Kanagawa, 239 8536 Japan Wade Wan Broadcom Corporation 5300 California Ave Irvine, CA 92617 | Tel: Email: | +46761153563 rickard sjoberg@ericsson com |
| Source: | AHG 15 |  |  |

_____________________________

# Abstract

This report summarizes the reference picture buffering and list construction ad-hoc activities between the 8th and the 9th JCT-VC meeting, and lists the input documents to this meeting related to this ad-hoc group.

# Mandate

Table 1	- AHG mandates

| AHG Title and Email Reflector | Chairs |
| --- | --- |
| Reference picture buffering and list construction (AHG21) (jct-vc@lists.rwth-aachen.de) - Finalize the test specification document for reference picture buffering and list construction proposals for the 9th JCT-VC meeting in a timely manner (JCTVC-H0725). - Provide source code that enables HM encoding of all test cases described in JCTVC-H0725 and produce anchor data. - Identify and work to resolve issues relating to the draft text description of reference picture handling and list construction, and the associated HM software functionality. - Study possible improvements related to reference picture buffering and list construction. - Study the loss resilience properties of reference picture handling and its support in the HM software (in coordination with the loss resilience AHG). | R. Sjöberg (chair), Y. Chen, Hendry, T. K. Tan, W. Wan, Y. -K. Wang (vice chairs) |

# Test specification for reference picture buffering proposals

## Test specification text

The specification text for reference picture buffering and list construction proposals was finalized and uploaded on February 22. It consists of eight "random access" test cases and five "low-delay" test cases. Test case 2.8 and 3.5 were added at the 8th meeting in San Jose. Short descriptions of these test cases are shown in Table 2.

Table 2	- H0725 test cases

| Test case | Description of picture structure |
| --- | --- |
| 2.1 | The JCT-VC 7B random access common condition picture structure |
| 2.2 | Hierarchical 7B with 1 reference picture per list |
| 2.3 | Hierarchical 7B with 2 reference picture per list |
| 2.4 | As 2.3 but additionally using multiple reference pictures in temporal layer 0 |
| 2.5 | Hierarchical 63B with 1 reference picture per list |
| 2.6 | Hierarchical 7B without CRA pictures and long-term pictures |
| 2.7 | Hierarchical 7B with 2 reference picture per list. Not nested. |
| 2.8 | As 2.1 but with reference picture list reordering |
| 3.1 | The JCT-VC low delay common condition picture structure |
| 3.2 | 1 reference picture low-delay structure with temporal scalability |
| 3.3 | Low-delay with one short-term and two long-term pictures |
| 3.4 | Low-delay using 5 picture pseudo-random adaptive reference picture selection. |
| 3.5 | Low-delay with reference picture list reordering for the first slice in each picture |

## Source code

Anchor source code supporting all H0725 test cases was made available on April 26 in the HM-6.1-dev-ahg15 branch. A config file for each test case is available in HM-6.1-dev-ahg15/cfg/JCTVC- H0725/. Most test cases are supported by only config file changes but cases 2.6, 3.3 and 3.4 contain source code changes as well.

Due to lack of time anchors were not generated.

The HM-6.1-dev-ahg15 source code contains bit count functionality. It reports the number of bits spent on reference picture set and reference picture lists in a bit stream. This includes SPS, PPS and slice header syntax. The following slice header syntax elements are included in the count:

IDR

- idr_pic_id

- no_output_of_prior_pics_flag

POC

- pic_order_cnt_lsb

RPS

- short_term_ref_pic_set_sps_flag

- parseShortTermRefPicSet()

- short_term_ref_pic_set_idx

- num_long_term_pics

- delta_poc_lsb_lt

- delta_poc_msb_present_flag

- delta_poc_msb_cycle_lt_minus1

- used_by_curr_pic_lt_flag

num_ref

- num_ref_idx_active_override_flag

- num_ref_idx_l0_active_minus1

- num_ref_idx_l1_minus1

modifications

- ref_pic_list_modification_flag_l0

- list_entry_l0

- ref_pic_list_modification_flag_l1

- list_entry_l1

- ref_pic_list_combination_flag

- num_ref_idx_lc_active_minus1

- ref_pic_list_modification_flag_lc

- pic_from_list_0_flag

- ref_idx_list_curr

The RPS cost below shows the average percentage of bits that are spent on RPS related syntax. The numbers were calculated by first running each test case on the common condition sequences. The RA sequence set was used for the 2.x test cases and the LD sequence set was used for the 3.x test cases. For each sequence, fixed QPs 23, 27, 32, 37 was used. Note that 1500 byte slices were turned on for the 3.x test cases. The HM-6.1-dev-ahg15 decoder reports "RPS related bits in total: ", this number was divided by the bitstream file size and averaged for all bitstreams in each test case.

Table 3	- RPS bit-cost for the H0725 test cases

| Case | Description of picture structure | QP22 | QP27 | QP32 | QP37 | Avg |
| --- | --- | --- | --- | --- | --- | --- |
| 2.1 | The JCT-VC 7B RA common condition picture structure | 0.0 | 0.1 | 0.1 | 0.2 | 0.1 |
| 2.2T1 | Hierarchical 7B with 1 reference picture per list | 0.0 | 0.0 | 0.1 | 0.2 | 0.1 |
| 2.2T1 | Hierarchical 7B with 1 reference picture per list | 0.0 | 0.0 | 0.1 | 0.2 | 0.1 |
| 2.3T3 | Hierarchical 7B with 2 reference picture per list | 0.0 | 0.1 | 0.1 | 0.2 | 0.1 |
| 2.3T4 | Hierarchical 7B with 2 reference picture per list | 0.0 | 0.1 | 0.1 | 0.2 | 0.1 |
| 2.4 | As 2.3, additionally using multiple ref pictures in t_id 0 | 0.0 | 0.1 | 0.1 | 0.2 | 0.1 |
| 2.5 | Hierarchical 63B with 1 reference picture per list | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| 2.6 | Hierarchical 7B without CRA pictures and long-term pictures | 0.0 | 0.1 | 0.2 | 0.3 | 0.1 |
| 2.7 | Hierarchical 7B with 2 reference picture per list. Not nested. | 0.0 | 0.1 | 0.1 | 0.2 | 0.1 |
| 2.8 | As 2.1 but with reference picture list reordering | 0.0 | 0.1 | 0.2 | 0.3 | 0.2 |
| 3.1 | The JCT-VC low delay common condition picture structure | 0.1 | 0.1 | 0.2 | 0.3 | 0.2 |
| 3.2 | 1 reference picture low-delay structure with temporal scalability | 0.1 | 0.1 | 0.1 | 0.2 | 0.1 |
| 3.3a | LD with 1 short-term and 2 long-term pictures (200ms RTT) | 0.2 | 0.2 | 0.2 | 0.4 | 0.2 |
| 3.3b | LD with 1 short-term and 2 long-term pictures (1000ms RTT) | 0.2 | 0.2 | 0.3 | 0.4 | 0.3 |
| 3.4 | Low-delay using 5 pseudo-random adaptive reference pictures. | 0.2 | 0.2 | 0.3 | 0.4 | 0.3 |
| 3.5 | Low-delay with picture list reordering for the first slice in each picture |  |  |  |  |  |

Simulations for the 3.5 test case were not finished in time before the meeting since a bug was found very close to the start of the meeting.

# List of AHG related input documents

The following contributions related to this AHG were submitted to the current JCT-VC meeting:

JCTVC-I0076 [Hendry, B. Jeon (LG)] ”AHG15: Signalling Long-term Reference Picture Set”

JCTVC-I0087 [Hendry, Y. Jeon, S. Park, B. Jeon (LG), Y. Chen, Y. Wang, W. Chien (Qualcomm)] “Comments on Reference Picture Lists Combination Syntax”

JCTVC-I0112 [K. Suehring, H. Schwarz, T. Wiegand (HHI)] “AHG15: Long-term picture signalling for error-free environments”

JCTVC-I0125 [T. Lee, J. Park (Samsung)] “On Reference List Combination”

JCTVC-I0131 [T. Lee, Y. Park, J. Park (Samsung)] “Syntax reordering of Reference List Modification and Combination”

JCTVC-I0135 [T. Sugio, T. Nishi, S. M. T. Naing, C. S. Lim (Panasonic)] “AHG15: Modification on picture marking process”

JCTVC-I0145 [I.-K Kim, Y. Park, J. H. Kim, J. H. Park (Samsung)] “Delta parameter derivation for inter reference picture set prediction”

JCTVC-I0220 [Y. He, Y. Ye, J. Dong (InterDigital)] “AHG15: Clarification of mapping process for reference picture lists combination in B slices”

JCTVC-I0234 [R. Sjöberg, J. Samuelsson (Ericsson)] ”AHG15: Fix for an unhandled long-term picture case”

JCTVC-I0340 [A. K. Ramasubramonian, Y. -K. Wang, Y. Chen (Qualcomm), C.S. Lim (Panasonic), S. Deshpande (Sharp)] “AHG15: Signalling of long-term reference pictures in the SPS”

JCTVC-I0342 [R. L. Joshi, Y. Chen, Y.-K. Wang, A. K. Ramasubramonian (Qualcomm)] “AHG15: On reference picture set derivation and LTRP signaling in slice header”

JCTVC-I0344 [R. L. Joshi, A. K. Ramasubramonian, Y.-K. Wang, Y. Chen (Qualcomm)] “AHG15: On reference picture set definition and signalling”

JCTVC-I0347 [A. K. Ramasubramonian, Y. Chen, Y.-K. Wang (Qualcomm)] “AHG15: On inter-RPS prediction”

JCTVC-I0348 [A. K. Ramasubramonian, Y. Chen, Y.-K. Wang (Qualcomm)] “AHG15: On reference picture list modification”

JCTVC-I0388 [J. Zhao, S. Deshpande, A. Segall (Sharp)] “AHG15: Parameterized RPS Models”

JCTVC-I0416 [M. Coban (Qualcomm)] “On definition of ref_pic_list_combination_flag”

# Recommendation

The Reference picture buffering and list construction AHG recommends for the JCT-VC to review all picture buffer management and list construction related input documents.