| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0021 |
| --- | --- |

| Title: | CE1: Summary report of Core Experiment on sample adaptive offset filtering |  |  |
| --- | --- | --- | --- |
| Status: | Input Document of JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Yu-Wen Huang Elena Alshina In Suk Chong Wade Wan Minhua Zhou |  | yuwen.huang@mediatek.com elena_a.alshina@samsung.com ichong@qualcomm.com wwan@broadcom.com zhou@ti.com |
| Source: | CE coordinators |  |  |

_____________________________

# Introduction

A summary of Core Experiment 1 (CE1) on sample adaptive offset (SAO) filtering is reported. All proposals are evaluated based on the common test conditions in JCTVC-H1100 [1] and additional conditions according to the CE1 description in JCTVC-H1101 [2]. There were two phases in CE1. The first phase was to test individual proposals. According to results of the first phase, promising proposals including non-CE1 proposals released by March 29, 2012, were chosen for integration in the second phase. Although the second phase was conducted in CE1, it should be regarded as AHG6 activities.

# Document list

## CE1 documents

- JCTVC-I0136 [Panasonic] CE1: Method of removing boundary artifacts for SAO

Crosscheck report: JCTVC-I0399 [Samsung]

Crosscheck report: JCTVC-I0318 [TI]

- JCTVC-I0160 [Samsung, Qualcomm, MediaTek] CE1, Test2.2: Reduced number of edge offsets per LCU

Crosscheck report: JCTVC-I0093 [Cisco]

- JCTVC-I0161 [MediaTek] CE1 Test1.5: Results of forcing chroma off when luma is off in interleaving mode, CE1 Test4.1: Removing run prediction in APS mode, and CE1 Test4.2: Region-based syntax in APS mode

Crosscheck report: JCTVC-I0122 [Panasonic]

Crosscheck report: JCTVC-I0319 [TI]

- JCTVC-I0195 [TI] CE1 Test 4.3 : Removal of pixel access constraints for SAO interleaving mode

- JCTVC-I0196 [TI] CE1 Test 4.4 and 4.5: Removal of SAO merge flags in interleaving mode

Crosscheck report: JCTVC-I0123 [Panasonic]

Crosscheck report: JCTVC-I0242 [Broadcom]

- JCTVC-I0377 [Motorola, Samsung] CE1 Test 5.1/5.2: Modified EO offset operation and coding of offsets

Crosscheck report: JCTVC-I0317 [TI]

Crosscheck report: JCTVC-I0398 [Ericsson]

- JCTVC-I0378 [Motorola, Samsung] CE1 Test 6.1: Harmonization of LCU-based SAO

- JCTVC-I0379 [Motorola, Samsung] CE1 Test 6.2: Coding of SAO type

Crosscheck report: JCTVC-I0481 [Cisco]

## Other CE1-related documents

- JCTVC-I0066 [Mitsubishi] Non-CE1: Improved edge offset coding for SAO

Crosscheck report: JCTVC-I0453 [NTT]

- JCTVC-I0073 [Ericsson] AHG6: SAO simplification

Crosscheck report: JCTVC-I0316 [TI]

- JCTVC-I0130 [Samsung] Simplification on SAO syntax

Crosscheck report: JCTVC-I0211 [MediaTek]

- JCTVC-I0137 [Panasonic] AHG6: Non-CE1: Removing boundary artifacts by weighted offset values for SAO

Crosscheck report: JCTVC-I0400 [Samsung]

- JCTVC-I0162 [Samsung, Qualcomm, MediaTek] Non-CE1, Test2.1: Reduced number of band offsets per LCU

- JCTVC-I0168 [MediaTek] Non-CE1: Bug-fix of offset coding in SAO interleaving mode

Crosscheck report: JCTVC-I0191 [Canon]

- JCTVC-I0171 [Panasonic] Refinement for SAO and ALF syntax in the APS

- JCTVC-I0172 [MediaTek] Non-CE1: Prohibition against merging across slice/tile boundaries in SAO

- JCTVC-I0183 [Canon] Non-CE1: On SAO parameters reduction for chroma

Crosscheck: JCTVC-I0413 [Ericsson]

- JCTVC-I0184 [Canon] Non-CE1: Encoder modification for SAO interleaving mode

Crosscheck: JCTVC-I0401 [Samsung]

- JCTVC-I0185 [Canon] Non-CE1: On SAO fixed offsets

Crosscheck: JCTVC-I0202 [MediaTek]

- JCTVC-I0193 [TI, Qualcomm] Non-CE1: LCU SAO enable flag coding

Crosscheck: JCTVC-I0095 [Cisco]

Crosscheck: JCTVC-I0405 [Samsung]

- JCTVC-I0199 [MediaTek, TI, Qualcomm] Non-CE1: Decoupling SAO on/off from SAO type with neighbor-based contexts

Crosscheck: JCTVC-I0492 [Samsung]

- JCTVC-I0200 [TI] Non-CE1: SAO parameter estimation using non-deblocked pixels in interleaving mode

Crosscheck: JCTVC-I0385 [MediaTek]

- JCTVC-I0246 [Sony] SAO type coding simplification

- JCTVC-I0247 [Sony] Continuity in band offset SAO

Crosscheck: JCTVC-I0431 [Qualcomm]

- JCTVC-I0261 [TI] Non-CE1: Edge offset index coding for LCU-based SAO

Crosscheck: JCTVC-I0096 [Cisco]

- JCTVC-I0292 [Sharp] Non-CE1: Relocation of SAO syntax in interleaving mode

Crosscheck: JCTVC-I0210 [MediaTek]

- JCTVC-I0295 [Samsung] Non-CE1: On positive/negative sign for category 1, 4 edge offset in SAO

Crosscheck report: JCTVC-I0192 [Canon]

- JCTVC-I0380 [Motorola, Samsung] Non-CE1: On SAO syntax unification

- JCTVC-I0384 [MediaTek] Non-CE1: Unified SAO interleaving syntax in APS and slice header

# CE1 activities and SAO-related AHG6 activities

## Phase 1 – Individual proposals

In this subsection, the anchor is CE1 anchor (HM-6.0 SAO-on with non-normative changes). When APS mode is considered, Main-RA and HE10-RA are tested with LCU size equal to 64x64. When interleaving mode is considered, Main-LDB and HE10-LDB are tested with three different LCU sizes (i.e., 64x64, 32x32, and 16x16). Experiments were conducted in parallel.

### CE1 Test1.5.1, JCTVC-I0161 [MediaTek]

Tool description: For interleaving mode, if luma is off in one LCU, two chroma components of the LCU must be off and do not send anything for chroma.

Purpose: Coding efficiency improvement

| Anchor: HM-6.0 SAO interleaving mode with CE1 changes | Overall BD-Rate w/o Class F |  |  | Class F BD-Rate |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y (%) | Cb (%) | Cr (%) | Y (%) | Cb (%) | Cr (%) |
| Main-LDB, LCU=64x64 | -0.4 | 0.9 | 1.1 | -0.5 | 0.7 | 0.4 |
| HE10-LDB, LCU=64x64 | -0.4 | 0.3 | 0.3 | -0.5 | 0.8 | 0.6 |
| Main-LDB, LCU=32x32 | -0.7 | 0.3 | 0.1 | -1.0 | 1.1 | 0.3 |
| HE10-LDB, LCU=32x32 | -0.7 | -0.3 | -0.1 | -0.7 | -0.3 | -0.1 |
| Main-LDB, LCU=16x16 | -1.4 | -1.2 | -1.0 | -1.8 | -1.5 | -0.9 |
| HE10-LDB, LCU=16x16 | -1.4 | -1.2 | -1.2 | -1.9 | -2.0 | -1.8 |

Comment(s) from CE1 members:

- Luma gains are good, but chroma losses are undesirable.

- Some other proposals related to SAO LCU on/off coding seem to be better.

Recommendation: No further action in CE1 before the 9th JCT-VC meeting

### CE1 Test1.5.2, JCTVC-I0161 [MediaTek]

Tool description: For interleaving mode, if luma is off in one LCU, two chroma components of the LCU must be off on the encoder side. CE1 Test1.5.2 is viewed as non-normative CE1 Test1.5.1.

Purpose: Information sharing

| Anchor: HM-6.0 SAO interleaving mode with CE1 changes | Overall BD-Rate w/o Class F |  |  | Class F BD-Rate |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y (%) | Cb (%) | Cr (%) | Y (%) | Cb (%) | Cr (%) |
| Main-LDB, LCU=64x64 | 0.0 | 1.2 | 1.4 | -0.1 | 1.1 | 0.8 |
| HE10-LDB, LCU=64x64 | 0.0 | 0.7 | 0.7 | 0.0 | 1.2 | 1.0 |
| Main-LDB, LCU=32x32 | 0.0 | 0.8 | 0.6 | -0.2 | 1.8 | 1.0 |
| HE10-LDB, LCU=32x32 | 0.0 | 0.3 | 0.5 | 0.1 | 0.4 | 0.6 |
| Main-LDB, LCU=16x16 | 0.0 | 0.0 | 0.2 | 0.0 | 0.1 | 0.7 |
| HE10-LDB, LCU=16x16 | 0.0 | 0.0 | -0.1 | -0.1 | -0.3 | -0.1 |

Comment(s) from CE1 members:

- Forcing chroma off when luma is off in a non-normative way may not a good idea.

Recommendation: No further action in CE1 before the 9th JCT-VC meeting

### CE1 Test2.2.1, JCTVC-I0160 [Samsung, Qualcomm, MediaTek]

Tool description: For interleaving mode, send one explicit edge offset per component per LCU and derive the rest three edge offsets

Purpose: Coding efficiency improvement

| Anchor: HM-6.0 SAO interleaving mode with CE1 changes | Overall BD-Rate w/o Class F |  |  | Class F BD-Rate |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y (%) | Cb (%) | Cr (%) | Y (%) | Cb (%) | Cr (%) |
| Main-LDB, LCU=64x64 | 0.2 | 0.6 | 0.7 | 0.2 | 0.6 | 0.8 |
| HE10-LDB, LCU=64x64 | -0.1 | -0.4 | -0.5 | 0.0 | 0.8 | 0.5 |
| Main-LDB, LCU=32x32 | 0.1 | -0.3 | -0.2 | 0.0 | 0.3 | 0.8 |
| HE10-LDB, LCU=32x32 | -0.3 | -0.2 | -0.1 | -0.4 | -0.5 | 0.3 |
| Main-LDB, LCU=16x16 | -0.1 | 0.3 | 0.3 | -0.1 | -2.7 | -1.0 |
| HE10-LDB, LCU=16x16 | -0.1 | 0.2 | -0.1 | -0.4 | -1.0 | 1.6 |

Comment(s) from CE1 members:

- The loss for 64x64 seems too high.

- The proponent also tested Main/HE10-AI/RA/LP and reported 0.1%, 0.3%, and 0.3% average luma gains for 64x64, 32x32, and 16x16, respectively.

Recommendation: Further study in CE1 phase 2

### CE1 Test2.2.2, JCTVC-I0160 [Samsung, Qualcomm, MediaTek]

Tool description: For interleaving mode, send two explicit edge offsets per component per LCU and derive the rest two edge offsets

Purpose: Coding efficiency improvement

| Anchor: HM-6.0 SAO interleaving mode with CE1 changes | Overall BD-Rate w/o Class F |  |  | Class F BD-Rate |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y (%) | Cb (%) | Cr (%) | Y (%) | Cb (%) | Cr (%) |
| Main-LDB, LCU=64x64 | 0.3 | 1.4 | 1.8 | 0.5 | 1.5 | 1.5 |
| HE10-LDB, LCU=64x64 | -0.1 | 0.3 | 0.3 | -0.1 | 0.8 | 2.1 |
| Main-LDB, LCU=32x32 | 0.0 | 0.1 | 0.1 | 0.2 | 0.8 | 1.3 |
| HE10-LDB, LCU=32x32 | -0.1 | 0.0 | -0.1 | -0.2 | 0.2 | -1.4 |
| Main-LDB, LCU=16x16 | -0.2 | -0.1 | -0.1 | 0.1 | -0.9 | -1.0 |
| HE10-LDB, LCU=16x16 | -0.1 | 0.0 | 0.2 | -0.2 | -1.5 | -1.3 |

Comment(s) from CE1 members:

- The loss for 64x64 seems too high.

- The proponent also tested Main/HE10-AI/RA/LP and reported 0.0%, 0.1%, and 0.2% average luma gains for 64x64, 32x32, and 16x16, respectively.

Recommendation: Further study in CE1 phase 2

### CE1 Test3.1, JCTVC-I0136 [Panasonic]

Tool description: Reduce the offset magnitude by half for BO, which is non-normative and encoder-only.

Purpose: Subjective quality improvement

| Anchor: HM-6.0 SAO APS mode for RA and interleaving mode with CE1 changes for LDB | Overall BD-Rate w/o Class F |  |  | Class F BD-Rate |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y (%) | Cb (%) | Cr (%) | Y (%) | Cb (%) | Cr (%) |
| Main-RA, LCU=64x64 | 0.0 | 0.0 | 0.1 | 0.3 | 0.2 | 0.5 |
| HE10-RA, LCU=64x64 | 0.0 | 0.0 | 0.0 | 0.2 | 0.0 | 0.4 |
| Main-LDB, LCU=64x64 | 0.0 | 0.1 | 0.0 | 0.5 | 0.5 | 0.8 |
| HE10-LDB, LCU=64x64 | 0.0 | 0.0 | 0.0 | 0.1 | 0.8 | 0.1 |
| Main-LDB, LCU=32x32 | 0.0 | 0.1 | 0.1 | 0.6 | 1.5 | 0.8 |
| HE10-LDB, LCU=32x32 | 0.0 | 0.2 | 0.1 | 0.3 | 0.6 | 1.1 |
| Main-LDB, LCU=16x16 | 0.0 | 0.0 | 0.0 | 0.3 | -0.3 | -0.2 |
| HE10-LDB, LCU=16x16 | 0.0 | -0.1 | 0.1 | 0.1 | 0.0 | 0.1 |

Comment(s) from CE1 members:

- The proponent requested to conduct subjective testing for this proposal in the 9th JCT-VC meeting and compare this proposal with JCTVC-I0137.

Recommendation: Further study in the 9th JCT-VC meeting

### CE1 Test3.2, JCTVC-I0136 [Panasonic]

Tool description: Weaken offset values near LCU boundaries for BO.

Purpose: Subjective quality improvement

| Anchor: HM-6.0 SAO APS mode for RA and interleaving mode with CE1 changes for LDB | Overall BD-Rate w/o Class F |  |  | Class F BD-Rate |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y (%) | Cb (%) | Cr (%) | Y (%) | Cb (%) | Cr (%) |
| Main-RA, LCU=64x64 | 0.0 | 0.1 | 0.1 | 0.1 | 0.2 | 0.3 |
| HE10-RA, LCU=64x64 | 0.0 | 0.1 | 0.1 | 0.1 | 0.2 | 0.3 |
| Main-LDB, LCU=64x64 | 0.1 | 0.2 | 0.3 | 0.2 | 0.8 | 0.7 |
| HE10-LDB, LCU=64x64 | 0.0 | 0.2 | 0.1 | 0.1 | 0.9 | 0.3 |
| Main-LDB, LCU=32x32 | 0.1 | 0.1 | 0.0 | 0.3 | 0.9 | -0.4 |
| HE10-LDB, LCU=32x32 | 0.0 | 0.1 | 0.0 | 0.1 | -0.8 | 0.2 |
| Main-LDB, LCU=16x16 | 0.0 | 0.1 | 0.1 | 0.2 | 0.3 | -0.5 |
| HE10-LDB, LCU=16x16 | 0.0 | 0.0 | 0.1 | 0.2 | 0.1 | 0.0 |

Comment(s) from CE1 members:

- The proponent requested to skip subjective testing for this proposal and conduct subjective testing for JCTVC-I0137 in the 9th JCT-VC meeting.

Recommendation: No further action in CE1 before the 9th JCT-VC meeting

### CE1 Test3.3, JCTVC-I0136 [Panasonic]

Tool description: Weaken offset values near band boundaries for BO.

Purpose: Subjective quality improvement

| Anchor: HM-6.0 SAO APS mode for RA and interleaving mode with CE1 changes for LDB | Overall BD-Rate w/o Class F |  |  | Class F BD-Rate |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y (%) | Cb (%) | Cr (%) | Y (%) | Cb (%) | Cr (%) |
| Main-RA, LCU=64x64 | 0.1 | 0.4 | 0.5 | 0.6 | 0.8 | 1.2 |
| HE10-RA, LCU=64x64 | 0.1 | 0.6 | 0.8 | 0.9 | 1.3 | 2.6 |
| Main-LDB, LCU=64x64 | 0.2 | 0.4 | 0.7 | 1.1 | 0.9 | 1.9 |
| HE10-LDB, LCU=64x64 | 0.1 | 0.4 | 0.5 | 1.0 | 1.8 | 1.7 |
| Main-LDB, LCU=32x32 | 0.1 | 0.0 | 0.0 | 0.6 | 0.3 | 0.7 |
| HE10-LDB, LCU=32x32 | 0.0 | 0.1 | 0.1 | 0.6 | 0.3 | 1.6 |
| Main-LDB, LCU=16x16 | 0.0 | 0.1 | 0.2 | 0.3 | 0.1 | -0.3 |
| HE10-LDB, LCU=16x16 | 0.0 | 0.0 | 0.2 | 0.1 | -0.4 | 0.5 |

Comment(s) from CE1 members:

- The proponent requested to skip subjective testing for this proposal and conduct subjective testing for JCTVC-I0137 in the 9th JCT-VC meeting.

Recommendation: No further action in CE1 before the 9th JCT-VC meeting

### CE1 Test3.4, JCTVC-I0136 [Panasonic]

Tool description: Weaken offset values near both LCU and band boundaries for BO.

Purpose: Subjective quality improvement

| Anchor: HM-6.0 SAO APS mode for RA and interleaving mode with CE1 changes for LDB | Overall BD-Rate w/o Class F |  |  | Class F BD-Rate |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y (%) | Cb (%) | Cr (%) | Y (%) | Cb (%) | Cr (%) |
| Main-RA, LCU=64x64 | 0.1 | 0.4 | 0.6 | 0.7 | 0.9 | 1.4 |
| HE10-RA, LCU=64x64 | 0.2 | 0.6 | 0.9 | 0.9 | 1.3 | 2.7 |
| Main-LDB, LCU=64x64 | 0.2 | 0.7 | 1.0 | 1.4 | 1.8 | 2.0 |
| HE10-LDB, LCU=64x64 | 0.2 | 0.4 | 0.5 | 1.2 | 2.3 | 3.4 |
| Main-LDB, LCU=32x32 | 0.2 | 0.1 | 0.3 | 1.0 | 1.3 | 1.7 |
| HE10-LDB, LCU=32x32 | 0.1 | 0.2 | 0.0 | 0.7 | 0.2 | 1.3 |
| Main-LDB, LCU=16x16 | 0.0 | 0.0 | 0.1 | 0.4 | -0.2 | 0.6 |
| HE10-LDB, LCU=16x16 | 0.0 | 0.0 | 0.1 | 0.2 | -0.1 | -0.2 |

Comment(s) from CE1 members:

- The proponent requested to skip subjective testing for this proposal and conduct subjective testing for JCTVC-I0137 in the 9th JCT-VC meeting.

Recommendation: No further action in CE1 before the 9th JCT-VC meeting

### CE1 Test4.1, JCTVC-I0161 [MediaTek]

Tool description: For APS mode, remove run prediction.

Purpose: Complexity reduction

| Anchor: HM-6.0 SAO APS mode | Overall BD-Rate w/o Class F |  |  | Class F BD-Rate |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y (%) | Cb (%) | Cr (%) | Y (%) | Cb (%) | Cr (%) |
| Main-RA, LCU=64x64 | 0.1 | 0.1 | 0.1 | 0.1 | 0.1 | 0.1 |
| HE10-RA, LCU=64x64 | 0.0 | 0.0 | 0.0 | 0.1 | 0.1 | 0.1 |

Comment(s) from CE1 members:

- The loss is small, and removing run prediction can save run line buffer.

Recommendation: Further study in CE1 phase 2

### CE1 Test4.2, JCTVC-I0161 [MediaTek]

Tool description: For APS mode, allow changing SAO parameters per region of MxN pixels, where M and N are signaled and the region size must be larger than or equal to 64x64, 128x128, and 256x256 for levels 1-4.3 (classes B-F), 5-5.2 (class A), 6-6.2, respectively.

Purpose: Complexity reduction

| Anchor: HM-6.0 SAO APS mode | Overall BD-Rate w/o Class F |  |  | Class F BD-Rate |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y (%) | Cb (%) | Cr (%) | Y (%) | Cb (%) | Cr (%) |
| Main-RA, LCU=64x64 | 0.0 | 0.2 | 0.2 | 0.0 | 0.0 | 0.0 |
| HE10-RA, LCU=64x64 | 0.0 | 0.2 | 0.2 | 0.0 | 0.0 | 0.0 |

Comment(s) from CE1 members:

- The loss is small, and this proposal can reduce at least 93.75% offset memory size of one picture.

Recommendation: Further study in CE1 phase 2

### CE1 Test4.3, JCTVC-I0195 [TI]

Tool description: Remove the pixel access constraint for the interleaving mode.

Purpose: Information sharing

| Anchor: HM-6.0 SAO interleaving mode with CE1 changes | Overall BD-Rate w/o Class F |  |  | Class F BD-Rate |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y (%) | Cb (%) | Cr (%) | Y (%) | Cb (%) | Cr (%) |
| Main-LDB, LCU=64x64 | -0.1 | -1.0 | -1.0 | -0.1 | -0.5 | -0.7 |
| HE10-LDB, LCU=64x64 | -0.4 | -1.4 | -1.6 | -0.3 | -0.6 | -0.9 |
| Main-LDB, LCU=32x32 | -0.3 | -1.1 | -1.4 | -1.0 | -1.8 | -3.6 |
| HE10-LDB, LCU=32x32 | -0.3 | -0.3 | -0.5 | -0.9 | -2.4 | -2.0 |
| Main-LDB, LCU=16x16 | -0.3 | 0.0 | -0.1 | -1.5 | -0.9 | -2.1 |
| HE10-LDB, LCU=16x16 | -0.2 | -0.1 | -0.1 | -1.2 | -1.8 | -2.2 |

Comment(s) from CE1 members:

- It is desirable to improve the CE1 encoder-only non-normative pixel access constraint, especially for 16x16.

Recommendation: No further action in CE1 before the 9th JCT-VC meeting

### CE1 Test4.4, JCTVC-I0196 [TI]

Tool description: For interleaving mode, remove merge-up.

Purpose: Complexity reduction

| Anchor: HM-6.0 SAO interleaving mode with CE1 changes | Overall BD-Rate w/o Class F |  |  | Class F BD-Rate |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y (%) | Cb (%) | Cr (%) | Y (%) | Cb (%) | Cr (%) |
| Main-LDB, LCU=64x64 | 0.1 | 0.4 | 0.5 | 0.1 | 0.5 | 0.3 |
| HE10-LDB, LCU=64x64 | 0.1 | 0.3 | 0.3 | 0.3 | 1.0 | 0.7 |
| Main-LDB, LCU=32x32 | 0.1 | 0.0 | 0.0 | 0.3 | 1.4 | 0.2 |
| HE10-LDB, LCU=32x32 | 0.1 | 0.0 | 0.0 | 0.3 | 0.3 | 0.4 |
| Main-LDB, LCU=16x16 | 0.0 | 0.0 | 0.0 | 0.2 | -0.2 | 0.4 |
| HE10-LDB, LCU=16x16 | 0.0 | -0.1 | 0.0 | 0.2 | -1.6 | -1.5 |

Comment(s) from CE1 members:

- Removing merge-up cannot reduce offset line buffer in all implementations.

- Results seem to be different on top of other proposals.

Recommendation: Further study in CE1 phase 2

### CE1 Test4.5, JCTVC-I0196 [TI]

Tool description: For interleaving mode, remove merge-left and merge-up.

Purpose: Complexity reduction

| Anchor: HM-6.0 SAO interleaving mode with CE1 changes | Overall BD-Rate w/o Class F |  |  | Class F BD-Rate |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y (%) | Cb (%) | Cr (%) | Y (%) | Cb (%) | Cr (%) |
| Main-LDB, LCU=64x64 | -0.3 | 0.4 | 0.6 | 0.4 | 0.7 | 1.0 |
| HE10-LDB, LCU=64x64 | -0.1 | 0.2 | 0.2 | 0.8 | 1.4 | 1.3 |
| Main-LDB, LCU=32x32 | -0.4 | -0.3 | -0.1 | 0.6 | 1.6 | 1.0 |
| HE10-LDB, LCU=32x32 | -0.3 | -0.1 | -0.3 | 0.7 | -0.8 | -0.5 |
| Main-LDB, LCU=16x16 | -0.5 | -0.5 | -0.4 | 0.3 | -0.3 | 0.3 |
| HE10-LDB, LCU=16x16 | -0.5 | -0.4 | -0.4 | 0.1 | -1.9 | -1.7 |

Comment(s) from CE1 members:

- Removing merge-up cannot reduce offset line buffer in all implementations.

- Results seem to be different on top of other proposals.

Recommendation: Further study in CE1 phase 2

### CE1 Test5.1, JCTVC-I0377 [Motorola, Samsung]

Tool description: For interleaving mode, four edge offset classes are reduced to only one; offset index equal to zero corresponds to non-zero offset value; pixel processing is described as follows.

- C: current pixel before applying EO
L: one of the two neighbors
R: one of the two neighbors
O: offset
V: current pixel after applying EO

- If 2C>(L+R), I=(L+R)>>1
If 2C<(L+R), I=(L+R+1)>>1
If 2C=(L+R), I=C

- When C!=I,
if O applied to C brings the pixel value away from I, V=C+O
if O applied to C brings the pixel value towards I but does not pass I, V=C+O
if O applied to C brings the pixel value towards I and it passes I, V=I

- When C=I, V=C

Purpose: Coding efficiency improvement

| Anchor: HM-6.0 SAO interleaving mode with CE1 changes | Overall BD-Rate w/o Class F |  |  | Class F BD-Rate |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y (%) | Cb (%) | Cr (%) | Y (%) | Cb (%) | Cr (%) |
| Main-LDB, LCU=64x64 | 0.4 | -0.5 | -0.7 | 0.4 | -0.5 | 0.0 |
| HE10-LDB, LCU=64x64 | -0.2 | -1.7 | -2.0 | -0.8 | -0.5 | -1.2 |
| Main-LDB, LCU=32x32 | 0.4 | -1.4 | -1.7 | 0.0 | -0.2 | -0.9 |
| HE10-LDB, LCU=32x32 | -0.3 | -0.8 | -0.8 | -0.9 | -1.1 | -2.4 |
| Main-LDB, LCU=16x16 | 0.4 | 0.6 | 0.8 | 0.0 | -2.2 | -1.6 |
| HE10-LDB, LCU=16x16 | -0.1 | 0.3 | 0.6 | -0.7 | -1.9 | 0.0 |

Comment(s) from CE1 members:

- This proposal is not mature for 8-bit videos.

- The proponent requested to consider this proposal in phase 2, but there was no other support.

Recommendation: No further action in CE1 before the 9th JCT-VC meeting

Note: The proponent updated their software and results in JCTVC-I0377. This is not regarded as CE1 phase 1 activity because the updated software has not been released or crosschecked.

### CE1 Test5.2, JCTVC-I0377 [Motorola, Samsung]

Tool description: For interleaving mode, four edge offset classes are reduced to only one; offset index equal to zero corresponds to non-zero offset value.

Purpose: Coding efficiency improvement

| Anchor: HM-6.0 SAO interleaving mode with CE1 changes | Overall BD-Rate w/o Class F |  |  | Class F BD-Rate |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y (%) | Cb (%) | Cr (%) | Y (%) | Cb (%) | Cr (%) |
| Main-LDB, LCU=64x64 | 0.1 | -0.6 | -0.7 | 0.0 | -0.1 | -0.2 |
| HE10-LDB, LCU=64x64 | 0.0 | -0.3 | -0.2 | 0.0 | 0.2 | -0.3 |
| Main-LDB, LCU=32x32 | 0.0 | -0.3 | -0.5 | -0.1 | 0.8 | -0.9 |
| HE10-LDB, LCU=32x32 | 0.0 | 0.1 | -0.1 | -0.1 | -0.5 | -1.2 |
| Main-LDB, LCU=16x16 | 0.0 | 0.0 | 0.4 | 0.0 | -0.6 | 0.4 |
| HE10-LDB, LCU=16x16 | 0.0 | 0.1 | 0.1 | -0.1 | -0.1 | -0.8 |

Comment(s) from CE1 members:

- The gains are not very interesting.

- The proponent requested to consider this proposal in phase 2, but there was no other support.

Recommendation: No further action in CE1 before the 9th JCT-VC meeting

### CE1 Test6.1, JCTVC-I0378 [Motorola, Samsung]

Tool description: For interleaving mode, send N offsets in BO (N=4 for luma and N=2 for chroma) where each offset applies to a set of pixels values (offset bands). The N offset bands are derived from the statistics of a subset of pixels inside the LCU, where the subset of pixels are not affected by deblocking.

Purpose: Coding efficiency improvement

| Anchor: HM-6.0 SAO interleaving mode with CE1 changes | Overall BD-Rate w/o Class F |  |  | Class F BD-Rate |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y (%) | Cb (%) | Cr (%) | Y (%) | Cb (%) | Cr (%) |
| Main-LDB, LCU=64x64 | 0.2 | -1.1 | -1.9 | 2.8 | 0.4 | 3.6 |
| HE10-LDB, LCU=64x64 | 0.1 | -1.0 | -1.4 | 0.9 | 0.2 | 2.3 |
| Main-LDB, LCU=32x32 | 0.1 | -1.2 | -1.8 | 1.7 | -0.2 | 1.2 |
| HE10-LDB, LCU=32x32 | 0.0 | -0.4 | -0.6 | 0.4 | -1.5 | 0.2 |
| Main-LDB, LCU=16x16 | 0.0 | -0.1 | 0.1 | 0.4 | -0.5 | -0.1 |
| HE10-LDB, LCU=16x16 | 0.0 | 0.0 | 0.1 | 0.2 | -1.2 | -0.7 |

Comment(s) from CE1 members:

- Luma coding efficiency is sacrificed to achieve chroma gains, and big losses are shown for class F.

- The proponent requested to consider this proposal in phase 2, but there was no other support.

Recommendation: No further action in CE1 before the 9th JCT-VC meeting

### CE1 Test6.2, JCTVC-I0379 [Motorola, Samsung]

Tool description: For interleaving mode, send SAO LCU on/off flag first to select between on and off; then send SAO type selection flag to select between BO and EO.

Purpose: Coding efficiency improvement

| Anchor: HM-6.0 SAO interleaving mode with CE1 changes | Overall BD-Rate w/o Class F |  |  | Class F BD-Rate |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y (%) | Cb (%) | Cr (%) | Y (%) | Cb (%) | Cr (%) |
| Main-LDB, LCU=64x64 | 0.0 | -0.7 | -0.8 | -0.2 | -0.9 | -1.0 |
| HE10-LDB, LCU=64x64 | 0.0 | -0.4 | -0.5 | 0.0 | -0.3 | -0.7 |
| Main-LDB, LCU=32x32 | 0.0 | -0.4 | -0.6 | -0.2 | 0.0 | -1.4 |
| HE10-LDB, LCU=32x32 | 0.0 | -0.1 | -0.2 | -0.1 | -0.4 | -0.7 |
| Main-LDB, LCU=16x16 | -0.1 | 0.0 | -0.1 | -0.2 | -0.6 | -0.9 |
| HE10-LDB, LCU=16x16 | 0.0 | 0.0 | 0.1 | -0.1 | 1.0 | 0.6 |

Comment(s) from CE1 members:

- Gains are small, but the changes are also very small.

- The proponent requested to consider this proposal in phase 2, and one non-proponent showed support.

Recommendation: Further study in CE1 phase 2

### AHG6 Test2, JCTVC-I0261 [TI]

Tool description: For interleaving mode, use an index to represent a pair of edge offsets, which is a kind of parameterized offset coding. Use CABAC for RDO.

Purpose: Coding efficiency improvement

| Anchor: HM-6.0 SAO interleaving mode with CE1 changes | Overall BD-Rate w/o Class F |  |  | Class F BD-Rate |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y (%) | Cb (%) | Cr (%) | Y (%) | Cb (%) | Cr (%) |
| Main-LDB, LCU=64x64 | 0.0 | -1.3 | -1.6 | 0.0 | -0.6 | -0.3 |
| HE10-LDB, LCU=64x64 | -0.2 | -1.0 | -1.1 | -0.3 | 0.3 | -0.1 |
| Main-LDB, LCU=32x32 | -0.1 | -0.6 | -0.9 | -0.2 | 0.9 | 0.0 |
| HE10-LDB, LCU=32x32 | -0.2 | -0.4 | -0.7 | -0.3 | -1.3 | -1.9 |
| Main-LDB, LCU=16x16 | -0.1 | 0.0 | 0.1 | -0.1 | -0.6 | 0.6 |
| HE10-LDB, LCU=16x16 | -0.1 | 0.2 | 0.3 | -0.3 | -1.5 | 1.1 |

Comment(s) from CE1 members:

- Chroma gains are good.

- The index design could be an over-fitting to common test conditions.

Recommendation: Further study in CE1 phase 2

### AHG6 Test8, JCTVC-I0193 [TI]

Tool description: For interleaving mode, first send an index to represent SAO LCU on/off flags of three color components; avoid sending merge-left or merge-up flag when the left or upper neighbor is off.

Purpose: Coding efficiency improvement

| Anchor: HM-6.0 SAO interleaving mode with CE1 changes | Overall BD-Rate w/o Class F |  |  | Class F BD-Rate |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y (%) | Cb (%) | Cr (%) | Y (%) | Cb (%) | Cr (%) |
| Main-LDB, LCU=64x64 | -0.5 | -0.9 | -0.9 | -0.5 | -0.4 | -0.7 |
| HE10-LDB, LCU=64x64 | -0.5 | -0.5 | -0.5 | -0.4 | 0.2 | -0.6 |
| Main-LDB, LCU=32x32 | -0.8 | -0.9 | -1.0 | -0.9 | -0.5 | -1.3 |
| HE10-LDB, LCU=32x32 | -0.8 | -0.7 | -0.7 | -0.8 | -0.7 | -0.6 |
| Main-LDB, LCU=16x16 | -1.5 | -1.3 | -1.2 | -1.7 | -2.0 | -1.6 |
| HE10-LDB, LCU=16x16 | -1.4 | -1.2 | -0.9 | -1.8 | -1.0 | -1.2 |

Comment(s) from CE1 members:

- Both luma and chroma gains are good.

- The index design could be an over-fitting to common test conditions.

Recommendation: Further study in CE1 phase 2

## Phase 2 – Preliminary integration

In this subsection, the anchor is SAO-off. All conditions including Main-AI/RA/LDB/LDP and HE10-AI/RA/LDB/LDP were tested with three different LCU sizes (i.e., 64x64, 32x32, and 16x16). Experiments were conducted sequentially. In the following, stages of phase 2 are described. The outcome of phase 2 is named CE1 Test7.1, which contains an integrated CD and an integrated software package. However, every recommendation in phase 1 and phase 2 should be reviewed in the 9th JCT-VC meeting. Moreover, there are non-CE1 proposals competing with some proposals in phase 2. After all the recommendations of the two phases and the competitive non-CE1 proposals are discussed, CE1 Test7.1 should be modified accordingly in the 9th JCT-VC meeting as a starting point of a BoG on SAO, if the BoG is established by a JCT-VC chair.

### S1: CE1 Test4.2 [MediaTek]

Tool description: For APS mode, allow changing SAO parameters per region of MxN pixels, where M and N are signaled and the region size must be larger than or equal to 64x64, 128x128, and 256x256 for levels 1-4.3 (classes B-F), 5-5.2 (class A), 6-6.2, respectively.

Purpose: Complexity reduction

| Anchor: SAO-off | CE1 SAO (APS) |  |  | S1 (APS) |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y (%) | Cb (%) | Cr (%) | Y (%) | Cb (%) | Cr (%) |
| 64x64, all conditions, all classes | -2.7 | -5.1 | -6.4 | -2.7 | -5.0 | -6.3 |
| 32x32, all conditions, all classes | -2.6 | -5.4 | -6.7 | -2.7 | -5.4 | -6.7 |
| 16x16, all conditions, all classes | -2.4 | -7.3 | -9.4 | -2.5 | -7.2 | -9.3 |

Comment(s) from CE1 members:

- A non-proponent supported to enable this stage for later stages, and nobody objected.

Recommendation: Keep S1 in CE1 Test7.1

### S2: CE1 Test4.1 [MediaTek]

Tool description: For APS mode, remove run prediction.

Purpose: Complexity reduction

| Anchor: SAO-off | S1 (APS) |  |  | S2 (APS) |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y (%) | Cb (%) | Cr (%) | Y (%) | Cb (%) | Cr (%) |
| 64x64, all conditions, all classes | -2.7 | -5.0 | -6.3 | -2.6 | -5.0 | -6.2 |
| 32x32, all conditions, all classes | -2.7 | -5.4 | -6.7 | -2.6 | -5.3 | -6.6 |
| 16x16, all conditions, all classes | -2.5 | -7.2 | -9.3 | -2.5 | -7.2 | -9.2 |

Comment(s) from CE1 members:

- A non-proponent supported to enable this stage for later stages, and nobody objected.

Recommendation: Keep S2 in CE1 Test7.1

### S3: AHG6 Test8 [TI and Qualcomm]

Tool description: For interleaving mode, first send an index to represent SAO LCU on/off flags of three color components; avoid sending merge-left or merge-up flag when the left or upper neighbor is off.

Purpose: Coding efficiency improvement

| Anchor: SAO-off | CE1 SAO (interleaving) |  |  | S3 (interleaving) |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y (%) | Cb (%) | Cr (%) | Y (%) | Cb (%) | Cr (%) |
| 64x64, all conditions, all classes | -2.0 | -2.7 | -3.5 | -2.3 | -3.1 | -4.0 |
| 32x32, all conditions, all classes | -0.7 | 0.0 | -0.3 | -1.3 | -0.5 | -0.8 |
| 16x16, all conditions, all classes | 1.2 | 1.6 | 1.5 | 0.2 | 0.7 | 0.7 |

Comment(s) from CE1 members:

- Coding gains are improved.

- The index design could be an over-fitting to common test conditions.

- A non-proponent supported to enable this stage for later stages, and nobody objected.

Recommendation: Keep S3 in CE1 Test7.1

### S4.1: CE1 Test4.4

Tool description: For interleaving mode, remove merge-up.

Purpose: Complexity reduction

Note(s):

- S4.1 is in parallel with S4.2.

- Only S4.1 or S4.2 can be included in CE1 Test7.1 or both of them cannot be included.

| Anchor: SAO-off | S3 (interleaving) |  |  | S4.1 (interleaving) |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y (%) | Cb (%) | Cr (%) | Y (%) | Cb (%) | Cr (%) |
| 64x64, all conditions, all classes | -2.3 | -3.1 | -4.0 | -2.2 | -2.8 | -3.6 |
| 32x32, all conditions, all classes | -1.3 | -0.5 | -0.8 | -1.1 | -0.4 | -0.6 |
| 16x16, all conditions, all classes | 0.2 | 0.7 | 0.7 | 0.3 | 0.7 | 0.7 |

Comment(s) from CE1 members:

- The coding efficiency loss cannot justify the complexity reduction.

Recommendation: Remove S4.1 from CE1 Test7.1

### S4.2: CE1 Test4.5

Tool description: For interleaving mode, remove merge-left and merge-up.

Purpose: Complexity reduction

Note(s):

- S4.1 is in parallel with S4.2.

- Only S4.1 or S4.2 can be included in CE1 Test7.1 or both of them cannot be included.

| Anchor: SAO-off | S3 (interleaving) |  |  | S4.2 (interleaving) |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y (%) | Cb (%) | Cr (%) | Y (%) | Cb (%) | Cr (%) |
| 64x64, all conditions, all classes | -2.3 | -3.1 | -4.0 | -2.0 | -2.5 | -3.3 |
| 32x32, all conditions, all classes | -1.3 | -0.5 | -0.8 | -0.9 | -0.2 | -0.5 |
| 16x16, all conditions, all classes | 0.2 | 0.7 | 0.7 | 0.4 | 0.6 | 0.6 |

Comment(s) from CE1 members:

- The coding efficiency loss cannot justify the complexity reduction.

Recommendation: Remove S4.2 from CE1 Test7.1

### S5: RDO bug-fix [MediaTek, Motorola, Qualcomm, Samsung, TI]

Tool description: For APS mode, fix rate estimation of an edge offset in RDO, which was signed and should be modified to unsigned; for interleaving mode, fix rate estimation in RDO, which was VLC and should be modified to CABAC.

Purpose: Bug-fix

| Anchor: SAO-off | S2 (APS) |  |  | S5 (APS) |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y (%) | Cb (%) | Cr (%) | Y (%) | Cb (%) | Cr (%) |
| 64x64, all conditions, all classes | -2.6 | -5.0 | -6.2 | -2.6 | -4.9 | -6.2 |
| 32x32, all conditions, all classes | -2.6 | -5.3 | -6.6 | -2.6 | -5.3 | -6.6 |
| 16x16, all conditions, all classes | -2.5 | -7.2 | -9.2 | -2.5 | -7.1 | -9.2 |

| Anchor: SAO-off | S3 (interleaving) |  |  | S5 (interleaving) |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y (%) | Cb (%) | Cr (%) | Y (%) | Cb (%) | Cr (%) |
| 64x64, all conditions, all classes | -2.3 | -3.1 | -4.0 | -2.3 | -3.5 | -4.4 |
| 32x32, all conditions, all classes | -1.3 | -0.5 | -0.8 | -1.4 | -0.8 | -1.2 |
| 16x16, all conditions, all classes | 0.2 | 0.7 | 0.7 | 0.1 | 0.7 | 0.6 |

Comment(s) from CE1 members:

- These are reasonable things to do, even though the coding efficiency improvement is small.

Recommendation: Keep S5 in CE1 Test7.1

### S7: CE1 Test6.2 [Motorola, Samsung]

Tool description: For APS mode and interleaving mode, send SAO type selection flag to select between BO and EO.

Purpose: Coding efficiency improvement

Note: S6 and S8 were moved after S7 because S6 and S8 are competitive.

| Anchor: SAO-off | S5 (APS) |  |  | S7 (APS) |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y (%) | Cb (%) | Cr (%) | Y (%) | Cb (%) | Cr (%) |
| 64x64, all conditions, all classes | -2.6 | -4.9 | -6.2 | -2.6 | -5.1 | -6.4 |
| 32x32, all conditions, all classes | -2.6 | -5.3 | -6.6 | -2.6 | -5.5 | -6.9 |
| 16x16, all conditions, all classes | -2.5 | -7.1 | -9.2 | -2.5 | -7.4 | -9.5 |

| Anchor: SAO-off | S3 (interleaving) |  |  | S7 (interleaving) |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y (%) | Cb (%) | Cr (%) | Y (%) | Cb (%) | Cr (%) |
| 64x64, all conditions, all classes | -2.3 | -3.5 | -4.4 | -2.3 | -3.8 | -4.9 |
| 32x32, all conditions, all classes | -1.4 | -0.8 | -1.2 | -1.4 | -1.0 | -1.2 |
| 16x16, all conditions, all classes | 0.1 | 0.7 | -0.6 | 0.1 | 0.6 | 0.6 |

Comment(s) from CE1 members:

- One new context model is added and does not seem to justify the 0.3% chroma gain.

- Since later stages have been built on top of S7 and related simulation has been launched, in order not to delay later stages, S7 is agreed to be kept for the moment and will be reviewed more carefully in the 9th JCT-VC meeting.

Recommendation: Keep S7 in CE1 Test7.1

### S6: AHG6 Test2

Tool description: For interleaving mode, use an index to represent a pair of edge offsets, which is a kind of parameterized offset coding.

Purpose: Coding efficiency improvement

| Anchor: SAO-off | S7 (interleaving) |  |  | S6 (interleaving) |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y (%) | Cb (%) | Cr (%) | Y (%) | Cb (%) | Cr (%) |
| 64x64, all conditions, all classes | -2.3 | -3.8 | -4.9 | -2.3 | -4.4 | -5.5 |
| 32x32, all conditions, all classes | -1.4 | -1.0 | -1.2 | -1.3 | -1.6 | -2.0 |
| 16x16, all conditions, all classes | 0.1 | 0.6 | 0.6 | 0.6 | 1.3 | 1.1 |

Comment(s) from CE1 members:

- Even though S6 still shows gain for LCU size equal to 64x64, especially for chroma and LDP, the gain might be big enough.

Recommendation: Remove S6 from CE1 Test7.1

### S8.1: CE1 Test2.2.1

Tool description: For interleaving mode, send one explicit edge offset per component per LCU and derive the rest three edge offsets.

Purpose: Coding efficiency improvement

| Anchor: SAO-off | S7 (interleaving) |  |  | S8.1 (interleaving) |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y (%) | Cb (%) | Cr (%) | Y (%) | Cb (%) | Cr (%) |
| 64x64, all conditions, all classes | -2.3 | -3.8 | -4.9 | -2.3 | -3.4 | -4.3 |
| 32x32, all conditions, all classes | -1.4 | -1.0 | -1.2 | -1.4 | -1.2 | -1.4 |
| 16x16, all conditions, all classes | 0.1 | 0.6 | 0.6 | -0.1 | 0.6 | 0.8 |

Comment(s) from CE1 members:

- Not available before this document is written.

Recommendation: Not known

### S8.2: CE1 Test2.2.1 & CE1 Test2.1.1

Tool description: For interleaving mode, send one explicit edge offset per component per LCU and derive the rest three edge offsets, send only one explicit band offset, where the coded band offset can only be nonzero, encode BO sign flag with CABAC bypass mode, both EO and BO magnitude are encoded as unsigned values.

Purpose: Complexity reduction

| Anchor: SAO-off | S7 (interleaving) |  |  | S8.2 (interleaving) |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y (%) | Cb (%) | Cr (%) | Y (%) | Cb (%) | Cr (%) |
| 64x64, all conditions, all classes | -2.3 | -3.8 | -4.9 | -2.2 | -4.1 | -5.2 |
| 32x32, all conditions, all classes | -1.4 | -1.0 | -1.2 | -1.4 | -1.6 | -1.9 |
| 16x16, all conditions, all classes | 0.1 | 0.6 | 0.6 | 0.0 | 0.7 | 0.8 |

Comment(s) from CE1 members:

- Not available before this document is written.

Recommendation: Not known

## Summary of other CE1-related proposals

### JCTVC-I0066 [Mitsubishi]

- For APS mode, replace exponential Golomb codes by truncated unary codes for edge offsets.

Short length results show no BD-rate differences.

- For interleaving mode, replace unary binarization by truncated unary binarization for edge offsets.

Short length results show 0.01-0.08% coding efficiency gain.

### JCTVC-I0073 [Ericsson]

- This proposal reduces memory for offsets by 75% and requires no clipping in SAO filtering process.

- For BO, keep 5-bit band position, double band width, send only one offset for the band indicated by the band position, and the coded offset is always nonzero and signed with a power-of-two magnitude.

The offset is restricted to keep SAO filtered value within allowed range of pixel values.

Compared with CE1 anchor, luma BD-rate is 0%, and chroma BD-rate is -0.3%.

- For EO, restrict offsets to 0 or (1<<(bitDepth-8)) when edgeIdx is equal to 0 or 1, and restrict edge offsets to 0 or –(1<<(bitDepth-8)) when edgeIdx is equal to 3 or 4.

Due to restricted offsets filtering with EO do not overshoot or undershoot edges, e.g. valley cannot turn into a hill.

- When the EO modification is combined with the BO modification, compared with CE1 anchor, luma BD-rate is 0%, and chroma BD-rate is -1.2%.

### JCTVC-I0130 [Samsung]

- For APS mode, removing one unit flag suffers 0-0.1% coding efficiency loss.

- For APS mode, removing repeat row flag suffers 0-0.5% coding efficiency loss.

- For APS mode, removing one unit flag and repeat row flag suffers 0.1-0.7% coding efficiency loss.

### JCTVC-I0137 [Panasonic]

- This proposal is highly related to CE1 Test3.x.

- For BO, add a weighted offset to reconstructed samples along with LCU and band boundaries.

- The proponent requested to skip subjective testing for CE1 Test3.2, CE1 Test3.3, and CE1 Test3.4 and to conduct subjective testing for CE1 Test3.1 and JCTVC-I0137 in the 9th JCT-VC meeting.

### JCTVC-I0162 [Samsung, Qualcomm, MediaTek]

- This proposal is highly related to S6 and S8.

- For interleaving mode, on top of CE1 Test2.2.1 (i.e., send one explicit edge offset per component per LCU and derive the rest three edge offsets), send only one explicit band offset, where the coded band offset can only be nonzero.

- On average, compared with CE1 anchor, luma BD-rate is 0.0%, -0.3%, and -0.4% when the LCU size is 64x64, 32x32, and 16x16, respectively, without averaging class F.

- Class E and class F sometimes have large losses (up to 1.5%) when the LCU size is 64x64.

### JCTVC-I0168 [MediaTek]

- In HM-6.0, after removing signs for edge offsets, CABAC initialization, binarization, and context formation processes for BO are different from those for EO.

- In CD, both BO and EO are described by a same syntax table and share the same CABAC processes.

- Changing CD according to HM-6.0 needs to change hundreds of lines in CD, and nobody fixed it.

- It is proposed to decouple the BO sign from sao_offset, group BO signs of one LCU, code BO signs with bypass coding, and unify BO magnitude with unsigned EO, which changes 8 lines in HM-6.0 and 23 lines in CD.

- Results show no BD-rate differences.

### JCTVC-I0171 [Panasonic]

- This proposal is highly related to CE1 Test4.2.

- For APS mode, change LCU-based signaling to region-based signaling, where the number of regions in picture width and the number of regions in picture height are signaled.

- The concept of this proposal is the same as CE1 Test4.2. Only syntax design is a bit different.

### JCTVC-I0172 [MediaTek]

- Disallow merging across slice or tile boundaries.

### JCTVC-I0183 [Canon]

- Send Cr offsets and derive Cb offsets from Cr offsets

- Compared with CE1 anchor, coding gain is 0.2%, 0.9%, and 1.5% for Y, Cb, and Cr, respectively, when LCU size 64x64.

### JCTVC-I0184 [Canon]

- Non-normative and encoder-only

- Turn on/off the SAO tool for the current picture based on a statistical criterion of the previous picture.

- Can reduce average complexity by skipping SAO encoding processes for some pictures

- Compared with CE1 anchor, the BD-rate is -0.5%/-0.4%/-0.3% for Y/Cb/Cr when the LCU size is 64x64; the BD-rate is -1.6%/-1.5%/-1.4% for Y/Cb/Cr when the LCU size is 16x16.

### JCTVC-I0185 [Canon]

- This proposal is highly related to S6 (i.e., AHG6 Test2, JCTVC-I0261 [TI]).

- Use an index to represent four edge offsets.

- 16 sets of edge offsets are allowed, and 4-bit fixed length coding is used for the index.

- Results of JCTVC-I0185 are very close to those of JCTVC-I0261 and are as follows.

| Anchor: HM-6.0 SAO interleaving mode with CE1 changes | Overall BD-Rate w/o Class F |  |  | Class F BD-Rate |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y (%) | Cb (%) | Cr (%) | Y (%) | Cb (%) | Cr (%) |
| Main-LDB, LCU=64x64 | 0.1 | -1.3 | -1.5 | 0.1 | -0.9 | -0.5 |
| HE10-LDB, LCU=64x64 | -0.2 | -1.0 | -1.3 | -0.3 | -0.1 | -0.5 |
| Main-LDB, LCU=32x32 | 0.1 | -1.0 | -1.2 | -0.1 | -0.4 | -0.6 |
| HE10-LDB, LCU=32x32 | -0.3 | -0.4 | -0.4 | -0.4 | -1.0 | -1.5 |
| Main-LDB, LCU=16x16 | 0.3 | 0.6 | 0.5 | 0.0 | -1.4 | -1.1 |
| HE10-LDB, LCU=16x16 | -0.1 | 0.2 | 0.4 | -0.5 | -1.4 | 2.4 |

### JCTVC-I0199 [MediaTek, TI, Qualcomm]

- This proposal is highly related to S3.

- JCTVC-I0199 was proposed for improving S3. Instead of being first jointly coded with an index, each of the three SAO LCU on/off flags of three color components is coded separately with a neighbor-based context model design before corresponding merge flags are coded. The efficient merge flag coding (i.e., avoid sending merge-left or merge-up flag when the left or upper neighbor is off) in S3 is still applied.

- The index design of three color components’ on/off flags in S3 was trained using common test conditions and might not work well for all sequences in all conditions, while JCTVC-I0199 can let CABAC automatically fit statistics of each slice.

- Average results of Main-AI/RA/LDB/LDP and HE10-AI/RA/LDB/LDP for three different LCU sizes are summarized as follows. Anchor is SAO-off.

|  | S3 |  |  | JCTVC-I0199, 2 ctx models |  |  | JCTVC-I0199, 3 ctx models |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y (%) | Cb (%) | Cr (%) | Y (%) | Cb (%) | Cr (%) | Y (%) | Cb (%) | Cr (%) |
| 64x64 | -2.3 | -3.1 | -4.0 | -2.4 | -3.2 | -4.0 | -2.4 | -3.2 | -4.1 |
| 32x32 | -1.3 | -0.5 | -0.8 | -1.2 | -0.5 | -0.8 | -1.3 | -0.6 | -0.9 |
| 16x16 | 0.2 | 0.7 | 0.7 | 0.7 | 1.2 | 1.2 |  |  |  |

### JCTVC-I0200 [TI]

- Non-normative and encoder-only.

- For interleaving mode, using pre-deblocked pixels of the bottom rows and right columns in a current LCU during parameter estimation.

- Results are as follows.

| Anchor: HM-6.0 SAO interleaving mode with CE1 changes | Overall BD-Rate w/o Class F |  |  | Class F BD-Rate |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y (%) | Cb (%) | Cr (%) | Y (%) | Cb (%) | Cr (%) |
| Main-LDB, LCU=64x64 | 0.0 | -0.2 | -0.3 | 0.0 | -0.2 | -0.3 |
| HE10-LDB, LCU=64x64 | 0.0 | -0.1 | -0.2 | -0.1 | -0.2 | -0.8 |
| Main-LDB, LCU=32x32 | 0.0 | -0.3 | -0.6 | -0.2 | -0.1 | -1.8 |
| HE10-LDB, LCU=32x32 | 0.0 | -0.1 | -0.2 | -0.1 | -0.9 | -2.5 |
| Main-LDB, LCU=16x16 | -0.1 | -0.1 | -0.2 | -0.3 | -0.6 | -1.2 |
| HE10-LDB, LCU=16x16 | 0.0 | 0.0 | 0.1 | 0.0 | -1.3 | -0.4 |

The proponent found a bug in the software and is rerunning simulation now. The above result will be updated.

### JCTVC-I0246 [Sony]

- A 1-bit flag is used to signal the SAO type (i.e. BO or EO).

- The SAO on/off switch is also coded using a 1-bit flag with a separate context.

- The rest of the side information is coded using fixed length codes and bypass mode in CABAC.

- Full results are not yet obtained.

- Preliminary results show very tiny gains (0.01-0.04%).

### JCTVC-I0247 [Sony]

- This proposal is highly related to CE1 Test3.x.

- Reduce offset differences at band boundaries.

- Reduce offset differences at LCU boundaries.

- The claimed visual quality improvement has to be verified by subjective testing.

- BD-rate results are as follows.

| Anchor: HM-6.0 SAO APS mode for RA and interleaving mode with CE1 changes for LDB | Overall BD-Rate w/o Class F |  |  | Class F BD-Rate |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y (%) | Cb (%) | Cr (%) | Y (%) | Cb (%) | Cr (%) |
| Main-RA, LCU=64x64 | -0.1 | -0.1 | -0.1 | -0.1 | 0.0 | 0.0 |
| HE10-RA, LCU=64x64 | 0.0 | -0.1 | 0.0 | 0.0 | 0.0 | 0.2 |
| Main-LDB, LCU=64x64 | 0.0 | 0.1 | 0.3 | -0.2 | 0.2 | 0.3 |
| HE10-LDB, LCU=64x64 | 0.0 | -0.2 | -0.1 | -0.1 | 0.5 | -0.2 |
| Main-LDB, LCU=32x32 | 0.0 | 0.1 | -0.1 | -0.2 | 0.4 | 0.3 |
| HE10-LDB, LCU=32x32 | 0.0 | 0.2 | 0.0 | 0.0 | -0.3 | 1.0 |
| Main-LDB, LCU=16x16 | 0.0 | 0.0 | 0.0 | 0.0 | -0.1 | -0.4 |
| HE10-LDB, LCU=16x16 | 0.0 | -0.1 | 0.1 | 0.0 | 0.5 | 0.8 |

### JCTVC-I0292 [Sharp]

- For interleaving mode, move SAO data from the beginning of each LCU to the end of each LCU.

- Not compatible with fine granularity slices (i.e., non-LCU-aligned slices)

### JCTVC-I0295 [Samsung]

- Propose to add signs back for edge offsets in order to allow sharpening.

- An additional offset restriction is applied to avoid salt and pepper noise.

- Almost no change in BD-rate

- Reported 2-6% decoding time reduction

### JCTVC-I0380 [Motorola, Samsung]

- Non-CE1: On SAO syntax unification

- This proposal was not uploaded before this document was written.

### JCTVC-I0384 [MediaTek]

- Send SAO enable flag before interleaving flag in both APS and slice header

# References

- F. Bossen, “Common test conditions and software reference configurations,” Document of Joint Collaborative Team on Video Coding, JCTVC-H1100, February, 2012.

- Y.-W. Huang, E. Alshina, I. S. Chong, W. Wan, M. Zhou, “Description of Core Experiment 1 (CE1): Sample adaptive offset filtering,” Document of Joint Collaborative Team on Video Coding, JCTVC-H1101, February, 2012.