| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0022 WG11 Number: m25054 |
| --- | --- |

| Title: | CE2: Summary report of Core Experiment on Adaptive Loop Filtering |  |  |
| --- | --- | --- | --- |
| Status: | Input Document of JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Tomoo Yamakage1 Madhukar Budagavi2 In Suk Chong3 Matthias Narroschke4 Yu-Wen Huang5 |  | tomoo.yamakage@toshiba.co.jp madhukar@ti.com ichong@qualcomm.com Matthias.Narroschke@eu.panasonic.com yuwen.huang@mediatek.com |
| Source: | CE coordinators (Toshiba1, TI2, Qualcomm3, Panasonic4, MediaTek5) |  |  |

_____________________________

# Introduction

A summary of core experiment 2 (CE2) on Adaptive Loop Filtering (ALF) is reported. There are nine Subtests in CE2 as follows:

CE2.a: Signalling mode change from "switch between APS mode and slice header mode" to "switch between APS mode and interleaving mode"

CE2.b: Simplification of ALF signalling

CE2.c: Number of classes in BA mode

CE2.d: Number of regions in RA mode

CE2.e: 	Depth of LCU division for filtering control

CE2.f: 	Line memory reduction fix at independent slice and tile boundaries

CE2.g: Encoder complexity reduction for LCU-based optimization

CE2.h: Reuse luma filter coefficients in chroma filtering for LCU-based optimization

CE2.i: Combination subtests

These are evaluated based on the common test conditions in JCTVC-H1100 and additional conditions in JCTVC-H1102_r1.

Informal subjective viewing for CE2.f will be conducted during this Geneva meeting.

# Document list and cross-check assignment

- Summary document (this document)

  - JCTVC-I0022 [CE coordinators] CE2: Summary report of Core Experiment on Adaptive Loop Filtering

- Proposals and verification/study documents

Subtest a: Signalling mode change from "switch between APS mode and slice header mode" to "switch between APS mode and interleaving mode"

  - JCTVC-I0249 [Qualcomm/MediaTek/Toshiba] CE2.a.1: Signalling mode change from slice header mode to interleaving mode

    - Cross-check: JCTVC-I0243 [Broadcom]

  - JCTVC-I0042 [Toshiba /MediaTek/Qualcomm ] CE2.a.2: Additional signalling of picture-level ALF coefficients on top of CE2.a.1 (LCU interleaved signalling)

    - Cross-check: JCTVC-I0222 [TI]

Subtest b: Simplification of ALF signalling

  - JCTVC-I0173 [MediaTek] CE2.b.1: Results of removing run prediction in picture-based optimization

    - Cross-check: JCTVC-I0089 [Cisco]

  - JCTVC-I0248 [Qualcomm/MediaTek/Toshiba] CE2.b.2: Removing explicit K table signaling

    - Cross-check: JCTVC-I0089 [Cisco]

  - JCTVC-I0038 [Toshiba] CE2.b.3: Necessity of alf_pred_flag

    - Cross-check: JCTVC-I0497 [Nokia]

  - JCTVC-I0039 [Toshiba] CE2.b.4 Necessity of alf_nb_pred_luma_flag

    - Cross-check: JCTVC-I0497 [Nokia]

  - JCTVC-I0040 [Toshiba] CE2.b.5: Qp dependent quantization of ALF coefficients

    - Cross-check: JCTVC-I0241 [Broadcom]

Subtest c: 	Number of classes in BA mode

  - JCTVC-I0250 [Qualcomm] CE2.c.1-1: Reducing number of filters of BA (Encoder only)

    - Cross-check: JCTVC-I0098[Toshiba], JCTVC-I0365 [Sharp], JCTVC-I0089[Cisco]

  - JCTVC-I0212 [Samsung] CE2.c.1-2: Reduction of number of filter classes in ALF BA mode, by changing class mapping table and using HM6.0 encoding algorithm

    - Cross-check: JCTVC-I0365 [Sharp]

  - JCTVC-I0054 [NTT] CE2.c.2: Reduction of number of filter classes in BA mode with having flexible directional features

    - Cross-check: JCTVC-I0064 [Mitsubishi]

Subtest d: Number of regions in RA mode

  - JCTVC-I0041 [Toshiba] CE2.d.1: Number of regions in RA mode

    - Cross-check: JCTVC-I0240 [Broadcom]

Subtest e: Depth of LCU division for filtering control

  - JCTVC-I0213 [Samsung] CE2.e.1 Reduction of depth of LCU division for filtering control

    - Cross-check: JCTVC-I0099 [Toshiba]

Subtest f: Line memory reduction fix at independent slice and tile boundaries

  - JCTVC-I0167 [Panasonic] CE2.f: Line memory reduction fix at tile boundaries

    - Cross-check: JCTVC-I0223 [TI]

Subtest g: 	Encoder complexity reduction for LCU-based optimization

  - JCTVC-I0173 [MediaTek] CE2.g.1: Applying pixel access constraint in LCU-based optimization

    - Cross-check: JCTVC-Ixxxx [Qualcomm]

Subtest h: 	Reuse luma filter coefficients in chroma filtering for LCU-based optimization

  - JCTVC-I0173 [MediaTek]CE2.h.1: Reusing luma filter coefficients for chroma in LCU-based optimization

    - Cross-check: JCTVC-I0089 [Cisco]

Subtest i: Combination subtests

  - JCTVC-I0089 [Cisco] CE2: Results for CE2.b.1, CE2.b.2, CE2.c.1, CE2.h.1, CE2.i.1

    - Cross-check: JCTVC-I0100 [Toshiba]

# Experimental results

Based on the common test conditions in JCTVC-H1100, CE2 specified test conditions as described in JCTVC-H1102_r1. Results of CE2 are evaluated on the basis of BD-measures and/or complexity.

## Subtest a: Signalling mode change from "switch between APS mode and slice header mode" to "switch between APS mode and interleaving mode"

### Modification points and results

In CD/HM6, there are two signalling mode about ALF as shown in the following figure:

In CE2.a, the following signalling modes are tested to replace “slice header mode”:

The syntax of CE2.a.1 is similar to CTVC-H0066. The syntax of CE2.a.2 is allowing additional signalling ALF coefficients in APS, where the coefficients signalled in APS are designed using previous frame’s coding results in order to provide low delay operation.

Please note that the unification of “APS mode” and “slice header mode” has been studied in AHG6.

The following table summarizes the coding results (Y BD-rate, encoding time and decoding time in %) for LCU-based encoding in HE10 configuration (without Class F):

| Proposal | HE10-AI | HE10-RA | HE10-LDB | HE10-LDP |
| --- | --- | --- | --- | --- |
| CE2.a.1 | 0.0/98/100 | 0.0/98/100 | -0.1/97/100 | -0.2/97/100 |
| CE2.a.2 (16) | -0.4/101/102 | -0.7/100/102 | -0.3/100/102 | -0.6/100/102 |
| CE2.a.2 (1) | -0.1/101/100 | -0.3/100/101 | -0.1/100/101 | -0.3/100/101 |

### Comments from cross-checkers

- CE2.a.1 (JCTVC-I0249) :

  - Cross-check: JCTVC-I0243 [Broadcom]

    - The cross-checker’s results completely match with those of proponents. It is recommended to interleave ALF control flags in each LCU and remove merge left and merge up flags.

- CE2.a.2 (JCTVC-I0042) :

  - Cross-check: JCTVC-I0222 [TI]

### Recommendation

Either syntax structures of CE2.a.1, CE2.a.2, or Option2 studied in AHG6 are recommended to replace the current syntax structure of slice header mode.

Further study in AHG6 is also recommended for variants of syntax structure.

In addition, it is recommended to study unification of syntax structure of APS mode and slice header mode in AHG6.

## Subtest b: Simplification of ALF signalling

### Modification points and results

CE2.b.1 studied removal of run prediction in picture-based optimization (APS mode).

CE2.b.2 studied removal of explicit K table signaling.

CE2.b.3 studied necessity (removal) of alf_pred_flag that enables prediction between filter coefficient sets when signalling more than one filter.

CE2.b.4 studied necessity (removal) of alf_nb_pred_luma_flag that enables prediction of the (N-1)-th coefficient from other coefficients of a filter.

CE2.b.5 studied the effectiveness of Qp dependent quantization of ALF coefficients.

The following table summarizes the coding results (Y BD-rate, encoding time and decoding time in %) in HE10 configuration (without Class F):

| Proposal | HE10-AI | HE10-RA | HE10-LDB | HE10-LDP |
| --- | --- | --- | --- | --- |
| CE2.b.1 (APS) | 0.0/100/100 | 0.0/100/100 | 0.0/100/100 | 0.0/99/98 |
| CE2.b.2 (APS) | 0.0/100/ | 0.0/100/ | 0.0/100 | 0.1/100/ |
| CE2.b.2 (slice) | 0.0/99/ | 0.0/100/ | 0.0/99 | 0.0/99 |
| CE2.b.3 (0, APS) | 0.0/100/100 | 0.0/101/100 | 0.0/101/100 | 0.0/101/101 |
| CE2.b.3 (0, slice) | 0.0/99/100 | 0.0/100/100 | 0.0/101/100 | 0.0/100/101 |
| CE2.b.3 (1, APS) | 0.0/100/100 | 0.0/101/100 | 0.0/101/100 | 0.0/101/101 |
| CE2.b.3 (1, slice) | 0.0/99/100 | 0.0/100/100 | 0.0/101/100 | 0.0/100/101 |
| CE2.b.4 (0, APS) | 0.0/100/100 | 0.0/100/100 | 0.0/100/100 | 0.0/100/101 |
| CE2.b.4 (0, slice) | 0.0/100/100 | 0.0/100/100 | 0.0/100/100 | 0.0/100/101 |
| CE2.b.4 (1, APS) | 0.0/100/100 | 0.0/100/100 | 0.0/100/100 | 0.0/100/101 |
| CE2.b.4 (1, slice) | 0.0/100/100 | 0.0/100/100 | 0.0/100/100 | 0.0/100/101 |
| CE2.b.5 (APS) | 0.0/101/100 | 0.0/100/101 | 0.0/100/101 | 0.1/100/101 |
| CE2.b.5 (slice) | 0.0/100/99 | 0.1/100/100 | 0.0/100/100 | 0.3/100/100 |

Note: In CE2.b.4 results, “0” means “with prediction” and “1” means “without prediction”, which is consistent behavior with HM6 (but inconsistent with CD).

### Comments from cross-checkers

- CE2.b.1 (JCTVC-I0173):

  - Cross-check: JCTVC-I0089 [Cisco]

    - Coding results are exactly matched, but runtime has some discrepancy.

- CE2.b.2 (JCTVC-I0248):

  - Cross-check: JCTVC-I0089 [Cisco]

    - Coding results are exactly matched, but runtime has some discrepancy.

- CE2.b.3 (JCTVC-I0038):

  - Cross-check: JCTVC-I0497 [Nokia]

- CE2.b.4 (JCTVC-I0039):

  - Cross-check: JCTVC-I0497 [Nokia]

- CE2.b.5 (JCTVC-I0241):

  - Cross-check: JCTVC-I0241 [Broadcom]

    - The cross-checker’s results completely match with those of the proponent. Since the loss of using universal 8 bit precision is negligible and the extra complexity introduced by the QP dependence in both parsing and filtering, it is recommended to remove QP dependent quantization of ALF coefficients.

### Recommendation

All simplification is recommended for adoption as follows:

According to CE2.b.1 results, remove run prediction in picture-based optimization (APS mode).

According to CE2.b.2 results, remove explicit K table signaling and use implicit K table.

According to CE2.b.3 results, remove alf_pred_flag and no prediction between filter coefficient sets.

According to CE2.b.4 results, remove alf_nb_pred_luma_flag and noprediction of the (N-1)-th coefficient from other coefficients of a filter.

According to CE2.b.5 results, ALF coefficients are quantized independently from Qp value.

## Subtest c: Number of classes in BA mode

### Modification points and results

CE2.c.1-1 studied the reduction of number of filter coefficients in non-normative means (i.e., encoder restriction).

CE2.c.1-2 studied the reduction of number of filter coefficients in normative means (i.e., specification change). In addition, additional results by reducing computation for filter classification are provided.

CE2.c.2 studied the reduction of number of filter coefficients in normative means (i.e., specification change), where the number is adaptively selected from 1, 4, or 6.

The following table summarizes the coding results (Y BD-rate, encoding time and decoding time in %) for picture-based encoding in HE10 configuration (without Class F):

| Proposal | HE10-AI | HE10-RA | HE10-LDB | HE10-LDP |
| --- | --- | --- | --- | --- |
| CE2.c.1-1 (10) | 0.0/100 | 0.1/98 | 0.1/99 | 0.3/100 |
| CE2.c.1-1 (7) | 0.0/100 | 0.1/100 | 0.1/99 | 0.3/100 |
| CE2.c.1-1 (4) | 0.1/100 | 0.2/99 | 0.2/99 | 0.5/100 |
| CE2.c.1-1 (1) | 0.5/98 | 0.9/99 | 0.7/99 | 1.4/100 |
| CE2.c.1-2 (7) | 0.1/96/93 | 0.1/97/94 | 0.1/97/93 | 0.2/99/96 |
| CE2.c.1-2 w/ HV3 (7) | 0.2/96/93 | 0.3/97/94 | 0.2/97/92 | 0.3/99/96 |
| CE2.c.1-2 (4) | 0.1/96/95 | 0.2/98/97 | 0.1/98/96 | 0.2/100/100 |
| CE2.c.1-2 w/ HV3 (4) | 0.2/97/96 | 0.3/98/97 | 0.2/98/96 | 0.4/100/100edlts bym 1, 4, or 7nve means (i.e.,ructure of CE2.a.2.syntax structure. lter scoefficients signalled in APS are desi |
| CE2.c.2 (1, 4, or 6) | 0.0/101/100 | 0.3/100/100 | 0.0/99/101 | 0.0/99/101 |

Note: HE10-LDP results of CE2.c.2 contain Class F results.

### Comments from cross-checkers

- CE2.c.1-1 (JCTVC-I0250):

  - Cross-check: JCTVC-I0098[Toshiba]

    - The cross-checker’s results for BA10 perfectly match with those of software provider.

  - Cross-check: JCTVC-I0365 [Sharp]

    - The cross-checker’s results for BA7 perfectly match with those of software provider.

  - Cross-check: JCTVC-I0089[Cisco]

    - The cross-checker’s results for BA4 and no BA perfectly match with those of software provider.

  - Cross-check: JCTVC-Ixxxx[Sony]

  - Cross-check: JCTVC-Ixxxx[Samsung]

- CE2.c.1-2 (JCTVC-I0212):

  - Cross-check: JCTVC-I0365 [Sharp]

    - The cross-checker’s results for BA7 with HV3 perfectly match with those of software provider.

  - Cross-check: JCTVC-Ixxxx[Qualcomm]

- CE2.c.2 (JCTVC-I0054):

  - Cross-check: JCTVC-I0064 [Mitsubishi]

    - It has been confirmed that objective R-D performance produced in this work perfectly matches the results from the proponent. Similar tendency on encoding and decoding time was also observed.

### Recommendation

Decision should be made based on the further study of the Option 2 syntax in AHG6.

## Subtest d: Number of regions in RA mode

### Modification points and results

This subtest studied the effectiveness of region adaptive (RA) mode by dividing a picture up to 1/16 size or 1/64 size by replacing the signalling scheme of block adaptive (BA) in CD/HM6. In this subtest, number of filters in each region is forced to be 1 and syntax related to BA is removed.

The following table summarizes the coding results (Y BD-rate, encoding time and decoding time in %) for picture-based encoding in HE10 configuration (without Class F):

| Proposal | HE10-AI | HE10-RA | HE10-LDB | HE10-LDP |
| --- | --- | --- | --- | --- |
| CE2.d.1 (RA16) | 0.3/101/100 | 0.6/100/99 | 0.4/100/99 | 0.7/100/99 |
| CE2.d.1 (RA64) | 0.3/100/99 | 0.5/100/99 | 0.3/100/98 | 0.4/100/98 |

### Comments from cross-checkers

- CE2.d.1 (JCTVC-I0041):

  - Cross-check: JCTVC-I0240 [Broadcom]

    - The BD rate results match the ones reported by proponents for RA64. One possible reason for the loss is that the regions are specified by run coding and copy row flag.

### Recommendation

In AHG6, decision should be made based on the further study of the Option 2 syntax and implementation of RA syntax similar to WD5/HM5.

## Subtest e: Depth of LCU division for filtering control

### Modification points and results

This subtest investigated reduction of LCU quadtree division depth for ALF filtering control, where PIC-mode of ALF in CD/HM6 has filtering on/off control up to the 3rd-level of LCU quadtree division depth.

The following table summarizes the coding results (Y BD-rate, encoding time and decoding time in %) for picture-based encoding in HE10 configuration (without Class F):

| Proposal | HE10-AI | HE10-RA | HE10-LDB | HE10-LDP |
| --- | --- | --- | --- | --- |
| CE2.e.1 (depth=2) | 0.0/97/99 | 0.0/99/99 | 0.0/99/100 | 0.0/99/101 |
| CE2.e.1 (depth=1) | 0.0/94/100 | 0.0/99/100 | 0.0/99/101 | 0.0/98/101 |
| CE2.e.1 (depth=0) | 0.0/91/97 | 0.1/97/99 | 0.1/98/101 | 0.1/97/101 |

### Comments from cross-checkers

- CE2.e.1 (JCTVC-I0213):

  - Cross-check: JCTVC-I0099 [Toshiba]

    - Coding efficiency results are exactly matched with the proponent’s results and relative encoding and decoding time are similar to those of proponent’s for maximum depth of 0 and 2.

  - Cross-check: JCTVC-I0099 [Qualcomm]

### Recommendation

LCU level control instead of CU level control is recommended.

## Subtest f: Line memory reduction fix at independent slice and tile boundaries

### Modification points and results

The proposal can be described with the following rule:

“If the loop_filter_across_tile_flag is set to 0, Virtual Boundary (VB) padding is not applied at the horizontal tile boundaries in ALF operation.”

According to the CD text, the VB padding is not applied at the bottom frame boundary. Therefore with the proposal the operation at bottom tile boundary is aligned with the operation at the bottom frame boundary. The proposal eliminates the double padding at tile boundaries for the ALF process. Note that there is no line memory requirement at horizontal tile boundaries. Therefore switching VB padding off at horizontal tile boundaries does not increase the hardware complexity.

The following table summarizes the coding results (Y BD-rate, encoding time and decoding time in %) for picture-based encoding in HE10 configuration (without Class F):

| Proposal | HE10-AI | HE10-RA | HE10-LDB | HE10-LDP |
| --- | --- | --- | --- | --- |
| CE2.f.1 | -0.1/101/100 | -0.1/100/101 | -0.1/103/95 | -0.1/105/100 |

### Comments from cross-checkers

- CE2.f.1 (JCTVC-I0167):

  - Cross-check: JCTVC-I0223 [TI]

### Recommendation

Consider this proposal for adoption after the subjective viewing during this Geneva meeting.

## Subtest g: Encoder complexity reduction for LCU-based optimization

### Modification points and results

The bottom four luma pixel rows, the bottom two chroma pixel rows, the rightmost five luma pixel columns, and the rightmost three chroma pixel columns in each LCU are not used during ALF coefficient derivation for slice header mode (LCU-based optimization).

The following table summarizes the coding results (Y BD-rate, encoding time and decoding time in %) for LCU-based encoding in HE10 configuration (without Class F):

| Proposal | HE10-AI | HE10-RA | HE10-LDB | HE10-LDP |
| --- | --- | --- | --- | --- |
| CE2.g.1 | 0.1/100/97 | 0.2/100/99 | 0.2/99/97 | 0.3/100/93 |

### Comments from cross-checkers

- CE2.g.1 (JCTVC-I0173):

  - Cross-check: JCTVC-Ixxxx [Qualcomm]

### Recommendation

No action taken if Option 2 in the AHG6 study is decided to replace the current slice header mode.

## Subtest h: Reuse luma filter coefficients in chroma filtering for LCU-based optimization

### Modification points and results

alf_chroma_coeff_from_luma_flag is added to indicate whether luma filter coefficients are reused in chroma filtering or not. When alf_chroma_coeff_from_luma_flag is equal to zero, chroma filter coefficients should be decoded. If alf_chroma_coeff_from_luma_flag is equal to one, luma filter coefficients are used in chroma filtering and no other information should be decoded for chroma ALF. Note that when luma ALF is disabled, alf_chroma_coeff_from_luma_flag is inferred as zero.

The following table summarizes the coding results (Y BD-rate, encoding time and decoding time in %) for LCU-based encoding in HE10 configuration (without Class F):

| Proposal | HE10-AI | HE10-RA | HE10-LDB | HE10-LDP |
| --- | --- | --- | --- | --- |
| CE2.h.1 | 0.1/99/100 | 0.1/100/100 | 0.1/100/100 | 0.2/100/101 |

### Comments from cross-checkers

- CE2.h.1 (JCTVC-I0173):

  - Cross-check: JCTVC-I0089 [Cisco]

    - The cross-checker’s results perfectly match with those of software provider.

### Recommendation

No action taken if Option 2 in the AHG6 study is decided to replace the current slice header mode.

This subtest is related to Options 1 and 3.  If Option 1 or 3 is recommended by AHG6, it is recommended to adopt CE2.h, since chroma provides 0.7% to 3.1% gain which means overall gain (Y/U/V) seems “gain”.

## Subtest i: Combination subtests

### Modification points and results

All signalling and signal processing modification in JCTVC-H0066 are tested. These modifications include combination of CE2.a.1, CE2.b.1-4, CE2.c.1, CE2.e.1 and CE2.h.1 conceptually.

The following table summarizes the coding results (Y BD-rate, encoding time and decoding time in %) for LCU-based encoding in HE10 configuration (without Class F):

| Proposal | HE10-AI | HE10-RA | HE10-LDB | HE10-LDP |
| --- | --- | --- | --- | --- |
| CE2.i.1 | 0.8/77/74 | 1.0/108/99 | 0.4/108/100 | 0.8/113/99 |

### Comments from cross-checkers

- CE2.i.1 (JCTVC-I0089):

  - Cross-check: JCTVC-I0100 [Toshiba]

    - Coding efficiency results are exactly matched. Relative encoding and decoding time are different (20% or more discrepancy) in some cases, however, it would be due to the test platform issue.

### Recommendation

No action taken, however, the concept of simplification and signalling should be further studied in AHG6.

# References

[1]	“Common test conditions and software reference configurations,” JCTVC-H1100, San Jose, CA, US, February 2012.

[2]	“Description of CE2: Adaptive Loop Filtering”, JCTVC-H1102_r1, San Jose, CA, US, February 2012.