| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0031 |
| --- | --- |

| Title: | AHG5- Coefficient coding – significance map clean-up |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Gergely Korodi, Dake He, Gaëlle Martin-Cocher | Tel: Email: | gkorodi @ rim . com |
| Source: | Research In Motion Ltd |  |  |

_____________________________

# Abstract

This proposal provides a simplification of HM6 significance map coding by applying the HM6 4x4 luma context assignment table to 4x4 chroma and to 8x8 luma and chroma. The design is simplified by using a single table instead of three in the current HM6.  Initialisation values have not been changed. An additional context is used for 8x8 DC coefficient. BD Rate changes for normal QP are 0.02% for Intra and Intra10, 0.01% for RA and RA10, 0.03% for LD and -0.02% for LD10. Low QP results are also provided with a worst case average at 0.02%

# A single context assignment table for 4x4 and 8x8 luma&chroma

During the break-out on coefficient coding [1], a desire to simplify and harmonise significance map coding across TU sizes was expressed, while keeping changes to a minimum in order to consolidate the WD/CD text and software.

One aspect where simplification and harmonization was sought is the context assignment for small TU sizes. The desire to keep position based coding for small TUs in order to deal with the worst case scenario was noted as part of the BoG discussion.

In current HM6, context assignment is defined by look-up tables, one for 4x4 luma, one for 4x4 chroma and one for 8x8 luma & chroma.

This proposal uses the current HM6 4x4 context assignment table and applies it to 4x4 chroma and 8x8 luma & chroma. For the 8x8 cases the table is up-sampled, and an additional context is used for the DC (top-left) coefficient. The benefits of the proposal are the harmonisation and simplification of context assignment for small TUs with a single table instead of 3 in HM6 and virtually no BD rate losses. The drawback is the use of one more context than in HM-6.0.

# Results

## Common condition results with HM6 initialisation values

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A (8bit) | 0.01% | 0.01% | 0.00% | 0.01% | -0.03% | -0.05% |
| Class B | 0.02% | -0.01% | 0.01% | 0.02% | -0.04% | -0.03% |
| Class C | 0.02% | 0.03% | 0.07% | 0.01% | 0.01% | 0.03% |
| Class D | 0.04% | 0.09% | 0.07% | 0.02% | 0.02% | 0.12% |
| Class E | 0.02% | -0.02% | 0.03% | 0.02% | -0.07% | 0.10% |
| Overall | 0.02% | 0.02% | 0.04% | 0.02% | -0.02% | 0.03% |
|  | 0.02% | 0.03% | 0.04% | 0.02% | -0.01% | 0.03% |

|  | Random Access Main |  |  | Random Access HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A (8bit) | 0.00% | 0.03% | -0.01% | 0.00% | 0.08% | 0.12% |
| Class B | 0.02% | 0.00% | 0.13% | 0.01% | 0.01% | -0.06% |
| Class C | 0.01% | 0.15% | 0.18% | 0.01% | 0.04% | 0.09% |
| Class D | 0.01% | 0.25% | 0.32% | 0.03% | 0.40% | 0.33% |
| Class E |  |  |  |  |  |  |
| Overall | 0.01% | 0.11% | 0.17% | 0.01% | 0.13% | 0.11% |
|  | 0.01% | 0.11% | 0.14% | 0.01% | 0.15% | 0.09% |

|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.02% | 0.06% | 0.21% | 0.00% | 0.02% | 0.50% |
| Class C | 0.04% | 0.29% | 0.03% | 0.02% | 0.42% | 0.26% |
| Class D | 0.00% | 0.74% | 0.57% | -0.09% | 0.39% | 0.21% |
| Class E | 0.07% | -0.33% | 0.58% | -0.01% | -0.20% | 0.29% |
| Overall | 0.03% | 0.21% | 0.32% | -0.02% | 0.17% | 0.33% |
|  | 0.03% | 0.19% | 0.36% | -0.02% | 0.18% | 0.36% |

## Low QP results with HM6 initialisation values

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.00% | 0.01% | 0.01% | 0.00% | -0.01% | 0.01% |
| Class B | 0.05% | 0.05% | 0.05% | 0.06% | 0.03% | 0.02% |
| Class C | 0.01% | 0.03% | 0.02% | 0.01% | 0.01% | -0.01% |
| Class D | 0.01% | 0.01% | 0.01% | 0.01% | -0.01% | -0.02% |
| Class E | 0.01% | -0.03% | 0.01% | 0.01% | -0.06% | -0.04% |
| Overall | 0.02% | 0.02% | 0.02% | 0.02% | 0.00% | -0.01% |
|  | 0.02% | 0.03% | 0.02% | 0.02% | 0.00% | 0.00% |

|  | Random Access Main |  |  | Random Access HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.00% | 0.04% | 0.03% | 0.01% | 0.16% | 0.03% |
| Class B | 0.02% | 0.01% | 0.05% | 0.03% | -0.04% | -0.04% |
| Class C | 0.01% | 0.16% | 0.08% | 0.00% | 0.15% | 0.09% |
| Class D | 0.00% | 0.08% | 0.16% | -0.02% | 0.21% | 0.22% |
| Class E |  |  |  |  |  |  |
| Overall | 0.01% | 0.07% | 0.08% | 0.01% | 0.11% | 0.07% |
|  | 0.01% | 0.05% | 0.08% | 0.01% | 0.08% | 0.07% |

|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.01% | 0.00% | 0.04% | 0.02% | -0.03% | 0.25% |
| Class C | 0.00% | 0.14% | -0.15% | 0.00% | 0.23% | 0.23% |
| Class D | 0.04% | 0.23% | 0.25% | 0.00% | 0.19% | 0.32% |
| Class E | 0.05% | -0.45% | 0.20% | 0.02% | -0.41% | -0.03% |
| Overall | 0.02% | 0.01% | 0.07% | 0.01% | 0.02% | 0.21% |
|  | 0.02% | -0.05% | 0.08% | 0.01% | 0.03% | 0.25% |

# Conclusion

It is proposed to adopt the single table simplification in HM7 and HEVC CD/DIS.

# Change to the CD text

##### Derivation process of ctxIdxInc for the syntax element significant_coeff_flag

Inputs to this process are the colour component index cIdx, the current coefficient scan position ( xC , yC ), the transform block width log2TrafoWidth and the transform block height log2TrafoHeight.

Output of this process is ctxIdxInc.

The variable sigCtx depends on the current position ( xC, yC ), the colour component index cIdx, the transform block size and previsously decoded bins of the syntax element significant_coeff_flag. For the derivation of sigCtx, the following applies.

- If log2TrafoWidth is equal to log2TrafoHeight and log2TrafoWidth is equal to 2, sigCtx is derived using ctxIdxMap[ ] specified in Table 9-1 as follows.

sigCtx  =  ctxIdxMap[   (yC << 2) + xC ]	(9-1)

- Otherwise if log2TrafoWidth is equal to log2TrafoHeight and log2TrafoWidth is equal to 3, sigCtx is derived using ctxIdxMap[ ] specified in Error: Reference source not found0 as follows.

sigCtx  =  ((xC + yC) = = 0) ? 9 : ctxIdxMap[ ((yC >> 1 ) << 2) + (xC >> 1) ]	(9-2)

sigCtx  +=  9		(9-3)

- Otherwise if xC + yC is equal to 0, sigCtx is derived as follows.

sigCtx  =19		(9-4)

- Otherwise, if (xC>>2) + (yC>>2) is less than (3 << (max(log2TrafoWidth, log2TrafoHeight) − 4)), sigCtx is derived using previously decoded bins of the syntax element significant_coeff_flag as follows.

- The variable sigCtx is initialized as follows.

sigCtx  =  0		(9-5)

- When xC is less than ( 1 << log2TrafoWidth ) − 1, the following applies.

sigCtx  =  sigCtx  +  significant_coeff_flag[ xC + 1 ][ yC ]	(9-6)

- When xC is less than ( 1 << log2TrafoWidth ) − 1 and yC is less than ( 1 << log2TrafoHeight ) − 1, the following applies.

sigCtx  =  sigCtx  +  significant_coeff_flag[ xC + 1 ][ yC + 1 ]	(9-7)

- When xC is less than ( 1 << log2Width ) − 2, the following applies.

sigCtx  =  sigCtx  +  significant_coeff_flag[ xC + 2 ][ yC ]	(9-8)

- When all of the following conditions are true,

  - yC is less than ( 1 << log2TrafoHeight ) − 1,

  - xC % 4 is not equal to 0 or yC % 4 is not equal to 0,

  - xC % 4 is not equal to 3 or yC % 4 is not equal to 2,

the following applies.

sigCtx  =  sigCtx  +  significant_coeff_flag[ xC ][ yC + 1 ]	(9-9)

- When yC is less than ( 1 << log2TrafoHeight ) − 2 and sigCtx is less than 4, the following applies.

sigCtx  =  sigCtx  +  significant_coeff_flag[ xC ][ yC + 2 ]	(9-10)

- The variable sigCtx is modified as follows.

  - If cIdx is equal to 0 and (xC>>2) + (yC>>2) are greater than 0, the following applies.

sigCtx = ( (sigCtx + 1) >> 1 ) + 23		(9-11)

  - Otherwise, the following applies.

sigCtx = ( (sigCtx + 1) >> 1 ) + 20	(9-12)

- Otherwise( (xC>>2) + (yC>>2) is equal to or greater than (3 << (max(log2TrafoWidth, log2TrafoHeight) − 4)) ), sigCtx is derived as follows.

sigCtx  =  ( cIdx > 0) ? 20:23

The context index increment ctxIdxInc is derived using the colour component index cIdx and sigCtx as follows.

- If cIdx is equal to 0, ctxIdxInc is derived as follows.

ctxIdxInc  =  sigCtx		(9-13)

- Otherwise (cIdx is greater than 0), ctxIdxInc is derived as follows.

ctxIdxInc  =  26  +  sigCtx		(9-14)

Table 9-1 – Specification of ctxIdxMap[ i ]

| i | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ctxIdxMap[ i ] | 0 | 1 | 4 | 5 | 2 | 3 | 4 | 5 | 6 | 6 | 8 | 8 | 7 | 7 | 8 |

# References

[1]  JCTVC-H0719, BoG report on significance map. G. Martin-Cocher.

# Patent rights declaration(s)

Research In Motion Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).