| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0033 |
| --- | --- |

| Title: | Bug fix to Ticket 366 on incorrect definition of intra luma prediction mode of I_PCM |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Keiichi Chono 1753, Shimonumabe, Nakahara-Ku Kawasaki, Kanagawa, 211-8666 JAPAN | Tel: Email: | +81 50 3757 5439 chono@ct.jp.nec.com |
| Source: | NEC Corporation |  |  |

_____________________________

# Abstract

This contribution reviews Ticket 366 on intra luma prediction mode of I_PCM and provides its bug fix text as well as software patch. It is recommended that the software patch is integrated into the HM software and that the bug fix text or its equivalent one is integrated into the DIS text.

# Background and problem statement

Ticket366 reports that when neighbour is I_PCM, the derivation process of intra luma mode predictor in the current HM software does not return DC mode since "2" no longer corresponds to it. It suggests that the default intra luma mode is initialized to "DC_IDX" rather than the magic number "2" and/ that checking of I_PCM is added to the derivation process of intra luma mode predictor.

We recommend that the following are integrated into the HM software and text along with the suggested bug fixes:

- The default intra chroma mode is initialized to "DC_IDX", like the default intra luma mode, in the HM software;

- Add bug fixes of the related portions in the text as follows:

  - Specify intra luma and chroma modes of I_PCM in the semantics.

  - Specify the additional case in the derivation process of intra luma mode predictor to check for I_PCM.

# Suggested bug fixes in the text

2.1	Bug fix in the semantics of pcm_flag

Replace the following text

"pcm_flag specifies whether the associated coding unit with PART_2Nx2N is coded by I_PCM: If the pcm_flag is equal to 1, the associated coding unit with PART_2Nx2N is coded by I_PCM. When the pcm_flag is not present, it shall be infered to be equal to 0."

by

"pcm_flag specifies whether the associated coding unit with PART_2Nx2N is coded by I_PCM: If the pcm_flag is equal to 1, the associated coding unit with PART_2Nx2N is coded by I_PCM and its intra prediction modes for luma and chroma samples are set equal to Intra_DC. When the pcm_flag is not present, it shall be inferred to be equal to 0."

2.2	Bug fix in "8.4.1	Derivation process for luma intra prediction mode"

Replace the following text

"IntraPredMode[ xB ][ yB ] is derived as the following ordered steps.

- The neighbouring locations ( xBA, yBA ) and ( xBB, yBB ) are set equal to ( xB−1, yB ) and ( xB, yB−1 ), respectively.

- The minimum coding block addresses cbAddrA and cbAddrB of the coding treeblocks covering the locations ( xBA,  yBA ) and ( xBB, yBB ) respectively where are derived as follows.

cbAddrA =MinCbAddrZS[ xBA << Log2MinCbSize ][ yBA << Log2MinCbSize ]	(8-1)

cbAddrB =MinCbAddrZS[ xBB << Log2MinCbSize ][ yBB << Log2MinCbSize ]	(8-2)

- The availability process for a minimum coding block address specified in subclause Error: Reference source not found is invoked once with the minmum coding block address cbAddrA as the input and the output assigned to availableA and once with the minmum coding block address cbAddrB as the input and the output assigned to availableB.

- For N being either replaced A or B, the variables intraPredModeN are derived as follows.

- If availableN is equal to FALSE, intraPredModeN is set equal to Intra_DC.

- Otherwise, if the coding unit covering ( xBN,  yBN ) is not coded as intra mode, intraPredModeN is set equal to Intra_DC,

- Otherwise, if yB−1 is less than yCtb, intraPredModeA is set equal to IntraPredMode[ xBA ][ yBA ] and intraPredModeB is set equal to Intra_DC.

- Otherwise, intraPredModeN is set equal to IntraPredMode[ xBN ][ yBN ], where IntraPredMode is the variable array assigned to the coding unit covering the luma location ( xBN, yBN )."

by

"IntraPredMode[ xB ][ yB ] is derived as the following ordered steps.

- The neighbouring locations ( xBA, yBA ) and ( xBB, yBB ) are set equal to ( xB−1, yB ) and ( xB, yB−1 ), respectively.

- The minimum coding block addresses cbAddrA and cbAddrB of the coding treeblocks covering the locations ( xBA,  yBA ) and ( xBB, yBB ) respectively where are derived as follows.

cbAddrA =MinCbAddrZS[ xBA << Log2MinCbSize ][ yBA << Log2MinCbSize ]	(8-3)

cbAddrB =MinCbAddrZS[ xBB << Log2MinCbSize ][ yBB << Log2MinCbSize ]	(8-4)

- The availability process for a minimum coding block address specified in subclause Error: Reference source not found is invoked once with the minimum coding block address cbAddrA as the input and the output assigned to availableA and once with the minimum coding block address cbAddrB as the input and the output assigned to availableB.

- For N being either replaced A or B, the variables intraPredModeN are derived as follows.

- If availableN is equal to FALSE, intraPredModeN is set equal to Intra_DC.

- Otherwise, if the coding unit covering ( xBN,  yBN ) is not coded as intra mode, intraPredModeN is set equal to Intra_DC,

- Otherwise, if the coding unit covering ( xBN,  yBN ) is coded as intra mode with I_PCM, intraPredModeN is set equal to Intra_DC,

- Otherwise, if yB−1 is less than yCtb, intraPredModeA is set equal to IntraPredMode[ xBA ][ yBA ] and intraPredModeB is set equal to Intra_DC.

- Otherwise, intraPredModeN is set equal to IntraPredMode[ xBN ][ yBN ], where IntraPredMode is the variable array assigned to the coding unit covering the luma location ( xBN, yBN )."

# Conclusions

This contribution has presented bug fixes to Ticket 366 on intra luma prediction mode of I_PCM. Software patch to HM-6.0-dev rev.2157 is attached "JCTVC-I0033-IPCM-Ticket366". It is recommended that the software patch is integrated into the HM software and that the bug fixes or their equivalent ones are integrated into the DIS text.

# Patent rights declarations

NEC Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).