| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0034-r1 |
| --- | --- |

| Title: | Suggested texts for Tickets 347, 356, 366, 434, 441, 443, 463, 471 and 483 on I_PCM |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Keiichi Chono 1753, Shimonumabe, Nakahara-Ku Kawasaki, Kanagawa, 211-8666 JAPAN | Tel: Email: | +81 50 3757 5439 chono@ct.jp.nec.com |
| Source: | NEC Corporation |  |  |

_____________________________

# Abstract

This contribution reviews editorial issues related to I_PCM and provides their suggested revisions. The suggested revisions resolve the issues in Tickets 347, 356, 366, 434, 441, 443, 463, 471, and 483 as well as Ballot comments JP 29, 34, 42, 43, 47, 67 and 71. It is recommended that the revisions or equivalent ones are reflected onto the DIS text.

# List of editorial issues

- In 7.3.7.2, non-break hyphen instead of minus is used for "NumPCMBlock--" (Ticket 347).

- In 7.4.2.1, there is inconsistency in SPS syntax and semantic names between "pcm_bit_depth_XXX_minus1" and "pcm_sample_bit_depth_xxx_minus1", (The 1st item in Ticket 443); the requirement in Log2MinIPCMCUSiz is incorrect when Log2MinCbSize is greater than 3.

- In 7.4.6, the semantics of PCM related PU syntaxes are defined in CU syntax (Ticket 356 and the 3rd item in Ticket 434); the inference of pcm_flag value is incorrect when the pcm_flag is not present due to the presence of num_subsequent_pcm in a previously coded coding unit (The 1st item in Ticket 471); the default intra luma and chroma predicton modes of I_PCM are not defined (Ticket 366); the meanings of "subsequent coding units" and "subsequent I_PCM coding units" are a little bit ambiguous (The 2nd item in Ticket 471).

- In 7.4.8, the inference of split_tranform_flag is not correct when an I_PCM size is larger than 1<<Log2MaxTrafoSize, and it results in redundant deblocking process in inner regions of I_PCM (Ticket 483).

- In 8.4, Cr sample location in pcm_sample_chroma in equation (8-18) is wrong.

- In 8.7.1.4, "0" QP setting for I_PCM regions are missed; instead, qPL is set equal to 0 if q0,0 is coded by I_PCM (Ticket 463).

- In deblocking and SAO processes, there is an editor note "how can we know a sample is coded by I_PCM mode?".

# Suggested revisions

2.1	Revisions in "7.3.7.2 PCM sample syntax"

Replace the table

| pcm_sample( x0, y0, log2CbSize ) { | Descriptor |
| --- | --- |
| for( i = 0; i < 1 << ( log2CbSize << 1 ); i++ ) |  |
| pcm_sample_luma[ i ] | u(v) |
| for( i = 0; i < ( 1 << ( log2CbSize << 1 ) ) >> 1; i++ ) |  |
| pcm_sample_chroma[ i ] | u(v) |
| NumPCMBlock-- |  |
| } |  |

by

| pcm_sample( x0, y0, log2CbSize ) { | Descriptor |
| --- | --- |
| for( i = 0; i < 1 << ( log2CbSize << 1 ); i++ ) |  |
| pcm_sample_luma[ i ] | u(v) |
| for( i = 0; i < ( 1 << ( log2CbSize << 1 ) ) >> 1; i++ ) |  |
| pcm_sample_chroma[ i ] | u(v) |
| NumPCMBlock−− |  |
| } |  |

2.2	Revisions in "7.4.2.1 Sequence parameter set RBSP semantics"

Replace the following text

" pcm_sample_bit_depth_luma_minus1 + 1 specifies the number of bits used to represent each of PCM sample values of luma component. The value of pcm_sample_bit_depth_luma_minus1 + 1 shall be less than or equal to the value of BitDepthY.

PCMBitDepthY = 1 + pcm_sample_bit_depth_luma_minus1	(7-1)

pcm_sample_bit_depth_chroma_minus1 + 1 specifies the number of bits used to represent each of PCM sample values of chroma components. The value of pcm_sample_bit_depth_chroma_minus1 + 1 shall be less than or equal to the value of BitDepthC.

PCMBitDepthC = 1 + pcm_sample_bit_depth_chroma_minus1	(7-2)"

by

" pcm_sample_bit_depth_luma_minus1 + 1 specifies the number of bits used to represent each of PCM sample values of luma component. The value of pcm_sample_bit_depth_luma_minus1 + 1 shall be less than or equal to the value of BitDepthY.

PCMBitDepthY = 1 + pcm_sample_bit_depth_luma_minus1	(7-3)

pcm_sample_bit_depth_chroma_minus1 + 1 specifies the number of bits used to represent each of PCM sample values of chroma components. The value of pcm_sample_bit_depth_chroma_minus1 + 1 shall be less than or equal to the value of BitDepthC.

PCMBitDepthC = 1 + pcm_sample_bit_depth_chroma_minus1	(7-4)".

Replace the following text

"log2_min_pcm_coding_block_size_minus3 + 3 specifies the minimum size of I_PCM coding blocks.

The variable Log2MinIPCMCUSize is set equal to log2_min_pcm_coding_block_size_minus3 + 3. The variable Log2MinIPCMCUSize shall be equal or less than Min( Log2CtbSize, 5 )."

by

"log2_min_pcm_coding_block_size_minus3 + 3 specifies the minimum size of I_PCM coding blocks.

The variable Log2MinIPCMCUSize is set equal to log2_min_pcm_coding_block_size_minus3 + 3. The variable Log2MinIPCMCUSize shall be in the range of Log2MinCbSize to Min( Log2CtbSize,5), inclusive.".

2.3	Revisions in "7.4.6 Coding unit semantics"

Replace the following text

"pcm_flag specifies whether the associated coding unit with PART_2Nx2N is coded by I_PCM: If the pcm_flag is equal to 1, the associated coding unit with PART_2Nx2N is coded by I_PCM. When the pcm_flag is not present, it shall be infered to be equal to 0.

num_subsequent_pcm specifies the number of subsequent I_PCM coding units with the current log2CbSize that successively follow the current I_PCM coding unit in the same depth of a TB. The values of pcm_flags of the subsequent coding units are set equal to 1. It is a requirement of bitstream conformance that the immediate roots of the current and subsequent I_PCM coding units are identical. The value num_subsequent_pcm shall be in the range of 0 to 3, inclusive."

by

"pcm_flag specifies whether the associated coding unit with PART_2Nx2N is coded by I_PCM: If the pcm_flag is equal to 1, the associated coding unit with PART_2Nx2N is coded by I_PCM and its intra prediction modes for luma and chroma samples are set equal to Intra_DC. When the pcm_flag is not present, it shall be inferred as follows: If NumPCMBlock is equal to 0, it shall be inferred to be equal to 0; otherwise, it shall be inferred to be equal to 1.

num_subsequent_pcm specifies the number of subsequent PART_2Nx2N coding units coded by I_PCM with the current log2CbSize that share the same parent node as the current I_PCM coding unit and successively follow the current I_PCM coding unit in decoding order. The value num_subsequent_pcm shall be in the range of 0 to 3, inclusive.".

And move the semantics of pcm_flag, num_subsequent_pcm, pcm_alignment_zero_bit, pcm_sample_luma[ i ], and pcm_sample_chroma[ i ] to 7.4.7 Prediction unit semantics.

2.4	Revision in "7.4.8	Transform tree semantics"

Replace the following text

"split_transform_flag[ x0 ][ y0 ][ trafoDepth ] specifies whether a block is split into four blocks with smaller horizontal or vertical size for the purpose of transform coding. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered block relative to the top-left luma sample of the picture. The array index trafoDepth specifies the current subdivision level of a coding unit into blocks for the purpose of transform coding. trafoDepth is equal to 0 for blocks that correspond to coding units.

When split_transform_flag[ x0 ][ y0 ][ trafoDepth ] is not present, it is inferred as follows:

- If log2TrafoSize is greater than Log2MaxTrafoSize or intraSplitFlag is equal to 1 or interSplitFlag is equal to 1, the value of split_transform_flag[ x0 ][ y0 ][ trafoDepth ] is inferred to be equal to 1.

- Otherwise (log2TrafoSize is less than or equal to Log2MaxTrafoSize, intraSplitFlag is equal to 0 and interSplitFlag is equal to 0), the value of split_transform_flag[ x0 ][ y0 ][ trafoDepth ] is inferred to be equal to 0."

by

"split_transform_flag[ x0 ][ y0 ][ trafoDepth ] specifies whether a block is split into four blocks with smaller horizontal or vertical size for the purpose of transform coding. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered block relative to the top-left luma sample of the picture. The array index trafoDepth specifies the current subdivision level of a coding unit into blocks for the purpose of transform coding. trafoDepth is equal to 0 for blocks that correspond to coding units.

When split_transform_flag[ x0 ][ y0 ][ trafoDepth ] is not present, it is inferred as follows:

- If pcm_flag is equal to 1, the value of split_transform_flag[ x0 ][ y0 ][ trafoDepth ] is inferred to be equal to 0.

- Otherwise, if log2TrafoSize is greater than Log2MaxTrafoSize or intraSplitFlag is equal to 1 or interSplitFlag is equal to 1, the value of split_transform_flag[ x0 ][ y0 ][ trafoDepth ] is inferred to be equal to 1.

- Otherwise (pcm_flag is equal to 0, log2TrafoSize is less than or equal to Log2MaxTrafoSize, intraSplitFlag is equal to 0 and interSplitFlag is equal to 0), the value of split_transform_flag[ x0 ][ y0 ][ trafoDepth ] is inferred to be equal to 0.".

2.5	Revision in "8.4 Decoding process for coding units coded in intra prediction mode"

Replace the equation (8-18)

"recSamplesCr[ xB/2 + i, yB/2 + j ] = 
		pcm_sample_chroma[ ( nS/2 * ( j + nS ) ) + i ] << ( BitDepthC – PCMBitDepthC ) with i, j = 0..nS/2-1 						(8-5)"

by

"recSamplesCr[ xB/2 + i, yB/2 + j ] = 
		pcm_sample_chroma[ ( nS/2 * ( j + nS/2 ) ) + i ] << ( BitDepthC – PCMBitDepthC ) with i, j = 0..nS/2-1 						(8-6)".

2.6	Revisions in "8.7.1.4 Filtering process for coding unit"

In "8.7.1.4.1	Decision process for luma block edge," replace the following text

"Let s’ represent the luma sample array recPictureL of the current picture.

A variable qPL is derived as follows:

qPL = ( ( QPY + QPP + 1 ) >> 1 )

where QPP is specified by the QPY value of the coding unit containing the sample p0,0.	(8-7)"

by

"Let s’ represent the luma sample array s’ of the current picture.

The sample value q0,0 is derived as follows:

q0,0 = s’ [ xC + xB, yC + yB ]

If verticalEdgeFlag is equal to 1, the sample value p0,0 is derived as follows:

p0,0 = s’ [ xC + xB – 1, yC + yB ]

Otherwise (verticalEdgeFlag is equal to 0),

p0,0 = s’ [ xC + xB, yC + yB – 1]

The variables QPQ and QPP are set equal to the QPY values of the coding units containing the sample q0,0 and p0,0, respectively.

If pcm_enabled_flag is equal to 1, the variables QPQ and QPP are modified as follows:

–	If pcm_flag of the coding unit containing the sample q0,0 is equal to 1, QPQ is set equal to 0;

- If pcm_flag of the coding unit containing the sample p0,0 is equal to 1, QPP is set equal to 0.

A variable qPL is derived as follows:

qPL = ( ( QPQ + QPP + 1 ) >> 1 ) 		 					(8-8)

A variable qPL is derived as follows:

qPL = ( ( QPQ + QPP + 1 ) >> 1 )

where QPP is specified by the QPY value of the coding unit containing the sample p0,0.	(8-9)"

In "8.7.1.4.2	Filtering process for luma block edge", replace the following text

"Depending on pcm_flag, a variable β is specified as follows:

–	If pcm_flag is equal to 1, the variables β is specified as Error: Reference source not found with luma quantization parameter 0 as input.

–	Otherwise, the variables β is specified as Error: Reference source not found with luma quantization parameter Clip3( 0, 51, qPL +  ( beta_offset_div2 << 1 ) ) as input."

by

"The variables β is specified as Error: Reference source not found with luma quantization parameter Clip3( 0, 51, qPL +  ( beta_offset_div2 << 1 ) ) as input.

Depending on pcm_flag, a variable β is specified as follows:

–	If pcm_flag is equal to 1, the variables β is specified as Error: Reference source not found with luma quantization parameter 0 as input.

–	Otherwise, the variables β is specified as Error: Reference source not found with luma quantization parameter Clip3( 0, 51, qPL +  ( beta_offset_div2 << 1 ) ) as input."

In "8.7.1.4.3	Filtering process for chroma block edge", replace the following text

"Let s’ be a variable specifying chroma sample array which is derived as follows.

–	If cIdx is equal to 1, s’ represents the chroma sample array recPictureCb of the current picture.

–	Otherwise (cIdx is equal to 2), s’ represents the chroma sample array recPictureCr of the current picture.

A variable qPL is derived as follows:

qPL = ( ( QPY + QPP + 1 ) >> 1 )

where QPP is specified by the QPY value of the coding unit containing the sample p0,0.	(8-10)"

by

" Let s’ be a variable specifying chroma sample array which is derived as follows.

–	If cIdx is equal to 1, s’ represents the chroma sample array recPictureCb of the current picture.

–	Otherwise (cIdx is equal to 2), s’ represents the chroma sample array recPictureCr of the current picture.

The sample value q0,0 is derived as follows:

q0,0 = s’ [ xC + xB, yC + yB ]

If verticalEdgeFlag is equal to 1, the sample value p0,0 is derived as follows:

p0,0 = s’ [ xC + xB – 1, yC + yB ]

Otherwise (verticalEdgeFlag is equal to 0),

p0,0 = s’ [ xC + xB, yC + yB – 1]

The variables QPQ and QPP are set equal to the QPY values of the coding units containing the sample q0,0 and p0,0, respectively.

If pcm_enabled_flag is equal to 1, the variables QPQ and QPP are modified as follows:

–	If pcm_flag of the coding unit containing the sample q0,0 is equal to 1, QPQ is set equal to 0;

- If pcm_flag of the coding unit containing the sample p0,0 is equal to 1, QPP is set equal to 0.

A variable qPL is derived as follows:

qPL = QPUV( ( QPQ + QPP + 1 ) >> 1 ) 		(8-11)

A variable qPL is derived as follows:

qPL = ( ( QPY + QPP + 1 ) >> 1 )

where QPP is specified by the QPY value of the coding unit containing the sample p0,0.	(8-12)

where QPUV as specified in Table 8-XX based on the index qPL.

- Table 8-xx – Specification of QPUV as a function of qPL

| qPL | <30 | 30 | 31 | 32 | 33 | 34 | 35 | 36 | 37 | 38 | 39 | 40 | 41 | 42 | 43 | 44 | 45 | 46 | 47 | 48 | 49 | 50 | 51 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| QPUV | = qPL | 29 | 30 | 31 | 32 | 32 | 33 | 34 | 34 | 35 | 35 | 36 | 36 | 37 | 37 | 37 | 38 | 38 | 38 | 39 | 39 | 39 | 39 |

"

2.7	Solutions to the editor note

In "8.7.1.4.5	Filtering process for a luma sample", replace the following text

"Each of the filtered sample values, pi’ with i = 0..nDp−1, is substituted by the corresponding input sample value pi if all of the following conditions are true.

–	pi is a sample of an I_PCM block.

–	pcm_loop_filter_disable_flag value is equal to 1.

Similarly, each of the filtered sample values, qj’ with j = 0..nDq−1, is substituted by the corresponding input sample value qj if all of the following conditions are true.

–	qj is a sample of an I_PCM block.

–	pcm_loop_filter_disable_flag value is equal to 1."

by

"Each of the filtered sample values, pi’ with i = 0..nDp−1, is substituted by the corresponding input sample value pi if all of the following conditions are true.

–	pcm_flag of the coding unit containing the sample pi is equal to 1.

–	pcm_loop_filter_disable_flag value is equal to 1.

Similarly, each of the filtered sample values, qj’ with j = 0..nDq−1, is substituted by the corresponding input sample value qj if all of the following conditions are true.

–	pcm_flag of the coding unit containing the sample qj is equal to 1.

–	pcm_loop_filter_disable_flag value is equal to 1.".

In "8.7.1.4.6	Filtering process for a chroma sample", replace the following text

"The filtered sample value, p0’ is substituted by the corresponding input sample value p0 if all of the following conditions are true.

–	p0 is a sample of an I_PCM block.

–	pcm_loop_filter_disable_flag value is equal to 1.

Similarly, the filtered sample value, q0’ is substituted by the corresponding input sample value q0 if all of the following conditions are true.

–	q0 is a sample of an I_PCM block.

–	pcm_loop_filter_disable_flag value is equal to 1."

by

"The filtered sample value, p0’ is substituted by the corresponding input sample value p0 if all of the following conditions are true.

–	pcm_flag of the coding unit containing the sample p0 is equal to 1.

–	pcm_loop_filter_disable_flag value is equal to 1.

Similarly, the filtered sample value, q0’ is substituted by the corresponding input sample value q0 if all of the following conditions are true.

–	pcm_flag of the coding unit containing the sample q0 is equal to 1.

–	pcm_loop_filter_disable_flag value is equal to 1.".

In "8.7.2.1.1 Modification process for luma and chroma samples	", replace the following text

"If pcm_loop_filter_disable_flag is equal to 1 and the recPicture[ xC + i, yC + j ] is a reconstructed sample of an I_PCM block or qpprime_y_zero_transquant_bypass_flag is equal to 1 and QP’Y is equal to 0, the recSaoPicture[ xC + I yC + j ] is derived as"

by

"If pcm_loop_filter_disable_flag is equal to 1 and pcm_flag of the coding unit containing the recPicture[ xC + i, yC + j ] is equal to 1 or qpprime_y_zero_transquant_bypass_flag is equal to 1 and QP’Y is equal to 0, the recSaoPicture[ xC + I yC + j ] is derived as".

# Recommendation

The suggested revisions resolve the issues in Ticket 347, 356, 366, 434, 441, 443, 463, 471 and 483. It is recommended that the suggested revisions are reflected onto the DIS text.

# Patent rights declarations

NEC Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).