| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0035-r1 |
| --- | --- |

| Title: | AHG6: Report on unified QP derivation process in deblocking of I_PCM regions (draft) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report/proposal |  |  |
| Author(s) or Contact(s): | Keiichi Chono 1753, Shimonumabe, Nakahara-Ku Kawasaki, Kanagawa, 211-8666 JAPAN Geert Van der Auwera, Marta Karczewicz 5775 Morehouse Drive San Diego, CA 92121, USA | Email: | chono@ct.jp.nec.com geertv@qualcomm.com martak@qualcomm.com |
| Source: | NEC Corporation, Qualcomm Inc. |  |  |

_____________________________

# Abstract

This contribution investigates two unified QP derivation schemes in the deblocking process for I_PCM regions: one scheme (Scheme1) is to use QPY regardless of pcm_flag and pcm_loop_filter_disable_flag; another one (Scheme2) is to signal QPY at each I_PCM block. Simulation shows that:

- Scheme1 with pcm_loop_filter_disable_flag = 0 degrades the picture fidelity of lossless coded regions at high bit rates;

- Scheme1 with pcm_loop_filter_disable_flag = 1 avoids the picture fidelity degradation without introducing any additional side information increases;

- Scheme2 also avoids the picture fidelity degradation by sending QP=0 at each I_PCM block while introducing additional side information of 20kbps (in CIF video coding).

It is recommended that:

- Revise defects related to the deblocking process in the CD text.

- If the unification of QP derivation in the deblocking is considered, keep pcm_loop_filter_disable_flag and include Scheme1 in the DIS text.

# Background

The current HEVC specifies pcm_loop_filter_disable_flag in the SPS to skip the deblocking, SAO, and ALF on samples of I_PCM regions. When pcm_loop_filter_disable_flag is equal to 1, the deblocking does not change samples of I_PCM regions. Use of pcm_loop_filter_disable_flag allows encoders to keep the fidelity of I_PCM regions containing the original samples, i.e., lossless coded regions. (See JCTVC-E192 for more details.)

The current HEVC also specifies qpprime_y_zero_transquant_bypass_flag in the SPS. qpprime_y_zero_transquant_bypass_flag equal to 1 specifies that coding units associated with QP′Y equal to 0 are reconstructed without using scaling and transform processes. In addition, the deblocking, SAO, and ALF on the reconstructed regions are skipped. Use of qpprime_y_zero_transquant_bypass_flag also allows encoders to keep the fidelity of lossless coded regions. (See JCTVC-G0528 for more details.)

If we keep the functionality of skipping the deblocking, SAO, and ALF on samples of lossless coded regions, it may be desirable to unify other processes in the deblocking, SAO, and ALF. JCTVC-H0448 proposed to unify the QP derivation processes in the deblocking process for I_PCM and other regions by using reconstructed QPs in I_PCM regions. During its review session, sending QPs in the I_PCM regions was also suggested as an alternative unification scheme.

This contribution studies the two unification schemes and is organized as follows: Section 2 reviews the current QP derivation process in deblocking in HM. Section 3 describes the two unification schemes and gives analysis on those as well as on the current HM. Section 4 shows simulation results.

# Review of QP derivation process in deblocking in HM

The QP (qPL) derivation process in deblocking in HM is based on that in AVC: QPP and QPQ associated with block boundary pixels, p0 and q0 respectively, are determined first; if p0 is coded by I_PCM, then QPP is set equal to 0; similarly, if q0 is coded by I_PCM, then QPQ is set equal to 0; finally qPL is set equal to (QPP + QPP + 1)>>1. (Note that as reported in Ticket 463, the current HEVC text misses the "0" QP setting for I_PCM regions; instead, qPL is set equal to 0 if q0 is coded by I_PCM. )

It is likely that the "0" QP setting for I_PCM regions is introduced into the AVC design since it does not have the functionality of skipping the deblocking on I_PCM regions (as well as signaling QPs of I_PCM regions).

# Descriptions and analysis of unification schemes

This section describes the following two unification schemes that enable to unify QP derivation process in the deblocking process:

- Scheme1: Use QPY values of CUs at every block boundary in the deblocking process

- Scheme2: Signal QP values in I_PCM regions on top of Scheme1: If I_PCM block size is larger than or equal to MinCUDQPSize, its QP value is coded by using FLC with the minimum number of bits needed for the QP range.

Scheme1 removes the "0" QP setting for I_PCM regions. Thus it enables video codecs to determine QP in the deblocking process without discriminating I_PCM and other regions. The text of Scheme1 is attached in Appendix A. As it can be seen from Appendix A, Scheme1 simplifies the current text.

Scheme2 on top of Scheme1 accords with FLC one in the following starting basis suggested in the 8th JCTVC meeting:

- Never deblock inside of I_PCM regions;

- Send a QP for the I_PCM region (terminate CABAC, then send number of successive blocks n, then send n QP values using FLC with the minimum number of bits needed for the QP range or as delta QP with se(v) or as ue(v), then send byte alignment bits, then send I_PCM samples);

- Outside of the I_PCM region, apply deblocking as usual.

Scheme2 introduces a new syntax element in PU, and it requires an additional text in PU semantics. The additional text is attached in Appendix B. Notice that there is a constraint that QP is signaled only when  its I_PCM block size is larger than or equal to the minimum dQP CU size. This constraint is needed for keeping the current QP related specification, as there is a similar constraint on FGS size for the minimum dQP CU size (i.e., FGS size is larger than or equal to the minimum dQP CU size). While Scheme2 with the constraint does not affect the current QP related specification, it requires additional checking of QP parsing for I_PCM CUs.

The advantages and disadvantages of HM, Scheme1, and Scheme2 are summarized as follows:

[HM]

Advantage:

- Stable design because the same scheme is used in AVC

- No change in the text (except bug fix).

Disadvantage:

- No unification of QP derivation process in deblocking.

[Scheme1]

Advantage:

- Unification of QP derivation process in deblocking.

- Simplification in the text.

Disadvantage:

- In lossless coding applications, low-fidelity at high bit rates (when pcm_loop_filter_disable_flag is equal to 0).

[Scheme2]

Advantage:

- Unification of QP derivation process in deblocking.

- Freedom in adjusting the deblocking strengths for I_PCM regions.

Disadvantage:

- Additional processing and side information in I_PCM regions.

- Additional checking of QP parsing.

- Additional descriptions in the text.

In simulation, we investigate Scheme1's impacts on picture fidelity at high bit rates, and Scheme2's impacts on processing time and overall bit rates.

# Simulation

3.1	Experimental setup

Both Scheme1 and Scheme2 described in Section 2 are implemented into the HM-6.0-dev rev. 2170. ("20120409-HM-6.0-dev-rev2170-AHG6-IPCM.patch" is the patch.) Their 64-bit encoder and decoder executables were generated by using Microsoft Visual C++ 2008. They ran on Windows-based computing platform listed below.

| Computing platform | Usage |
| --- | --- |
| Windows 7 64-bit Professional Intel Xeon 3.33GHz and Memory 32GB | Up to 12 processes for encoding and decoding ran at the same time. |

Simulation for common test sequences were conducted according to JCTVC-H1100 using parameters PCMEnabledFlag=1, PCMLog2MaxSize=5, PCMLog2MinSize=3, and MaxCuDQPDepth=3. The results are described in 3.2. Since I_PCM is not selected in the common test sequences with the common coding conditions, additional simulation results for a synthesized sequence using lower QP values are described in 3.3.

3.2	Results for common test sequences

Table 1 and Table 2 show Scheme1 and Scheme2 results, respectively. Since I_PCM is not selected, their PSNR values and bit rates are identical to Anchor's.

Table1: Anchor versus Scheme1. (Detailed results are found in "JCTVC-I0035-HM-6.0-dev-rev2170-Scheme1.xls." Decoding time is measured with a fix of Ticket 468. )

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| Class B | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| Class C | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| Class D | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| Class E | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| Overall | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
|  | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| Class F | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 99% |  |  |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| Class B | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| Class C | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| Class D | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| Class E |  |  |  |  |  |  |
| Overall | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
|  | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| Class F | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| Enc Time[%] | 100% |  |  | 99% |  |  |
| Dec Time[%] | 99% |  |  | 100% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| Class C | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| Class D | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| Class E | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| Overall | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
|  | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| Class F | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 99% |  |  | 99% |  |  |
|  | Low delay P Main |  |  | Low delay P HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| Class C | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| Class D | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| Class E | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| Overall | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| Class F | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

Table2: Anchor versus Scheme2. (Detailed results are found in "JCTVC-I0035-HM-6.0-dev-rev2170-Scheme2.xls." Decoding time is measured with a fix of Ticket 468.)

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| Class B | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| Class C | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| Class D | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| Class E | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| Overall | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
|  | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| Class F | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| Enc Time[%] | 101% |  |  | 101% |  |  |
| Dec Time[%] | 100% |  |  | 99% |  |  |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| Class B | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| Class C | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| Class D | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| Class E |  |  |  |  |  |  |
| Overall | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
|  | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| Class F | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| Enc Time[%] | 100% |  |  | 99% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| Class C | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| Class D | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| Class E | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| Overall | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
|  | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| Class F | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| Enc Time[%] | 99% |  |  | 100% |  |  |
| Dec Time[%] | 99% |  |  | 99% |  |  |
|  | Low delay P Main |  |  | Low delay P HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| Class C | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| Class D | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| Class E | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| Overall | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| Class F | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

3.3	Additional results for synthesized noisy sequence

This subsection provides additional results for a synthesized sequence depicted in Figure 2. The synthesized sequence is Gaussian noise, which is difficult to compress by using prediction, transform coding, and entropy encoding. Some videos can partially contain textures whose nature is similar to Gaussian noise. The synthesized sequence is generated by using software available from http://phenix.int-evry.fr/jct/doc_end_user/documents/5_Geneva/wg11/JCTVC-E057-v5.zip.

Figure 2: Synthesized sequence, Sandstorms, (CIF 30Hz 30frames)

We encoded Sandstorms by using the RA Main setting with QPs equal to 2, 6, 10, and 14. (An attached txt "JCTVC-IXXXX-sandstorms-enc-batch.txt" describes MS-DOS script to run the RA Main coding for Sandstorms.) All regions in the coded sequences with QP=2, 6, and 10 are coded by I_PCM, whereas in the coded sequence with QP=14, some regions (associated QP = 16 or 17) are coded by I_PCM and the rest are coded by non I_PCM. The I_PCM regions associated with QP equal to 16 or 17 are modified by the deblocking process unless the "0" QP setting for I_PCM regions is used. Therefore, we observe PSNR drops in the Scheme 1 and Scheme2 results for QP=14.

Table 3 shows the RA Main coding results of Scheme1( with pcm_loop_filter_disable_flag=0), Scheme2( with QP=QPSlice signaling), Scheme1 with pcm_loop_filter_disable_flag=1, and Scheme2 with QP=0 signaling. In Table 3, "Rate kbps", "Y dB", "U dB", and "V dB" shows the averaged differences in bitrates, Y PSNR, U, PSNR, and V PSNR, respectively, between Anchor and Tested. Negative value in Rate kbps means reduction in bit rates. Negative value in Y, U, or V dB means decreases in PSNR.

Table 3: Summary of RA Main coding results. (Detailed results are found in "JCTVC-I0035-HM-6.0-dev-rev2170-additional.xls.")

From Table 3, the following are observed:

- Scheme1 degrades the picture fidelity of the lossless coded regions at high bit rates; in other words, the current AVC-style specification is well designed for high picture fidelity applications.

- Scheme2 degrades the picture fidelity of the lossless coded regions at high bit rates, too; however, sending 0 QPs in I_PCM regions avoids the degradation with additional overheads associated with emulation_prevention_three_byte (due to the successive FLCs of QP=0).

- pcm_loop_filter_disable_flag enables encoders to keep the picture fidelity of the lossless coded regions without increasing bit rates.

Based on the above observations and the analysis in Section 2, the following actions are suggested:

- Revise defects related to the deblocking process in the CD text;

- If the unification of QP derivation in the deblocking is considered, keep pcm_loop_filter_disable_flag and include Scheme1 in the DIS text.

# Conclusions

This contribution has reviewed the QP derivation of the HM deblocking process, and the related defects in the CD text as reported in Ticket 463. It also has analyzed the following two unified QP derivation schemes:

- Scheme1: Use QPY regardless of pcm_flag and pcm_loop_filter_disable_flag.

- Scheme2: Signal QPY at each I_PCM block.

Simulation results showed that:

- Scheme1 with pcm_loop_filter_disable_flag = 0 degrades the picture fidelity of lossless coded regions at high bit rates;

- Scheme1 with pcm_loop_filter_disable_flag = 1 avoids the picture fidelity degradation without introducing any additional side information increases;

- Scheme2 avoids the picture fidelity degradation by sending QP=0 at each I_PCM block while introducing additional side information of 20kbps (in CIF video coding).

Based on the analysis and the simulation results, the following are recommended:

- Revise defects related to the deblocking process in the CD text;

- If the unification of QP derivation in the deblocking is considered, keep pcm_loop_filter_disable_flag and include Scheme1 in the DIS text.

# Reference

- K. Chono, H. Aoki (NEC), V. Wahadaniah, C. Lim (Panasonic), “Proposal of enhanced PCM coding in HEVC,” JCTVC-E192, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th  Meeting: Geneva, Switzerland, March 2011.

- W. Gao, M. Jiang, H. Yu, ”AHG19: A QP-based enabling method for lossless coding in HEVC,” JCTVC-H0528, , Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San Jose, CA, USA, 1-10 February, 2012.

- G. Van der Auwera, X. Wang, M. Karczewicz, “AHG6: Deblocking of IPCM Blocks Containing Reconstructed Samples,” JCTVC-H0448, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San Jose, CA, USA, 1-10 February, 2012.

# Patent rights declarations

NEC Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Appendix A: Text for Scheme1

A.1  Bug fix to 8.7.1.4.1 Decision process for luma block edge

Replace the following text

" Let s’ represent the luma sample array recPictureL of the current picture.

A variable qPL is derived as follows:

qPL = ( ( QPY + QPP + 1 ) >> 1 )

where QPP is specified by the QPY value of the coding unit containing the sample p0,0.	(8-1)"

by

"Let s’ represent the luma sample array s’ of the current picture.

The sample value q0,0 is derived as follows:

q0,0 = s’ [ xC + xB, yC + yB ]

If verticalEdgeFlag is equal to 1, the sample value p0,0 is derived as follows:

p0,0 = s’ [ xC + xB – 1, yC + yB ]

Otherwise (verticalEdgeFlag is equal to 0),

p0,0 = s’ [ xC + xB, yC + yB – 1]

The variables QPQ and QPP are set equal to the QPY values of the coding unit containing the sample q0,0 and p0,0, respectively.

A variable qPL is derived as follows:

qPL = ( ( QPQ + QPP + 1 ) >> 1 ) 		 					(8-2)

A variable qPL is derived as follows:

qPL = ( ( QPQ + QPP + 1 ) >> 1 )

where QPP is specified by the QPY value of the coding unit containing the sample p0,0.	(8-3)"

A.2	Bug fix to 8.7.1.4.2 Filtering process for luma block edge

Replace the following text

" Depending on pcm_flag, a variable β is specified as follows:

–	If pcm_flag is equal to 1, the variables β is specified as Error: Reference source not found with luma quantization parameter 0 as input.

–	Otherwise, the variables β is specified as Error: Reference source not found with luma quantization parameter Clip3( 0, 51, qPL +  ( beta_offset_div2 << 1 ) ) as input."

by

" The variables β is specified as Error: Reference source not found with luma quantization parameter Clip3( 0, 51, qPL +  ( beta_offset_div2 << 1 ) ) as input.

Depending on pcm_flag, a variable β is specified as follows:

–	If pcm_flag is equal to 1, the variables β is specified as Error: Reference source not found with luma quantization parameter 0 as input.

–	Otherwise, the variables β is specified as Error: Reference source not found with luma quantization parameter Clip3( 0, 51, qPL +  ( beta_offset_div2 << 1 ) ) as input."

A.3	Bug fix to 8.7.1.4.3 Filtering process for chroma block edge

Replace the following text

" Let s’ be a variable specifying chroma sample array which is derived as follows.

–	If cIdx is equal to 1, s’ represents the chroma sample array recPictureCb of the current picture.

–	Otherwise (cIdx is equal to 2), s’ represents the chroma sample array recPictureCr of the current picture.

A variable qPL is derived as follows:

qPL = ( ( QPY + QPP + 1 ) >> 1 )

where QPP is specified by the QPY value of the coding unit containing the sample p0,0.	(8-4)"

by

" Let s’ be a variable specifying chroma sample array which is derived as follows.

–	If cIdx is equal to 1, s’ represents the chroma sample array recPictureCb of the current picture.

–	Otherwise (cIdx is equal to 2), s’ represents the chroma sample array recPictureCr of the current picture.

The sample value q0,0 is derived as follows:

q0,0 = s’ [ xC + xB, yC + yB ]

If verticalEdgeFlag is equal to 1, the sample value p0,0 is derived as follows:

p0,0 = s’ [ xC + xB – 1, yC + yB ]

Otherwise (verticalEdgeFlag is equal to 0),

p0,0 = s’ [ xC + xB, yC + yB – 1]

The variables QPQ and QPP are set equal to the QPY values of the coding unit containing the sample q0,0 and p0,0, respectively.

A variable qPL is derived as follows:

qPL = QPUV( ( QPQ + QPP + 1 ) >> 1 ) 		(8-5)

A variable qPL is derived as follows:

qPL = ( ( QPY + QPP + 1 ) >> 1 )

where QPP is specified by the QPY value of the coding unit containing the sample p0,0.	(8-6)

where QPUV as specified in Table 8-XX based on the index qPL.

- Table 8-xx – Specification of QPUV as a function of qPL

| qPL | <30 | 30 | 31 | 32 | 33 | 34 | 35 | 36 | 37 | 38 | 39 | 40 | 41 | 42 | 43 | 44 | 45 | 46 | 47 | 48 | 49 | 50 | 51 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| QPUV | = qPL | 29 | 30 | 31 | 32 | 32 | 33 | 34 | 34 | 35 | 35 | 36 | 36 | 37 | 37 | 37 | 38 | 38 | 38 | 39 | 39 | 39 | 39 |

"

# Appendix B: Additional text to Appendix A

B.1. Changes in 7.3.7 Prediction unit syntax

| prediction_unit( x0, y0, log2CbSize ) { | Descriptor |
| --- | --- |
| if( skip_flag[ x0 ][ y0 ] ) { |  |
| if( MaxNumMergeCand > 1 ) |  |
| merge_idx[ x0 ][ y0 ] | ae(v) |
| } else if( PredMode = = MODE_INTRA ) { |  |
| if( PartMode = = PART_2Nx2N && pcm_enabled_flag && log2CbSize >= Log2MinIPCMCUSize && log2CbSize <= Log2MaxIPCMCUSize ) |  |
| pcm_flag | ae(v) |
| if( pcm_flag ) { |  |
| num_subsequent_pcm | tu(3) |
| NumPCMBlock = num_subsequent_pcm + 1 |  |
| if( (max_cu_qp_delta_depth > 0 ) && log2CbSize >= log2MinCUDQPSize ) |  |
| for( i = num_subsequent_pcm; i >= 0; i−− ) { |  |
| cu_qp_plus_qp_bd_offset_y [ i ] | u(v) |
| } |  |
| while( !byte_aligned( ) ) |  |
| pcm_alignment_zero_bit | u(v) |
| pcm_sample( x0, y0, log2CbSize ) |  |
| } else { |  |
| prev_intra_luma_pred_flag[ x0 ][ y0 ] | ae(v) |
| if( prev_intra_luma_pred_flag[ x0 ][ y0 ] ) |  |
| mpm_idx[ x0 ][ y0 ] | ae(v) |
| else |  |
| rem_intra_luma_pred_mode[ x0 ][ y0 ] | ae(v) |
| intra_chroma_pred_mode[ x0 ][ y0 ] | ae(v) |
| SignalledAsChromaDC = ( chroma_pred_from_luma_enabled_flag ? intra_chroma_pred_mode[ x0 ][ y0 ] = = 3 : intra_chroma_pred_mode[ x0 ][ y0 ] = = 2 ) |  |
| } |  |
| } else { /* MODE_INTER */ |  |
| merge_flag[ x0 ][ y0 ] | ae(v) |
| if( merge_flag[ x0 ][ y0 ] ) { |  |
| if( MaxNumMergeCand > 1 ) |  |
| merge_idx[ x0 ][ y0 ] | ae(v) |
| } else { |  |
| if( slice_type = = B ) |  |
| inter_pred_flag[ x0 ][ y0 ] | ae(v) |
| if( inter_pred_flag[ x0 ][ y0 ] = = Pred_LC ) { |  |
| if( num_ref_idx_lc_active_minus1 > 0 ) |  |
| ref_idx_lc[ x0 ][ y0 ] | ae(v) |
| mvd_coding(mvd_lc[ x0 ][ y0 ][ 0 ], mvd_lc[ x0 ][ y0 ][ 1 ]) |  |
| mvp_lc_flag[ x0 ][ y0 ] | ae(v) |
| } else { /* Pred_L0 or Pred_BI */ |  |
| if( num_ref_idx_l0_active_minus1 > 0 ) |  |
| ref_idx_l0[ x0 ][ y0 ] | ae(v) |
| mvd_coding(mvd_l0[ x0 ][ y0 ][ 0 ], mvd_l0[ x0 ][ y0 ][ 1 ]) |  |
| mvp_l0_flag[ x0 ][ y0 ] | ae(v) |
| } |  |
| if( inter_pred_flag[ x0 ][ y0 ] = = Pred_BI ) { |  |
| if( num_ref_idx_l1_active_minus1 > 0 ) |  |
| ref_idx_l1[ x0 ][ y0 ] | ae(v) |
| if( mvd_l1_zero_flag ) { |  |
| mvd_l1[ x0 ][ y0 ][ 0 ] = 0 |  |
| mvd_l1[ x0 ][ y0 ][ 1 ] = 0 |  |
| } else |  |
| mvd_coding( mvd_l1[ x0 ][ y0 ][ 0 ], mvd_l1[ x0 ][ y0 ][ 1 ] ) |  |
| mvp_l1_flag[ x0 ][ y0 ] | ae(v) |
| } |  |
| } |  |
| } |  |
| } |  |

B.2. Changes in 7.3.7.2 PCM sample syntax

| pcm_sample( x0, y0, log2CbSize ) { | Descriptor |
| --- | --- |
| if( (max_cu_qp_delta_depth > 0 ) && log2CbSize >= log2MinCUDQPSize ) |  |
| QPY[ x0 ][ y0 ]= cu_qp_plus_qp_bd_offset_y [ NumPCMBlock − 1 ] − QpBdOffsetY |  |
| for( i = 0; i < 1 << ( log2CbSize << 1 ); i++ ) |  |
| pcm_sample_luma[ i ] | u(v) |
| for( i = 0; i < ( 1 << ( log2CbSize << 1 ) ) >> 2; i++ ) |  |
| pcm_sample_chroma[ i ] | u(v) |
| NumPCMBlock−− |  |
| } |  |

B.3. Addition of cu_qp_plus_qp_bd_offset_y semantics to 7.4.7 Prediction unit semantics

cu_qp_plus_qp_bd_offset_y specifies a luma quantization parameter minus QpBdOffsetY. The decoded value of cu_qp_plus_qp_bd_offset_y shall be in the range of 0 to 51+ QpBdOffsetY, inclusive. The number of bits representing cu_qp_plus_qp_bd_offset_y is specified in Table XXX based on BitDepthY.

The luma quantization parameter QP’Y and the chroma quantization parameters QP’Cb and QP’Cr are derived by invoking the process specified in subclause Error: Reference source not found with ( x0 ,y0 ) as inputs.

- Table X-xx – Specification of the number of bits of cu_qp_plus_qp_bd_offset_y as a function of BitDepthY

| BitDepthY | 8 | 9 | 10 | 11 | 12 | 13 | 14 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Number of bits | 6 | 6 | 7 | 7 | 7 | 7 | 7 |

B.4. Changes in 8.6.1 Derivation process for quantization parameters

The following text

"The value of QPY is derived as

QPY = ( ( ( QPY_PRED + cu_qp_delta +52+ 2*QpBdOffsetY )%( 52 + QpBdOffsetY ) ) − QpBdOffsetY	(8-7)

The value of luma quantization parameter QP’Y is derived as

QP’Y = QPY + QpBdOffsetY		(8-8)"

is replaced by

"If all of the following conditions are true,

–	the current max_cu_qp_delta_depth is greater than 0

–	the size of coding unit covering ( xBN,  yBN ) is larger than or equal to the current (1<<log2MinCUDQPSize)

–	the coding unit covering ( xBN,  yBN ) is associated with pcm_flag equal to 1

then the value of QPY is derived as follows:

QPY = cu_qp_plus_qp_bd_offset_y − QpBdOffsetY		(8-253)

Otherwise, the value of QPY is derived as follows:

QPY = ( ( ( QPY_PRED + cu_qp_delta +52+ 2*QpBdOffsetY )%( 52 + QpBdOffsetY ) ) − QpBdOffsetY	(8-254)."