| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0039 WG11 Number: m24277 |
| --- | --- |

| Title: | CE2.b.4 Necessity of alf_nb_pred_luma_flag |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tomoo Yamakage Takayuki Itoh Takashi Watanabe | Tel: Email: | tomoo.yamakage@toshiba.co.jp takayuki2.itoh@toshiba.co.jp takashi39.watanabe@toshiba.co.jp |
| Source: | TOSHIBA Corporation |  |  |

_____________________________

# Abstract

This subtest is to study the necessity of alf_nb_pred_luma_flag. This test will compare the current design with restricting the value of alf_nb_pred_luma_flag to 0 or 1 (alf_nb_pred_luma_flag will not be signalled). It is reported that in comparison with the CE2 anchors, little coding loss is observed by restricting the value of alf_nb_pred_luma_flag to 0 or 1. Therefore, removal of alf_nb_pred_luma_flag and no prediction of the (N-1)-th coefficient are recommended.

# Introduction

In current ALF in the CD text, the (N-1)-th coefficient in luma filter can be predicted by other filter coefficients according to equation 1 or 2. Whether the (N-1)-th coefficient in i-th filter are predicted or not is indicated by alf_nb_pred_luma_flag[i] (if alf_nb_pred_luma_flag[i] equals to 1, the (N-1)-th coefficient is predicted from the 0-th to (N-2)-th coefficients.)

    - If alf_pred_flag is equal to 0 or the value of i is equal to 0,

cL[ i ][ N-1 ] = alf_filt_coeff [ 0 ][ ry ][ rx ][ i ][ N-1 ] + ( 1 << alfPrecisionBit ) –
					( k2 * alf_filt_coeff[ 0 ][ ry ][ rx ][ i ][ k ] ) >> 2	(1)
				with k=0..N-2

    - Otherwise,

alf_coeff_luma_nb[ i ] = alf_filt_coeff[ 0 ][ ry ][ rx ][ i ][ N-1 ] –
				( k2* alf_filt_coeff[ 0 ][ ry ][ rx ][ i ][ k ] ) >> 2

cL[ i ][ N-1 ] = alf_coeff_luma_nb[ i ] + cL[ i − 1 ][ N-1 ] 	(2)
				with k=0..N−2

Note: Equations (1) and (2) differ intended for the derivation process of the (N-1)-th coefficient. Whole process of coefficients derivation is described in JCTVC-I0039_WD.doc .

In this subtest, the necessity of alf_nb_pred_luma_flag is studied by restricting the value of alf_nb_pred_luma_flag to 0 or 1.

Note that in the CD text, the (N-1)-th coefficient is predicted if alf_nb_pred_luma_flag[i] equals to 1, but in HM-6.0 software, it is predicted if alf_nb_pred_luma_flag[i] equals to 0 (see ticket #475).  Therefore, please note that the distributed software and the experimental results followed the HM6 implementation, but the draft CD/DIS text followed the CD text.

# WD modifications

See JCTVC-I0039_WD.doc

# Simulation results

The CE2 anchor is used for comparison. Tables 1 and 2 show results for restricting the value of alf_nb_pred_luma_flag to 0, whereas tables 3 and 4 show results for restricting the value of alf_nb_pred_luma_flag to 1. Tables 1 and 3 are results from LCU-based encoder, whereas tables 2 and 4 are results from picture-based encoder. The results show that the coding efficiency loss by restricting the value of alf_nb_pred_luma_flag is almost 0. Therefore, it is recommended to remove alf_nb_pred_luma_flag and no prediction is used to the (N-1)-th coefficient.

# Patent rights declaration(s)

TOSHIBA corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

Table 1 Results of LCU-based encoder, alf_nb_pred_luma_flag = 0. (w/ prediction)

|  | All Intra HE10 |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | 0.0% | 0.0% | 0.0% |
| Class B | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | 0.0% |
| Class E | 0.0% | 0.0% | 0.0% |
| Overall | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% |
| Class F | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 100% |  |  |
|  | Random Access HE10 |  |  |
|  | Y | U | V |
| Class A | 0.0% | 0.0% | -0.1% |
| Class B | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | -0.1% |
| Class E |  |  |  |
| Overall | 0.0% | 0.0% | -0.1% |
|  | 0.0% | 0.0% | -0.1% |
| Class F | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 100% |  |  |
|  | Low delay B HE10 |  |  |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | 0.0% | 0.0% | -0.1% |
| Class C | 0.0% | 0.0% | 0.1% |
| Class D | 0.0% | 0.4% | -0.2% |
| Class E | 0.0% | -0.3% | -0.1% |
| Overall | 0.0% | 0.1% | -0.1% |
|  | 0.0% | 0.0% | -0.1% |
| Class F | 0.1% | -0.2% | 0.1% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 100% |  |  |
|  | Low delay P HE10 |  |  |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | 0.0% | 0.3% | 0.4% |
| Class C | 0.0% | 0.1% | -0.5% |
| Class D | 0.0% | 0.2% | 0.1% |
| Class E | 0.0% | -0.1% | 1.0% |
| Overall | 0.0% | 0.2% | 0.2% |
|  | 0.0% | 0.2% | 0.3% |
| Class F | 0.0% | -0.4% | 0.5% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 101% |  |  |

Table 2 Results of Picture-based encoder, alf_nb_pred_luma_flag = 0. (w/ prediction)

|  | All Intra HE10 |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | 0.0% | 0.0% | 0.0% |
| Class B | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | 0.0% |
| Class E | 0.0% | 0.0% | 0.0% |
| Overall | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% |
| Class F | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 100% |  |  |
|  | Random Access HE10 |  |  |
|  | Y | U | V |
| Class A | 0.0% | 0.1% | 0.1% |
| Class B | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | 0.0% |
| Class E |  |  |  |
| Overall | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% |
| Class F | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 100% |  |  |
|  | Low delay B HE10 |  |  |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | 0.0% | 0.1% | 0.1% |
| Class C | 0.0% | 0.2% | -0.2% |
| Class D | 0.0% | -0.5% | -0.3% |
| Class E | 0.0% | 0.3% | -0.3% |
| Overall | 0.0% | 0.0% | -0.2% |
|  | 0.0% | 0.0% | -0.1% |
| Class F | 0.1% | -0.1% | 0.2% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 100% |  |  |
|  | Low delay P HE10 |  |  |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | 0.0% | -0.1% | 0.2% |
| Class C | 0.0% | -0.2% | -0.1% |
| Class D | 0.1% | 0.1% | -0.3% |
| Class E | 0.0% | 0.2% | -0.3% |
| Overall | 0.0% | 0.0% | -0.1% |
|  | 0.0% | 0.0% | -0.1% |
| Class F | 0.0% | -0.4% | 0.3% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 101% |  |  |

Table 3 Results of LCU-based encoder, alf_nb_pred_luma_flag = 1. (w/o prediction)

|  | All Intra HE10 |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | 0.0% | 0.0% | 0.0% |
| Class B | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | 0.0% |
| Class E | 0.0% | 0.0% | 0.0% |
| Overall | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% |
| Class F | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 100% |  |  |
|  | Random Access HE10 |  |  |
|  | Y | U | V |
| Class A | 0.0% | -0.1% | 0.0% |
| Class B | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.1% |
| Class D | 0.0% | -0.1% | -0.1% |
| Class E |  |  |  |
| Overall | 0.0% | -0.1% | 0.0% |
|  | 0.0% | 0.0% | 0.0% |
| Class F | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 100% |  |  |
|  | Low delay B HE10 |  |  |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | 0.0% | 0.0% | -0.2% |
| Class C | 0.0% | 0.0% | 0.2% |
| Class D | 0.0% | -0.1% | 0.2% |
| Class E | 0.0% | 0.1% | 0.1% |
| Overall | 0.0% | 0.0% | 0.1% |
|  | 0.0% | 0.0% | 0.0% |
| Class F | 0.0% | 0.2% | -0.2% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 100% |  |  |
|  | Low delay P HE10 |  |  |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | 0.0% | 0.1% | 0.2% |
| Class C | 0.0% | 0.0% | -0.2% |
| Class D | 0.0% | -0.2% | -0.2% |
| Class E | 0.1% | -0.3% | 0.3% |
| Overall | 0.0% | -0.1% | 0.0% |
|  | 0.0% | -0.1% | 0.1% |
| Class F | 0.0% | 0.1% | 1.3% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 101% |  |  |

Table 4 Results of Picture-based encoder, alf_nb_pred_luma_flag = 1. (w/o prediction)

|  | All Intra HE10 |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | 0.0% | 0.0% | 0.0% |
| Class B | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | 0.0% |
| Class E | 0.0% | 0.0% | 0.0% |
| Overall | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% |
| Class F | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 100% |  |  |
|  | Random Access HE10 |  |  |
|  | Y | U | V |
| Class A | 0.0% | 0.1% | 0.0% |
| Class B | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.1% |
| Class D | 0.0% | 0.0% | 0.0% |
| Class E |  |  |  |
| Overall | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% |
| Class F | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 100% |  |  |
|  | Low delay B HE10 |  |  |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | 0.0% | -0.2% | -0.1% |
| Class C | 0.0% | 0.1% | -0.2% |
| Class D | 0.0% | -0.4% | -0.5% |
| Class E | 0.0% | 0.0% | -0.3% |
| Overall | 0.0% | -0.1% | -0.3% |
|  | 0.0% | -0.1% | -0.2% |
| Class F | 0.0% | 0.2% | -0.1% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 100% |  |  |
|  | Low delay P HE10 |  |  |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | 0.0% | -0.1% | 0.0% |
| Class C | 0.0% | -0.2% | -0.1% |
| Class D | 0.0% | 0.2% | -0.1% |
| Class E | 0.0% | 0.3% | 0.0% |
| Overall | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.1% | -0.1% |
| Class F | 0.0% | -0.4% | 0.3% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 101% |  |  |