[8.7.3.2	Derivation process for filter coefficients]

Input to this process is a variable cIdx specifying colour component index,

Outputs of this process are filter coefficients c for the samples for the colour component cIdx.

If cIdx is equal to 0, the derivation process for luma filter coefficients specified in subclause Error: Reference source not found is invoked.

Otherwise (cIdx is equal to 1 or 2), the derivation process for chroma coefficients specified in subclause Error: Reference source not found is invoked.

-

[8.7.3.2.1	Derivation process for luma filter coefficients]

Outputs of this process are the luma filter coefficients cL.

The luma filter coefficients cL with elements cL[ i ][ j ], with i = 0..AlfNumFilters − 1 and  j = 0..9 are derived as follows.

- If alf_nb_pred_luma_flag[ i ] is equal to 0,  the following ordered steps apply:

- The luma filter coefficients cL with elements cL[ i ][ j ], i = 0..AlfNumFilters−1, j = 0..8 are derived as follows:

- If alf_pred_flag is equal to 0 or the value of i is equal to 0,

cL[ i ][ j ] = alf_filt_coeff [ 0 ][ ry ][ rx ][ i ][ j ]	(8-1)

- Otherwise (alf_pred_flag is equal to 1 and the value of i is greater than 1),

cL[ i ][ j ] = alf_filt_coeff [ 0 ][ ry ][ rx ][ i ][ j ] + cL[ i − 1 ][ j ]	(8-2)

- The luma filter coefficients cL with elements cL[ i ][ j ] with i = 0..AlfNumFilters − 1 and j = 9 are derived as follows.

- If alf_pred_flag is equal to 0 or the value of i is equal to 0,

cL[ i ][ j ] = alf_filt_coeff[ 0 ][ ry ][ rx ][ i ][ j ] + ( 1 << 8 ) −
				k(2* alf_filt_coeff[ 0 ][ ry ][ rx ][ i ][ k ]) 	(8-3)
			with k=0..j−2

- Otherwise (alf_pred_flag is equal to 1 and the value of i is greater than 1),

cL[ i ][ j ] = alf_filt_coeff[ 0 ][ ry ][ rx ][ i ][ j ] + cL[ i − 1 ][ j ] −
				k(2* alf_filt_coeff[ 0 ][ ry ][ rx ][ i ][ k ]) 	(8-4)
			with k=0..j−2

- Otherwise (alf_nb_pred_luma_flag[i] is equal to 1), the following ordered steps apply:

- The luma filter coefficients cL with elements cL[ i ][ j ], i = 0..AlfNumFilters−1, j = 0..7 are derived as follows:

    - If alf_pred_flag is equal to 0 or the value of i is equal to 0,

cL[ i ][ j ] = alf_filt_coeff [ 0 ][ ry ][ rx ][ i ][ j ]	(8-5)

    - Otherwise (alf_pred_flag is equal to 1 and the value of i is greater than 1),

cL[ i ][ j ] = alf_filt_coeff [ 0 ][ ry ][ rx ][ i ][ j ] + cL[ i − 1 ][ j ]	(8-6)

- The luma filter coefficients cL with elements cL[ i ][ j ], i = 0..AlfNumFilters−1, j = 8 are derived as follows:

    - If alf_pred_flag is equal to 0 or the value of i is equal to 0,

cL[ i ][ j ] = alf_filt_coeff [ 0 ][ ry ][ rx ][ i ][ j ] + ( 1 << 8 ) –
					( k2 * alf_filt_coeff[ 0 ][ ry ][ rx ][ i ][ k ] ) >> 2	(8-7)
				with k=0..j−1

    - Otherwise,

alf_coeff_luma_nb[ i ] = alf_filt_coeff[ 0 ][ ry ][ rx ][ i ][ j ] –
				( k2* alf_filt_coeff[ 0 ][ ry ][ rx ][ i ][ k ] ) >> 2	(8-8)

cL[ i ][ j ] = alf_coeff_luma_nb[ i ] + cL[ i − 1 ][ j ] 	(8-9)
				with k=0..j−1

- The luma filter coefficients cL with elements cL[ i ][ j ], i = 0..AlfNumFilters−1, j = 9 are derived as follows:

    - If alf_pred_flag is equal to 0 or the value of i is equal to 0,

cL[ i ][ j ] = alf_filt_coeff[ 0 ][ ry ][ rx ][ i ][ j ] + ( 1 << 8) –
					k(2* alf_filt_coeff[ 0 ][ ry ][ rx ][ i ][ k ]) − 2*cL[ i ][ j − 1 ]	(8-10)
				with k=0..j−2

    - Otherwise,

cL[ i ][ j ] = alf_filt_coeff[ 0 ][ ry ][ rx ][ i ][ j ] + cL[ i − 1 ][ j ] –
					k(2* alf_filt_coeff[ 0 ][ ry ][ rx ][ i ][ k ]) − 2*alf_coeff_luma_nb[ i ]	(8-11)
				with k=0..j−2

Considering the symmetry of the filter, the luma filter coefficients cL with elements cL[ i ][ j ], i = 0..AlfNumFilters − 1 is derived as follows:

cL[ i ][ 18 ] = cL[ i ][ 9 ]		(8-12)

cL[ i ][ 17 – j ] = cL[ i ][ j ]		(8-13)
	with j = 0..8

[8.7.3.2.2	Derivation process for chroma filter coefficients]

Inputs to this process are

  - a variable cIdx specifying colour component index,

Outputs of this process are filter coefficients cc for the samples for the colour component cIdx.

The chroma filter coefficients cC with elements cC[ i ], i = 0..9 is derived as follows:

- If i is equal to 9, the coefficient cC[i] is derived as

sum = j( alf_filt_coeff [ cIdx ][ ry ][ rx ][ 0 ] [ j ] << 1 ) 	(8-14)
	with j = 0..8

cC[ i ] = ( 1 << 8 ) − ( sum – alf_filt_coeff [ cIdx ][ ry ][ rx ][ 0 ][ i ] )	(8-15)

- Otherwise (i is less than 9), considering the symmetry of the filter, the chroma filter coefficients cC with elements cC[ i ], i = 0..18 are derived as follows:

cC[ i ] = alf_filt_coeff [ cIdx ][ ry ][ rx ][ 0 ] [ i ]		(8-16)

cC[ i ][ 18 ] = cC[ i ][9 ]		(8-17)

cC[ i ][ 17 – j ] = cC[ i ][ j ]		(8-18)
	with j = 0..8