| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0041 WG11 Number: m24279 |
| --- | --- |

| Title: | CE2.d.1: Number of regions in RA mode |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tomoo Yamakage Takayuki Itoh Takashi Watanabe | Tel: Email: | tomoo.yamakage@toshiba.co.jp takayuki2.itoh@toshiba.co.jp takashi39.watanabe@toshiba.co.jp |
| Source: | TOSHIBA Corporation |  |  |

_____________________________

# Abstract

This subtest is to study the effectiveness of RA mode by dividing a picture up to 1/16 size or 1/64 size. In HM6/CD, the number of filters in each region is up to 16 in the context of BA mode. In this contribution, number of filters in each region is forced to be 1 and syntax related to BA is removed. Since picture-based ALF in HM6/CD has both BA and quandtree-based region adaptation, by removing BA, it is possible to test region adaptive (RA) ALF. By changing the depth of quadtree, different size of RA can be tested.

It is reported that in comparison with the CE2 anchors (both BA and RA modes), 1/16 size RA shows 0.3%, 0.6%, 0.4%, 0.7% coding loss for AI, RA, LDB, LDP, respectively. The loss becomes 0.3%, 0.5%, 0.3%, 0.4%, respectively, for 1/64 size RA. Therefore, it is not recommended the current specification by the tested RA mode.

# Introduction

In HM6/CD, both BA and quadtree-based RA are included in the picture-based ALF. In this contribution, in order to test the effectiveness of RA, BA mode in HM6/CD is removed by forcing 1 filter per each region and by removing BA related syntax.

# WD modifications

See JCTVC-I0041_WD.doc

# Simulation results

The CE2 anchor is used for comparison. This test uses picture-based encoding only.

Table 1 shows results for 1/16 size RA (depth of the quadtree is set to be less than 2). The BD-rate for luma is 0.3%, 0.6%, 0.4%, 0.7% for AI, RA, LDB, LDP, respectively. Table 2 shows results for 1/64 size RA (depth of the quadtree is set to be less than 3). The BD-rate for luma is 0.3%, 0.5%, 0.3%, 0.4% for AI, RA, LDB, LDP, respectively. Therefore, it is not recommended the current specification by the tested RA mode.

# Patent rights declaration(s)

TOSHIBA corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

Table 1 Results for 1/16 size RA.

|  | All Intra HE10 |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | 0.5% | 0.3% | 0.3% |
| Class B | 0.3% | 0.1% | 0.0% |
| Class C | 0.2% | 0.0% | 0.0% |
| Class D | 0.2% | -0.1% | -0.1% |
| Class E | 0.4% | 0.1% | 0.1% |
| Overall | 0.3% | 0.1% | 0.1% |
|  | 0.3% | 0.1% | 0.1% |
| Class F | 0.1% | -0.1% | 0.0% |
| Enc Time[%] | 101% |  |  |
| Dec Time[%] | 100% |  |  |
|  | Random Access HE10 |  |  |
|  | Y | U | V |
| Class A | 0.9% | 0.4% | 0.4% |
| Class B | 0.7% | 0.2% | 0.2% |
| Class C | 0.3% | 0.0% | 0.0% |
| Class D | 0.3% | -0.2% | -0.1% |
| Class E |  |  |  |
| Overall | 0.6% | 0.1% | 0.1% |
|  | 0.5% | 0.1% | 0.1% |
| Class F | 0.2% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 99% |  |  |
|  | Low delay B HE10 |  |  |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | 0.5% | 0.2% | 0.0% |
| Class C | 0.4% | -0.1% | -0.3% |
| Class D | 0.2% | -0.2% | -0.1% |
| Class E | 0.2% | 0.2% | 0.0% |
| Overall | 0.4% | 0.0% | -0.1% |
|  | 0.4% | 0.1% | -0.2% |
| Class F | 0.2% | 0.0% | 0.1% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 99% |  |  |
|  | Low delay P HE10 |  |  |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | 1.0% | 0.4% | 0.5% |
| Class C | 0.6% | -0.1% | 0.0% |
| Class D | 0.5% | 0.3% | 0.1% |
| Class E | 0.6% | 0.0% | -0.3% |
| Overall | 0.7% | 0.2% | 0.1% |
|  | 0.7% | 0.2% | 0.1% |
| Class F | 0.3% | -0.4% | 0.5% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 99% |  |  |

Table 2 Results for 1/64 size RA.

|  | All Intra HE10 |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | 0.5% | 0.2% | 0.1% |
| Class B | 0.2% | 0.1% | 0.1% |
| Class C | 0.2% | 0.1% | 0.0% |
| Class D | 0.2% | -0.1% | -0.1% |
| Class E | 0.3% | 0.2% | 0.2% |
| Overall | 0.3% | 0.1% | 0.1% |
|  | 0.3% | 0.1% | 0.1% |
| Class F | 0.1% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 99% |  |  |
|  | Random Access HE10 |  |  |
|  | Y | U | V |
| Class A | 0.9% | 0.0% | -0.2% |
| Class B | 0.5% | -0.2% | 0.0% |
| Class C | 0.3% | 0.0% | -0.1% |
| Class D | 0.3% | -0.2% | -0.1% |
| Class E |  |  |  |
| Overall | 0.5% | -0.1% | -0.1% |
|  | 0.5% | -0.1% | -0.1% |
| Class F | 0.1% | -0.1% | -0.1% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 99% |  |  |
|  | Low delay B HE10 |  |  |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | 0.4% | -0.3% | -0.3% |
| Class C | 0.4% | -0.3% | -0.6% |
| Class D | 0.2% | -0.2% | -0.1% |
| Class E | 0.1% | 0.6% | -0.6% |
| Overall | 0.3% | -0.1% | -0.4% |
|  | 0.3% | 0.0% | -0.4% |
| Class F | 0.1% | -0.2% | -0.5% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 98% |  |  |
|  | Low delay P HE10 |  |  |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | 0.6% | 0.0% | 0.2% |
| Class C | 0.4% | -0.3% | -0.2% |
| Class D | 0.5% | 0.3% | 0.1% |
| Class E | -0.2% | 0.5% | 0.1% |
| Overall | 0.4% | 0.1% | 0.1% |
|  | 0.4% | 0.1% | 0.1% |
| Class F | 0.0% | -0.4% | -0.1% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 98% |  |  |