- Modifications points of CE2.a.1 (from JCTVC-Ixxx)

[7.3.4	Slice data syntax]

| slice_data( ) { | Descriptor |
| --- | --- |
| CtbAddrRS = SliceCtbAddrRS |  |
| CtbAddrTS = CtbAddrRStoTS[ CtbAddrRS ] |  |
| moreDataFlag = 1 |  |
| if( adaptive_loop_filter_flag && alf_cu_control_flag ) |  |
| AlfCuFlagIdx = −1 |  |
| do { |  |
| xCtb = InverseRasterScan( CtbAddrRS, CtbSize, CtbSize, pic_width_in_luma_samples, 0 ) |  |
| yCtb = InverseRasterScan( CtbAddrRS, CtbSize, CtbSize, pic_width_in_luma_samples, 1 ) |  |
| NumPCMBlock = 0 |  |
| CtbAddrInSlice = CtbAddrRS − ( slice_address >> SliceGranularity ) |  |
| AddrUp= CtbAddrRS – PicWidthInCtbs |  |
| if( slice_sao_interleaving_flag ) { |  |
| if(slice_sample_adaptive_offset_flag) |  |
| sao_unit_cabac( xCtb, yCtb, 0 ) |  |
| if(sao_cb_enable_flag) |  |
| sao_unit_cabac( xCtb, yCtb, 1 ) |  |
| if(sao_cr_enable_flag) |  |
| sao_unit_cabac( xCtb, yCtb, 2 ) |  |
| } |  |
| if(alf_coef_in_slice_flag) |  |
| alf_unit_cabac( rx, ry, cIdx ) |  |
| moreDataFlag = coding_tree( xCtb, yCtb, Log2CtbSize, 0 ) |  |
| CtbAddrTS++ |  |
| if( moreDataFlag && ( ( tiles_or_entropy_coding_sync_idc = = 1 && TileId[ CtbAddrTS ] != TileId[ CtbAddrTS − 1 ] ) ¦ ¦ ( tiles_or_entropy_coding_sync_idc = = 2 && num_substream_minus1 > 0 && CtbAddrTS / PicWidthInCtbs <= num_substream_minus1 && CtbAddrTS % PicWidthInCtbs = = 0 ) ) ) { |  |
| rbsp_trailing_bits( ) |  |
| if( nextbits(24) = = 0x000002 ) { |  |
| entry_point_marker_two_3bytes | f(24) |
| tile_idx_minus_1 | u(v) |
| } |  |
| } |  |
| } while( moreDataFlag ) |  |
| } |  |

[7.3.3.x	Adaptive loop filter unit cabac syntax for LCU interleaved mode]

| alf_unit_cabac( rx, ry, cIdx ){ | Descriptor |
| --- | --- |
| if(cIdx ==0 ¦¦ (cIdx !=0 && alf_lcu_enabled_flag[0][ry][rx] ==1)) |  |
| { |  |
| alf_lcu_enable_flag[ cIdx ][ ry ][ rx ] | u(1) |
| if (alf_lcu_enable_flag[ cIdx ][ ry ][ rx ]) |  |
| alf_info_cabac( rx, ry, cIdx ) |  |
| } |  |
| } |  |

[7.3.3.6	Adaptive loop filter info cabac syntax for LCU interleaved mode]

| alf_info_cabac( rx, ry, cIdx ) { | Descriptor |
| --- | --- |
| if(NumALFFiltersInStoredBuffer[cIdx] > 0) |  |
| alf_new_filter_set_flag | u(1) |
| if( alf_new_filter_set_flag = = 0 && NumALFFiltersInStoredBuffer[cIdx] > 0) |  |
| { |  |
| if(NumALFFiltersInStoredBuffer[cIdx] > 1) |  |
| { |  |
| alf_use_prev_stored_filter_flag | u(1) |
| if(alf_use_prev_stored_filter_flag==0 && NumALFFiltersInStoredBuffer[cIdx] > 2) |  |
| alf_remaining_stored_filter_idx[ cIdx ] | u(v) |
| } |  |
| } |  |
| else { |  |
| if( cIdx = = 0 ) { |  |
| alf_nb_pred_luma_flag[ cIdx ][ ry ][ rx ] | u(1) |
| for( j = 0; j < AlfCodedLength; j++ ) |  |
| alf_filt_coeff[ cIdx ][ ry ][ rx ][ j ] | ge(v) |
| } else |  |
| for( j = 0; j < AlfCodedLength; j++ ) |  |
| alf_filt_coeff[ cIdx ][ ry ][ rx ][ 0 ][ j ] | se(v) |
| } |  |
| } |  |
| } |  |

[7.4.3.6	Adaptive loop filter info semantics]

alf_use_prev_stored_filter_flag specifies whether the most recently stored filter of stored filter set is used.

alf_new_filter_set_flag equal to 1 specifies that the current coding treeblock uses a new filter set; equal to 0 specifies that the current coding treeblock uses the stored filter set with the buffer index equal to 0 if alf_use_prev_stored_filter_flag =1, otherwise (alf_use_prev_stored_filter_flag =0) buffer index is set equal to alf_remaining_stored_filter_idx[cIdx ]+1.

When alf_new_filter_set_flag is equal to 1, NumALFFiltersInStoredBuffer[cIdx] is calculated as  NumALFFiltersInStoredBuffer[cIdx]  = max(16, NumALFFiltersInStoredBuffer[cIdx]+1).

- Modification point of CE2.a.2 (difference from CE2.a.1 are marked by “Track Changes” format)

[7.3.2.5	Adaptation parameter set RBSP syntax]

| aps_rbsp( ) { | Descriptor |
| --- | --- |
| aps_id | ue(v) |
| aps_scaling_list_data_present_flag | u(1) |
| if( aps_scaling_list_data_present_flag ) |  |
| scaling_list_param( ) |  |
| aps_deblocking_filter_flag | u(1) |
| if(aps_deblocking_filter_flag) { |  |
| disable_deblocking_filter_flag | u(1) |
| if( !disable_deblocking_filter_flag ) { |  |
| beta_offset_div2 | se(v) |
| tc_offset_div2 | se(v) |
| } |  |
| } |  |
| aps_sao_interleaving_flag | u(1) |
| if( !aps_sao_interleaving_flag ) { |  |
| aps_sample_adaptive_offset_flag | u(1) |
| if( aps_sample_adaptive_offset_flag ) |  |
| aps_sao_param( ) |  |
| } |  |
| alf_aps_filter_flag[0] | u(1) |
| if(alf_aps_filter_flag) |  |
| { |  |
| alf_picture_info(0) |  |
| for(cIdx=1;cIdx<3;cIdx++) |  |
| { |  |
| alf_aps_filter_flag[cIdx] | u(1) |
| if(alf_aps_filter_flag[cIdx]) |  |
| alf_picture_info(cIdx) |  |
| } |  |
| } |  |
| aps_adaptive_loop_filter_flag | u(1) |
| if( aps_adaptive_loop_filter_flag ) |  |
| alf_param( ) |  |
| aps_extension_flag | u(1) |
| if( aps_extension_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| aps_extension_data_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

alf_aps_filter_flag equal to 1 specifies that the picture-level ALF coefficients are present in the current APS; equal to 0 specifies that the picture-level ALF coefficients is not present in the current APS. When alf_aps_filter_flag[cIdx] equals to 1, NumALFFiltersInStoredBuffer[cIdx] is initialized to 1 in each slices referring to the current APS, otherwise, NumALFFiltersInStoredBuffer[cIdx] is initialized to 0 in each slices referring to the current APS.

[7.3.2.x Adaptive loop filter picture info syntax]

| alf_picture_info( cIdx ) { | Descriptor |
| --- | --- |
| if( cIdx = = 0 ) { |  |
| alf_no_filters_minus1 | ue(v) |
| if( alf_no_filters_minus1 = = 1 ) |  |
| alf_start_second_filter | ue(v) |
| else if( alf_no_filters_minus1 > 1 ) |  |
| for( i = 1; i < 16; i++ ) |  |
| alf_filter_pattern_flag[ cIdx ][ i ] | u(1) |
| if( alf_no_filters_minus1 > 0 ) |  |
| alf_pred_flag[ cIdx ] | u(1) |
| for( i = 0; i < AlfNumFilters; i++ ) |  |
| alf_nb_pred_luma_flag[ cIdx ][ i ] | u(1) |
| if( AlfNumFilters > 1 ) { |  |
| alf_min_kstart_minus1 | ue(v) |
| for( i = 1; i < 4; i++ ) |  |
| alf_golomb_index_flag[ i ] | u(1) |
| } |  |
| for( i = 0; i < AlfNumFilters; i++ ) |  |
| for( j = 0; j < AlfCodedLength; j++ ) |  |
| alf_filt_coeff[ cIdx ][ i ][ j ] | ge(v) |
| } else |  |
| for( j = 0; j < AlfCodedLength; j++ ) |  |
| alf_filt_coeff[ cIdx ][ 0 ][ j ] | se(v) |
| } |  |

Note that since alf_picture_param() signals filter coefficients in the same manner as alf_info(),the same semantics as alf_info() can be reused.

[7.4.3.6	Adaptive loop filter info semantics]

alf_use_prev_stored_filter_flag equals to 1 specifies that the picture-level filter coefficients signaled in APS are used in the current LCU; equals to 0 specifies the stored filter specified by alf_remaining_stored_filter_idx[cIdx] is used in the current LCU.

alf_new_filter_set_flag equal to 1 specifies that the current coding treeblock uses a new filter set; equal to 0 specifies that the current coding treeblock uses either the picture-level filter coefficients signaled in APS or a filter in the stored filter set.

When alf_new_filter_set_flag is equal to 1, NumALFFiltersInStoredBuffer[cIdx] is calculated as  NumALFFiltersInStoredBuffer[cIdx]  = max(16, NumALFFiltersInStoredBuffer[cIdx]+1).