| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0042 WG11 Number: m24280 |
| --- | --- |

| Title: | CE2.a.2: Additional signaling of picture-level ALF coefficients on top of CE2.a.1 (LCU interleaved signalling) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tomoo Yamakage1 Takayuki Itoh1 Takashi Watanabe1 Ching-Yeh Chen2 Chia-Yang Tsai2 Chih-Ming Fu2 Yu-Wen Huang2 Shawmin Lei2 In Suk Chong3 Marta Karczewicz3 | Tel: Email: | {tomoo.yamakage, takayuki2.itoh, takashi39.watanabe}@toshiba.co.jp {chingyeh.chen, chiayang.tsai, chihming.fu, yuwen.huang, shawmin.lei}@mediatek.com {ichong, martak}@qualcomm.com |
| Source: | TOSHIBA Corporation1, MediaTek2, Qualcomm3 |  |  |

_____________________________

# Abstract

This subtest is to study the coding efficiency improvement by additional signaling of picture-level ALF coefficients on top of CE2.a.1 (LCU interleaved signalling). The picture-level ALF coefficients will be derived by the previous pictures in the same manner as proposed in JCTVC-C082 in order to avoid additional frame memory scan to design ALF coefficients for the picture and provide low latency capability, where the coefficients are expected to be sub-optimal coefficients for the current picture. In each LCU, ALF control flag is signaled. If the flag is 1, ALF coefficients that have already been signalled are re-used. Otherwise, ALF coefficients will be newly signalled for the LCU.

In this contribution, two options were tested for LCU-based encoding: (1) allowing multiple (up to 16) filters per APS, and (2) allowing one filter per APS. Note that since CE2.a.1 allows only 1 filter signaling in an LCU, CE2.a.2 also allows 1 filter signaling in an LCU.

The CE2 LCU-based anchor is used for comparison. It is reported that in comparison with the LCU-based CE2 anchors, 0.4%, 0.7%, 0.3%, 0.6% coding gains are observed for AI, RA, LDB, LDP, respectively for (1), and 0.1%, 0.3%, 0.1%, 0.3% coding gains are observed for (2).

# Introduction

In the CD text and HM6, LCU-based encoder and syntax of ALF were introduced. CE2.a.1 studies to interleave the syntax of ALF into LCU in order to achieve LCU-level latency (see JCTVC-I0249 (CE2.a.1)).

In this subtest, additional signalling of picture-level ALF coefficients (signalled in APS) on top of CE2.a.1 is introduced to achieve better coding efficiency while keeping LCU-level latency. To keep LCU-level latency, the picture-level ALF coefficients derived by the previous pictures (in the same manner as in JCTVC-C082) are used for the current picture. In CE2.a.2 software, ALF control flag and filter information are signalled in the same manner as those in CE2.a.1, where each LCU may select picture-level ALF coefficients signalled in APS, newly signal filter information, select the stored filter in the codebook, or ALF off.

Two options are tested for LCU-based encoding: (1) allowing multiple (up to 16) filters per APS, and (2) allowing one filter per APS.

# WD modifications

See JCTVC-I0042_WD.doc

# Simulation results

The LCU-based CE2 anchor is used for comparison. Table 1 shows results for option (1), and table 2 shows results for option (2). It is reported that in comparison with the CE2 anchor, 0.4%, 0.7%, 0.3%, 0.6% coding gains are observed for AI, RA, LDB, LDP, respectively for (1), and 0.1%, 0.3%, 0.1%, 0.3% coding gains are observed for (2).

Additionally, the results for option (1) are compared to the results obtained by picture-based ALF that uses current picture to design filter coefficients (table 3). It is reported that in comparison with the picture-based ALF, 0.2%, 0.5%, 0.4%, 0.1% coding loss is observed for (1). If the current picture is used to design ALF coefficients, the loss would be reduced.

# Patent rights declaration(s)

TOSHIBA corporation, MediaTek and Qualcomm may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

Table 1 Results for CE2.a.2, allowing multiple (up to 16) filters per APS.

|  | All Intra HE10 |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | -0.7% | -1.6% | -1.3% |
| Class B | -0.4% | -2.1% | -2.0% |
| Class C | -0.3% | -2.1% | -2.6% |
| Class D | -0.1% | -2.0% | -2.4% |
| Class E | -0.4% | -6.0% | -7.5% |
| Overall | -0.4% | -2.6% | -2.9% |
|  | -0.4% | -2.5% | -2.8% |
| Class F | -0.2% | -0.9% | -1.1% |
| Enc Time[%] | 101% |  |  |
| Dec Time[%] | 102% |  |  |
|  | Random Access HE10 |  |  |
|  | Y | U | V |
| Class A | -1.4% | -4.4% | -3.3% |
| Class B | -0.7% | -2.1% | -1.5% |
| Class C | -0.4% | -0.7% | -0.7% |
| Class D | -0.4% | -0.3% | -0.3% |
| Class E |  |  |  |
| Overall | -0.7% | -1.9% | -1.4% |
|  | -0.7% | -1.9% | -1.4% |
| Class F | -0.2% | 0.1% | -0.2% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 102% |  |  |
|  | Low delay B HE10 |  |  |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | -0.6% | -3.6% | -3.7% |
| Class C | -0.3% | -1.2% | -1.5% |
| Class D | -0.3% | -0.3% | 0.0% |
| Class E | 0.0% | -5.1% | -5.1% |
| Overall | -0.3% | -2.5% | -2.5% |
|  | -0.3% | -2.4% | -2.4% |
| Class F | -0.2% | -1.4% | -0.3% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 102% |  |  |
|  | Low delay P HE10 |  |  |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | -1.4% | -6.1% | -6.3% |
| Class C | -0.5% | -1.2% | -2.0% |
| Class D | -0.1% | -0.3% | -0.2% |
| Class E | -0.2% | -8.1% | -7.7% |
| Overall | -0.6% | -3.8% | -4.0% |
|  | -0.7% | -3.7% | -3.8% |
| Class F | -0.5% | 0.2% | 0.2% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 102% |  |  |

Table 2 Results for CE2.a.2, allowing 1 filter per APS.

|  | All Intra HE10 |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | -0.2% | -1.5% | -1.3% |
| Class B | -0.1% | -2.0% | -1.9% |
| Class C | -0.1% | -2.0% | -2.4% |
| Class D | -0.1% | -2.0% | -2.4% |
| Class E | -0.1% | -5.9% | -7.4% |
| Overall | -0.1% | -2.5% | -2.8% |
|  | -0.1% | -2.4% | -2.7% |
| Class F | -0.1% | -0.7% | -1.0% |
| Enc Time[%] | 101% |  |  |
| Dec Time[%] | 100% |  |  |
|  | Random Access HE10 |  |  |
|  | Y | U | V |
| Class A | -0.5% | -4.1% | -3.1% |
| Class B | -0.2% | -1.9% | -1.3% |
| Class C | -0.2% | -0.7% | -0.6% |
| Class D | -0.3% | -0.3% | -0.4% |
| Class E |  |  |  |
| Overall | -0.3% | -1.7% | -1.4% |
|  | -0.3% | -1.8% | -1.4% |
| Class F | -0.2% | 0.1% | -0.1% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 101% |  |  |
|  | Low delay B HE10 |  |  |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | -0.3% | -3.2% | -3.2% |
| Class C | -0.1% | -1.2% | -1.7% |
| Class D | -0.2% | 0.0% | -0.3% |
| Class E | 0.2% | -4.5% | -4.7% |
| Overall | -0.1% | -2.2% | -2.4% |
|  | -0.1% | -2.0% | -2.3% |
| Class F | -0.2% | -0.5% | 0.1% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 101% |  |  |
|  | Low delay P HE10 |  |  |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | -0.7% | -5.5% | -5.8% |
| Class C | -0.3% | -1.4% | -1.7% |
| Class D | -0.1% | -0.5% | -0.7% |
| Class E | 0.1% | -8.0% | -7.5% |
| Overall | -0.3% | -3.7% | -3.8% |
|  | -0.3% | -3.6% | -3.6% |
| Class F | -0.3% | -0.1% | -0.3% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 101% |  |  |

Table 3 Results for CE2.a.2, allowing multiple (up to 16) filters per APS (vs. picture-based ALF in HM6)

|  | All Intra HE10 |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | 0.2% | 0.3% | 0.0% |
| Class B | 0.2% | 0.9% | 1.3% |
| Class C | 0.2% | 1.1% | 1.6% |
| Class D | 0.3% | 1.1% | 1.3% |
| Class E | 0.5% | 2.1% | 1.4% |
| Overall | 0.2% | 1.0% | 1.1% |
|  | 0.2% | 1.0% | 1.1% |
| Class F | 0.1% | 1.6% | 1.4% |
| Enc Time[%] | 97% |  |  |
| Dec Time[%] | 95% |  |  |
|  | Random Access HE10 |  |  |
|  | Y | U | V |
| Class A | 0.6% | -1.0% | -0.4% |
| Class B | 0.5% | 0.7% | 0.9% |
| Class C | 0.4% | 0.2% | 0.3% |
| Class D | 0.6% | 0.2% | 0.0% |
| Class E |  |  |  |
| Overall | 0.5% | 0.1% | 0.2% |
|  | 0.5% | 0.1% | 0.2% |
| Class F | 0.1% | 1.9% | 1.5% |
| Enc Time[%] | 99% |  |  |
| Dec Time[%] | 98% |  |  |
|  | Low delay B HE10 |  |  |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | 0.3% | 0.3% | -0.2% |
| Class C | 0.4% | 0.9% | 0.9% |
| Class D | 0.4% | 0.5% | 0.7% |
| Class E | 0.4% | 1.5% | 1.1% |
| Overall | 0.4% | 0.7% | 0.6% |
|  | 0.4% | 0.7% | 0.5% |
| Class F | -0.1% | 1.1% | 1.4% |
| Enc Time[%] | 99% |  |  |
| Dec Time[%] | 98% |  |  |
|  | Low delay P HE10 |  |  |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | 0.1% | -0.5% | -0.8% |
| Class C | 0.2% | 1.7% | 1.6% |
| Class D | 0.6% | 1.1% | 1.2% |
| Class E | -0.9% | 0.4% | -0.4% |
| Overall | 0.1% | 0.6% | 0.3% |
|  | 0.1% | 0.7% | 0.3% |
| Class F | -0.2% | 2.2% | 2.4% |
| Enc Time[%] | 98% |  |  |
| Dec Time[%] | 97% |  |  |