| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0043 |
| --- | --- |

| Title: | Deblocking across slice and tile boundaries |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Michael Horowitz Shilin Xu 1055 W. Hastings Street Vancouver, BC V6E 2E9 Canada | Tel: Email: | +1 778 731-3432 michael@ebriskvideo.com |
| Source: | eBrisk Video, Inc. |  |  |

_____________________________

# Abstract

This contribution presents two new deblocking filter control modes designed to reduce the visibility of slice and tile boundaries at low QP values.  In the first mode, the deblocking filter is not applied to any edge in a slice except those edges co-located with the top and left slice boundaries.  In the second, the deblocking filter is not applied to any edge in a slice except those edges co-located with top and left boundaries of tiles contained within the slice.

# Introduction

For certain sequences, slice and tile boundaries are visible at low QP values as shown for the slice boundary at the tip of the blue arrow in Figure 1.  Part of the reason is that at low QP values (i.e., QP values less than or equal to 15), without the use of the beta and tC offset parameters, the deblocking filter is effectively disabled due to the fact that the values for beta and tC as derived in Table 8-12 of the HEVC CD text are both equal to 0.  One way to mitigate the boundary artefacts is to use beta and tC offset values to ensure the deblocking filter remains active at low QP values, however, the resulting deblocking filter may result in over filtering. Figure 3 reveals reduced image detail as a result of setting the deblocking offset parameters LoopFilterBetaOffset_div2 equal to 10 and LoopFilterTcOffset_div2 equal to 10 compared to Figure 4 in which the deblocking filter is not active.

In order to reduce the visibility of slice and tile boundaries without reducing image detail, two new deblocking filter control modes are proposed. In the first mode, the deblocking filter is not applied to any edge in a slice except those edges co-located with the top and left slice boundaries. In the second, the deblocking filter is not applied to any edge in a slice except those edges co-located with top and left boundaries of tiles contained within the slice. When the new filter modes are enabled, the beta and tC offset values may be set such that the deblocking filter is active at low QP values in order to mitigate visual artefacts at slice and/or tile boundaries without introducing filter artefacts in other areas. Figure 2 shows deblocking only at slice boundaries (the slice boundary is present at the same location as in Figure 1) with LoopFilterBetaOffset_div2 equal to 10 and LoopFilterTcOffset_div2 equal to 10.

Figure 1. The 42nd frame from the Raven sequence encoded at QP 15 using HM6.0.

Figure 2. The 42nd frame from the Raven sequence encoded at QP 15 using HM6.0 modified to apply the deblocking filter only at slice boundaries.

Figure 3. The 12th frame from the Raven sequence encoded at QP 15 using HM6.0.

Figure 4. The 12th frame from the Raven sequence encoded at QP 15 using HM6.0 modified to apply the deblocking filter only at slice boundaries.

# Deblocking filter control

Two deblocking filter control flags are introduced to control the proposed filter modes.  The disable_deblocking_filter_except_across_slices_flag, when equal to 1, disables the deblocking filter for all edges within a slice except edges co-located with the top and left slice boundaries.  A second flag, the disable_deblocking_filter_except_across_tiles_flag, when equal to 1, disables deblocking filtering for all edges except those co-located with top and left tile boundaries for all tiles contained in the slice.  The two flags are active only if the disable_deblocking_filter_flag is equal to 0.  Syntax and semantics for the flags are presented in the following section.

# Syntax and semantics

In this section, syntax and semantics for the proposed deblocking filter modes are introduced.

## Syntax

It is proposed that the disable_deblocking_filter_except_across_slices_flag and disable_deblocking_filter_except_across_tiles_flag  syntax elements be placed in the aps rbsp and slice header, analogous to the disable_deblocking_filter_flag, as shown in Tables 1 and 2 respectively below.

| aps_rbsp( ) { | Descriptor |
| --- | --- |
| aps_id | ue(v) |
| aps_scaling_list_data_present_flag | u(1) |
| if( aps_scaling_list_data_present_flag ) |  |
| scaling_list_param( ) |  |
| aps_deblocking_filter_flag | u(1) |
| if(aps_deblocking_filter_flag) { |  |
| disable_deblocking_filter_flag | u(1) |
| if( !disable_deblocking_filter_flag ) { |  |
| beta_offset_div2 | se(v) |
| tc_offset_div2 | se(v) |
| disable_deblocking_filter_except_across_slices_flag | u(1) |
| disable_deblocking_filter_except_across_tiles_flag | u(1) |
| } |  |
| } |  |
| … |  |

Table 1. Addition of proposed syntax elements to the aps rbsp.

| slice_header( ) { | Descriptor |
| --- | --- |
| first_slice_in_pic_flag | u(1) |
| if( first_slice_in_pic_flag = = 0 ) |  |
| slice_address | u(v) |
| slice_type | ue(v) |
| entropy_slice_flag | u(1) |
| … |  |
| if( !entropy_slice_flag ) { |  |
| slice_qp_delta | se(v) |
| if( deblocking_filter_control_present_flag ) { |  |
| if( deblocking_filter_in_aps_enabled_flag ) |  |
| inherit_dbl_params_from_aps_flag | u(1) |
| if( !inherit_dbl_params_from_aps_flag ) { |  |
| disable_deblocking_filter_flag | u(1) |
| if( !disable_deblocking_filter_flag ) { |  |
| beta_offset_div2 | se(v) |
| tc_offset_div2 | se(v) |
| disable_deblocking_filter_except_across_slices_flag | u(1) |
| disable_deblocking_filter_except_across_tiles_flag | u(1) |
| } |  |
| … |  |

Table 2. Addition of proposed syntax elements to the slice header.

## Semantics

The semantics for the proposed disable_deblocking_filter_except_across_slices_flag and disable_deblocking_filter_except_across_tiles_flag syntax elements are presented in this section.

disable_deblocking_filter_except_across_slices_flag equal to 1 specifies that the deblocking filtering operations are performed only at edges co-located with the top and left boundaries of the associated slice.  disable_deblocking_filter_except_across_slices_flag equal to 0 specifies that the deblocking filtering operations are not exclusively performed at edges co-located with the top and left boundaries of the associated slice.

disable_deblocking_filter_except_across_tiles_flag equal to 1 specifies that the deblocking filtering operations are performed only at edges co-located with the top and left boundaries of all tiles contained within the associated slice.  disable_deblocking_filter_except_across_tiles_flag equal to 0 specifies that the deblocking filtering operations are not exclusively performed at edges co-located with the top and left boundaries of all tiles contained within the associated slice.

# Patent rights declaration(s)

eBrisk Video, Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).