| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0044 |
| --- | --- |

| Title: | Modification of recovery point SEI message |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kimihiko Kazui, Junpei Koyama, Satoshi Shimada and Akira Nakagawa 4-1-1 Kamikodanaka, Nakahara-ku 211-8588 Kawasaki JAPAN | Tel: Email: | +81-44-754-2639 kazui.kimihiko@jp.fujitsu.com koyama.junpei@jp.fujitsu.com sshimada@jp.fujitsu.com anaka@jp.fujitsu.com |
| Source: | FUJITSU LABORATORIES LTD. |  |  |

_____________________________

# Abstract

The contribution proposes to modify the specification of recovery point SEI message in HM6.0 text. Mainly, recovery_frame_cnt syntax element is replaced with recovery_poc_cnt syntax element for specifying a recovery point since frame_num syntax element is no more used in HM6.0.

# Problem Statement

The specification of recovery point SEI message in HM6.0 text is basically inherited from the AVC specification.

In the AVC specification, the position of a recovery point is specified by recovery_frame_cnt syntax element. The first picture in the decoding order with frame_num x is defined as a recovery point if x is equal to ( ( y + z ) % MaxFrameNum ), where y and z are frame_num of a random access picture associated with a recovery point SEI message and recovery_frame_cnt of the recovery point SEI message, respectively.

HM6.0 text does not use frame_num syntax element to specify a reference picture. Consequently, recovery_frame_cnt is not suitable to specify the position of a recovery point.

# Proposal

## POC-based specification of recovery point

It is proposed to use recovery_poc_cnt instead of recovery_frame_cnt.

The first picture in the decoding order with pic_order_cnt_lsb x is defined as a recovery point picture if x is equal to ( ( y + z ) % MaxPicOrderCntLsb ), where y and z are pic_order_cnt_lsb of a random access picture associated with a recovery point SEI message and recovery_poc_cnt of the recovery point SEI message, respectively.

The proposed modification allows a non-referenced picture to be a recovery point picture while the AVC specification does not. In some specific cases, a recovery point can be set earlier in the display order than that by the AVC specification. See Figure 1 for detail.

Figure 1  Recovery point according to the proposal

## Other modifications

It is normatively specified that prevPicOrderCntLsb and prevPicOrderCntMsb are reset to 0 when the decoding process starts from a random access picture associated with a recovery point SEI message.

Text describing the relationship with APS and CRA picture is added.

## Proposed text

The proposed text is in the attached word document.

# References

[1] "High Efficiency Video Coding (HEVC) text specification draft 6", JCTVC-H1003, February 2012, San Jose USA

# Patent rights declaration(s)

FUJITSU LIMITED may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).