| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0047 |
| --- | --- |

| Title: | CE3: Subtest 3 – Cross-check of JCTVC-H0228 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Chuohao Yeo Yih Han Tan 1 Fusionopolis Way #21-01 Connexis (South Tower) Singapore 138632 | Tel: Email: | +65 6408 2000 chyeo@i2r.a-star.edu.sg |
| Source: | Institute for Infocomm Research |  |  |

_____________________________

# Abstract

This contribution presents a summary of a cross-check performed by the Institute for Infocomm Research on HHI’s proposal for transform coefficient coding. The cross-check performance results reportedly match those provided by HHI. The implemented algorithm is also reportedly in line with HHI’s proposal.

# Introduction

In JCTVC-H0228 [3], the following major modifications are proposed:

- Using a single scan pass to code significant_coeff_flag, coeff_abs_level_greater1_flag, coeff_abs_level_greater2_flag, and coeff_abs_level_remaining (instead of in separate passes) within a coefficient group. coeff_sign_flag is still coded in a separate pass.

- Using the same context derivation method (based on a neighborhood template) and contexts for all transform block sizes when coding significant_coeff_flag

- Using the sum of absolute values in the neighborhood template and the diagonal band in which the coefficient is in to determine the context to use when coding significant_coeff_flag

- Using the same context derivation method (based on a neighborhood template) and contexts for all transform block sizes when coding both coeff_abs_level_greater1_flag and coeff_abs_level_greater2_flag

- Using the sum of absolute values minus 1 of significant coefficients in the neighborhood template and the diagonal band in which the coefficient is in to determine the context to use when coding coeff_abs_level_greater1_flag and coeff_abs_level_greater2_flag

With the proposed method as implemented in the code provided by the proponents, the number of context models required for coding significant_coeff_flag, coeff_abs_level_greater1_flag, coeff_abs_level_greater2_flag, and coeff_abs_level_remaining is:

| Syntax Element | Proposed |  | WD6/HM6 |  |
| --- | --- | --- | --- | --- |
|  | Luma | Chroma | Luma | Chroma |
| significant_coeff_flag | 18 | 12 | 27 | 21 |
| coeff_abs_level_greater1_flag | 16 | 6 | 16 | 8 |
| coeff_abs_level_greater2_flag |  |  | 4 | 2 |
| Total | 52 |  | 78 |  |

# Simulation results

The test conditions as specified in CE3 Subtest 3 [1] are followed. Tests were conducted on configurations as specified in the common test conditions [2].

The results for the coding efficiency test of Subtest 3 are as follows:

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.8% | -0.9% | -1.0% | -0.7% | -1.3% | -1.0% |
| Class B | -0.6% | -0.8% | -0.9% | -0.6% | -0.8% | -0.9% |
| Class C | -0.7% | -0.4% | -0.4% | -0.7% | -0.3% | -0.4% |
| Class D | -0.9% | -0.4% | -0.5% | -0.9% | -0.4% | -0.4% |
| Class E | -1.0% | -1.0% | -1.1% | -0.9% | -1.0% | -0.9% |
| Overall | -0.8% | -0.7% | -0.8% | -0.8% | -0.7% | -0.7% |
|  | -0.8% | -0.7% | -0.8% | -0.8% | -0.7% | -0.7% |
| Class F | -1.0% | -0.9% | -0.9% | -1.0% | -1.2% | -1.0% |
| Enc Time[%] | 102% |  |  | 102% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | -0.4% | 0.6% | 0.3% | -0.3% | 0.0% | 0.0% |
| Class B | -0.4% | -0.3% | -0.2% | -0.4% | -0.1% | -0.3% |
| Class C | -0.6% | 0.0% | -0.2% | -0.5% | -0.1% | -0.1% |
| Class D | -0.7% | 0.3% | 0.1% | -0.6% | 0.5% | 0.2% |
| Class E |  |  |  |  |  |  |
| Overall | -0.5% | 0.1% | 0.0% | -0.4% | 0.1% | -0.1% |
|  | -0.4% | 0.1% | 0.0% | -0.4% | 0.1% | -0.1% |
| Class F | -0.9% | -0.7% | -0.7% | -0.9% | -1.0% | -0.7% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 99% |  |  | 99% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.1% | 0.2% | -0.1% | -0.1% | 0.4% | 0.3% |
| Class C | -0.3% | 0.9% | 0.5% | -0.3% | 0.6% | 0.5% |
| Class D | -0.3% | 1.3% | 1.2% | -0.3% | 0.7% | 1.0% |
| Class E | -0.2% | -0.7% | -0.5% | -0.3% | 0.1% | 0.4% |
| Overall | -0.2% | 0.5% | 0.3% | -0.2% | 0.4% | 0.5% |
|  | -0.2% | 0.5% | 0.3% | -0.2% | 0.5% | 0.5% |
| Class F | -0.8% | -0.5% | -0.4% | -0.6% | -1.7% | -0.2% |
| Enc Time[%] | 101% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 99% |  |  |

These results match those provided by the proponents.

# Conclusions

We have cross-checked CE3 Subtest 3 H0228, studied the algorithm, and verified the coding simulation results.

# Reference

[1] V. Sze et al., “Description of Core Experiment (CE3): Coefficient scanning and coding,” JCTVC-H1103, San Jose, USA, Feb. 2012.

[2] F. Bossen, “Common test conditions and software reference configurations,” JCTVC-H1100, San Jose, USA, Feb. 2012.

[3] T. Nguyen et al., “Non-CE11: Proposed cleanup for transform coefficient coding,” JCTVC-H0228, San Jose, USA, Feb. 2012.