| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0049 |
| --- | --- |

| Title: | Non-CE3: Context reduction for coeff_abs_level_greater2_flag |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | C. Rosewarne 1 Thomas Holt Drive, North Ryde, NSW 2113, Australia. M. Maeda 30-2, Shimomaruko 3-chome, Ohta-ku, Tokyo 146-8501, Japan. | Tel: Email: Tel: Email: | +61 2 9805 2844 chris.rosewarne@cisra.canon.com.au +81-3-3758-2111 maeda.mitsuru@canon.co.jp |
| Source: | Canon Information Systems Research Australia, Canon Inc. |  |  |

_____________________________

# Abstract

This contribution presents two methods for context reduction for coeff_abs_level_greater2_flag.  Each method reduces the number of contexts available to the luma channel from four to two. The resulting luma coding efficiency for both methods in IA_Main and IA_HE10 is 0.01% BD-rate loss in luma, for RA_Main, RA_HE10 the respective luma results are 0.03%, 0.02% and 0.04%, 0.03%, for LD_Main, LD_HE10 the respective luma results are 0.05%, 0.01% and 0.03%, 0.03%.

# Introduction

In HM-6.0, the high-throughput binarisation scheme [1] was adopted.  This reduced the number of instances of coeff_abs_level_greater2_flag to at most one per sub-block.  The variable ‘LargerT1’ corresponds to the variable ‘uiNumOne’ in the HM-6.0 source code.

In HM-6.0, six contexts for coeff_abs_level_greater2_flag are allocated as shown in Table 1.

Table 1. Context usage for coeff_abs_level_greater2_flag in HM-6.0.

| Channel | Location | Previous sub-set |
| --- | --- | --- |
| Luma | Sub-set 0 | LargerT1 = 0 |
|  |  | LargerT1 > 0 |
|  | Other sub-sets | LargerT1 = 0 |
|  |  | LargerT1 > 0 |
| Chroma |  | LargerT1 = 0 |
|  |  | LargerT1 > 0 |

The variable ‘LargerT1’ is defined as a cumulative count of the number of larger-than-one residual coefficients from previous sub-sets, used for context selection and then right-shifted by one at each sub-set boundary. This provides an adaptive mechanism to the density of residual coefficients from previous sub-sets.

# Description of the proposed change

Two methods to reduce the context count for coeff_abs_level_greater2_flag are presented. Each method results in a reduction of two contexts for this syntax element (in both cases for the luma channel).

## Method 1

Table 2 shows the proposed context derivation method for coeff_abs_level_greater2_flag. In this method, the dependence on the sub-set location, is removed which results in a reduction of two contexts. Context derivation for chroma is harmonised with context derivation for luma.

Table 2. Method 1 - Proposed context allocation for coeff_abs_level_greater2_flag.

| Channel | Previous sub-set |
| --- | --- |
| Luma | LargerT1 = 0 |
|  | LargerT1 > 0 |
| Chroma | LargerT1 = 0 |
|  | LargerT1 > 0 |

## Method 2

Table 3 shows an alternative proposed context derivation for coeff_abs_level_greater2_flag.  In this method, the dependence on the LargerT1 value, derived from previous sub-sets, is removed which results in a reduction of two contexts. Context derivation for chroma is harmonised with context derivation for luma.

Table 3. Method 2 - Proposed context allocation for coeff_abs_level_greater2_flag.

| Channel | Location |
| --- | --- |
| Luma | Sub-set 0 |
|  | Other sub-sets |
| Chroma | Sub-set 0 |
|  | Other sub-sets |

# Experiment Results

Results are presented in Table 4 and Table 5 below. Simulations were performed under the common conditions [2]. Refer to the accompanying spreadsheet for results with greater precision. Execution took place on a Linux cluster using GCC 3.4.6.

Table 4. Simulation results in HM-6.0 for Method 1.

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.022% | 0.117% | 0.088% | 0.023% | 0.078% | 0.109% |
| Class B | 0.005% | 0.100% | 0.087% | 0.016% | 0.059% | 0.031% |
| Class C | 0.005% | 0.029% | 0.077% | 0.004% | 0.037% | 0.077% |
| Class D | 0.007% | 0.072% | -0.005% | 0.014% | 0.042% | 0.038% |
| Class E | 0.001% | 0.083% | 0.095% | 0.005% | 0.018% | 0.055% |
| Overall | 0.008% | 0.081% | 0.068% | 0.013% | 0.049% | 0.061% |
|  | 0.009% | 0.082% | 0.065% | 0.013% | 0.054% | 0.067% |
| Class F | -0.010% | 0.110% | -0.009% | 0.002% | 0.056% | 0.023% |
| Enc Time[%] | 100% |  |  | 99% |  |  |
| Dec Time[%] | 100% |  |  | 99% |  |  |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0.096% | 0.142% | 0.177% | 0.031% | 0.061% | -0.042% |
| Class B | 0.018% | -0.016% | 0.228% | 0.004% | 0.054% | 0.078% |
| Class C | 0.021% | -0.009% | 0.028% | 0.011% | -0.196% | 0.025% |
| Class D | -0.005% | 0.116% | 0.034% | 0.052% | 0.077% | 0.089% |
| Class E |  |  |  |  |  |  |
| Overall | 0.031% | 0.054% | 0.123% | 0.023% | 0.002% | 0.040% |
|  | 0.017% | 0.065% | 0.089% | 0.026% | 0.019% | 0.024% |
| Class F | 0.024% | 0.096% | 0.086% | -0.015% | 0.035% | 0.072% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 99% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.019% | -0.203% | 0.041% | 0.003% | -0.060% | 0.448% |
| Class C | 0.045% | -0.020% | -0.149% | 0.041% | 0.501% | 0.101% |
| Class D | 0.059% | 0.566% | 0.246% | -0.016% | -0.054% | 0.026% |
| Class E | 0.114% | -0.890% | 0.671% | 0.018% | 0.167% | 0.601% |
| Overall | 0.053% | -0.094% | 0.163% | 0.011% | 0.124% | 0.284% |
|  | 0.052% | -0.006% | 0.134% | 0.013% | 0.135% | 0.238% |
| Class F | -0.121% | -0.196% | -0.163% | 0.015% | -0.746% | 0.322% |
| Enc Time[%] | 100% |  |  | 101% |  |  |
| Dec Time[%] | 99% |  |  | 101% |  |  |

Table 5. Simulation results in HM-6.0 for Method 2.

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.027% | 0.150% | 0.122% | 0.025% | 0.143% | 0.090% |
| Class B | 0.009% | 0.094% | 0.093% | 0.013% | 0.035% | 0.053% |
| Class C | 0.001% | 0.015% | 0.095% | 0.001% | 0.048% | 0.060% |
| Class D | 0.009% | 0.080% | 0.019% | 0.013% | 0.006% | 0.034% |
| Class E | 0.001% | 0.067% | 0.115% | 0.005% | 0.029% | 0.116% |
| Overall | 0.010% | 0.083% | 0.088% | 0.012% | 0.053% | 0.067% |
|  | 0.010% | 0.085% | 0.088% | 0.012% | 0.054% | 0.067% |
| Class F | -0.001% | 0.097% | 0.074% | 0.014% | 0.051% | 0.015% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0.101% | 0.210% | 0.147% | 0.044% | 0.295% | 0.233% |
| Class B | 0.021% | 0.066% | 0.042% | -0.006% | 0.026% | 0.084% |
| Class C | 0.008% | -0.017% | 0.118% | 0.032% | 0.025% | -0.047% |
| Class D | 0.017% | 0.028% | 0.017% | 0.058% | 0.013% | 0.135% |
| Class E |  |  |  |  |  |  |
| Overall | 0.036% | 0.071% | 0.079% | 0.030% | 0.086% | 0.100% |
|  | 0.021% | 0.070% | 0.082% | 0.034% | 0.070% | 0.064% |
| Class F | 0.051% | 0.103% | 0.157% | 0.035% | -0.006% | 0.065% |
| Enc Time[%] | 100% |  |  | 101% |  |  |
| Dec Time[%] | 99% |  |  | 100% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.007% | -0.209% | 0.133% | 0.000% | -0.231% | 0.267% |
| Class C | 0.039% | 0.278% | -0.297% | 0.042% | 0.242% | 0.063% |
| Class D | 0.059% | 0.184% | 0.315% | 0.035% | -0.237% | 0.132% |
| Class E | -0.009% | -0.534% | -0.081% | 0.037% | -0.455% | 0.633% |
| Overall | 0.025% | -0.050% | 0.031% | 0.026% | -0.156% | 0.251% |
|  | 0.027% | -0.052% | 0.030% | 0.027% | -0.102% | 0.161% |
| Class F | -0.065% | -0.518% | 0.028% | 0.081% | 0.246% | 1.698% |
| Enc Time[%] | 100% |  |  | 102% |  |  |
| Dec Time[%] | 99% |  |  | 101% |  |  |

# Working Draft changes

Note: Changes relative to WD fix for ticket 384.

## Method 1

9.2.3.1.6 Derivation process of ctxIdxInc for the syntax element coeff_abs_level_greater2_flag

Inputs to this process are the colour component index cIdx, the 16 coeffcient subset index i and the current coefficient scan index n within the current subset.

Output of this process is ctxIdxInc.

The variable ctxSet specifies the current context set and is set to the variable ctxSet that has been derived in subclause Error: Reference source not found for the same subset i.

The context index increment ctxIdxInc is derived set equal to the variable ctxSet as follows.

- When  numGreater1 available in 9-33 is greater than 0, ctxSet is incremented by one as follows.

ctxSet  =  ctxSet  +  1		(9-1)

When cIdx is greater than 0, ctxIdxInc is modified as follows.

ctxIdxInc =  ctxIdxInc  +  24		(9-2)

## Method 2

9.2.3.1.6 Derivation process of ctxIdxInc for the syntax element coeff_abs_level_greater2_flag

Inputs to this process are the colour component index cIdx, the 16 coeffcient subset index i and the current coefficient scan index n within the current subset.

Output of this process is ctxIdxInc.

The variable ctxSet specifies the current context set and is set to the variable ctxSet that has been derived in subclause Error: Reference source not found for the same subset i.

The context index increment ctxIdxInc is derived set equal to the variable ctxSet as follows.

- If the current subset index i is equal to 0 or cIdx is greater than 0, the following applies.

ctxSetInc  =  0			(9-3)

- Otherwise (i is greater than 0 and cIdx is equal to 0), the following applies.

ctxSetInc  =  1			(9-4)

When cIdx is greater than 0, ctxIdxInc is modified as follows.

ctxIdxInc =  ctxIdxInc  +  24		(9-5)

# Conclusion

Results are presented a harmonisation of coeff_abs_level_greater2_flag context selection.  A reduction of 2 contexts for the coeff_abs_level2_greater_flag syntax element is also achieved.

# References

- J. Chen, W. Chien, J. Rajan, J. Sole, M. Karczewicz, “Non-CE1: throughput improvement on CABAC coefficients level coding”, Joint Collaborative Team on Video Coding, JCTVC-H0554, January 2012.

- F. Bossen, “Common HM test conditions and software reference configurations,” Joint Collaborative Team on Video Coding, JCTVC-H1100, February 2012.

# Patent rights declaration(s)

Canon Information Systems Research Australia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Canon Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).