| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0050 |
| --- | --- |

| Title: | Non-CE3: Rice parameter initialization and update modifications. |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | C. Rosewarne, V.Kolesnikov 1 Thomas Holt Drive, North Ryde, NSW 2113, Australia M. Maeda 30-2, Shimomaruko 3-chome, Ohta-ku, Tokyo 146-8501, Japan. | Tel: Email: Tel: Email: | +61-2-9805-2844 {chris.rosewarne, vladimir.kolesnikov}@cisra.canon.com.au +81-3-3758-2111 maeda.mitsuru@canon.co.jp |
| Source: | Canon Information Systems Research Australia (CiSRA), Canon Inc. |  |  |

_____________________________

# Abstract

A method of initialisation of the rice parameter K for the Golomb-Rice coding the coeff_abs_level_remaining values of a sub-set is presented. The method reportedly shows a coding gain under the common conditions while using low-QP conditions of -0.3%, -0.3% and  -0.3% in AI-main and -0.1%, -0.1% and -0.1% in RA-main.  In normal QP conditions, the measured coding impact 0.0%, 0.0% and 0.0% in AI-main and 0.0%, -0.1% and 0.1% in RA-main. A method of encoding coeff_abs_level_remaining data for a TU is presented. The method is asserted to enable hardware implementations to realise improved throughput of TU processing by avoiding data dependencies between individual coeff_abs_level_remaining values.  The coding impact results under common conditions are. 0.0%, 0.0% and 0.0% in AI-main and 0.0%, 0.0% and 0.0% in RA-main.

# Problem statement

## K parameter initialization

In HM-6.0, a Rice parameter K [1] is used to perform Golomb-Rice coding of the coeff_abs_level_remaining values [2]. K is initialized to zero at the beginning of every sub-set within the transform unit. We assert that initialization to zero is not always optimal.

## Adaptive K parameter

In HM-6.0 the Rice parameter K is used to perform Golomb-Rice coding of the coeff_abs_level_remaining using CABAC bypass mode. After encoding of each coeff_abs_level_remaining value a new value for K is estimated using a lookup table. The updated K parameter is used to encode the next coeff_abs_level_remaining value. This scheme is shown in Figure 1:

Figure 1. Coding of coef_abs_level_remaining.

Steps other than step 1 depend on the information decoded by previous steps. For example decoding step 3 depends on the updated value K1 which is updated on the step 2 which in turn depends on the data decoded on the step 1.

The input stream contains bypass data which can be randomly accessed. However it’s not possible to fetch codeword2 from the stream prior to the point where K1 becomes known as the starting offset of codeword2 depends on K1.

# Proposed solution

## K parameter initialization

This contribution proposes to initialize the K parameter based on the number of significant coefficients in each sub-set. If the number of significant coefficients is greater than a threshold value T then K=1, otherwise K=0. In HM-6.0, the number of significant coefficients is calculated during encoding of the significant_coeff_flag values [2] and therefore is already available at the point where K is initialized. Therefore only one check per sub-set is required to test the condition. For HM-6.0 it is proposed to use T=14. Therefore for sub-sets with the number of significant coefficients > 14 it is proposed to initialize K=1, and K=0 otherwise.

## Restricted K Parameter Adaptivity

This contribution proposes to avoid updating the K parameter in sub-sets where the adaptivity of the parameter K doesn’t influence coding efficiency. For the subsets where K is not updated, simpler logic is required to decode the coeff_abs_level_remaining values, enabling higher throughput implementations to be realised in hardware. This scheme is shown in Figure 2

Figure 2. Coding of coeff_abs_level_remaining without K updating.

In this case the codewords can be fetched from the input stream and decoded independently of each other.

It is proposed to disable K parameter adaptivity if the sub-set index value T is equal to or greater than a certain threshold value. For HM-6.0 it is proposed to use T=6.  A sample 16x16 TU is presented on Figure 3 illustrating the proposed solution.

Figure 3. TU with fixed K

# Experiment Results

## K parameter initialization

Results presented in Table 1 and Table 2 below refer to the accompanying spreadsheet for results with greater precision. Execution took place on a Linux cluster using GCC 3.4.6.

Table 1. Simulation results for low QP (1,5, 9, 13).

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.2% | -0.3% | -0.3% | -0.3% | -0.3% | -0.3% |
| Class B | -0.1% | -0.2% | -0.2% | -0.1% | -0.1% | -0.1% |
| Class C | -0.3% | -0.3% | -0.3% | -0.3% | -0.3% | -0.3% |
| Class D | -0.5% | -0.5% | -0.5% | -0.4% | -0.5% | -0.5% |
| Class E | 0.0% | -0.1% | -0.1% | 0.0% | -0.1% | -0.1% |
| Class F | -0.5% | -0.5% | -0.5% | -0.6% | -0.5% | -0.5% |
| Overall | -0.3% | -0.3% | -0.3% | -0.3% | -0.3% | -0.3% |
|  | -0.3% | -0.3% | -0.3% | -0.3% | -0.3% | -0.3% |
| Enc Time[%] | 100% |  |  | 98% |  |  |
| Dec Time[%] | 100% |  |  | 98% |  |  |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | -0.1% | -0.1% | -0.1% | -0.1% | -0.2% | -0.2% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.1% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | -0.1% | -0.1% | 0.0% | 0.0% | 0.0% |
| Class E |  |  |  |  |  |  |
| Class F | -0.2% | -0.3% | -0.2% | -0.3% | -0.3% | -0.4% |
| Overall | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% |
|  | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% |
| Enc Time[%] | 100% |  |  | 102% |  |  |
| Dec Time[%] | 100% |  |  | 101% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | 0.1% | 0.0% | 0.0% | 0.1% | 0.1% |
| Class C | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class E | 0.0% | 0.1% | 0.0% | 0.1% | 0.1% | 0.1% |
| Class F | -0.3% | -0.3% | -0.2% | -0.4% | 0.6% | -0.2% |
| Overall | 0.0% | 0.0% | 0.0% | -0.1% | 0.2% | 0.0% |
|  | 0.0% | 0.0% | -0.1% | -0.1% | 0.2% | 0.0% |
| Enc Time[%] | 102% |  |  | 101% |  |  |
| Dec Time[%] | 102% |  |  | 101% |  |  |
|  | Low delay P Main |  |  | Low delay P HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.1% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class E | 0.0% | 0.1% | 0.0% | 0.1% | 0.1% | 0.1% |
| Class F | -0.3% | -0.3% | -0.2% | -0.3% | -0.3% | -0.9% |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | -5.0% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 99% |  |  | 100% |  |  |

Table 2. Simulation results for normal QP.

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% |
| Class E | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class F | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0.1% | -0.2% | 0.1% | 0.0% | 0.0% | 0.4% |
| Class B | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | -0.2% | 0.0% | 0.0% | 0.0% | 0.2% |
| Class E |  |  |  |  |  |  |
| Class F | 0.1% | 0.0% | 0.1% | 0.1% | 0.0% | 0.1% |
| Overall | 0.0% | -0.1% | 0.1% | 0.0% | 0.0% | 0.1% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 99% |  |  | 100% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | 0.1% | -0.1% | 0.0% | -0.2% | 0.0% |
| Class C | 0.0% | 0.2% | 0.1% | 0.0% | 0.0% | 0.1% |
| Class D | 0.1% | 0.2% | 0.5% | 0.0% | 0.1% | -0.3% |
| Class E | 0.0% | -0.6% | 0.6% | 0.0% | -0.4% | -0.6% |
| Class F | 0.0% | 0.2% | 0.0% | 0.1% | -0.7% | 1.8% |
| Overall | 0.0% | 0.1% | 0.2% | 0.0% | -0.2% | 0.2% |
|  | 0.0% | 0.1% | 0.2% | 0.0% | -0.2% | 0.2% |
| Enc Time[%] | 100% |  |  | 101% |  |  |
| Dec Time[%] | 99% |  |  | 101% |  |  |
|  | Low delay P Main |  |  | Low delay P HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | 0.4% | 0.1% | 0.0% | 0.0% | -0.3% |
| Class C | 0.0% | -0.3% | -0.3% | 0.0% | 0.0% | -0.1% |
| Class D | 0.0% | 0.3% | -0.7% | 0.0% | 0.3% | 0.2% |
| Class E | 0.0% | -0.6% | 0.3% | 0.1% | 0.8% | -0.4% |
| Class F | 0.1% | 0.3% | 0.2% | 0.0% | 1.1% | -1.6% |
| Overall | 0.0% | 0.1% | -0.1% | 0.0% | 0.4% | -0.4% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.4% | -0.4% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 99% |  |  | 100% |  |  |

## Restricted K parameter adaptivity

Table 3. Simulation results for normal QP

|  |  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Y | U | V | Y | U | V |
|  | Class A | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% |
|  | Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | Class D | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | Class E | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | Class F | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  |  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | Enc Time[%] | 99% |  |  | 100% |  |  |
|  | Dec Time[%] | 99% |  |  | 100% |  |  |
|  |  | Random Access Main |  |  | Random Access HE10 |  |  |
|  |  | Y | U | V | Y | U | V |
|  | Class A | 0.0% | 0.1% | 0.0% | 0.0% | -0.1% | 0.4% |
|  | Class B | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.1% |
|  | Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% |
|  | Class D | 0.0% | 0.0% | -0.1% | 0.0% | -0.1% | -0.1% |
|  | Class E |  |  |  |  |  |  |
|  | Class F | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% |
|  | Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% |
|  |  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% |
|  | Enc Time[%] | 100% |  |  | 99% |  |  |
|  | Dec Time[%] | 100% |  |  | 99% |  |  |
|  |  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  |  | Y | U | V | Y | U | V |
|  | Class A |  |  |  |  |  |  |
|  | Class B | 0.0% | -0.2% | -0.3% | 0.0% | -0.3% | 0.2% |
|  | Class C | 0.0% | 0.0% | -0.2% | 0.0% | 0.1% | 0.2% |
|  | Class D | 0.0% | -0.1% | 0.3% | 0.0% | 0.1% | -0.4% |
|  | Class E | 0.0% | -0.6% | -0.9% | -0.1% | -0.2% | -0.4% |
|  | Class F | -0.1% | -0.4% | 0.4% | 0.0% | 0.2% | 0.0% |
|  | Overall | 0.0% | -0.2% | -0.1% | 0.0% | 0.0% | 0.0% |
|  |  | 0.0% | -0.2% | -0.1% | 0.0% | 0.0% | 0.0% |
|  | Enc Time[%] | 101% |  |  | 101% |  |  |
|  | Dec Time[%] | 101% |  |  | 101% |  |  |
|  |  | Low delay P Main |  |  | Low delay P HE10 |  |  |
|  |  | Y | U | V | Y | U | V |
|  | Class A |  |  |  |  |  |  |
|  | Class B | 0.0% | 0.0% | -0.4% | 0.0% | 0.0% | 0.3% |
|  | Class C | 0.0% | -0.3% | -0.2% | 0.0% | -0.2% | -0.1% |
|  | Class D | 0.0% | 0.3% | -0.3% | 0.0% | 0.4% | 0.2% |
|  | Class E | 0.0% | -0.5% | -0.5% | -0.1% | 0.0% | -0.3% |
|  | Class F | 0.0% | 0.1% | 0.2% | 0.0% | 0.4% | -0.3% |
|  | Overall | 0.0% | 0.0% | -0.2% | 0.0% | 0.1% | 0.0% |
|  |  | 0.0% | -0.1% | -0.2% | 0.0% | 0.2% | 0.0% |
|  | Enc Time[%] | 99% |  |  | 101% |  |  |
|  | Dec Time[%] | 99% |  |  | 100% |  |  |

# Working draft text

## K Parameter Initialization

#### 9.2.2.8 Binarization process for coeff_abs_level_remaining

Input to this process is a request for a binarization for the syntax element coeff_abs_level_remaining[ n ].

Output of this process is the binarization of the syntax element.

The variables cLastSE and cLastRiceParam are derived as follows.

- If n is equal to 0 or all previous syntex elements coeff_abs_level_remaining[ pos ] with pos less than n are derived to be equal to 0 instead of being explicitly parsed, cLastSE is set equal to 0 and cLastRiceParam is derived as follows:are set equal to 0.

  - If the number of transform coefficient levels of the coefficient group at location ( xCG, yCG ) having a non zero value in the current subset is greater than 14, cLastRiceParam is set equal to 1.

  - Otherwise (the number of significant coefficients in the current subset is less than or equal to 14), cLastRiceParam is set equal to 0.

- Otherwise ( n is not equal to 0 ), cLastSE is set equal to coeff_abs_level_remaining[ n  − 1 ] and cLastRiceParam is set equal to the value of cRiceParam that has been derived during the invocation of this subclause for the syntax element coeff_abs_level_remaining[ n − 1 ] of the same transform block.

The variable cRiceParam is derived from cLastSE and cLastRiceParam equivalent to the following pseudo-code

cRiceParam = cLastRiceParam

+ (cLastSE >= 3   &&  cLastRiceParam = = 0)

+ (cLastSE >= 6   &&  cLastRiceParam <= 1)

+ (cLastSE >= 12  &&  cLastRiceParam <= 2)

+ (cLastSE >= 23  &&  cLastRiceParam <= 3)

The variable cTRMax is derived from cRiceParam as given in Table 9-3.

Table 9-3 – Specifcation of cTRMax depending on cRiceParam

| cRiceParam | cTRMax |
| --- | --- |
| 0 | 7 |
| 1 | 14 |
| 2 | 26 |
| 3 | 46 |
| 4 | 78 |

The binarization of coeff_abs_level_remaining consists of a prefix part and (when present) a suffix part. The prefix part of the binarization is derived by invoking the TR binarization process as specified in subclause Error: Reference source not found for the syntax element coeff_abs_level_remaining[ n ] with the variables cRiceParam and cTRMax as the inputs.

When the prefix bit string is not equal to the bin string that consists of ( cTRMax >> cRiceParam ) + cRiceParam ones, the bin string consists of a prefix bin string and a suffix bin string. The suffix bin string is derived using the EG0 binarization as specified in subclause Error: Reference source not found.

## Restricted K Parameter Adaptivity

#### 9.2.2.8 Binarization process for coeff_abs_level_remaining

Input to this process is a request for a binarization for the syntax element coeff_abs_level_remaining[ n ] and the subset index i.

Output of this process is the binarization of the syntax element.

The variables cLastSE and cLastRiceParam are derived as follows.

- If n is equal to 0 or all previous syntex elements coeff_abs_level_remaining[ pos ] with pos less than n are derived to be equal to 0 instead of being explicitly parsed, cLastSE and cLastRiceParam are set equal to 0.

- Otherwise ( n is not equal to 0 ), cLastSE is set equal to coeff_abs_level_remaining[ n  − 1 ] and cLastRiceParam is set equal to the value of cRiceParam that has been derived during the invocation of this subclause for the syntax element coeff_abs_level_remaining[ n − 1 ] of the same transform block.

The variable cRiceParam is derived from cLastSE, and cLastRiceParam and the subset index i as follows.equivalent to the following pseudo-code

- If the subset index i is greater than or equal to 6, cRiceParam is set equal to 0.

- Otherwise (the subset index i is less than 6), cRiceParam is derived according to the following pseudo-code.

cRiceParam = cLastRiceParam

+ (cLastSE >= 3   &&  cLastRiceParam = = 0)

+ (cLastSE >= 6   &&  cLastRiceParam <= 1)

+ (cLastSE >= 12  &&  cLastRiceParam <= 2)

+ (cLastSE >= 23  &&  cLastRiceParam <= 3)

The variable cTRMax is derived from cRiceParam as given in Table 9-3.

Table 9-4 – Specifcation of cTRMax depending on cRiceParam

| cRiceParam | cTRMax |
| --- | --- |
| 0 | 7 |
| 1 | 14 |
| 2 | 26 |
| 3 | 46 |
| 4 | 78 |

The binarization of coeff_abs_level_remaining consists of a prefix part and (when present) a suffix part. The prefix part of the binarization is derived by invoking the TR binarization process as specified in subclause Error: Reference source not found for the syntax element coeff_abs_level_remaining[ n ] with the variables cRiceParam and cTRMax as the inputs.

When the prefix bit string is not equal to the bin string that consists of ( cTRMax >> cRiceParam ) + cRiceParam ones, the bin string consists of a prefix bin string and a suffix bin string. The suffix bin string is derived using the EG0 binarization as specified in subclause Error: Reference source not found.

# Conclusion

## K Parameter Initialization

Presented is an approach to reduce the bitrate and increase coding efficiency of the coeff_abs_level_remaining values. Results were presented using HM-6.0.

## Restricted K Parameter Adaptivity

Presented is an approach to increase the throughput of coeff_abs_level_remaining processing by avoiding the Rice parameter K adaptation for the sub-sets where it does not influence coding efficiency. Results were presented using HM-6.0.

# References

[1] T. Nguyen et al., “Reduced-complexity entropy coding of transform coefficient levels using a combination of VLC and PIPE” JCT-VC document JCTVC-D336, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, KR, 20-28 January, 2011

[2] B. Bross, et al., “High efficiency video coding (HEVC) text specification draft 6”, JCT-VC document JCTVC-H1003, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21–30 November, 2011

# Patent rights declaration(s)

CiSRA may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Canon Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).