| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0054 |
| --- | --- |

| Title: | CE2.c.2: Reduction of number of filter classes in BA mode with having flexible directional features |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Masaaki Matsumura Seishi Takamura Atsushi Shimizu 1-1, Hikari-no-oka, Yokosuka-shi, Kanagawa 239-0847, JAPAN | Tel: Email: | +81-46-859-3521 matsumura.masaaki@lab.ntt.co.jp +81-46-859-2371 takamura.seishi@lab.ntt.co.jp +81-46-859-2448 shimizu.atsushi@lab.ntt.co.jp |
| Source: | NTT (Nippon Telegraph and Telephone) Corporation |  |  |

_____________________________

# Abstract

This contribution presents an adaptive classification framework for block-based adaptation (BA) on adaptive loop filter (ALF). By switching prepared three classification patterns each of which consists of 1, 4 or 6 filters, the average BD-rate losses were 0.0, 0.3, 0.0 and 0.0% for HE10-AI, HE10-RA, HE10-LB, HE10-LP cases, respectively. The average encoding time was 98.5 - 101.2% and the average decoding time was 99.8 - 101.0%.

# Introduction

In the current ALF of HM6.0 [1], BA mode employs initial 16 filters consist of fixed six activity domains with one (for the lowest activity domain) or three directional classification patterns (Figure 1), and designs each filter coefficient for minimizing square difference between decoded picture and original picture. Although the merging process restricts the number of filters to necessary minimum number, how to classify the initial filters is one of the important factors that decide coding efficiency of ALF [2]. Especially, for HM6.0 with LCU based ALF syntax, allocation of 16 filters to small region composed by few LCU will be excessive. Therefore, losses of the coding gain by reducing the number of initial filters might be small.

In this contribution, adaptive allocation of initial filters for each LCU is proposed. The encoder derives the index of a suitable classification pattern from prepared three patterns each of which consists of 1, 4 or 6 filters (Figure 1) by analysing the activity of the texture in LCU. The benefit of this approach is reduction of the generating-bits related to the number of filters.

Figure 1: Comparison between conventional classification and proposed one.

# Semantics and syntax

In this section, semantics and syntax related to this contribution are described. The modified syntax is specified in Table 1. The difference between conventional ALF and proposal set highlighted in yellow and colored in red. The modified semantics used in the syntax are specified below:

alf_no_filters_minus1 is used to derive the number of filter sets for the current coding treeblock. It is a requirement of bitstream conformance that the values of alf_no_filters_minus1 are in the range of 0 to 2, inclusive.

alf_start_second_filter specifies the block adaptive (BA) mode index of luma samples for which the second filter is applied.

alf_multiple_filters_flag equal to 0 specifies that the number of filters is assigned to 1: equal to 1 specifies that the number of filters is assigned to either 4 or 6.

alf_six_filters_flag equal to 0 specifies that alf_no_filters is assigned to 4: equal to 1 specifies that alf_no_filters is assigned to 6.

| Table 1: Adaptive loop filter info syntax |  |
| --- | --- |
| alf_info( rx, ry, cIdx ) { |  |
| if(NumALFFiltersInStoredBuffer[cIdx] > 0) |  |
| alf_new_filter_set_flag | u(1) |
| if( alf_new_filter_set_flag = = 0 && NumALFFiltersInStoredBuffer[cIdx] > 0) |  |
| alf_stored_filter_set_idx[ cIdx ] | u(v) |
| else { |  |
| if( cIdx = = 0 ) { |  |
| alf_no_filters_minus1 | ue(v) |
| if( alf_no_filters_minus1 = = 1 ) |  |
| alf_start_second_filter | ue(v) |
| else if( alf_no_filters_minus1 > 1 ) |  |
| for( i = 1; i < 15; i++ ) |  |
| alf_filter_pattern_flag[ cIdx ][ ry ][ rx ][ i ] | u(1) |
| alf_multiple_filters_flag | u(1) |
| AlfNoFilters = alf_multiple_filters_flag + 1 |  |
| if( AlfNoFilters != 1 ) { |  |
| alf_six_filters_flag | u(1) |
| if( !alf_six_filters_flag ) |  |
| AlfNoFilters = 4 |  |
| else |  |
| AlfNoFilters = 6 |  |
| for( i = 1; i < AlfNoFilters; i++ ) |  |
| alf_filter_pattern_flag[ cIdx ][ ry ][ rx ][ i ] | u(1) |
| } |  |
| … |  |
| } |  |
| … |  |
| } |  |
| } |  |

# Decoder description

## Overview for subclauses of this section

In this section, the decoding process of proposed ALF is described. In this contribution, subclause 8.7.3.3 described in Commitee Draft [1] are modified. On subclause 3.2 (as 8.7.3.3), the differences between conventional ALF process and proposed one set highlighted in yellow.

## Derivation process for filter index array for luma samples (as 8.7.3.3)

Inputs of this process are:

- a luma location ( xC, yC ) specifying the top-left luma sample of the current coding unit relative to the top left luma sample of the current picture,

- a variable log2CbSize specifying the size of the current coding unit.

Output of this process is the two-dimensional filter index array of (nS)x(nS), fIdx.

A variable nS is set equal to ( 1 << log2CbSize ).

The boundary padding process specified in subclause Error: Reference source not found is invoked with the luma location ( xC, yC ), the size of coding unit log2CbSize and the chroma component index cIdx set equal to 0, and the output is assigned to the luma sample array s”.

The filter index array fIdx is specified in the follows:

- If AlfNoFilters is equal to 1, the filter index array fIdx is assigned to 0.

- Otherwise, if AlfNoFilters is equal to 1, the filter index array fIdx is specified in the follows:

- The variables varTempH[ x ][ y ], varTempV[ x ][ y ] and varTemp1[ x ][ y ] with x, y = 0, 2, 4,...(nS) is derived as

varTempH[ x ][ y ] = | ( s”[ xC+x, yC+y ] << 1 ) – s”[ xC+x−1, yC+y ] – s”[ xC+x+1, yC+y ] |		(8-375)
varTempV[ x ][ y ] = | ( s”[ xC+x, yC+y ] << 1 ) – s”[ xC+x, yC+y−1 ] – s”[ xC+x, yC+y+1 ] |		(8-376)

- The variables varTempH1[ x ][ y ], varTempV1[ x ][ y ], and varTemp3[ x ][ y ] with x, y = 0..( (nS) − 1 ) >> 2 are derived as

varTempH1[ x ][ y ] = ij varTempH[ (x << 2 ) + i ][ (y << 2) + j ] with i, j = 1, 2		(8-377)
varTempV1[ x ][ y ] = ij varTempV[ (x << 2 ) + i ][ (y << 2) + j ] with i, j = 1, 2		(8-378)
varTemp3[ x ][ y ] = ( varTempH1[ x ][ y ] + varTempV1[ x ][ y ] ) >> 3			(8-379)

- If varTemp3[ x ][ y ] is smaller than 8, the filter index array fIdx is assigned to 0.

- Otherwise,

- If varTempH1[ x >> 2 ][ y >> 2 ] is greater than varTempV1[ x >> 2 ][ y >> 2 ] << 1, fIdx[ x ][ y ] is assigned to 2.

- Otherwise, if varTempH1[ x >> 2 ][ y >> 2 ] is greater than varTempV1[ x >> 2 ][ y >> 2 ] << 1, fIdx[ x ][ y ] is assigned to 3.

- Otherwise, fIdx[ x ][ y ] is assigned to 1.

- Othewise, the filter index array fIdx is specified in the follows:

- The variables varTempH[ x ][ y ], varTempV[ x ][ y ] and varTemp1[ x ][ y ] with x, y = 0, 2, 4,...(nS) is derived as

varTempH[ x ][ y ] = | ( s”[ xC+x, yC+y ] << 1 ) – s”[ xC+x−1, yC+y ] – s”[ xC+x+1, yC+y ] |		(8-380)
varTempV[ x ][ y ] = | ( s”[ xC+x, yC+y ] << 1 ) – s”[ xC+x, yC+y−1 ] – s”[ xC+x, yC+y+1 ] |		(8-381)

- The variables varTempH1[ x ][ y ], varTempV1[ x ][ y ], and varTemp3[ x ][ y ] with x, y = 0..( (nS) − 1 ) >> 2 are derived as

varTempH1[ x ][ y ] = ij varTempH[ (x << 2 ) + i ][ (y << 2) + j ] with i, j = 1, 2		(8-382)
varTempV1[ x ][ y ] = ij varTempV[ (x << 2 ) + i ][ (y << 2) + j ] with i, j = 1, 2		(8-383)
varTemp3[ x ][ y ] = ( varTempH1[ x ][ y ] + varTempV1[ x ][ y ] ) >> 3			(8-384)

- If varTemp3[ x ][ y ] is smaller than 6, the filter index array fIdx is assigned to 0.

- Otherwise, If varTemp3[ x ][ y ] is smaller than 16,

- If varTempH1[ x >> 2 ][ y >> 2 ] is greater than varTempV1[ x >> 2 ][ y >> 2 ], fIdx[ x ][ y ] is assigned to 2.

- Otherwise, fIdx[ x ][ y ] is assigned to 4.

- Otherwise,

- If varTempH1[ x >> 2 ][ y >> 2 ] is greater than varTempV1[ x >> 2 ][ y >> 2 ] << 1, fIdx[ x ][ y ] is assigned to 3.

- Otherwise, if varTempH1[ x >> 2 ][ y >> 2 ] is greater than varTempV1[ x >> 2 ][ y >> 2 ] << 1, fIdx[ x ][ y ] is assigned to 5.

- Otherwise, fIdx[ x ][ y ] is assigned to 1.

# Simulation results

In this section, objective evaluations are described. Table 2 and 3 present coding efficiencies and computational complexities based on common condition compared to HM6.0 anchors (bug-fixed). The source code was compiled on 64-bit Windows 7 environment by using Microsoft Visual Studio 2010 Professional edition, and the experiments were performed on the cluster of PCs with 64-bit Windows 7, Intel Core i5 2500K and 16GB RAM.

| Table 2: Experimental results (mandatory). |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | All Intra HE10 |  |  |  |  |  | Random Access HE10 |  |  |  |  |
|  |  | Y |  | U |  | V |  | Y |  | U |  | V |
| Class A |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.5% |  | 0.1% |  | 0.1% |
| Class B |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.4% |  | -0.1% |  | 0.0% |
| Class C |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.2% |  | -0.3% |  | -0.2% |
| Class D |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.1% |  | -0.1% |  | -0.3% |
| Class E |  | 0.0% |  | 0.0% |  | 0.0% |  |  |  |  |  |  |
| Overall |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.3% |  | -0.1% |  | -0.1% |
|  |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.3% |  | -0.1% |  | -0.1% |
| Class F |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.1% |  | -0.2% |  | -0.1% |
| Enc Time[%] |  | 101.19% |  |  |  |  |  | 99.81% |  |  |  |  |
| Dec Time[%] |  | 100.00% |  |  |  |  |  | 99.63% |  |  |  |  |
|  |  | Low delay B HE10 |  |  |  |  |  | Low delay P HE10 |  |  |  |  |
|  |  | Y |  | U |  | V |  | Y |  | U |  | V |
| Class A |  |  |  |  |  |  |  | -- |  | -- |  | -- |
| Class B |  | 0.0% |  | 0.0% |  | 0.0% |  | -- |  | -- |  | -- |
| Class C |  | 0.0% |  | 0.1% |  | 0.2% |  | -- |  | -- |  | -- |
| Class D |  | 0.0% |  | 0.1% |  | 0.0% |  | -- |  | -- |  | -- |
| Class E |  | 0.0% |  | 0.1% |  | 0.0% |  | -- |  | -- |  | -- |
| Overall |  | 0.0% |  | 0.1% |  | 0.0% |  | -- |  | -- |  | - |
|  |  | 0.0% |  | 0.1% |  | 0.0% |  | -- |  | -- |  | -- |
| Class F |  | 0.0% |  | -0.5% |  | -0.8% |  | -- |  | -- |  | -- |
| Enc Time[%] |  | 98.53% |  |  |  |  |  | -- |  |  |  |  |
| Dec Time[%] |  | 100.63% |  |  |  |  |  | -- |  |  |  |  |
| Table 3: Experimental results (full). |  |  |  |  |  |  |  |  |  |  |  |  |
|  | All Intra HE10 |  |  |  |  |  | Random Access HE10 |  |  |  |  |  |
|  | Y |  | U |  | V |  | Y |  | U |  | V |  |
| Class A | 0.0% |  | 0.0% |  | 0.0% |  | 0.5% |  | 0.1% |  | 0.1% |  |
| Class B | 0.0% |  | 0.0% |  | 0.0% |  | 0.4% |  | -0.1% |  | 0.0% |  |
| Class C | 0.0% |  | 0.0% |  | 0.0% |  | 0.2% |  | -0.3% |  | -0.2% |  |
| Class D | 0.0% |  | 0.0% |  | 0.0% |  | 0.1% |  | -0.1% |  | -0.3% |  |
| Class E | 0.0% |  | 0.0% |  | 0.0% |  |  |  |  |  |  |  |
| Class F | 0.0% |  | 0.0% |  | 0.0% |  | 0.1% |  | -0.2% |  | -0.1% |  |
| Overall | 0.0% |  | 0.0% |  | 0.0% |  | 0.3% |  | -0.1% |  | -0.1% |  |
|  | 0.0% |  | 0.0% |  | 0.0% |  | 0.3% |  | -0.1% |  | -0.1% |  |
| Enc Time[%] | 101.19% |  |  |  |  |  | 99.66% |  |  |  |  |  |
| Dec Time[%] | 100.00% |  |  |  |  |  | 99.80% |  |  |  |  |  |
|  | Low delay B HE10 |  |  |  |  |  | Low delay P HE10 |  |  |  |  |  |
|  | Y |  | U |  | V |  | Y |  | U |  | V |  |
| Class A |  |  |  |  |  |  |  |  |  |  |  |  |
| Class B | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.3% |  | 0.4% |  |
| Class C | 0.0% |  | 0.1% |  | 0.2% |  | 0.0% |  | -0.1% |  | -0.3% |  |
| Class D | 0.0% |  | 0.1% |  | 0.0% |  | 0.0% |  | 0.5% |  | 0.0% |  |
| Class E | 0.0% |  | 0.1% |  | 0.0% |  | 0.0% |  | 0.5% |  | 0.0% |  |
| Class F | 0.0% |  | -0.5% |  | -0.8% |  | 0.0% |  | -0.2% |  | 1.1% |  |
| Overall | 0.0% |  | -0.1% |  | -0.1% |  | 0.0% |  | 0.2% |  | 0.4% |  |
|  | 0.0% |  | -0.1% |  | -0.2% |  | 0.0% |  | 0.1% |  | 0.4% |  |
| Enc Time[%] | 98.48% |  |  |  |  |  | 99.02% |  |  |  |  |  |
| Dec Time[%] | 100.67% |  |  |  |  |  | 101.03% |  |  |  |  |  |

# Conclusion

In this contribution, adaptive allocation of initial filters for each LCU is proposed. By switching prepared three classification patterns each of which consists of 1, 4 or 6 filters, the average BD-rate losses were 0.0, 0.3, 0.0 and 0.0% for HE10-AI, HE10-RA, HE10-LB, HE10-LP cases, respectively. The average encoding time was 98.5 - 101.2% and the average decoding time was 99.8 - 101.0%. Adoption of this simplification is recommended.

Thanks to Mitsubishi’s cross-verification [3] for confirming the coding results.

# References

- B. Bross, W.-J. Han, J.-R. Ohm, G. J. Sullivan, T. Wiegand, “High efficiency video coding (HEVC) text specification draft 6,” JCTVC-H1003_dK.doc, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012.

- M. Matsumura, S. Takamura, A. Shimizu, H. Jozawa, “Non-CE8: Modification of directional classification for block-based adaptation on ALF,” JCTVC-H0094.doc, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012.

- K. Sugimoto, A. Minezawa, S. Sekiguchi, “CE2 C.2: crosscheck on reduction of number of filter classes in BA mode (JCTVC-I0054),” JCTVC-I0064.doc, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012.

# Patent rights declaration(s)

NTT may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).