| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0055 |
| --- | --- |

| Title: | Picture quality evaluation of non-local means post filtering by SSIM |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Masaaki Matsumura Seishi Takamura Atsushi Shimizu 1-1, Hikari-no-oka, Yokosuka-shi, Kanagawa 239-0847, JAPAN | Tel: Email: | +81-46-859-3521 matsumura.masaaki@lab.ntt.co.jp +81-46-859-2371 takamura.seishi@lab.ntt.co.jp +81-46-859-2448 shimizu.atsushi@lab.ntt.co.jp |
| Source: | NTT (Nippon Telegraph and Telephone) Corporation |  |  |

_____________________________

# Abstract

This contribution reports the picture quality evaluation of non-local means post filtering by SSIM. Especially, SSIM improves up to 0.0043 in the condition of low-rate (QP: 37). Average BD-rate improvements of 1.1% calculated by SSIM instead of PSNR were obtained and the improvement of subjective quality in various regions was confirmed. SSIM-based RDO might be useful for improving the subjective quality. The maximum gain was 5.1% for the sequence “SlideEditing”.

# Picture quality evaluation by SSIM

In this section, how to evaluate the picture quality by SSIM [1] is described. SSIM with two input signals (original image x and compared image y) consists of three components luminance comparison l(x, y), contrast comparison c(x, y) and structure comparison s(x, y) as follows:

SSIM(x, y) = [l(x, y)] + [c(x, y)] + [s(x, y)]						(1-1).

where  > 0,  > 0 and  > 0 are parameters used to adjust the relative importance of the three components. In general, to simplify the function,  =  =  = 1 is assigned to them. Simplified SSIM is defined as:

SSIMx,y=(2μxμy+C1) (2σxy+C2)(μx2+μy2+C1) (σx2+σy2+C2)						(1-2).

where xand y are mean intensities of each signal. xand y are deviations of each signal. C1 is assigned to (K1L)2 and C2 is assigned to (K2L)2. Here, K1 is assigned to 0.01 and K2 is assigned to 0.03. L is the dynamic range of the signal value (255 for 8-bit grayscale images).

Since SSIM evaluates the similarity of geometrical structure, mosquito noise, block noise or excessive diffusion frequently detected as distortions by video coding cause debasement of SSIM (Figure 1). Therefore, many encoders such as JM and x264 employ SSIM metric.

Figure 1: Examples of SSIM evaluation.

# Non-local means filter

In this section, non-local means (NLM) filter [2] utilized as a post filter is described. In the basic algorithm of conventional NLM filter, for derivation of denoised luma picture recFiltPictureL with i = 0.. pic_width_in_luma_samples – 1 and j = 0..pic_height_in_luma_samples – 1, the following ordered steps apply:

- Initially, SumFltCoeffs is assigned to 0 and SumSamples is assigned to s”[iT][jT], where s” with two-dimensional elements specifies a padded luma sample array and a location (iT, jT) specifies the coordinate of the sample to be denoised.

- For each tap element (generically, odd numbered NxN block around the sample is constructed as the tap element) with location (iS, jS), a SSD (Sum of Square Difference) between the template at a location (iT, jT) and the template at the location (iS, jS) is derived (an example of 9x9 tap with 3x3 template is described in Figure 2). SumFltCoeffs and SumSamples are derived by:

FltCoeff = exp(–SSD  2)								(2-1)
SumFltCoeffs += FltCoeff								(2-2)
SumSamples += FltCoeff * s”[iS][jS]							(2-3)

- recFiltPictureL[iT][jT] is derived by SumSamples SumWeights.

NOTE – Function exp() indicates mathematical exponential function.  specifies filter strength of NLM.

Figure 2: Relationship between the templates, tap elements and target.

In this contribution, 5x5 square-shared tap and 5 samples cross-shaped template (formed by t0, t2, t4, t6 and t8) are implemented. The parameter DenoiseCoeff, that controls filter strength of NLM, shall be in the range of 1 to 16, inclusive. Using DenoiseCoeff and exponential function modified into LUT, FltCoeff described in (2-1) is derived by:

expLUT[i]=Round16383*exp(−i÷164)				(2-4)
FltCoeff = expLUT[Min(SSD  (5 * DenoiseCoeff , 1023)]				(2-5).

Since NLM filter smoothes the areas with similar geometric structures, low-to-middle filter strength will reduce mosquito noises by smoothing of maintaining the textural edges, and middle-to-high filter strength will diffuse also block boundaries. Therefore, NLM filter removes the noises efficiently; visible improvements of subjective quality are reported in [2, 3, 4].

# Simulation results

## NLM post filtering and associated overhead

In the simulation, NLM filter is applied to the decoded luma component in quadtree-basis. The NLM parameter, DenoiseCoeff (4-bit), is signaled for header of the overhead, whether NLM filter is applied to the block or not is decided according to the RD-cost based on SSIM [5]. This overhead (FltBit) is added to the anchor bit rate for BD-rate evaluations. When NLM filter does not improve the picture quality, 1-bit filter header disables NLM filter.

## Objective evaluation

Table 1 presents example of the PSNR and SSIM comparisons between HM6.0 anchor (Main Profile All-Intra) and NLM post filtered. Although the improvements of BD-rate (PSNR) are small,  favorable improvements of SSIM are obtained in particular for lower bit rate.

| Table 1: SSIM comparisons |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Reference (MainProfile All-Intra) |  |  |  | NLM post filtering |  |  |  |  | BD-rate (PSNR) | BD-rate (SSIM) |
|  |  | QP | kbps | Y psnr | SSIM |  | kbps | Y psnr | SSIM | FltBit/frame |  | Y | Y |
| Class A 4K | PeopleOnStreet | 22 | 104595.62 | 43.24 | 0.9825 |  | 104595.65 | 43.24 | 0.9825 | 1.00 |  | -0.5% | -1.3% |
|  |  | 27 | 60768.17 | 39.83 | 0.9639 |  | 60847.34 | 39.83 | 0.9641 | 2639.16 |  |  |  |
|  |  | 32 | 34760.80 | 36.72 | 0.9337 |  | 34852.43 | 36.78 | 0.9347 | 3054.52 |  |  |  |
|  |  | 37 | 20373.29 | 33.86 | 0.8966 |  | 20460.43 | 34.00 | 0.8997 | 2904.76 |  |  |  |
| Class B 1080p | Kimono | 22 | 22261.04 | 42.70 | 0.9659 |  | 22285.29 | 42.65 | 0.9660 | 1010.15 |  | 1.2% | -1.8% |
|  |  | 27 | 12119.17 | 41.01 | 0.9546 |  | 12155.68 | 40.95 | 0.9550 | 1521.55 |  |  |  |
|  |  | 32 | 6817.14 | 38.96 | 0.9361 |  | 6850.46 | 38.95 | 0.9371 | 1388.18 |  |  |  |
|  |  | 37 | 3795.37 | 36.51 | 0.9048 |  | 3828.51 | 36.56 | 0.9069 | 1380.79 |  |  |  |
| Class C WVGA | BasketballDrill | 22 | 20724.66 | 41.75 | 0.9692 |  | 20730.08 | 41.75 | 0.9692 | 108.55 |  | -0.1% | -1.0% |
|  |  | 27 | 11201.07 | 38.37 | 0.9377 |  | 11218.79 | 38.36 | 0.9380 | 354.34 |  |  |  |
|  |  | 32 | 5982.81 | 35.41 | 0.8930 |  | 6000.85 | 35.44 | 0.8941 | 360.77 |  |  |  |
|  |  | 37 | 3302.37 | 32.78 | 0.8421 |  | 3319.97 | 32.86 | 0.8447 | 352.02 |  |  |  |
| Class D WQVGA | BQSquare | 22 | 12962.45 | 41.18 | 0.9765 |  | 12965.20 | 41.17 | 0.9766 | 45.82 |  | -0.3% | -0.5% |
|  |  | 27 | 8219.97 | 36.80 | 0.9420 |  | 8224.93 | 36.81 | 0.9422 | 82.53 |  |  |  |
|  |  | 32 | 5047.25 | 32.99 | 0.8935 |  | 5052.74 | 33.03 | 0.8941 | 91.46 |  |  |  |
|  |  | 37 | 2988.97 | 29.44 | 0.8388 |  | 2994.74 | 29.50 | 0.8402 | 96.12 |  |  |  |
| Class E 720p | FourPeople | 22 | 30153.57 | 43.81 | 0.9790 |  | 30186.48 | 43.76 | 0.9791 | 548.46 |  | 0.0% | -2.6% |
|  |  | 27 | 18425.97 | 41.26 | 0.9708 |  | 18463.14 | 41.23 | 0.9712 | 619.45 |  |  |  |
|  |  | 32 | 11306.02 | 38.39 | 0.9569 |  | 11343.52 | 38.44 | 0.9581 | 625.03 |  |  |  |
|  |  | 37 | 6884.67 | 35.29 | 0.9320 |  | 6923.28 | 35.41 | 0.9348 | 643.56 |  |  |  |
| Class F WQVGA | SlideEditing | 22 | 37876.15 | 46.37 | 0.9976 |  | 37877.00 | 46.37 | 0.9976 | 28.50 |  | -1.9% | -5.1% |
|  |  | 27 | 28379.30 | 41.83 | 0.9950 |  | 28393.94 | 42.16 | 0.9957 | 487.89 |  |  |  |
|  |  | 32 | 21184.69 | 37.04 | 0.9880 |  | 21200.15 | 37.40 | 0.9898 | 515.37 |  |  |  |
|  |  | 37 | 15101.99 | 32.07 | 0.9681 |  | 15118.73 | 32.47 | 0.9725 | 558.07 |  |  |  |

## Subjective evaluation

Each of Figure 3 to 8 presents one of the frames which confirmed visible improvement.

Figure 3: PeopleOnStreet, SSIM (HM6.0: 0.89800, NLM: 0.90126), 
PSNR (HM6.0: 33.8615 [dB], NLM: 34.0082 [dB]), QP37, POC#100.

Figure 4: Kimono, SSIM (HM6.0: 0.92321, NLM: 0.92611), 
PSNR (HM6.0: 37.5249 [dB], NLM: 37.5876 [dB]), QP37, POC#121.

Figure 5: BasketballDrill, SSIM (HM6.0: 0.84562, NLM: 0.84895), 
PSNR (HM6.0: 32.7240 [dB], NLM: 32.8322 [dB]), QP37, POC#179.

Figure 6: BQSquare, SSIM (HM6.0: 0.82674, NLM: 0.82902), 
PSNR (HM6.0: 29.8169 [dB], NLM: 29.8732 [dB]), QP37, POC#588.

Figure 7: FourPeople, SSIM (HM6.0: 0.93184, NLM: 0.93495), 
PSNR (HM6.0: 35.2584 [dB], NLM: 35.3902 [dB]), QP37, POC#101.

Figure 8: SlideEditing, SSIM (HM6.0: 0.96423, NLM: 0.96961), 
PSNR (HM6.0: 31.6031 [dB], NLM: 32.0183 [dB]), QP37, POC#141.

# Conclusion

In this contribution, the picture quality enhancement by non-local means post filtering is demonstrated. Especially, SSIM improves up to 0.0043 in the condition of low-rate (QP: 37). Average BD-rate improvements of 1.1% calculated by SSIM instead of PSNR were obtained and the improvement of subjective quality in various regions was confirmed. SSIM-based RDO might be useful for improving the subjective quality. The maximum gain was 5.1% for the sequence “SlideEditing”.

# References

- Zhou Wang, A C Bovik, H R Sheikh, and E P Simoncelli, “Image quality assessment: From error visibility to structural similarity,” IEEE Transactions on Image Processing, vol. 13, no. 4, pp. 600–612, Apr. 2004.

- M. Matsumura, Y. Bandoh, S. Takamura, H. Jozawa, “In-loop filter based on non-local means filter,” JCTVC-E206, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, 16-23 March, 2011.

- M. Matsumura, Y. Bandoh, S. Takamura, H. Jozawa, “Modifications of in-loop filter based on non-local means filter,” JCTVC- F047, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011.

- M. Matsumura, S. Takamura, H. Jozawa, “CE8.h: CU-based ALF with non-local means filter,” JCTVC-G235, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011.

- C. Yeo, H. L. Tan, Y. H. Tan, “On Rate Distortion Optimization Using SSIM,” in proc. of the IEEE ICASSP-2012, Kyoto, Japan, Mar. 25 - 30, 2012.

# Patent rights declaration(s)

NTT may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).