| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0056 |
| --- | --- |

| Title: | Bitstream restriction flag to enable tile split |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ohji Nakagami Teruhiko Suzuki Sony City Osaki. 2-10-1 Osaki Shinagawa-ku , Tokyo, 141-8610 JAPAN | Tel: Email: | +81-50-3750-2746 ohji.nakagami@jp.sony.com teruhikos@jp.sony.com |
| Source: | Sony Corporation |  |  |

_____________________________

# Abstract

The contribution proposes to add 1-bit flag in VUI as tile_splittable_flag. The proposed flag represents bitstream restriction when tile coding is used. The flag enables decoders to decode tiles independently not only in picture level but also in bitstream level. When the flag is set to true, it is possible to extract any tile from the bitstream without entire decoding process. It is asserted that such flag enhances usability of tile coding in some application field. Eg. Frame packing stereo encoding, TV-conference systems etc.

# Introduction

In the current draft, a tile in a picture can be decoded independently from other tiles in the same picture. When the following bitstream restrictions are satisfied by encoders, a tile in a bitstream can be decoded independently from other tiles in the same bistream.

- Tile structure is kept through the sequence

- Motion vectors pointes inside the current tile.

- Temporal mv in the different tile is not used.

- Loop filtering is disabled between tile boundaries.

- No SAO/ALF filter parameters are copied between tiles.

- Tiles are separated in the entropy coding point of view.

By sending a 1-bit flag to represent such restrictions are satisfied, the decoders can decode only the required tile. In other words, when the flag is true, tiles can be split to the independent bitstream without entire decode and encode process. Such flag enhances usability of tile coding in some application field. Eg. Frame packing stereo (side-by-side, top-bottom) coding, TV-conference systems etc.

# Proposal

Two methods are proposed.

In method1, it is proposed to send a 1-bit flag in a bitstream.

In method2, it is proposed to send a 1-bit flag for each tile in a bitstream.

The difference between two proposed methods is flexibility to represent the restriction. In method1, when the flag is true, any tile in the bitstream can be decoded independently. In method2, only a tile for which the flag is true can be decoded independently. It is noted that the flexibility of method2 enhances usability of the tile coding.

The detail syntax and semantics about the restrictions are described below.

E.1 VUI syntax

E.1.1 VUI parameters syntax

Method1

| vui_parameters( ) { | Descriptor |
| --- | --- |
| Abbrev. |  |
| bitstream_restriction_flag | u(1) |
| if( bitstream_restriction_flag ) { |  |
| motion_vectors_over_pic_boundaries_flag | u(1) |
| tile_splittable_flag | u(1) |
| max_bytes_per_pic_denom | ue(v) |
| max_bits_per_mincu_denom | ue(v) |
| log2_max_mv_length_horizontal | ue(v) |
| log2_max_mv_length_vertical | ue(v) |
| } |  |
| } |  |

Method2

| vui_parameters( ) { | Descriptor |
| --- | --- |
| Abbrev. |  |
| bitstream_restriction_flag | u(1) |
| if( bitstream_restriction_flag ) { |  |
| motion_vectors_over_pic_boundaries_flag | u(1) |
| if( tiles_or_entropy_coding_sync_idc = = 1 ) |  |
| for( j = 0, tileId = 0; j <= num_tile_columns_minus1; j++ ) |  |
| for( i = 0; i <= num_tile_rows_minus1; i++, tileId++ ) |  |
| tile_splittable_flag | u(1) |
| max_bytes_per_pic_denom | ue(v) |
| max_bits_per_mincu_denom | ue(v) |
| log2_max_mv_length_horizontal | ue(v) |
| log2_max_mv_length_vertical | ue(v) |
| } |  |
| } |  |

E.2 VUI semantics

E.2.1 VUI parameters semantics

tile_splittable_flag equal to 1 indicates the bitstream (Method1) / the tile and the bitstream (Method2) confirms the following all restrictions.

- tiles_or_entropy_coding_sync_idc is set equal to 1

- tile_info_present_flag is set equal to 0 or (tile_info_present_flag is set equal to 1 and ColumnWidth[ i ] and RowHeight[ i ] are the same value through the sequence for all i )

NOTE – Tile structure is kept through the sequence

- For all CU in a tile, no sample outside the current tile boundaries and no sample at a fractional sample position for which the sample value is derived using one or more samples outside the current tile boundaries is used for inter prediction of any sample.

NOTE – Motion vectors pointes inside the same tile of the current CU. This definition is similar to the definition of motion_vectors_over_pic_boundaries_flag in the current draft.

- For all CU in a tile, temporal mv in the different tile is not used

- One of the following two conditions should be satisfied.

  - the size of a slice on a tile is equal to or less than the size of the tile

  - num_entry_point_offsets is set equal to the number of tiles in a frame.

NOTE–Tiles are separated in the entropy coding point of view.

- sao_offset_vlc() for SAO or alf_unit() for ALF should be used at the most top or the most left LCU in a tile.

NOTE– No SAO/ALF filter parameters are copied between tiles..

- loop_filter_across_tiles_enabled_flag==0

NOTE – Loop filtering is disabled between tile boundaries.

tile_splittable_flag equal to 0 indicate the bitstream (Method1) / the tile and the bitstream (Method2) may not confirm such restrictions.

When the tile_splittable_flag syntax element is not present, tile_splittable_flag syntax value is inferred to be equal to 0.

# Patent rights declaration(s)

Sony Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).