| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0057 |
| --- | --- |

| Title: | Frame packing arrangement SEI extension for HEVC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ohji Nakagami Teruhiko Suzuki Sony City Osaki. 2-10-1 Osaki Shinagawa-ku , Tokyo, 141-8610 JAPAN | Tel: Email: | +81-50-3750-2746 ohji.nakagami@jp.sony.com teruhikos@jp.sony.com |
| Source: | Sony Corporation |  |  |

_____________________________

# Abstract

In the current draft, frame packing arrangement SEI is defined as the same as SEI defined in AVC (Rec. ITU-T H.264 | ISO/IEC 14496-10). This contribution proposes to modify the frame packing arrangement SEI for HEVC to support coding tools in HEVC.

Two extensions to the frame packing arrangement SEI are proposed. The proposed extension links the view information and coding structure. One is an extension for frame sequential coding using temporal_id. The other is an extension for frame packing coding using tile.

The proposed extensions enable to represent view information with fewer bits. Furthermore, decoders can output only the base view without entire decoding process since the additional view is independent of the base view in the coding structure point of view.

# Introduction

Temporal_id and tile are beneficial coding tools for frame packing stereo 3D. For frame sequential coding, it is possible to separate the base and the additional view by using lower and higher temporal_id, respectively. For side-by-side and top-bottom coding, tile is suitable coding tool since decoders are enabled to decode each view coded by in independent tile.

The contribution proposes to extend the frame packing arrangement SEI to support such features. In the current draft, the SEI is defined as the same as SEI defined in AVC (Rec. ITU-T H.264 | ISO/IEC 14496-10) specification. For temporal and spatial packing stereo 3D, two extensions are proposed. One is for frame sequential coding using temporal_id. The other is for frame packing coding using tile. The proposed extensions link view information and coding tools. It decreases burdens for decoders to parse and process the SEI.

# Proposal

The following two extensions are proposed to Frame packing arrangement SEI in the current draft are proposed.

## Extension for frame sequential coding

It is proposed to add two syntax elements (frame0_max_temporal_id_minus1 and frame1_max_temporal_id_minus1) to support temporal_id in Frame packing arrangement SEI.

frame0_max_temporal_id_minus1 represents the max temporal id minus 1 value for the frame0, and frame1_max_temporal_id_minus1 represents the max temporal id minus 1 value for the frame1.

With this syntax, decoder can link view information and temporal_id. When only frame0 is necessary, decoders can drop NAL for frame1 similar to temporal scalability.

## Extension for frame packing coding

It is proposed to send a 1-bit control flag (tile_coding_flag) and a 1-bit view information flag (frame_id) for each tile.

tile_coding_flag represents the proposed extension is used or not. When the flag is false, the conventional pixel position syntax is used to send view position.

frame_id represents view information for each tile. When frame_id is equal to X, the tile belongs to frameX, where X is 0 or 1.

With these syntaxes, decoder can link view information and tile_id. When only frame0 is necessary, decoders can drop tiles for frame1 in picture level if the bitstream is encoded with some restrictions.

The proposal also reduces the bits in the SEI message. According to the frame packing arrangement SEI in AVC, it is necessary to send the SEI message for each AU to represent view information. One reason is that there is no link between view and coding information. By using temporal_id or tile_id as described in this section, such link is created. Thus at least one SEI message is enough to send, as long as the coding structure is kept. This decreases burden for decoders to handle the SEI messages.

# Syntax change

The detail syntax and semantic changes are as follows. The change part is yellow highlighted.

Frame packing arrangement SEI message syntax

| frame_packing_arrangement( payloadSize ) { | C | Descriptor |
| --- | --- | --- |
| frame_packing_arrangement_id | 5 | ue(v) |
| frame_packing_arrangement_cancel_flag | 5 | u(1) |
| if( !frame_packing_arrangement_cancel_flag ) { |  |  |
| frame_packing_arrangement_type | 5 | u(7) |
| quincunx_sampling_flag | 5 | u(1) |
| content_interpretation_type | 5 | u(6) |
| spatial_flipping_flag | 5 | u(1) |
| frame0_flipped_flag | 5 | u(1) |
| field_views_flag | 5 | u(1) |
| current_frame_is_frame0_flag | 5 | u(1) |
| frame0_self_contained_flag | 5 | u(1) |
| frame1_self_contained_flag | 5 | u(1) |
| if (frame_packing_arrangement_type == 5) { |  |  |
| frame0_max_temporal_id_minus1 | 5 | u(3) |
| frame1_max_temporal_id_minus1 | 5 | u(3) |
| }else{ |  |  |
| if( !quincunx_sampling_flag && frame_packing_arrangement_type != 5 ) { |  |  |
| tile_coding_flag | 5 | u(1) |
| if( tile_coding_flag ) |  |  |
| for( j = 0, tileId = 0; j <= num_tile_columns_minus1; j++ ) |  |  |
| for( i = 0; i <= num_tile_rows_minus1; i++, tileId++ ) |  |  |
| frame_id | 5 | u(1) |
| else { |  |  |
| frame0_grid_position_x | 5 | u(4) |
| frame0_grid_position_y | 5 | u(4) |
| frame1_grid_position_x | 5 | u(4) |
| frame1_grid_position_y | 5 | u(4) |
| } |  |  |
| } |  |  |
| } |  |  |
| frame_packing_arrangement_reserved_byte | 5 | u(8) |
| frame_packing_arrangement_repetition_period | 5 | ue(v) |
| } |  |  |
| frame_packing_arrangement_extension_flag | 5 | u(1) |
| } |  |  |

Frame packing arrangement SEI message semantics

frame0_max_temporal_id_minus1 specifies ,when frame_packing_arrangement is equal to 5, the maximum number of temporal layers present in the sequence for frame0. The value of frame0_max_temporal_layers_minus1 shall be in the range of 0 to 6, inclusive.

frame1_max_temporal_id_minus1 specifies ,when frame_packing_arrangement is equal to 5, the maximum number of temporal layers present in the sequence for frame1. The value of frane1_max_temporal_layers_minus1 shall be in the range of (frame0_max_temporal_id_minus1 +1) to 7, inclusive

tile_coding_flag equal to 1 ,when frame_packing_arrangement_type is equal to 3 or equal to 4, indicates that tile coding is used for side-by-side or top-bottom coding, and frame_id for each tile exist in the bitstream. When frame_packing_arrangement_type is less than 3, it is a requirement of bitstream conformance that tile_coding_flag shall be equal to 0.

frame_id equal to 0 specifies that a tile having the corresponding tileID belongs to frame0. frame_id equal to 1 specifies that a tile having the corresponding tileID belongs to frame1.

# Patent rights declaration(s)

Sony Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).