| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0059 |
| --- | --- |

| Title: | Improvement of Scaling List |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yoshitaka Morigami Junichi Tanaka Teruhiko Suzuki 2-10-1, Osaki, Shinagawa-ku , Tokyo, 141-8610 JAPAN | Tel: Email: | +81-50-3750-2740 Yoshitaka.Morigami@jp.sony.com Tanaka.Junichi@jp.sony.com teruhikos@jp.sony.com |
| Source: | Sony Corporation |  |  |

_____________________________

# Abstract

HEVC supports scaling list as AVC, proposed by JCTVC-G434. At the last meeting it was proposed by JCTVC-G0230 that 16x16- and 32x32- matrices are transmitted in the form of 8x8 matrices and in encoding/decoding process they are up-sampled with zero-order hold to store memory, and for 16x16- and 32x32- matrices DC components are separately transmitted. In addition, JCTVC-H0237 proposed to switch default matrix depending on the value of (0,0) component of the matrix. However, especially for default matrix coding, the following two problems can be observed.

- Complexity of default matrix coding is increased (by checking the status of (0,0))

- Minor bug in copying matrix from other matrices

In addition,, we observed coding efficiency reduced in DPCM coding for 16x16 and 32x32 matrices because the initial value of DPCM coding is set to 8.

In order to fix those problems, this document proposes the following two methods:

- Simplification of coding of default matrices,

- Improvement of DPCM coding for 16x16 and 32x32 matrices,

# Problems

## Coding of Default Matrices

### Switch to use the dedault matrix

In HM-6.0, it is possible to switch the default matrix and user defined matrix for each matrix size (signalled by SizeID: 4x4, 8,8, 16x16, 32x32) and matrix type (signalled by MatrixID: Intra Y/Cb/Cr, Inter Y/Cb/Cr), which was proposed by JCTVC-H0237. For each matrix, the condition to use the default matrix is as follows:

- Default matrix is used if the value of (0,0) component is 0 in the case that the size of the matrix is 8x8 or smaller

- Default matrix is used if the value of DC is 0 in the case that the size of the matrix is 16x16 or larger

However, this method needs to check the first component of matrix and increases the complexity of decoding process. (The syntax and the source code became complicated due to this. The logic size is increased by this process)

### Problem in “copy mode”

HM6 adopted “copy mode” in which the current matrix is coped from the previously decoded matrix with the same matrix size when scaling_list_pred_mode_flag is equal to 0. (See mode detail in JCTVC-G434) In this copy mode, the reference matrix is derived by the following formula. (RefMatrixID is the reference id of the target matrix)

RefMatrixID = MatrixID – ( 1 +  scaling_list_pred_matrix_id_delta )

However, the value of RefMatrixID can be “-1”, when MatrixID=0, scaling_list_pred_mode_flag=0, and scaling_list_pred_matrix_id=0. However, this “-1” has no meaning. Therefore, a restriction is necessary to fix this problem.

## Coding efficiency of DPCM coding for 16x16 and 32x32 matrices

In HM-6.0, 16x16- and 32x32- matrices are transmitted as 8x8- matrices and DC components. In decoding process, 8x8- matrices are up-converted into 16x16 or 32x32. Then (0, 0) component is replaced by corresponding DC components. In encoding process, DC components of 16x16- and 32x32- matrices are subtracted by 8 and encoded by signed golomb coding. To encode other components of each 8x8 matrix, (0, 0) components is subtracted by 8 and this value is used as the initial value of DPCM coding of 8x8 matrix. Figure 1 shows the reconstruction of 16x16 matrix. As this figure shows, DC component and other components in 8x8 matrix are correlated. Therefore, it is possible to reduce the generated bits by encoding DC component and other 8x8 components together. (rather than encoding independently)

Example:

Assume that the value of DC component is 12, and the value of (0, 0) component of 8x8- matrices for transmission is 16.

According to the current HM specification (the initial value of DPCM coding is 8), the value to encode (0, 0) coefficient is 16-8=8. 9-bit is needed by encoding signed exponential golomb coding. However, if DC component is used as the initial value of DPCM, the value to encode (0, 0) component is 16-12=4 and only 7-bit is needed.

Figure 1: Generation method of 16x16 matrix

# Proposed Method

## Simplification of Default matrices coding

As explained in sub-clause 1.1, in HM6, the default matrix and user defined matrix are switched by the first component of matrix. To reduce the complexity, our proposal uses “RefMatrixID”, which is the reference to the other matrix in “copy mode”.

In our proposed method, default matrices are coded as “copy mode”, that is, copy of the matrix with the same size.

In “copy mode”, RefMatrixID is obtained with the following formula:

RefMatrixID = MatrixID – ( 1 +  scaling_list_pred_matrix_id_delta )

Then the matrix whose MatrixID is RefMatrixID is copied. However, in HM6, RefMatrixID == MatrixID is not allowed, because this means the reference matrix is the current matrix. In our proposal, using of default matrix is assigned to this “RefMatrixID==MatrixID”. Error: Reference source not found shows how this method is implemented. (Default matrix is associated with RefMatrixID equal to MatrixID (current))

To fulfill this operation, RefMatrixID is calculated with the following formula:

RefMatrixID = MatrixID –  scaling_list_pred_matrix_id_delta

In this formula, RefMatrixID == MatrixID is allowed to indicate the default matrix is used.

In addition, by using this formula, the problem in 1.1.2 can be resolved, because RefMatrixID is always limited to 0 or above.

## Improved DPCM coding for 16x16 and 32x32 matrices

Figure Error: Reference source not found shows the proposed method of DPCM coding. This contribution proposes to encode DC component and other 8x8 matrix component together by DPCM coding. In this case, the initial value of DPCM coding is the value of DC component, not 8 as in HM6.

Figure 2: Method of coding DC and 8x8 matrix

# Conclusion

In HM6, the following problems can be observed.

- Complexity of default matrix coding is increased (by checking the status of (0,0))

- Minor bug in copying matrix from other matrices

- Coding efficiency loss in DPCM coding

In order to fix those problems, this document proposes the following two methods:

- Simplification of coding of default matrices,

- Improvement of DPCM coding for 16x16 and 32x32 matrices

These modifications be included in HEVC specification.

# Reference

- “CE4 subtest 3: Quantization matrix for HEVC based on JCTVC-F362 and F475”
	JCTVC-G434, Geneva, November 2011

- “CE4 subtest 2.1: Enhancements of HEVC quantization matrix”
	JCTVC-H0230, San José, February, 2012

- “Non-CE4 Subtest 2: Improvement on quantization matrix signaling”
	JCTVC-H0237, San José, February, 2012

# Patent rights declaration(s)

Sony Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Annex A: Proposed syntax and semantics

The detail syntax and semantics of proposed method is summarized in this Annex.

| scaling_list_param( ) { | Descriptor |
| --- | --- |
| scaling_list_present_flag | u(1) |
| if( scaling_list_present_flag ) |  |
| for( sizeID = 0; sizeID < 4; sizeID++ ) |  |
| for( matrixID = 0; matrixID < (sizeID = = 3) ? 2 : 6; matrixID++ ) { |  |
| scaling_list_pred_mode_flag | u(1) |
| if( !scaling_list_pred_mode_flag ) |  |
| scaling_list_pred_matrix_id_delta | ue(v) |
| else |  |
| scaling_list( ScalingList[ sizeID ][ matrixID ], sizeID , matrixID ) |  |
| } |  |
| } |  |

| scaling_list( scalingList, sizeID , matrixID ) { | Descriptor |
| --- | --- |
| nextCoef = 8 | u(1) |
| coefNum = Min( 64, ( 1 << ( 4 + ( sizeID << 1) ) ) ) |  |
| UseDefaultScalingMatrix = 0 |  |
| if( sizeID > 1 ) { |  |
| scaling_list_dc_coef_minus8[ sizeID − 2 ][ matrixID ] | se(v) |
| if( scaling_list_dc_coef_minus8[ sizeID − 2 ][ matrixID ] = = −8 ) |  |
| UseDefaultScalingMatrixFlag = 1 |  |
| nextCoef = scaling_list_dc_coef_minus8[ sizeID – 2 ][ matrixID ] + 8 |  |
| } |  |
| if( UseDefaultScalingMatrixFlag = = 0 ) { |  |
| stopNow = 0 |  |
| for( i=0; i < coefNum && !stopNow; i++) { |  |
| scaling_list_delta_coef | se(v) |
| nextCoef = ( nextCoef + scaling_list_delta_coef + 256 ) % 256 |  |
| if( sizeID < 2 ) { |  |
| useDefaultScalingMatrixFlag = (i = = 0 && nextCoef = = 0 ) |  |
| if( useDefaultScalingMatrixFlag ) |  |
| stopNow = 1 |  |
| } |  |
| if( !stopNow ) |  |
| scalingList[ i ] = nextCoef |  |
| } |  |
| } |  |
| } |  |

Scaling list data semantics

scaling_list_pred_matrix_id_delta specifies the target reference matrix to copy the value of scaling list. When scaling_list_pred_mode_flag is equal to 0, scaling_list_pred_matrix_id_delta specifies which matrix should be used in the current matrix by the following:

if scaling_list_pred_matrix_id_delta is equal to 0, 
		UseDefaultScalingMatrixFlag  =  1
other
		RefMatrixID = MatrixID – ( scaling_list_pred_matrix_id_delta )