| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0060 |
| --- | --- |

| Title: | AhG5: Unified neighbour-based context selection |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | C. Rosewarne 1 Thomas Holt Drive, North Ryde, NSW 2113, Australia. M. Maeda 30-2, Shimomaruko 3-chome, Ohta-ku, Tokyo 146-8501, Japan. | Tel: Email: Tel: Email: | +61 2 9805 2844 chris.rosewarne@cisra.canon.com.au +81-3-3758-2111 maeda.mitsuru@canon.co.jp |
| Source: | Canon Information Systems Research Australia, Canon Inc. |  |  |

_____________________________

# Abstract

This contribution presents a method for context selection for the significance map of transform units (TUs) which results in a uniform scheme being applied regardless of the TU size. The presented method is to apply a neighbour-based dependency map context selection scheme to the 4x4 and 8x8 TUs, replacing the existing position-based scheme. Changes to the dependency map are made for horizontal and vertical scan patterns in order to enable high throughput (2 bins per cycle) implementations to be realised. A consequence of substituting the position-based scheme with the neighbour-based scheme is a reduction of 2 contexts in the context model.

# Introduction

In HM-6.0, two context selection schemes are used for the significance map. For 4x4 and 8x8 TUs, a position-based scheme is used, where the context for a given significant coefficient flag in the significance map is fixed according to the present location within the TU. For other TU sizes, a neighbour-based scheme is used, where the context is selected based on a dependency map of several neighbouring significant coefficient flag values.

A drawback of this scheme is the need for two independent context selection mechanisms in HEVC.

Table 1 shows the number of contexts required for each channel of the 4x4 and 8x8 TU sizes by the position-based scheme of HM-6.0.

Table 1. HM-6.0 context allocation for 4x4 and 8x8 TUs.

| TU size | Channel | Number of contexts |
| --- | --- | --- |
| 4x4 | Luma | 9 |
|  | Chroma | 6 |
| 8x8 | Luma | 11 |
|  | Chroma | 11 |
| Others | Luma | 7 |
|  | Chroma | 4 |
| Total |  | 27 (luma) + 21 (chroma) = 48 |

# Description of the proposed change

Mode dependent coefficient scanning [1] applies to 4x4 and 8x8 TUs, resulting in horizontal and vertical scans also being applied to these TU sizes.

The dependency map used in HM-6.0 is optimised to enable high throughput for a diagonal scan only [2].

When horizontal or vertical scans are applied to a TU using the neighbour-based dependency map context selection scheme, modified versions of the diagonal scan dependency map are selected to enable high throughput implementations for all scan pattern.

Figure 1 shows the dependency map required for each scan pattern. An 8x8 TU is depicted, but the proposed scheme is also applied to 4x4 TUs. An exemplary current significant coefficient flag is highlighted in yellow and the neighbouring significant coefficient flags, which comprise the dependency map, are shown in purple.

Figure 1. Dependency map for each scan pattern.

The distribution of residual coefficients within a TU differs between areas covering low frequency residual coefficients and areas covering high frequency residual coefficients. This is exploited in [3] for 16x16 and 32x32 TUs by utilizing separate sets of contexts for low frequency and high frequency sub-blocks of the TU.

In this proposal, this approach is extended to the 4x4 and 8x8 TU to enable context adaption to this statistical property. The regions are depicted in Figure 2.

Figure 2. Regions for context selection.

Table 2 shows the proposed allocation of contexts to regions for each channel of the 4x4 and 8x8 TUs. The proposed scheme results in a reduction of 6 contexts.

Note that in each non-DC region, five contexts are used instead of three.

Table 2. Proposed context allocation for 4x4 and 8x8 TUs.

| TU size | Channel | Region | Number of contexts |
| --- | --- | --- | --- |
| 4x4 | Luma | DC | 1 |
|  |  | Non-DC | 5 |
|  | Chroma | DC | 1 |
|  |  | Non-DC | 5 |
| 8x8 | Luma | DC | 1 |
|  |  | LF | 5 |
|  |  | HF | 5 |
|  | Chroma | DC | 1 |
|  |  | Non-DC | 5 |
| Others | Luma | All | 11 |
|  | Chroma | All | 6 |
| Total: |  |  | 28 (luma) + 18 (chroma) = 46 |

# Experiment Results

Results are presented in Table 3 below. Simulations were performed under the common conditions [4]. Refer to the accompanying spreadsheet for results with greater precision. Execution took place on a Linux cluster using GCC 3.4.6.

Note that context initialization values were not updated in this simulation.

Table 3. Simulation results in HM-6.0.

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | 1.5% | 1.2% | 0.0% | 1.0% | 1.1% |
| Class B | 0.1% | 1.1% | 0.9% | 0.0% | 0.9% | 0.8% |
| Class C | 0.1% | 0.7% | 0.7% | 0.0% | 0.6% | 0.5% |
| Class D | 0.1% | 0.8% | 0.7% | 0.0% | 0.7% | 0.6% |
| Class E | 0.3% | 0.8% | 0.8% | 0.2% | 0.6% | 0.7% |
| Overall | 0.1% | 1.0% | 0.9% | 0.0% | 0.8% | 0.8% |
|  | 0.1% | 1.0% | 0.9% | 0.0% | 0.8% | 0.8% |
| Class F | 0.0% | 0.5% | 0.2% | -0.1% | 0.1% | -0.1% |
| Enc Time[%] | 102% |  |  | 99% |  |  |
| Dec Time[%] | 102% |  |  | 99% |  |  |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0.1% | 2.2% | 2.2% | 0.1% | 1.6% | 1.9% |
| Class B | 0.2% | 0.5% | 0.7% | 0.2% | 0.6% | 0.7% |
| Class C | 0.1% | 0.5% | 0.6% | 0.1% | 0.4% | 0.6% |
| Class D | 0.1% | 0.4% | 0.8% | 0.2% | 0.6% | 0.5% |
| Class E |  |  |  |  |  |  |
| Overall | 0.1% | 0.9% | 1.0% | 0.2% | 0.8% | 0.9% |
|  | 0.2% | 0.9% | 1.0% | 0.2% | 0.8% | 0.9% |
| Class F | 0.0% | 0.2% | 0.3% | 0.0% | 0.1% | 0.2% |
| Enc Time[%] | 102% |  |  | 101% |  |  |
| Dec Time[%] | 101% |  |  | 100% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.2% | 0.6% | 0.1% | 0.5% | 0.6% | 0.9% |
| Class C | 0.1% | 0.3% | 0.0% | 0.3% | 0.5% | 0.5% |
| Class D | 0.4% | 0.6% | 0.5% | 0.4% | -0.4% | -0.1% |
| Class E | 0.5% | -0.2% | 0.3% | 0.6% | -0.6% | 0.1% |
| Overall | 0.3% | 0.4% | 0.2% | 0.4% | 0.1% | 0.4% |
|  | 0.3% | 0.4% | 0.2% | 0.4% | 0.2% | 0.4% |
| Class F | -0.3% | 0.1% | -0.5% | 0.2% | 0.6% | 1.0% |
| Enc Time[%] | 101% |  |  | 101% |  |  |
| Dec Time[%] | 100% |  |  | 101% |  |  |

# Working Draft changes

# Conclusion

Results are presented a harmonisation of significance map context selection, resulting in exclusive use of a neighbour-based context selection scheme for all TU sizes. Necessary modifications to enable high-throughput implementation are made. A reduction of 2 contexts for the significance map coding is also achieved.

# References

- Y. Zheng, M. Coban, J. Sole, R. Joshi, M. Karczewicz, “CE11: Mode Dependent Coefficient Scanning”, Joint Collaborative Team on Video Coding, JCTVC-D393, January 2011.

- C. Auyeung, W. Liu, “Parallel processing friendly simplified context selection of significance map”, Joint Collaborative Team on Video Coding, JCTVC-D260, January 2011.

- K. Terada, H. Sasai, T. Nishi, “Non-CE11:Simplification of context selection for significant_coeff_flag”, Joint Collaborative Team on Video Coding, JCTVC-H0290, January 2012.

- F. Bossen, “Common HM test conditions and software reference configurations,” Joint Collaborative Team on Video Coding, JCTVC-H1100, February 2012.

# Patent rights declaration(s)

Canon Information Systems Research Australia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Canon Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).