| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0061 WG11 Number: m24300 |
| --- | --- |

| Title: | Simplified APS Partial Update through Referencing |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ming Li and Ping Wu No. 50 Ruanjiandadao Road, ZTE R&D Center District II, Nanjing, China 210012 Stephan Wenger and Jill Boyce 433 Hackensack Ave., Hackensack, N.J. 07601, USA David Flynn BBC R&D, Centre House, 56 Wood Lane, W12 7SB London, UK | Tel: Email: | Ming Li, +86-25-88014644 {li.ming42, ping.wu}@zte.com.cn Stephan Wenger, +1-415-713-5473 {stephan,jill}@vidyo.com davidf@rd.bbc.co.uk |
| Source: | ZTE, Vidyo and BBC Research & Development |  |  |

_____________________________

# Abstract

Proposed is a simplified partial update mechanism for parameter sets, particular for the adaptation parameter set, based on parameter set referencing approach enabling partial referencing some of the parameters in one existing adaptation parameter set (APS) without requiring retransmission of unchanged APS information, which helps to save bits from coding repeated tool parameters.

# Introduction

At the 6th JCT-VC Meeting, adaptation parameter set (APS) was developed and adopted to HEVC based on the contribution JCTVC-F747 [1], which is intended for picture-adaptive data. Although APS provides an effective approach to achieve slice level common information sharing, there exists a problem of duplicating of the coded APS information when only a part of the APS parameters changes among pictures while the other ones are kept the same.

To tackle this problem, an APS referencing approach [2] is proposed, which allows the codec to update partial parameters in the current APS referred to by a slice with the counterpart data in an existing APS. The decoder parses the syntax elements in these APSs to get the tool parameters (including the ON/OFF flag and matrix or filter parameters) of scaling list, deblocking filter, sample adaptive offset (SAO) and adaptive loop filter (ALF), then combines the obtained parameters together and activates them to initialize the tools for decoding the current slice.

At discussed at San Jose meeting, the approach presented in JCTVC-H0069-v4 [2] allowing self-referencing updating operations was somewhat complex. Such complexity mainly came from the fact that an update referencing a different APS ID would act as a pointer (resolved at activation time) rather than as a conditional-replacement-replace update (resolved at the time of receipt of the APS NAL unit). Therefore, the following modifications are performed to make the approach in JCTVC-H0069-v4 be recognized as a simple and decent solution as:

- The self-referencing operations on the condition of (aps_ref_flag == 1 && aps_ref_id == aps_id) are removed by a constraint that aps_ref_id shall not be equal to aps_id in an APS.

- The maximum length of the APS chain is restricted to be 2 by a constraint that an APS with its aps_ref_flag equal to 1 cannot be referenced by other APSs.

# Modified Syntax and Semantics

The modified APS syntax based on the current CD document [3] is presented in the table below.

Table 1.  Modified APS Syntax

| aps_rbsp( ) { | Descriptor |
| --- | --- |
| aps_id | ue(v) |
| aps_ref_flag | u(1) |
| if(aps_ref_flag) |  |
| aps_ref_id | ue(v) |
| aps_scaling_list_data_present_flag | u(1) |
| if( aps_scaling_list_data_present_flag ) |  |
| scaling_list_param( ) |  |
| aps_deblocking_filter_data_present_flag | u(1) |
| if(aps_deblocking_filter_data_present_flag) { |  |
| aps_deblocking_filter_flag | u(1) |
| if(aps_deblocking_filter_flag) { |  |
| disable_deblocking_filter_flag | u(1) |
| if( !disable_deblocking_filter_flag ) { |  |
| beta_offset_div2 | se(v) |
| tc_offset_div2 | se(v) |
| } |  |
| } |  |
| aps_sample_adaptive_offset_data_present_flag | u(1) |
| if(aps_sample_adaptive_offset_data_present_flag) { |  |
| aps_sao_interleaving_flag | u(1) |
| if( !aps_sao_interleaving_flag ) { |  |
| aps_sample_adaptive_offset_flag | u(1) |
| if( aps_sample_adaptive_offset_flag ) |  |
| aps_sao_param( ) |  |
| } |  |
| } |  |
| aps_adapitve_loop_filter_data_present_flag | u(1) |
| if(aps_adapitve_loop_filter_data_present_flag) { |  |
| aps_adaptive_loop_filter_flag | u(1) |
| if( aps_adaptive_loop_filter_flag ) |  |
| alf_param( ) |  |
| } |  |
| aps_extension_flag | u(1) |
| if( aps_extension_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| aps_extension_data_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

In semantics:

7.4.2.5	Adaptation parameter set RBSP semantics

Adaptation parameter set supports partial updating utilizing the aps_id aps_ref_flag and aps_ref_id syntax elements.

aps_id identifies the adaptation parameter set that is referred to in the slice header. The value of aps_id shall be in the range of 0 to TBD, inclusively. On receipt of an adaptation parameter set RBSP, the contents of this RBSP shall be known as APS[ aps_id ].

aps_ref_flag equal to 1 specifies that this adaptation parameter set references another adaptation parameter set. aps_ref_flag equal to 0 specifies that this adaptation parameter set does not reference any other adaptation parameter set.

An APS with the value of ref_aps_flag equal to 0 can be referenced by other APSs. Otherwise, an APS with the value of ref_aps_flag equal to 1 cannot be referenced by other APSs.

aps_ref_id identifies a subordinate adaptation parameter set for use in construction process of the active adaptation parameter set. The value of aps_ref_id shall be in the range of 0 to TBD, inclusively, excluding the value of aps_id.

aps_scaling_list_data_present_flag equal to 1 specifies that the scaling list parameters exist in this adaptation parameter set, equal to 0 specifies that scaling list parameters do not exist in this adaptation parameter set.

aps_deblocking_filter_data_present_flag equal to 1 specifies that deblocking parameters are present in this adaptation parameter set. aps_deblocking_filter_flag equal to 0 specifies that deblocking parameters do not exist in this adaptation parameter set.

aps_sample_adaptive_offset_data_present_flag equal to 1 specifies that SAO parameters are present in this adaptation parameter set, equal to 0 specifies that SAO parameters are not present in this adaptation parameter set.

aps_sao_interleaving_flag equal to 1 specifies that the SAO parameters are interleaved in slice data for slices referring to the active adaptation parameter set; equal to 0 specifies that the SAO parameters are in adaptation parameter set for slices referring to the active adaptation parameter set. When aps_sao_interleaving_flag do not exist or there is no active adaptation parameter set, the value of aps_sao_interleaving_flag is inferred to be 0.

aps_sample_adaptive_offset_flag equal to 1 specifies that the SAO is on for slices referring to the active adaptation parameter set; equal to 0 specifies that the SAO is off for slices referring to the active adaptation parameter set.  When aps_sample_adaptive_offset_flag do not exist or there is no active adaptation parameter set, the value of aps_sample_adaptive_offset_flag is inferred to be 0.

aps_adaptive_loop_filter_data_present_flag equal to 1 specifies that the ALF parameters are present in this adaptation parameter set, equal to 0 specifies that the ALF parameters are not present in this adaptation parameter set.

aps_adaptive_loop_filter_flag equal to 1 specifies that the ALF is on for slices referring to the active adaptation parameter set; equal to 0 specifies that the ALF is off for slices referring to the active adaptation parameter set. When aps_adaptive_loop_filter_flag do not exist or there is no active adaptation parameter set, the value of aps_adaptive_loop_filter_flag is inferred to be 0.

aps_extension_flag equal to 0 specifies that no aps_extension_data_flag syntax elements are present in the adaptation parameter set RBSP syntax structure. aps_extension_flag shall be equal to 0 in bitstreams conforming to this Recommendation | International Standard. The value of 1 for aps_extension_flag is reserved for future use by ITU T | ISO/IEC. Decoders shall ignore all data that follow the value 1 for aps_extension_flag in a picture parameter set NAL unit.

aps_extension_data_flag may have any value. Its value does not affect decoder conformance to profiles specified in this Recommendation | International Standard.

7.4.1.2.1	Order of sequence, picture and adaptation parameter set RBSPs and their activation

This subclause specifies the activation process of picture, sequence and adaptation parameter sets for coded video sequences that conform to one or more of the profiles specified in Annex A that are decoded using the decoding process specified in clauses 2-9.

- NOTE 1 – The sequence, picture and adaptation parameter set mechanism decouples the transmission of infrequently changing information from the transmission of coded macroblock data. Sequence, picture and adaptation parameter sets may, in some applications, be conveyed "out-of-band".

An adaptation parameter set RBSP includes parameters that can be referred to by the coded slice NAL units of one or more coded pictures when at least one of sample_adaptive_offset_enabled_flag or adaptive_loop_filter_enabled_flag are equal to 1. Each adaptation parameter set may contain data relating to a class of parameters, each indicated by a presence flag. A construction process is performed at the start of each coded picture to populate the active adaptation parameter set from a list of adaptation parameter set structures that have been previously received. The resulting active adaptation parameter set is only considered active for a single coded picture.

When a coded slice NAL unit references (using a particular value of aps_id) an adaptation parameter set (with that value of aps_id), the adaptation parameter set RBSP does not itself become activated, rather, the active adaptation parameter set is constructed using the coalesceAPS(aps_id) function.

coalesceAPS(apsId) {

if( APS[ apsId ].aps_ref_flag )

coalesceAPS( APS[ apsId ].aps_ref_id );

mergeAPS( activeAPS, APS[ apsId ] )

}

Where:

  - activeAPS is the active adaptation parameter set being constructed; and,

  - mergeAPS( a, b ) is a function that combines the contents of APS a and APS b, such that the contents of APS b overwrites that of APS a for those classes of parameters marked by data presence flags in APS b.

A picture parameter set RBSP includes parameters that can be referred to by the coded slice NAL units or coded slice data partition A NAL units of one or more coded pictures. Each picture parameter set RBSP is initially considered not active at the start of the operation of the decoding process. At most one picture parameter set RBSP is considered active at any given moment during the operation of the decoding process, and the activation of any particular picture parameter set RBSP results in the deactivation of the previously-active picture parameter set RBSP (if any).

When a picture parameter set RBSP (with a particular value of pic_parameter_set_id) is not active and it is referred to by a coded slice NAL unit or coded slice data partition A NAL unit (using that value of pic_parameter_set_id), it is activated. This picture parameter set RBSP is called the active picture parameter set RBSP until it is deactivated by the activation of another picture parameter set RBSP. A picture parameter set RBSP, with that particular value of pic_parameter_set_id, shall be available to the decoding process prior to its activation.

Any picture parameter set NAL unit containing the value of pic_parameter_set_id for the active picture parameter set RBSP for a coded picture shall have the same content as that of the active picture parameter set RBSP for the coded picture unless it follows the last VCL NAL unit of the coded picture and precedes the first VCL NAL unit of another coded picture.

A sequence parameter set RBSP includes parameters that can be referred to by one or more picture parameter set RBSPs or one or more SEI NAL units containing a buffering period SEI message. Each sequence parameter set RBSP is initially considered not active at the start of the operation of the decoding process. At most one sequence parameter set RBSP is considered active at any given moment during the operation of the decoding process, and the activation of any particular sequence parameter set RBSP results in the deactivation of the previously-active sequence parameter set RBSP (if any).

When a sequence parameter set RBSP (with a particular value of seq_parameter_set_id) is not already active and it is referred to by activation of a picture parameter set RBSP (using that value of seq_parameter_set_id) or is referred to by an SEI NAL unit containing a buffering period SEI message (using that value of seq_parameter_set_id), it is activated. This sequence parameter set RBSP is called the active sequence parameter set RBSP until it is deactivated by the activation of another sequence parameter set RBSP. A sequence parameter set RBSP, with that particular value of seq_parameter_set_id and contained within an access unit with temporal_id equal to 0, shall be available to the decoding process prior to its activation. An activated sequence parameter set RBSP shall remain active for the entire coded video sequence.

- NOTE 2 – Because an IDR access unit begins a new coded video sequence and an activated sequence parameter set RBSP must remain active for the entire coded video sequence, a sequence parameter set RBSP can only be activated by a buffering period SEI message when the buffering period SEI message is part of an IDR access unit.

Any sequence parameter set NAL unit containing the value of seq_parameter_set_id for the active sequence parameter set RBSP for a coded video sequence shall have the same content as that of the active sequence parameter set RBSP for the coded video sequence unless it follows the last access unit of the coded video sequence and precedes the first VCL NAL unit and the first SEI NAL unit containing a buffering period SEI message (when present) of another coded video sequence.

- NOTE 3 – If picture parameter set RBSP, sequence parameter set RBSP or adaptation parameter set RBSP are conveyed within the bitstream, these constraints impose an order constraint on the NAL units that contain the picture parameter set RBSP or sequence parameter set RBSP, respectively. Otherwise (picture parameter set RBSP, sequence parameter set RBSP or adaptation parameter set RBSP are conveyed by other means not specified in this Recommendation | International Standard), they must be available to the decoding process in a timely fashion such that these constraints are obeyed.

All constraints that are expressed on the relationship between the values of the syntax elements (and the values of variables derived from those syntax elements) in sequence parameter sets, picture parameter sets and adaptation parameter sets and other syntax elements are expressions of constraints that apply only to the active sequence parameter set, the active picture parameter set and the active adaptation parameter set. If any sequence parameter set RBSP is present that is not activated in the bitstream, its syntax elements shall have values that would conform to the specified constraints if it were activated by reference in an otherwise conforming bitstream. If any picture parameter set RBSP is present that is not ever activated in the bitstream, its syntax elements shall have values that would conform to the specified constraints if it were activated by reference in an otherwise-conforming bitstream. If any adaptation parameter set RBSP is present that is not ever activated in the bitstream, its syntax elements shall have values that would conform to the specified constraints if it were activated by reference in an otherwise-conforming bitstream.

During operation of the decoding process (see clause 8), the values of parameters of the active picture parameter set and the active sequence parameter set shall be considered in effect. For interpretation of SEI messages, the values of the parameters of the picture parameter set and sequence parameter set that are active for the operation of the decoding process for the VCL NAL units of the coded picture in the same access unit shall be considered in effect unless otherwise specified in the SEI message semantics.

# Conclusions

In order to facilitate flexible signalling of scaling list, SAO and ALF parameters in APS, an APS referencing approach is proposed. The recommendation is to adopt this approach to HEVC.

# References

- Stephan Wenger, Jill Boyce, Yu-Wen Huang, Chia-Yang Tsai, Ping Wu and Ming Li, “Adaptation Parameter Set (APS)”, JCT-VC Document JCTVC-F747, July 2011, Torino, Italy.

- Ming Li, Ping Wu, Stephan Wenger, Jill Boyce and David Flynn, “APS partial update through Referencing”, JCT-VC Document JCTVC-H0069-v4, February 2012, San Jose, USA.

- Benjamin Bross, Woo-Jin Han, Jens-Rainer Ohm, Gary J. Sullivan and Thomas Wiegand, “High efficiency video coding (HEVC) text specification draft 6”, JCT-VC Document JCTVC-H1003-v22, February 2012, San Jose, USA.

# Patent rights declaration(s)

ZTE Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Vidyo Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

The British Broadcasting Corporation (BBC) may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).