| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0062 WG11 Number: m24301 |
| --- | --- |

| Title: | Comments on Field Indication SEI Message |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ming Li Ping Wu No. 50 Ruanjiandadao Road, ZTE R&D Center District II, Nanjing, China 210012 Chad Fogg 4300 N. First Street San Jose 95134 | Tel: Email: | +86-25-88014644 li.ming42@zte.com.cn ping.wu@zte.com.cn +1 408 542 2500 chadfogg@gmail.com |
| Source: | ZTE Corporation and Harmonic Inc. |  |  |

_____________________________

# Abstract

In this document, some modifications on field indication SEI message are proposed. Different from the flag array structure in the field indication SEI message in JCTVC-H1003-v22, the flags for field applications are classified into two groups according to the sequence type information, which copes with features of field sequence and frame sequence, respectively.

# Introduction

To indicate source type and some relative information for field applications, field indication SEI message of a flag array structure was proposed and adopted in HEVC at San Jose meeting [1]. To cope with different features of field sequence and frame sequence, it is proposed to employ tree syntax structure and group the indication flags according to the sequence type information, i.e. field sequence and frame sequence, for clear interpretation.

# Proposed Field Indication SEI Message

The proposed field indication SEI message is presented in the table as below:

Table 1.  Proposed Field Indication SEI Message

| field_indication( payloadSize ) { | Descriptor |
| --- | --- |
| field_sequence_flag | u(1) |
| progressive_source_flag | u(1) |
| if (field_sequence_flag) { |  |
| bottom_field_flag | u(1) |
| duplicate_flag | u(1) |
| } else { |  |
| field_output_flag | u(1) |
| top_field_first_flag | u(1) |
| } |  |
| reserved_zero_4bits /* equal to 0 */ | u(3) |
| } |  |

The semantics and combination interpretations of field_sequence_flag and progressive_source_flag are the same as those of sequence_type_flag and progressive_source_flag in the current CD document [1].

bottom_field_flag indicates the parity of the field contained within the access unit when sequence_type_flag is equal to 1. bottom_field_flag equal to 1 indicates bottom field parity. Otherwise, bottom_field_flag equal to 0 indicates top field parity.

duplicate_flag equal to 1 indicates that the picture contained within the access unit is indicated to be a duplicate of a previous picture in output order. duplicate_flag equal to 0 indicates that the picture contained within the access unit is not indicated to be a duplicate picture.

NOTE 1 – This flag should be used to mark coded pictures known to have originated from a repetition process such as 3:2 pull-down or other duplication and interpolation methods. This flag would commonly be used when a video feed is encoded in a "transport pass-through" fashion, with known duplicate pictures tagged by setting duplicate_flag equal to 1.

NOTE 2 – When sequence_type_flag is equal to 1 and duplicate_flag is equal to 1, it is assumed the access unit contains a duplicated field of the previous field in output order with the parity indicated by bottom_field_flag.

field_output_flag equal to 0 indicates the frame contained within the access unit will be outputted as a progressive frame. Otherwise, the frame contained within the access unit will be split into two fields, which will be outputted in an order indicated by top_field_first_flag.

top_field_first_flag indicates field output order when field_output_flag is equal to 1. When top_field_first_flag is equal to 1, the top field is indicated to be temporally first followed by the bottom field. When top_field_first_flag is equal to 0, the bottom field is indicated to be temporally first, followed by the top field. When field_output_flag is equal to 0, top_field_first_flag has no meaning and shall be equal to 0.

The application scenarios of the proposed field indication SEI message are illustrated in the Figures 1-8.

Figure 1.  Use case of “Pure Progressive”.

Figure 2.  Use case of “Interleaved fields (PAL)”.

Figure 3.  Use case of “Interleaved fields (NTSC)”.

Figure 4.  Use case of “Pure field sequence (PAL)”.

Figure 5.  Use case of “Pure field sequence (NTSC)”.

Figure 6.  Use case of “PFS (PAL)”.

Figure 7.  Use case of “PFS (NTSC)”.

Figure 8.  Use case of “3:2 pull-down”.

# Conclusions

In this document, a field indication SEI message using tree structure is proposed. It is recommended to adopt the proposed to HEVC.

# References

- Benjamin Bross, Woo-Jin Han, Jens-Rainer Ohm, Gary J. Sullivan and Thomas Wiegand, “High efficiency video coding (HEVC) text specification draft 6”, JCT-VC Document JCTVC-H1003-v22, February 2012, San Jose, USA.

# Patent rights declaration(s)

ZTE Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Harmonic Inc. does not have any current or pending patent rights relating to the technology described in this contribution.