| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0063 |
| --- | --- |

| Title: | On HEVC profiles |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jani Lainema Kemal Ugur Miska Hannuksela Justin Ridge | Email: | jani.lainema@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

This contribution proposes to keep the number of profiles as small as feasible in order to maximize interoperability of devices and services implementing the standard. It is proposed to define two profiles for the version 1 of HEVC; Main profile for generic video use cases and a Still picture profile for still picture coding. The contribution further recognizes that there may be different needs for different service scenarios and JCT-VC should provide the hooks to enable external organizations to further profile usage of the standard when necessary.

# Introduction

JCT-VC is defining a standard for sharing video content in an interoperable fashion. The way to succeed in that is to provide a mechanism making sure that the receiving end has the same toolset available as the one used in creating the content. We can roughly separate the HEVC use cases into three categories, each with different means to achieve interoperability:

- Usage in open environment where devices share content with others between various and versatile ecosystems (e.g. video recording with a mobile device, sharing to different networks and playback on different displays)

- Usage in systems with capability negotiation possibilities (e.g. video conferencing)

- Usage in closed environment (e.g. a traditional TV broadcasting network where content is encoded for a homogenous device population)

In the first category, having a widely deployed multi-purpose profile is very important as the user/device creating the content needs to be confident that the content can be accepted by the receiving ends. Having a large number of profiles for roughly the same use cases risks fragmentation and in practice content has to be created using the smallest subset of the coding tools common to the foreseeable services. Thus, a single multi-purpose profile guaranteeing interoperability would be highly beneficial for this category of use cases. This appears to be especially critical in Internet environment and minimization of the number of profiles has been strongly encouraged e.g. by Google in their recent webcast [1].

For the second and third category of use cases, the profile definitions at the coding standard level appear to be less critical as the tools can be either negotiated at the beginning of the session or the party controlling the system may simply define the desired tool sets for a specific service. Thus, it appears that minimizing the number of profiles would not have negative effects for these categories of services as long as certain hooks are present in the specification so that system standards could allow certain subset of the tools to be enabled or negotiated.

We believe profiles should be created for clearly distinct use cases with a strong requirement for different sets of coding tools. In the case of HEVC there appears to be two such use cases: coding of video content and coding of still pictures. The next section discusses the proposed configurations for these usages.

# Proposal

We propose version 1 of HEVC to have the following profiles:

- Main profile for generic video services

- Still picture profile for coding single still pictures

The motivation and toolset for each profile are discussed further below

## Main profile

Main profile is proposed as the generic video coding profile providing optimal interoperability in open environments. We believe the Main profile defined in the current HEVC committee draft provides a good basis for this profile and ask the community to consider the following aspects when deciding on possible changes to the toolset defined for the profile:

- ALF has to be significantly simplified with respect to the CD design in order to be included to the profile

- AMP appears as a straight-forward extension of the current PU split design with some coding efficiency gains (-0.7 % / -1.0 % for RA-Main and LB-Main, respectively) as reported in Section 3. Adding that tool to the Main profile looks feasible.

- NSQT appears to complicate the transform decoding design by introducing new logic in TU splitting, coefficient scanning and transform selection. Although it provides some coding efficiency gains when applied on top of AMP (-0.4 % / -1.1 % for RA-Main and LB-Main, respectively) it looks not worth complicating the design with the tool.

- Picture level parallel processing: we should have only one tool for the functionality unless significant benefits for having multiple tools are demonstrated

- Parallelism for high resolution content: we should consider mandating maximum tile size for the bitstreams in order to guarantee possibility to parallelize the decoder operation (e.g. tile should not contain more than 2048x1024 pixels)

## Still picture profile

The still picture coding capabilities of HEVC significantly exceeds those of the earlier standards. Also if we force one slice per tile coding, HEVC is able to provide a very useful “random access within picture” feature at configurable granularity. This allows fast decoding of specific parts of potentially huge images without the need to decode the whole image (or even scan through the slice headers in the case implementing similar features using less flexible and less efficient slice based approaches).

We propose the Still picture profile to be identical to the Main profile with the following restrictions:

- Sequence contains one picture which consists of I slices only

- Every tile contains one and only one slice

# Additional information

The tables below report the objective coding efficiency benefits of AMP and NSQT tools in HM 6.0 according to the JCT-VC common test conditions. In the case of NSQT the gain over HM with AMP is measured by enabling AMP in both anchor and in the tested configuration. Thus, the reported configurations are as follows:

| Test | Anchor config | Tested config |
| --- | --- | --- |
| AMP test | HM 6.0 | HM 6.0 + AMP |
| NSQT test | HM 6.0 + AMP | HM 6.0 + AMP + NSQT |

The simulations were run on a cluster with heterogeneous nodes. As a result, the encoding and decoding times are not comparable.

AMP test:

|  | Random Access Main |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | -0.5% | -1.1% | -0.9% |
| Class B | -0.6% | -0.9% | -0.9% |
| Class C | -0.9% | -1.1% | -1.2% |
| Class D | -0.8% | -1.2% | -1.1% |
| Class E |  |  |  |
| Class F | -0.7% | -0.8% | -0.8% |
| Overall | -0.7% | -1.0% | -1.0% |
|  | -0.7% | -1.0% | -1.0% |
|  | Low delay B Main |  |  |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | -0.9% | -1.6% | -1.3% |
| Class C | -0.9% | -1.2% | -1.4% |
| Class D | -1.0% | -1.5% | -1.1% |
| Class E | -1.4% | -2.0% | -1.7% |
| Class F | -1.0% | -1.4% | -1.3% |
| Overall | -1.0% | -1.5% | -1.3% |
|  | -1.0% | -1.5% | -1.4% |

NSQT test:

|  | Random Access Main |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | -0.3% | -0.8% | -0.6% |
| Class B | -0.5% | -1.6% | -1.3% |
| Class C | -0.5% | -0.7% | -0.7% |
| Class D | -0.4% | -0.2% | -0.2% |
| Class E |  |  |  |
| Class F | -0.3% | -0.4% | -0.3% |
| Overall | -0.4% | -0.8% | -0.7% |
|  | -0.4% | -0.8% | -0.7% |
|  | Low delay B Main |  |  |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | -1.1% | -3.6% | -3.7% |
| Class C | -1.0% | -2.0% | -2.0% |
| Class D | -0.9% | -1.5% | -0.8% |
| Class E | -1.5% | -3.3% | -2.6% |
| Class F | -1.0% | -2.5% | -1.9% |
| Overall | -1.1% | -2.6% | -2.3% |
|  | -1.1% | -2.6% | -2.1% |

# References

[1] http://www.streamingmedia.com/Articles/Editorial/Featured-Articles/WebM-Taking-Another-Look-81428.aspx, Discussion between 07:00 – 08:20 minutes and 9:30 – 10:40 minutes, March 17, 2012.

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).