| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0065 |
| --- | --- |

| Title: | Modification on CABAC initialization |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kazuo Sugimoto Akira Minezawa Shun-ichi Sekiguchi Tokumichi Murakami Information Technology R&D Center Mitsubishi Electric Corporation 5-1-1, Ofuna, Kamakura, JAPAN | Tel: Email: | +81-467-41-2463 Sugimoto.Kazuo@ak.MitsubishiElectric.co.jp |
| Source: | Mitsubishi Electric Corporation |  |  |

_____________________________

This contribution proposes a modification on initialization table selection for CABAC context. The proposed method uses only two initialization tables instead of three tables in HM6. In P-slices and B-slices, initializationType=0 and 1 can be selected instead of 1 and 2 in HM6, while initializationType=0 is used for I-slices same as in HM6. To verify the performance, sequences with frequent scene changes are used with applying initializationType=0 in slices just after scene change. Simulation results show that performance gain is 0.1%, 0.1%, 0.1%, 0.1%, 0.1%, and 0.1% for RA-main, LB-main, LP-main, RA-HE10, LB-HE10 and LP-HE10 settings respectively using HM-6.0 in multiple 1500 byte slice mode with QP=12, 17 22 and 27. It is reported that the performance gain with QP=32, 37, 42 and 47 is 0.1%, 0.2%, 0.1%, 0.1%, 0.1%, and 0.2% respectively.

# Introduction

The cabac_init_flag in the current HEVC design is adopted in the last meeting[1], and three different tables are used. On the other hand, the benefit of average 0.2% coding gain by selecting from two tables in P and B slices can be obtained by using slice level 2-pass encoding. This contribution proposes a modification on initialization table selection using only two tables.

# Proposal

We propose two changes on the cabac_init_flag syntax element used for CABAC context initialization:

- Reduce the number of tables for CABAC initialization to two instead of three in the current design.

- Modify the definition of cabac_init_flag to be a flag to allow for P and B slice types to be initialized with either the “Intra slice type” table or “Inter slice type” table.

A primary benefit of the proposed method is allowing to switch intra slice type initialization also for inter slices. When intra CU are majority in an inter slice such as in a scene changed frame, intra slice type initialization is preferable to be used instead of inter slice type initialization.

# Simulation and results

The proposed setting was evaluated using HM-6.0 with the settings described in JCTVC-H1100 [2] with multiple 1500byte slice mode. Test sequences used for this evaluation are produced by concatenating first several frames of each common test sequence in the same class as described in Table 1. All sequences are coded with intra period of 32 frames.

Table 1: Test sequences

|  | Class A | Class B | Class C | Class D | Class E |
| --- | --- | --- | --- | --- | --- |
| 1st part | Traffic 21 frames | BasketballDrive 21 frames | BasketballDrill 21 frames | BasketballPass 21 frames | Vidyo1 21 frames |
| 2nd part | PeopleOnStreet 43 frames | Cactus 32 frames | BQMall 32 frames | BlowingBubbles 32 frames | Vidyo3 32 frames |
| 3rd part |  | Kimono 32 frames | RaceHorses 32 frames | RaceHorses 32 frames | Vidyo4 32 frames |
| 4th part |  | ParkScene 32 frames | PartyScene 43 frames | BQSquare 43 frames | FourPeople 32 frames |
| 5th part |  | BQTerrace 43 frames |  |  | KristenAndSara 32 frames |
| 6th part |  |  |  |  | Johnny 43 frames |
| Total frames | 64 frames | 160 frames | 128 frames | 128 frames | 192 frames |
| Number of scene changes | 1 | 4 | 3 | 3 | 5 |

Table 2: Performance of the proposed method with QP=12, 17, 22, 27

|  | Random Access Main |  |  | Random Access HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | -0.2% |
| Class B | 0.0% | -0.1% | -0.2% | -0.1% | -0.2% | -0.2% |
| Class C | -0.1% | -0.4% | -0.2% | -0.1% | -0.2% | -0.2% |
| Class D | -0.1% | -0.1% | -0.3% | -0.1% | -0.1% | -0.2% |
| Class E | -0.1% | -0.3% | -0.5% | -0.1% | -0.1% | -0.1% |
| Overall | -0.1% | -0.2% | -0.2% | -0.1% | -0.1% | -0.2% |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | -0.1% | -0.2% | -0.4% | -0.1% | -0.1% | -0.1% |
| Class B | -0.0% | -0.4% | -0.2% | -0.1% | 0.1% | 0.0% |
| Class C | -0.1% | -0.2% | -0.8% | -0.2% | -0.9% | -0.2% |
| Class D | -0.1% | -0.1% | -0.4% | -0.1% | -0.2% | -0.2% |
| Class E | -0.1% | -0.3% | 0.1% | -0.1% | -0.2% | -0.3% |
| Overall | -0.1% | -0.2% | -0.3% | -0.1% | -0.2% | -0.2% |
|  | Low delay P Main |  |  | Low delay P HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | -0.1% | -0.3% | -0.3% | -0.1% | -0.1% | -0.1% |
| Class B | -0.1% | 0.0% | 0.0% | -0.1% | -0.2% | -0.4% |
| Class C | -0.1% | 0.0% | -0.3% | -0.2% | 0.1% | -1.0% |
| Class D | -0.1% | 0.0% | -0.2% | -0.1% | -0.2% | -0.2% |
| Class E | 0.0% | -0.1% | 0.2% | -0.1% | -0.1% | 0.0% |
| Overall | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | -0.3% |

Table 3: Performance of the proposed method with QP=32, 37, 42, 47

|  | Random Access Main |  |  | Random Access HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | -1.3% | 0.0% | 0.0% | -0.3% | -0.3% |
| Class B | -0.1% | -0.1% | -0.6% | -0.1% | -0.3% | -0.3% |
| Class C | -0.1% | -0.9% | 0.0% | 0.0% | 1.1% | 0.5% |
| Class D | -0.2% | -0.5% | -0.5% | -0.3% | -0.3% | -0.6% |
| Class E | -0.1% | -0.8% | -1.6% | 0.0% | -0.8% | -0.1% |
| Overall | -0.1% | -0.7% | -0.5% | -0.1% | -0.1% | -0.1% |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | -0.2% | -0.6% | 0.6% | 0.0% | -0.2% | -0.5% |
| Class B | -0.2% | 0.0% | 0.9% | -0.1% | -0.6% | 1.0% |
| Class C | -0.2% | 0.9% | 1.0% | -0.2% | -1.0% | 1.6% |
| Class D | 0.0% | -0.9% | -0.2% | 0.0% | -0.2% | -0.4% |
| Class E | -0.5% | -1.8% | -2.3% | -0.1% | 0.7% | 1.9% |
| Overall | -0.2% | -0.5% | 0.0% | -0.1% | -0.3% | 0.7% |
|  | Low delay P Main |  |  | Low delay P HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | -0.1% | 0.2% | -0.4% | -0.1% | -0.3% | -0.8% |
| Class B | -0.3% | -0.9% | -1.0% | 0.1% | 0.0% | 0.6% |
| Class C | -0.2% | 0.0% | -0.8% | -0.5% | -0.3% | -0.4% |
| Class D | 0.1% | 0.7% | -0.1% | -0.1% | 1.1% | -1.0% |
| Class E | -0.2% | 0.7% | 0.0% | -0.4% | 3.1% | 3.2% |
| Overall | -0.1% | 0.1% | -0.5% | -0.2% | 0.7% | 0.3% |

# Conclusion

In this contribution reduction of cabac initialization table and modification of the definition of cabac_init_flag is proposed. It is proposed to adopt the described method in this contribution to HM7 and DIS.

# Reference

- Kiran Misra, Andrew Segall, “CE1 subtest B4 – On CABAC Init IDC”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-H0540, 8th Meeting: San Jose, CA, US, February, 2012.

- Frank Bossen, “Common test conditions and software reference configurations”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-H1100, 8th Meeting: San Jose, CA, US, February, 2012.

# Patent rights declaration(s)

Mitsubishi Electric Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).