## CABAC parsing process for slice data

This process is invoked when parsing syntax elements with descriptor ae(v) in subclauses Error: Reference source not found and Error: Reference source not found.

Inputs to this process are a request for a value of a syntax element and values of prior parsed syntax elements.

Output of this process is the value of the syntax element.

When starting the parsing of the slice data of a slice in subclause Error: Reference source not found, the initialization process of the CABAC parsing process is invoked as specified in subclause 9.1.1. When tiles_or_entropy_coding_sync_idc is equal to 2 and num_substreams_minus1 is greater than 0, a mapping table BitStreamTable with num_substreams_minus1 + 1 entries specifying a bitstream pointer table to use for later current bitstream pointer derivation is derived as follows.

–	BitStreamTable[ 0 ] is initialized to contain the bitstream pointer.

–	For all indices i greater than 0 and less than num_substreams_minus1+1, BitStreamTable[ i ] contains a bitstream pointer to entry_point_offset[ i ] bytes after BitStreamTable[ i − 1 ].

The current bitstream pointer is set to BitStreamTable[ 0 ].

Figure 9-1 – Spatial neighbour T that is used to invoke the treeblock availability derivation process relative to the current largest coding unit (informative)

The mininum coding block address of the coding treeblock containing the spatial neighbor block T (Figure 9-1), ctbMinCbAddrT, is derived using the location ( x0, y0 ) of the top-left luma sample of the current coding treeblock as follows.

x = x0 + 2 << Log2CtbSize − 1
y = y0 − 1
ctbMinCbAddrT = MinCbAddrZS[ x >> Log2MinCbSize ][ y >> Log2MinCbSize ]	(9-1)

The variable availableFlagT is obtained by invoking the coding block availability derivation process as specified in subclause Error: Reference source not found with ctbMinCbAddrT as input.

When starting the parsing of a coding tree as specified in subclause Error: Reference source not found and tiles_or_entropy_coding_sync_idc is equal to 2 and num_substreams_minus1 is greater than 0 , the following applies.

–	If CtbAddrRS % PicWidthInCtbs is equal to 0, the following applies.

–	When availableFlagT is equal to 1, the synchronization process of the CABAC parsing process is invoked as specified in subclause 9.1.1.3.

–	The decoding process for binary decisions before termination specified in subclause Error: Reference source not found is invoked, followed by the initialisation process for the arithmetic decoding engine specified in subclause 9.1.1.4.

–	The current bitstream pointer is set to indicate BitStreamTable[ i ] with the index i derived as follows.

i = ( CtbAddrRS / PicWidthInCtbs ) % ( num_substreams_minus1 + 1 )	(9-2)

–	Otherwise, if CtbAddrRS % PicWidthInCtbs is equal to 2, the memorization process of the CABAC parsing process is invoked as specified in subclause 9.1.1.2.

The parsing of syntax elements proceeds as follows:

For each requested value of a syntax element a binarization is derived as described in subclause 9.1.2.

The binarization for the syntax element and the sequence of parsed bins determines the decoding process flow as described in subclause Error: Reference source not found.

For each bin of the binarization of the syntax element, which is indexed by the variable binIdx, a context index ctxIdx is derived as specified in subclause Error: Reference source not found.

For each ctxIdx the arithmetic decoding process is invoked as specified in subclause Error: Reference source not found.

The resulting sequence ( b0..bbinIdx ) of parsed bins is compared to the set of bin strings given by the binarization process after decoding of each bin. When the sequence matches a bin string in the given set, the corresponding value is assigned to the syntax element.

In case the request for a value of a syntax element is processed for the syntax element pcm-flag and the decoded value of pcm_flag is equal to 1, the decoding engine is initialised after the decoding of any pcm_alignment_zero_bit, num_subsequent_pcm, and all pcm_sample_luma and pcm_sample_chroma data as specified in subclause 9.1.1.2.

### Initialization process

Outputs of this process are initialised CABAC internal variables.

The processes in subclauses 9.1.1.1 and 9.1.1.2 are invoked when starting the parsing of the slice data of a slice in subclause Error: Reference source not found or when starting the parsing of the data of a coding tree in subclause 7.3.5 and the coding tree is the first coding tree in a tile.

#### Initialization process for context variables

Outputs of this process are the initialised CABAC context variables indexed by ctxIdxTable and ctxIdx.

Table 9-7 to Table 9-28 contain the values of the 8 bit variable initValue used in the initialization of context variables that are assigned to all syntax elements in subclauses Error: Reference source not found to Error: Reference source not found except for the end-of-slice flag.

For each context variable, the two variables pStateIdx and valMPS are initialised.

NOTE 1 – The variable pStateIdx corresponds to a probability state index and the variable valMPS corresponds to the value of the most probable symbol as further described in subclause Error: Reference source not found.

From the 8 bit table entry initValue, the two 4 bit variables slopeIdx and intersecIdx are derived according to the following pseudo-code process:

slopeIdx = initValue >> 4
intersecIdx = initValue & 15

Slope m and Intersec n are derived from the indices as follows:

m = slopeIdx*5 - 45
		n = (intersecIdx<<3)-16

The two values assigned to pStateIdx and valMPS for the initialization are derived from SliceQPY, which is derived in Equation Error: Reference source not found. Given the variable m and n, the initialization is specified by the following pseudo-code process:

Clip3( 1, 126, ( ( m * Clip3( 0, 51, SliceQPY ) ) >> 4 ) + n )
valMPS     = ( preCtxState  <=  63)? 0: 1
pStateIdx = valMPS? (preCtxState − 64): (63 – preCtxState)	(9-3)

The variable initialisationType in  is derived as follows:

if( slice_type = = I )

initialisatonType = 0

else

initialisatonType = 1

if( cabac_init_flag && slice_type != I )

initialisationType = ( initialisationType = = 1 ) ? 1 : 0

Table 9-1 – Association of ctxIdx and syntax elements for each slice type in the initialization process

|  | Syntax element | ctxIdxTable | initialisationType |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | 0 |  | 1 |
| sao_unit_cabac() | sao_merge_left_flag | Table 9-2 | 0..2 |  | 3..5 |
|  | sao_merge_up_flag | Table 9-3 | 0 |  | 1 |
| sao_offset_cabac() | sao_type_idx | Table 9-4 | 0..1 |  | 2..3 |
|  | sao_offset | Table 9-5 | 0..2 |  | 3..5 |
| alf_cu_control_param() | alf_cu_flag | Table 9-6 | 0 |  | 1 |
| coding_tree() | split_coding_unit_flag | Table 9-7 | 0..2 |  | 3..5 |
| coding_unit() | skip_flag | Table 9-8 | 0..2 |  | 0..2 |
|  | cu_qp_delta | Table 9-9 | 0..3 |  | 4..7 |
|  | pred_mode_flag | Table 9-10 | 0 |  | 0 |
|  | part_mode | Table 9-10 | 1..4 |  | 5..7 |
| prediction_unit() | prev_intra_luma_pred_flag | Table 9-11 | 0 |  | 0 |
|  | intra_chroma_pred_mode | Table 9-12 | 0..1 |  | 2..3 |
|  | merge_flag | Table 9-13 | 0 |  | 0 |
|  | merge_idx | Table 9-14 | 0 |  | 0 |
|  | inter_pred_flag | Table 9-15 | 0..3 |  | 0..3 |
|  | ref_idx_lc, ref_idx_l0, ref_idx_l1 | Table 9-16 | 0..2 |  | 0..2 |
|  | abs_mvd_greater0_flag | Table 9-17 | 0 |  | 0 |
|  | abs_mvd_greater1_flag | Table 9-17 | 1 |  | 1 |
|  | mvp_lc_flag, mvp_l0_flag, mvp_l1_flag | Table 9-18 | 0 |  | 0 |
| transform_tree() | no_residual_data_flag | Table 9-19 | 0 |  | 0 |
|  | split_transform_flag | Table 9-20 | 0..3 |  | 4..7 |
|  | cbf_luma | Table 9-21 | 0..1 |  | 2..3 |
|  | cbf_cb, cbf_cr | Table 9-22 | 0..2 |  | 3..5 |
| residual_coding() | last_significant_coeff_x_prefix | Table 9-23 | 0..17 |  | 18..35 |
|  | last_significant_coeff_y_prefix | Table 9-24 | 0..17 |  | 18..35 |
|  | significant_coeff_group_flag | Table 9-25 | 0..3 |  | 4..7 |
|  | significant_coeff_flag | Table 9-26 | 0..47 |  | 48..95 |
|  | coeff_abs_level_greater1_flag | Table 9-27 | 0..23 |  | 24..47 |
|  | coeff_abs_level_greater2_flag | Table 9-28 | 0..5 |  | 6..11 |

NOTE 2 – ctxIdxTable equal to 0 and ctxIdx equal to 0 are associated with the end_of_slice_flag. The decoding process specified in subclause Error: Reference source not found applies to ctxIdxTable equal to 0 and ctxIdx equal to 0. This decoding process, however, may also be implemented by using the decoding process specified in subclause Error: Reference source not found. In this case, the initial values associated with ctxIdxTable equal to 0 and ctxIdx equal to 0 are specified to be pStateIdx = 63 and valMPS = 0, where pStateIdx = 63 represents a non-adapting probability state.

Table 9-2 – Values of variable initValue for sao_merge_left_flag ctxIdx

| Initialization variable | sao_merge_left_flag ctxIdx |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 |  |  |  |
| initValue | 118 | 118 | 118 | 102 | 102 | 102 |  |  |  |

Table 9-3 – Values of variable initValue for sao_merge_up_flag ctxIdx

| Initialization variable | sao_merge_up_flag ctxIdx |  |  |
| --- | --- | --- | --- |
|  | 0 | 1 |  |
| initValue | 109 | 102 |  |

Table 9-4 – Values of variable initValue for sao_type_idx ctxIdx

| Initialization variable | sao_type_idx ctxIdx |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 |  |  | 2 | 3 |
| initValue | 61 | 104 |  |  | 184 | 120 |

Table 9-5 – Values of variable initValue for sao_offset ctxIdx

| Initialization variable | sao_offset ctxIdx |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 |  |  |  |
| initValue | 171 | 119 | 199 | 169 | 119 | 151 |  |  |  |

Table 9-6 – Values of variable initValue for alf_cu_flag ctxIdx

| Initialization variable | alf_cu_flag ctxIdx |  |  |
| --- | --- | --- | --- |
|  | 0 | 1 |  |
| initValue | 109 | 102 |  |

Table 9-7 – Values of variable initValue for split_coding_unit_flag ctxIdx

| Initialization variable | split_coding_unit_flag ctxIdx |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 |  |  |  |
| initValue | 87 | 74 | 107 | 84 | 103 | 105 |  |  |  |

Table 9-8 – Values of variable initValue for skip_flag ctxIdx

| Initialization variable | skip_flag ctxIdx |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 |  |  | 2 | 3 |
| initValue | 165 | 168 |  |  | 168 | 154 |

Table 9-9 – Values of variable initValue for cu_qp_delta ctxIdx

| Initialization variable | cu_qp_delta ctxIdx |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 |  |  |  |
| initValue | 119 | 119 | 119 | 119 | 119 | 119 |  |  |  |

Table 9-10 – Values of variable initValue for pred_mode and part_mode

| Initialization variable | pred_type ctxIdx |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 |  |  | 3 | 4 | 5 |  |  |  |
| initValue | 114 |  |  | 119 | 87 | 119 |  |  |  |

Table 9-11 – Values of variable initValue for prev_intra_luma_pred_flag ctxIdx

| Initialization variable | prev_intra_luma_pred_flag ctxIdx |  |  |
| --- | --- | --- | --- |
|  | 0 |  | 1 |
| initValue | 167 |  | 150 |

Table 9-12 – Values of variable initValue for intra_chroma_pred_mode ctxIdx

| Initialization variable | intra_chroma_pred_mode ctxIdx |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 |  |  | 2 | 3 |
| initValue | 53 | 103 |  |  | 101 | 87 |

Table 9-13 – Value of variable initValue for merge_flag ctxIdx

| Initialization variable | merge_flag ctxIdx |  |
| --- | --- | --- |
|  |  | 0 |
| initValue |  | 119 |

Table 9-14 – Values of variable initValue for merge_idx ctxIdx

| Initialisation variable | merge_idx ctxIdx |  |
| --- | --- | --- |
|  |  | 0 |
| initValue |  | 116 |

Table 9-15 – Values of variable initValue for inter_pred_flag ctxIdx

| Initialisation variable | inter_pred_flag ctxIdx |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | 0 | 1 | 2 | 3 |
| initValue |  |  |  |  | 41 | 39 | 38 | 36 |

Table 9-16 – Values of variable initValue for ref_idx_lc, ref_idx_l0, ref_idx_l1 ctxIdx

| Initialization variable | ref_idx_lc, ref_idx_l0, ref_idx_l1 ctxIdx |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 0 | 1 | 2 |
| initValue |  |  |  | 118 | 118 | 134 |

Table 9-17 – Values of variable initValue for abs_mvd_greater0_flag and abs_mvd_greater1_flag ctxIdx

| Initialization variable | abs_mvd_greater0_flag, abs_mvd_greater1_flag ctxIdx |  |  |  |
| --- | --- | --- | --- | --- |
|  |  | 0 |  | 1 |
| initValue |  | 166 |  | 166 |

Table 9-18 – Values of variable initValue for mvp_lc_flag, mvp_l0_flag, mvp_l1_flag ctxIdx

| Initialization variable | mvp_lc_flag, mvp_l0_flag, mvp_l1_flag ctxIdx |  |
| --- | --- | --- |
|  | 0 |  |
| initValue | 134 |  |

Table 9-19 – Values of variable initValue for no_residual_data_flag ctxIdx

| Initialization variable | no_residual_data_flag ctxIdx |  |
| --- | --- | --- |
|  | 0 |  |
| initValue | 39 |  |

Table 9-20 – Values of variable initValue for split_transform_flag ctxIdx

| Initialization variable | split_transform_flag ctxIdx |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 |  |  |  |  | 4 | 5 | 6 | 7 |
| initValue | 119 | 162 | 148 | 100 |  |  |  |  | 119 | 102 | 86 | 86 |

Table 9-21 – Values of variable initValue for cbf_luma ctxIdx

| Initialization variable | cbf_luma ctxIdx |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 |  |  |
| initValue | 73 | 74 | 102 | 89 |  |  |

Table 9-22 – Values of variable initValue for cbf_cb and cbf_cr ctxIdx

| Initialization variable | cbf_cb ctxIdx |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 |  |  |  | 3 | 4 | 5 |
| initValue | 55 | 86 | 133 |  |  |  | 114 | 68 | 117 |

Table 9-23 – Values of variable initValue for last_significant_coeff_x_prefix ctxIdx

| Initialisation variable | last_significant_coefficient_x_prefix ctxIdx |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 |
| initValue | 72 | 72 | 71 | 72 | 104 | 89 | 88 | 89 | 59 | 73 | 89 | 106 | 60 | 59 | 43 | 54 | 70 | 53 |
|  | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 | 32 | 33 | 34 | 35 |
| initValue | 88 | 72 | 71 | 72 | 57 | 72 | 88 | 73 | 73 | 72 | 103 | 73 | 89 | 73 | 57 | 54 | 70 | 69 |

Table 9-24 – Values of variable initValue for last_significant_coeff_y_prefix ctxIdx

| Initialisation variable | last_significant_coefficient_x_prefix ctxIdx |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 |
| initValue | 72 | 72 | 71 | 72 | 104 | 89 | 88 | 89 | 59 | 73 | 89 | 106 | 60 | 59 | 43 | 54 | 70 | 53 |
|  | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 | 32 | 33 | 34 | 35 |
| initValue | 88 | 72 | 71 | 72 | 57 | 72 | 88 | 73 | 73 | 72 | 103 | 73 | 89 | 73 | 57 | 54 | 70 | 69 |

Table 9-25 – Values of variable initValue for significant_coeff_group_flag ctxIdx

| Initialization variable | significant_coeff_group_flag ctxIdx |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |  |  |  |  |
| initValue | 83 | 122 | 98 | 121 | 99 | 120 | 67 | 119 |  |  |  |  |

Table 9-26 – Values of variable initValue for significant_coeff_flag ctxIdx

| Initialization variable | significant_coeff_flag ctxIdx |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 |
| initValue | 74 | 73 | 88 | 72 | 72 | 55 | 71 | 54 | 71 | 88 | 103 | 71 | 53 | 87 | 134 | 86 |
|  | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 |
| initValue | 84 | 70 | 68 | 89 | 90 | 84 | 88 | 74 | 130 | 118 | 88 | 120 | 87 | 149 | 70 | 52 |
|  | 32 | 33 | 34 | 35 | 36 | 37 | 38 | 39 | 40 | 41 | 42 | 43 | 44 | 45 | 46 | 47 |
| initValue | 118 | 133 | 116 | 114 | 129 | 132 | 162 | 115 | 51 | 115 | 66 | 120 | 74 | 115 | 87 | 89 |
|  | 48 | 49 | 50 | 51 | 52 | 53 | 54 | 55 | 56 | 57 | 58 | 59 | 60 | 61 | 62 | 63 |
| initValue | 152 | 119 | 103 | 118 | 87 | 70 | 70 | 53 | 71 | 103 | 118 | 101 | 68 | 85 | 101 | 116 |
|  | 64 | 65 | 66 | 67 | 68 | 69 | 70 | 71 | 72 | 73 | 74 | 75 | 76 | 77 | 78 | 79 |
| initValue | 116 | 68 | 67 | 152 | 168 | 147 | 150 | 120 | 115 | 118 | 119 | 136 | 102 | 86 | 84 | 67 |
|  | 80 | 81 | 82 | 83 | 84 | 85 | 86 | 87 | 88 | 89 | 90 | 91 | 92 | 93 | 94 | 95 |
| initValue | 117 | 102 | 117 | 115 | 99 | 100 | 115 | 99 | 100 | 83 | 114 | 152 | 152 | 131 | 150 | 120 |

Table 9-27 – Values of variable initValue for coeff_abs_level_greater1_flag ctxIdx

| Initialisation variable | coeff_abs_level_greater1_flag ctxIdx |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 |
| initValue | 104 | 68 | 116 | 86 | 104 | 132 | 86 | 87 | 102 | 66 | 114 | 68 | 87 | 84 | 100 | 101 |
|  | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 |  |  |  |  |  |  |  |  |
| initValue | 104130 | 130 | 147 | 149 | 104 | 196 | 100 | 165 |  |  |  |  |  |  |  |  |
|  | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 | 32 | 33 | 34 | 35 | 36 | 37 | 38 | 39 |
| initValue | 119 | 179 | 148 | 164 | 119 | 85 | 117 | 149 | 133 | 98 | 114 | 115 | 118 | 99 | 115 | 100 |
|  | 40 | 41 | 42 | 43 | 44 | 45 | 46 | 47 |  |  |  |  |  |  |  |  |
| initValue | 135 | 177 | 147 | 164 | 119 | 132 | 148 | 149 |  |  |  |  |  |  |  |  |

Table 9-28 – Values of variable initValue for coeff_abs_level_greater2_flag ctxIdx

| Initialisation variable | coeff_abs_level_greater2_flag ctxIdx |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 |  |  |  |  |  |  |
| initValue | 86 | 102 | 115 | 117 | 101 | 101 |  |  |  |  |  |  |
|  | 6 | 7 | 8 | 9 | 10 | 11 |  |  |  |  |  |  |
| initValue | 84 | 117 | 83 | 84 | 84 | 117 |  |  |  |  |  |  |

#### Memorization process for context variables

Inputs of this process are the CABAC context variables indexed by ctxIdx.

Output of this process are variables TableStateSync and TableMPSSync containing the values of the variables m and n used in the initialization process of context variables that are assigned to all syntax elements in subclauses Error: Reference source not found and Error: Reference source not found except for the end-of-slice flag.

For each context variable, the corresponding entries n and m of tables TableStateSync and TableMPSSync are initialized to the corresponding pStateIdx and valMPS.

#### Synchronization process for context variables

Inputs of this process are variables TableStateSync and TableMPSSync containing the values of the variables n  and m used in the memorization process of context variables that are assigned to all syntax elements in subclauses Error: Reference source not found and Error: Reference source not found except for the end-of-slice flag.

Outputs of this process are the CABAC context variables indexed by ctxIdx..

For each context variable, the corresponding context variables pStateIdx and valMPS are initialized to the corresponding entries n and m of tables TableStateSync and TableMPSSync.

#### Initialization process for the arithmetic decoding engine

This process is invoked before decoding the first treeblock of a slice.

Outputs of this process are the initialised decoding engine registers codIRange and codIOffset both in 16 bit register precision.

The status of the arithmetic decoding engine is represented by the variables codIRange and codIOffset. In the initialization procedure of the arithmetic decoding process, codIRange is set equal to 510 and codIOffset is set equal to the value returned from read_bits( 9 ) interpreted as a 9 bit binary representation of an unsigned integer with most significant bit written first.

The bitstream shall not contain data that result in a value of codIOffset being equal to 510 or 511.

NOTE – The description of the arithmetic decoding engine in this Recommendation | International Standard utilizes 16 bit register precision. However, a minimum register precision of 9 bits is required for storing the values of the variables codIRange and codIOffset after invocation of the arithmetic decoding process (DecodeBin) as specified in subclause Error: Reference source not found. The arithmetic decoding process for a binary decision (DecodeDecision) as specified in subclause Error: Reference source not found and the decoding process for a binary decision before termination (DecodeTerminate) as specified in subclause Error: Reference source not found require a minimum register precision of 9 bits for the variables codIRange and codIOffset. The bypass decoding process for binary decisions (DecodeBypass) as specified in subclause Error: Reference source not found requires a minimum register precision of 10 bits for the variable codIOffset and a minimum register precision of 9 bits for the variable codIRange.

### Binarization process

Input to this process is a request for a syntax element.

Output of this process is the binarization of the syntax element, maxBinIdxCtx, ctxIdxOffset, and bypassFlag.

Table 9-29 specifies the type of binarization process, maxBinIdxCtx, ctxIdxTable, and ctxIdxOffset associated with each syntax element.

The specification of the unary (U) binarization process, the truncated unary (TU) binarization process, the truncated Rice (TR), the k-th order Exp-Golomb (EGk) binarization process, and the fixed-length (FL) binarization process are given in subclauses Error: Reference source not found to Error: Reference source not found, respectively. Other binarizations are specified in subclauses Error: Reference source not found to Error: Reference source not found.

The binarizations for the syntax element coeff_abs_level_remaining as specified in subclause Error: Reference source not found consist of bin strings given by a concatenation of prefix and suffix bit strings. For these binarization processes, the prefix and the suffix bit string are separately indexed using the binIdx variable as specified further in subclause Error: Reference source not found. The two sets of prefix bit strings and suffix bit strings are referred to as the binarization prefix part and the binarization suffix part, respectively.

Associated with each binarization or binarization part of a syntax element is a specific value of the context index table (ctxIdxTable) variable and the related context index offset (ctxIdxOffset) variable and a specific value of the maxBinIdxCtx variable as given in Table 9-29. When two values for each of these variables are specified for one syntax element in Table 9-29, the value in the upper row is related to the prefix part while the value in the lower row is related to the suffix part of the binarization of the corresponding syntax element.

The use of the DecodeBypass process and the variable bypassFlag is derived as follows.

–	If no value is assigned to ctxIdxOffset for the corresponding binarization or binarization part in Table 9-29 labelled as "na", all bins of the bit strings of the corresponding binarization or of the binarization prefix/suffix part are decoded by invoking the DecodeBypass process as specified in subclause Error: Reference source not found. In such a case, bypassFlag is set equal to 1, where bypassFlag is used to indicate that for parsing the value of the bin from the bitstream the DecodeBypass process is applied.

–	Otherwise, for each possible value of binIdx up to the specified value of maxBinIdxCtx given in Table 9-29, a specific value of the variable ctxIdx is further specified in subclause Error: Reference source not found. In such a case, bypassFlag is set equal to 0.

The possible values of the context index ctxIdx vary depending on the value of ctxIdxTable. The value assigned to ctxIdxOffset specifies the lower value of the range of ctxIdx assigned to the corresponding binarization or binarization part of a syntax element.

ctxIdxTable = 0 and ctxIdx = ctxIdxOffset = 0 are assigned to the syntax element end_of_slice_flag as further specified in subclause Error: Reference source not found. For parsing the value of the corresponding bin from the bitstream, the arithmetic decoding process for decisions before termination (DecodeTerminate) as specified in subclause Error: Reference source not found is applied.

| Table 9-29 – Syntax elements and associated types of binarization, maxBinIdxCtx, ctxIdxTable, and ctxIdxOffset |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Syntax element | initialisationType | Type of binarization | maxBinIdxCtx | ctxIdxTable | ctxIdxOffset |
| sao_merge_left_flag | 0 | FL, cMax = 1 | 0 | Table 9-2 | 0 |
|  | 1 |  | 0 | Table 9-2 | 3 |
|  |  |  |  | Table 9-2 |  |
| sao_merge_up_flag | 0 | FL, cMax = 1 | 0 | Table 9-3 | 0 |
|  | 1 |  | 0 | Table 9-3 | 1 |
|  |  |  |  | Table 9-3 |  |
| sao_type_idx | 0 | FL, cMax = 3 | 1 | Table 9-4 | 0 |
|  | 1 |  | 1 | Table 9-4 | 2 |
|  |  |  |  | Table 9-4 |  |
| sao_band_position | 0 | FL, cMax = 5 | na | na | na, (uses Decode Bypass) |
|  | 1 |  | na | na | na, (uses Decode Bypass) |
| sao_offset | 0 | EG0 | 2 | Table 9-5 | 0 |
|  | 1 |  | 2 | Table 9-5 | 3 |
|  |  |  |  | Table 9-5 |  |
| alf_cu_flag | 0 | Fl, cMax = 1 | 0 | Table 9-6 | 0 |
|  | 1 |  | 0 | Table 9-6 | 1 |
|  |  |  |  | Table 9-6 |  |
| end_of_slice_flag | all | FL, cMax = 1 | 0 | 0 | 0 |
| split_coding_unit_flag | 0 | FL, cMax = 1 | 0 | Table 9-7 | 0 |
|  | 1 |  | 0 | Table 9-7 | 3 |
|  |  |  |  | Table 9-7 |  |
| skip_flag | all | FL, cMax = 1 | 0 | Table 9-8 | 0 |
|  |  |  |  | Table 9-8 |  |
| cu_qp_delta | 0 | as specified in subclause Error: Reference source not found | 2 | Table 9-9 | 0 |
|  | 1 |  | 2 | Table 9-9 | 3 |
|  |  |  |  | Table 9-9 |  |
| pred_mode_flag | all | FL, cMax = 1 | 0 | Table 9-10 | 0 |
|  |  |  |  | Table 9-10 |  |
| part_mode | 0 | as specified in subclause Error: Reference source not found | 3 | Table 9-10 | 1 |
|  | 1 |  | 3 | Table 9-10 | 4 |
|  |  |  |  | Table 9-10 |  |
| pcm_flag | all | FL, cMax = 1 | 0 | 0 | 0 |
| prev_intra_luma_pred_flag | 0 | FL, cMax = 1 | 0 | Table 9-11 | 0 |
|  | 1 |  | 0 | Table 9-11 | 1 |
|  |  |  |  | Table 9-11 |  |
| mpm_idx | all | TU, cMax = 2 | na | na | na, (uses Decode Bypass) |
| rem_intra_luma_pred_mode | all | FL, cMax = 31 | na | na | na, (uses Decode Bypass) |
| intra_chroma_pred_mode | 0 | as specified in subclause Error: Reference source not found | 1 | Table 9-12 | 0 |
|  | 1 |  | 1 | Table 9-12 | 2 |
|  |  |  |  | Table 9-12 |  |
| merge_flag | all | FL, cMax = 1 | 0 | Table 9-13 | 0 |
|  |  |  |  | Table 9-13 |  |
| merge_idx | all | TU, cMax = MaxNumMergeCand − 1 | 0 | Table 9-14 | 0 |
|  |  |  |  | Table 9-14 |  |
| inter_pred_flag | all | FL, cMax = 1 | 0 | Table 9-15 | 0 |
|  |  |  |  | Table 9-15 |  |
| ref_idx_lc | all | TU, cMax = num_ref_idx_lC_active_minus1 | 2 | Table 9-16 | 0 |
|  |  |  |  | Table 9-16 |  |
| ref_idx_l0 | all | TU, cMax = num_ref_idx_l0_active_minus1 | 2 | Table 9-16 | 0 |
|  |  |  |  | Table 9-16 |  |
| ref_idx_l1 | all | TU, cMax = num_ref_idx_l1_active_minus1 | 2 | Table 9-16 | 0 |
|  |  |  |  | Table 9-16 |  |
| abs_mvd_greater0_flag[ ] | 1 | FL, cMax = 1 | 0 | Table 9-17 | 0 |
|  | 2 |  | 0 | Table 9-17 | 1 |
| abs_mvd_greater1_flag[ ] | 1 | FL, cMax = 1 | 0 | Table 9-17 | 2 |
|  | 2 |  | 0 | Table 9-17 | 3 |
| abs_mvd_minus2[ ] | all | EG1 | na | na | na, (uses Decode Bypass) |
| mvd_sign_flag[ ] | all | FL, cMax = 1 | na | na | na, (uses Decode Bypass) |
| mvp_lc_flag | all | FL, cMax = 1 | 0 | Table 9-18 | 0 |
|  |  |  |  | Table 9-18 |  |
| mvp_l0_flag | all | FL, cMax = 1 | 0 | Table 9-18 | 0 |
|  |  |  |  | Table 9-18 |  |
| mvp_l1_flag | all | FL, cMax = 1 | 0 | Table 9-18 | 0 |
|  |  |  |  | Table 9-18 |  |
| no_residual_data_flag | all | FL, cMax = 1 | 0 | Table 9-19 | 0 |
|  |  |  |  | Table 9-19 |  |
| split_transform_flag | 0 | FL, cMax = 1 | 0 | Table 9-20 | 0 |
|  | 1 |  | 0 | Table 9-20 | 4 |
|  |  |  |  | Table 9-20 |  |
| cbf_luma | 0 | FL, cMax = 1 | 0 | Table 9-21 | 0 |
|  | 1 |  | 0 | Table 9-21 | 2 |
|  |  |  |  | Table 9-21 |  |
| cbf_cb, cbf_cr | 0 | FL, cMax = 1 | 0 | Table 9-22 | 0 |
|  | 1 |  | 0 | Table 9-22 | 3 |
|  |  |  |  | Table 9-22 |  |
| last_significant_coeff_x_prefix | 0 | TU, cMax = ( log2TrafoWidth << 1 ) − 1 | 8 | Table 9-23 | 0 |
|  | 1 |  | 8 | Table 9-23 | 18 |
|  |  |  |  | Table 9-23 |  |
| last_significant_coeff_y_prefix | 0 | TU, cMax = ( log2TrafoHeight << 1 ) − 1 | 8 | Table 9-24 | 0 |
|  | 1 |  | 8 | Table 9-24 | 18 |
|  |  |  |  | Table 9-24 |  |
| last_significant_coeff_x_suffix | all | FL, cMax = (last_significant_coeff_x_prefix >> 1) − 1 | na | na | na, (uses Decode Bypass) |
| last_significant_coeff_y_suffix | all | FL, cMax = (last_significant_coeff_y_prefix >> 1) − 1 | na | na | na, (uses Decode Bypass) |
| significant_coeff_group_flag | 0 | FL, cMax = 1 | 0 | Table 9-25 | 0 |
|  | 1 |  | 0 | Table 9-25 | 4 |
|  |  |  |  | Table 9-25 |  |
| significant_coeff_flag | 0 | FL, cMax = 1 | 0 | Table 9-26 | 0 |
|  | 1 |  | 0 | Table 9-26 | 48 |
|  |  |  |  | Table 9-26 |  |
| coeff_abs_level_greater1_flag | 0 | FL, cMax = 1 | 0 | Table 9-27 | 0 |
|  | 1 |  | 0 | Table 9-27 | 24 |
|  |  |  |  | Table 9-27 |  |
| coeff_abs_level_greater2_flag | 0 | FL, cMax = 1 | 0 | Table 9-28 | 0 |
|  | 1 |  | 0 | Table 9-28 | 6 |
|  |  |  |  | Table 9-28 |  |
| coeff_abs_level_remaining | all | prefix and suffix as specified in subclause Error: Reference source not found | prefix: na suffix: na | prefix: na suffix: na | prefix: na, (uses Decode Bypass) suffix: na, (uses Decode Bypass) |
| coeff_sign_flag | all | FL, cMax = 1 | na | na | na, (uses Decode Bypass) |